- MIB file descriptions

(bcm-mib-files.zip is a zip file containing all MIB files.)

- Standard MIBs:

| MIB | File Name | Comments |
| --- | --- | --- |
| RFC1354-MIB | Rfc1354.mib | This MIB defines ipForwardTable. |
| SNMP-FRAMEWORK-MIB | Rfc2261.mib | The SNMP Management Architecture MIB |
| INET-ADDRESS-MIB | Rfc2851.mib | This MIB module defines textual conventions for representing Internet addresses. An Internet address can be an IPv4 address, an IPv6 address or a DNS domain name. |

- Nortel MIB’s:

| MIB | File Name | Comments |
| --- | --- | --- |
| Small Site MIB | Smallsite.mib | This defines the upper-level hierarchy of an enterprises(1).nortel(562) sub-branch called smallsite |
| Small Site Events | Smallsiteevents.mib | Defintion of events (traps) that are useable by any Small Site product or component |
| SYNOPTICS-ROOT-MIB | Synroxxx.mib | SynOptics root mib. The policy object identifier is added for policy mibs. |
| POLICY-FRAMEWORK-PIB | PibFramework.mib | A PIB (Policy Information Base) module containing the base set of policy rule classes that are required for support of all policies. This mibs under Synoptics branch. |
| QOS-POLICY-IP-PIB | PibIp.mib | The PIB module containing an initial set of policy rule classes that describe the quality of service (QoS) policies. It includes general classes that may be extended by other PIB specifications as well as an initial set of PIB classes related to IP processing. This mibs under Synoptics branch. |
| COPS-CLIENT-MIB | Copsclientmib.mib | The COPS Client MIB module. This mibs under Synoptics branch. |

MicroSoft MIB’s:

| MIB | File Name | Comments |
| --- | --- | --- |
| OSPF mibs | Wfospf.mib | This defines the OSPF mibs from Wellfleet. The MicroSoft adopted and released as part of MiscorSoft mibs and under wellfleet branch. |
| RIP2 mibs | Msiprip2.mib | This mibs define the RIP2 mibs. This mib is under MicroSoft branch. |
| Bootp mibs | Msipbtp.mib | This mibs define the BootP mibs and under MicroSoft branch. |

2. MIB file compilation and installation

Each MIB browser has its own mib compilation tool so just follow its procedure but the order of file should be same as following list.

The SmallSite Mibs have definition for binding values for BCM SNMP Traps.

The Policy mibs are branched out from Synoptic, so user need to install synro123.mib

before compile and install those mibs. If user already installed SYNOPTICS-ROOT-MIB for other MIBs, then add “policy OBJECT IDENTIFIER ::= { synoptics 4 }” in synroxxx.mib and recompile and reload it for policy mibs.

- For SmallSite mibs

- SmallSite.mib

- SmallSiteEvents.mib

- For Other mibs

- Rfc1354.mib

- Rfc2261.mib

- Rfc2851.mib

- Synro123.mib

- PibFramework.mib

- PibIp.mib

- Copsclientmib.mib

  - Wfospf.mib

  - Msiprip2.mib

  - Msipbtp.mib

- OSPF mibs

An OSPF Management Information Base has been published in RFC 1248.

The mibs defined in BCM is from MicroSoft and it is under Wellfleet branch. This mib is subset of mibs in RFC 1248.

The MIB consists of a general variables group and eight tables:

Group Name           Description    _____________________________________________________________

wfOspfGeneralGroup     General Global Variables

wfOspfAreaTable        Area Descriptions

wfOspfLsdbTable        Link State Database

wfOspfAreaRangeTable   Address Range Specifications

wfOspfIfTable          OSPF Interface Variables

wfOspfVirtIfTable      Virtual Links

wfOspfNbrTable         (Non-virtual) OSPF Neighbors

wfOspfVirtNbrTable     Virtual OSPF Neighbors

wfOspfDynNbrTable   OSPF Dynamic Neighbor Table

Section D.2 of the OSPF V2 specification (RFC 1247) lists a set of required

statistics that an implementation must maintain. These statistics have

been incorporated into the OSPF MIB. The MIB's thirteen Counters and

Gauges enable evaluation of the OSPF protocol's performance in an

operational environment. Most of the remainder of the MIB's variables

parameterize the many features that OSPF provides the network

administrator.

- RIP2 mibs

This MIB defines the management information for the Routing Information Protocol Version 2 ( RIP v2).  The information in this MIB is divided into 3 groups:

1.  Global Group

Contains the global information and statistics for the RIP protocol

Information contained in this group is independent of the interfaces

over which the protocol is enabled.

2.  Interface Group

Contains RIP configuration information and statistics specific

to each interface.

- Peer Group

Contains statistics pertaining to RIP peers.

5. Bootp mibs

This MIB defines the management information for the BOOTP  Protocol.  The information in this MIB is divided into 2 groups:

1. Global Group

Contains the global information BOOTP protocol

Information contained in this group is independent of the interfaces

over which the protocol is enabled.

2. Interface Group

Contains RIP configuration information and statistics specific

to each interface.

- Potential Issues

Followings are the possible problems which can be encountered during compilation and load.

- Our files are created and released under MicroSoft Windows environment so when these files are copied or transferred to UNIX environment the last carriage return can be deleted. This case user can get “END” is not found error during the compilation. The solution is simply open the mib file with UNIX text editor and add the carriage return at the end of “END”.

- If user already installed SYNOPTICS-ROOT-MIB for other MIBs, then add “policy OBJECT IDENTIFIER ::= { synoptics 4 }” in synroxxx.mib and recompile and reload it for policy mibs. The xxx in mib file name is version number of mib file.