RESTRICTED

RNxB25AAxx_SW1.0.doc	April 1, 2005

R2MFC SW release notes

For f/w Version 1.0

Proprietary		Issue 1.0

Author:	Jerry Freestone	Approved by:	Andres Canalizo

Department:	9D40

Keywords	BCM, R2MFC

Abstract	This document describes the firmware Release Notes for the R2MFC software release 1.0.

Security classification downgrading:		Refer to Northern Networks Corporate procedures

Declassify	Never

Funder

Prime

Case

(c) 2000 Northern Networks Ltd.

This document contains restricted, proprietary, copyrighted information, ownership of which is retained by Nortel Networks.  Nortel Networks consent is required prior to any reproduction, publication or other use of such information.

Table of Contents

Revision History	iii

1	Introduction	1

2	Firmware Version Codes	1

3	Equipment Requirements/Compatibility	1

4	Features Supported	1

4.1	Country Support	1

4.2	Custom Configuration	2

4.3	Second Dial Tone	2

4.4	CLI configuration tool	3

5	Known Issues	3

6	Known Limitations	3

# Revision History

Issue 1.0	April 25, 2005		First release.

# Introduction

This document contains release notes R2MFC software version 1.0.  These notes should be used (along with the quicksheet in the box) to determine compatibility and the best way to configure the MBM.

# Firmware Version Codes

The software release is 1.0 but in the CLI interface the software version (build version) will be 1.16.

# Equipment Requirements/Compatibility

This software is designed to work the R2MFC MBM in a BCM system.  The software is also designed to work in the Norstar R2MFC trunk module when it becomes available.

Please see the appropriate installer guide for minimum system level requirements to use an R2MFC MBM/trunk module.

# .C1.Features Supported

Release 1.0  software is designed to support of the functionality detailed in the user guide. Generic configuration of the module is done through the dipswitches on the front of the module.

Specific changes to R2 parameters are done through the CLI tool with a PC.

## Country Support

Release 1.0 software supports 5 different variants for R2MFC via the dipswitches on the front of the module.

Although these rules won’t always apply, (will depend on the specific switch configurations) the settings roughly translate as follows:

| Profile | Tested Telco E1 | City or Region |
| --- | --- | --- |
| Mexico1 | Telmex | South East Mexico |
| Mexico2 | Telmex | Mexico City, Guadalajara |
| Brazil1 | Embratel, Telefonica | Sao Paulo |

## Custom Configuration

Custom configurations are done through the CLI tool.  The dipswitches must be in custom configuration mode (below) to support a custom configuration.

NOTE:  The custom configuration must be saved in the CLI or the configuration will be lost with power down.

## Second Dial Tone

The second dial tone feature will provide an artificial dial tone if the if the feature is turned on and the dialing mode is set to overlap mode.  See the install guide for more details.

## CLI configuration tool

The Command Line Interface (CLI) tool is used to check configuration settings, modify configuration settings and access the on board diagnostic tools. See the installer guide for more information.

# Known Issues

TBD

# Known Limitations

Brazil-1 default configuration cannot work in a tandem BCM switch configuration because the inbound MFC dialing is limited to 4 digits.  The can be changed as a custom configuration through the CLI interface.  Please see the installation manual for details on how to change the number of incoming digits.