BCM

Call Center Demo

May 16, 2006

Issue 2.00

Authors         Jason Batai		Recommended by

Approved by

Dept.

Keywords

Abstract	This document describes a baseline for a Call Center Demo configuration.

Security classification downgrading

Reclassify	On

Never

Declassify	Never

© 2006 Nortel Networks

This document contains restricted, proprietary, copyrighted information, ownership of which is retained by Nortel. Nortel’s consent is required prior to any reproduction, publication or other use of such information.

# Revision History

Draft 1.00	July 09, 2003

New document based on original work by Luke Holden and David Whiteside

Draft 1.01	July 29, 2003

Updated Document – Added Https://… references and omitted Http://…:6800 references to accommodate for BCM 3.5

Draft 1.02	May 27, 2004

Updated Document – Updated for BCM 3.6, the new RCC package, and Dynamic demonstrations (CLID/DNIS, Caller Input Rules, EWT, and Activity Code reconfiguration on the fly)

Issue 1.00	April 19, 2006

Updated document to revert URLs from “iMMCC-html” back to “ivb-html”.  (Ian Olthof)

Issue 2.00	May 16, 2006

Updated document for refer to generic (Linux-based) BCMs rather than 3.x versions; changed Unified Manager references to Element Manager.  (Ian Olthof)

###### TABLE OF CONTENTS

Revision History	4

Introduction	6

Business Communications Manager Contact Center Demo	7

Hardware Requirements	7

Element Manager	10

Multimedia Contact Center Demo Files	11

Contact Center Administration Setup	12

Optional Configurations	15

Dynamic Demonstrations	17

Reporting for Contact Center	20

Outlook	22

LAN CTE	22

VCM	23

Notes	28

Call Back	28

IP Address Scheme	28

Sample CLID Numbers	28

DEMO HTML – MMCC test page.html	30

## Introduction

This document is indented to provide a baseline for BCM Call Center Demo’s.

Components suggested for demonstration include:

- Intelligent Call Routing

  - Based on CLID/ANI

  - Based on Basic Caller Input Rules

  - Based on Advanced Caller Input Rules

- Multimedia Call Center

  - Browser Only

  - Voice and Browser

  - Screen Push

  - Follow Me browsing

- Call Center Reporting

  - Real-time reports

  - Historical Reports

- Third Party applications

  - VCM by Convertec integrated with Microsoft Outlook 2000 for screen pops

- Call Center Features

  - Customizable CLID display on TUI

  - Auto-answer for agents with Headsets

  - Supervisor Monitoring

  - Supervisor Help

  - Expected Wait Time

  - F909

  - MWI

## Business Communications Manager Contact Center Demo

### Hardware Requirements

Before beginning the demo configuration, it is important that the correct hardware is available. The following components will be needed:

| Quantity | Item |
| --- | --- |
| 1 | 4-Port Switch/Hub |
| 1 | BCM (with Call Center/Multimedia Call Center Keycodes) |
| 1 | Breaker box for TDM Telsets |
| 1 | CTM 8 MBM |
| 1 | DSM 16+ MBM |
| 2 | PCs (with Monitor/Keyboard/Mouse) |
| 2 | Telephone Line Simulators |
| 3 | Analog Telephones |
| 3 | TDM Telsets |
| X | All applicable Networking, Telephony, Electrical Cables |

NOTE: It is advisable to use Internet Explorer 5.x or higher on both PCs.

NOTE: BCM50 has integrated CTM/DSM functionality.

Hardware Setup

With the above hardware, the following setup is recommended:

- Use the first telephone line simulator for the regular Contact Center calls

  - Connect the line simulator to your BCM’s CTM8 MBM at lines 1 and 2

  - Connect the remaining two ports on the line simulator to Analog Telsets 1 and 2.  Label these Telsets “General Caller” and “Upscale Caller”.

  - Make note of the Telephone numbers for CLID configurations

- Use the second telephone line simulator for the Multimedia Contact Center Call Center callback calls

  - Connect the line simulator to your BCM’s Caller ID Trunk Interface at lines 3 and 4

  - Ensure Line 3 and 4 are configured for callback (see the notes on page 28 for more details)

  - Make note of the Telephone numbers for CLID configurations

- Position the two PC’s so that your clients can view both ends of the demo MMCC session in progress

  - One PC will be the MMCC Caller

  - One PC will be the MMCC Agent

    - This PC will also be used for Convertec VMC, and Contact Center Reporting.

- Spread out the agent Telsets and the caller telephones, and make sure that the MMCC caller’s PC has an analog phone beside it and also that the MMCC agent’s PC has a Telset beside it.

  - Label the MMCC Callers Telset “Gold Caller”

- Connect both PCs and the BCM to the Hub/Switch to create a self contained environment independent of the CoreWan

  - Give each machine Static IP addresses for the Demo.  (See the notes on page 28 for example IP address scheme)

- Connect all TDM Telsets to the Breaker box, and the Breaker Box to the BCM’s DSM 16+ MBM.

Caller PC

192.168.1.12

255.255.255.0

Agent PC

192.168.1.11

255.255.255.0

BCM – Lan1

192.168.1.10

255.255.255.0

Machine

Sample        IP Address

Gold Caller

769-1002

Upscale Caller

769-1001

General Caller

769-1000

Telephone

Sample CLID

BCM

DSM 16+

CTM8

LAN

Analog Telephony

Digital Telephony

Breaker Box

General

Agent

Upscale

Agent

Gold

Agent

Switch

Telephone Line

Simulator 1

Telephone Line

Simulator 2

Analog Telephone

TDM Telset

General

Caller

Upscale

Caller

Gold

Caller

Agent PC

Caller PC

Basic Demo Configuration

### Element Manager

Log into Element Manager (http://192.168.1.10/mgmt.html)

Possible login password combinations

Username		Password

nnadmin		PlsChgMe!

Navigate the tree in Element manager:

-Telephony

-Lines

-Physical Lines

+Active Physical Lines

Expand the Enabled Physical Lines Key to display your physical lines, make note of the line numbers for your Auto Attendant Lines configuration.

While in Element Manager you may want to configure Call Back, please see the Notes section for more information.

### Multimedia Contact Center Demo Files

Now that the hardware has been setup, be certain that the correct Multimedia Contact Center software has been installed:

- On the caller PC, open a web browser window (Internet Explorer is preferable)

- The central demo website page is Mainpage.html and already resides on your BCM with the rest of the demo files at /srv/www/MMCC/html/demo.  The way to access the demo website is to type in the URL of Mainpage.html using the IP address of your BCM; if your BCM’s IP was 192.168.1.10 then in your web browser you would type in:

- https://192.168.1.10/ivb-html/demo/Mainpage.html

- When it appears, do not navigate the demo website just yet – minimize it for now as you will return to it during the Demo step

- Make sure the MMCC agent PC has the latest Multimedia Contact Center Client installed as well as Java Runtime Environment

- Ensure that the demo BCM has an equally up-to-date version of Multimedia Contact Center Server installed

- If required at the end of this document is HTML code for a MMCC test page, from this web page you can call skillsets 1 to 50.  It may be handy in custom configurations/demo.

### Contact Center Administration Setup

The next part of the demo configuration requires your Contact Center to be setup to receive the types of calls that will display certain key features:

- Ensure that the lines 1 and 2 are connected to your first telephone line simulator, which will answer regular Contact Center calls.

- Ensure that the lines 3 and 4 are connected to your second telephone line simulator, which will be used for Multimedia Contact Center callback calls.

- Login to your demo BCM’s CallPilot Manager

- Under the Auto Attendant menu select Lines Administration and scroll down to the line number corresponding to the each BCM line and make sure Answer Mode is set for ‘Contact Center’ and the skillsets match the configuration below

  - BCM line 1	-	Skillset 1	(Goes to Line simulator 1)

  - BCM line 2	-	Skillset 1	(Goes to Line simulator 1)

  - BCM line 3	-	Skillset 3	(Goes to Line simulator 2)

  - BCM line 4	-	Skillset 3	(Goes to Line simulator 2)

- Under the Contact Center menu select CLID/DNIS Routing Table and click the Add button.  Do not enter anything in the Line field; in the CLID/ANI field enter the phone number of your “Upscale caller”; for Action choose ‘Move to Skillset 2’

- Under the Contact Center menu select Caller Input Rules and for Table ID ‘1’ click the Change command.  For the Length field choose ‘Fixed’ and set the length to ‘4’ then submit.  Now click the Rules command for the same table and then click the Add button.  In the Match String field enter ‘1234’ and for Action choose ‘Move to Skillset 3’ then submit

- Under the Contact Center menu select Agent List and create three new agents:

    - The first agent will answer Voice only calls; name this agent “Voice-General”. Configure this Agent to use Auto-Answer.

    - The second agent will also answer Voice only calls; name this agent “Voice-Upscale”

    - The third agent will answer Both Voice and Multimedia Contact Center calls; name this agent “Both-Gold”

- Under the Contact Center menu select Greetings, configure each of the following greetings by clicking the voice command option

  - Greeting 1	-	 “Thank you for calling, you are in the General Skillset. Please wait for the next available agent to take your call”

  - Greeting 2	-	“Thank you for calling, you are in the Upscale Skillset.  Please enter your four-digit pass code to enter the Gold Skillset or wait on the line for the next available agent.

  - Greeting 3	-	“Thank you for holding.  All of our agents are currently unavailable at this time, please continue to hold and your call will be answered in the order it was received”

  - Greeting 10	-	“Thank you for holding, The expected wait time for your call is 1 Minute”

  - Greeting 11	-	“Thank you for holding, The expected wait time for your call is 5 Minutes”

  - Greeting 12	-	“Thank you for holding, The expected wait time for your call is 10 Minutes”

  - Greeting 13	-	“Thank you for holding, the expected wait time for your call is 15 Minutes. Please continue to hold or press 9 to leave a message”

  - Greeting 14	-	“Thank you for holding, the expected wait time for your call is 30 Minutes. Please continue to hold or press 9 to leave a message”

  - Greeting 15	-	“Thank you for holding, the expected wait time for your call exceeds 30 minutes.  Please continue to hold or Press nine to leave a message”

  - Greeting 148	-	“Welcome to the Gold Skillset, Please hold for the next available agent”

  - Greeting 149	-	“Gold Skillset Greeting 2, Please continue to hold or press 9 to leave a message”

  - Greeting 150	-	“Gold Skillset Greeting 3, Please continue to hold or press 9 to leave a message”

- Under the Contact Center menu select Expected Wait Time Table and for Table ID ‘1’ click the Change Command

  - Click the Add button and enter 00:01:00 for the time and 10 for the Greeting ID, then click Submit

  - Click the Add button and enter 00:05:00 for the time and 11 for the Greeting ID, then click Submit

  - Click the Add button and enter 00:10:00 for the time and 12 for the Greeting ID, then click Submit

  - Click the Add button and enter 00:15:00 for the time and 13 for the Greeting ID, then click Submit

  - Click the Add button and enter 00:30:00 for the time and 14 for the Greeting ID, then click Submit

  - Click the Change button beside the After All times Above, Change this value to 15 and Submit

- Under the Contact Center menu select Skillset List and create 3 new skillsets to answer your different demo calls:

    - For skillset 1, configure it to be your “General” queue where the only agent assigned to this queue is “Voice-General”. Delete all overflow rules and Proceed to configure the following Day routing table.

      - Greeting 1

      - Distribute 10 seconds

      - Greeting 3

      - Distribute 10 seconds

      - EWT Table 1	- with Basic Transfer

        - 9 Transfers to Skillset Mailbox

      - Go to Step 2

    - For skillset 2 configure it to be your “Upscale” queue where the only agent assigned to this queue is “Voice-Upscale”.  Delete all overflow rules and Proceed to configure the following Day routing table.

      - Greeting 2 (Forced) with Advanced Transfer

        - Use Caller Input Rules Table 1, 2 Retries

      - Distribute 10 seconds

      - Greeting 3

      - Distribute 10 seconds

      - EWT Table 1	- with Basic Transfer

        - 9 Transfers to Skillset Mailbox

      - Go to Step 2

    - For skillset 3 configure it to be your “Gold” queue where the only agent assigned to this queue is “Both-Gold”.

      - Greeting 148 (Forced)

      - Distribute 5 seconds

      - Greeting 149 with Basic Transfer

        - 9 Transfers to Skillset Mailbox

      - Distribute 5 seconds

      - EWT Table 1	- with Basic Transfer

        - 9 Transfers to Skillset Mailbox

      - Greeting 150 with Basic Transfer

        - 9 Transfers to Skillset Mailbox

      - Go to Step 2

Note: Skillset 3 is also being used to accept MMCC calls.  The MMCC file refresh.txt (on your BCM) has greetings 148, 149, and 150 pre-customized to reflect the canned MMCC Demo.  It is therefore advised to record and use these greeting numbers in your skillset 3 Day routing table, as these Web Refresher announcement numbers will be displayed to the waiting MMCC caller (see The Demo steps below)

    - Set up all required skillset mailboxes (F981) and then enable all skillsets.

### Optional Configurations

#### Supervisor Monitor

Get a fourth TDM Telset and connect it to the breaker box, or use an agent Telset to double for two agents.

Create Agent 04 and name this account ‘Super’.  Give this agent supervisor properties and save your settings.

Disable all three skillsets and add ‘Super’ to all skillsets agents lists.  Re-enable all skillsets.

Log in all of your agents and place some calls into your system and have them answered by agents normally. Have ‘Super’ invoke F905 and start to observe the info of various agents, verify that you can observe agents and join/mute yourself from the conference.

Release all calls but leave all agents logged in for the Supervisor Help verification.

#### Supervisor Help

On all agent Telsets Press Feature * 3 and then select a button with an indicator. Press Feature 906.

The Key you selected after pressing Feature *3 should now be configured with Feature 906 (supervisor help).

Place a call center call to a call center agent, answer the call and press the pre-programmed key. ‘Super’ should now get a Help Request Alert. Accept the session and try to join the conference. Verify voice path!

Place a call center call to the supervisor account, invoke F906 via pre-programmed key and check that you are presented with a different menu than the agent had.  While on a Call Center Call you should be able to select Request, Offer, or Cancel. While on any other type of call or not on call, F906 should make a supervisor offer help by default.

MWI

If you have extra Telsets configure MWI. From Call Pilot Manager click Contact Center and then Skillset List. Disable all skillsets and go to the properties page for each.  Specify a unique active DN for each MWI and re-enable the skillsets

Activity Codes

Activity Codes are a new feature to BCM 3.6. From Call Pilot Manager click Contact Center then Activity Codes.

There are two ways to configure Activity codes, you can click Insert and fill in the form or you can create a simple text document and use the import feature.

To demo the Import feature open Notepad. Type in one line of text:

1,Peg 1,Activity Code Peg 1

Now cut and paste the line multiple times

1,Peg 1,Activity Code Peg 1

1,Peg 1,Activity Code Peg 1

1,Peg 1,Activity Code Peg 1

1,Peg 1,Activity Code Peg 1

1,Peg 1,Activity Code Peg 1

1,Peg 1,Activity Code Peg 1

Now modify each line

1,Peg 1,Activity Code Peg 1

2,Peg 2,Activity Code Peg 2

3,Peg 3,Activity Code Peg 3

4,Peg 4,Activity Code Peg 4

5,Peg 5,Activity Code Peg 5

6,Peg 6,Activity Code Peg 6

Save the text file to your desktop as pegs.txt.  Click Import in the Call Pilot Manager interface and then browse to pegs.txt on your desktop.  Append these pegs to your list of pegs.

### Dynamic Demonstrations

A Dynamic Demonstration is when you manipulate the Contact Center’s configuration on the fly, meaning without disabling Skillsets or interrupting service.

#### CLID/DNIS Routing

CLID/DNIS Routing is a powerful feature to demonstrate when you re-configure everything on-the-fly during the demo.  Demonstrating this feature will require some modifications to the canned Demo.

Currently in the Canned Demo you can Call Skillset 1 with phone 1 and Skillset 2 with phone 2.  The Call Center will recognize the CLID of Phone 2 and route to skillset 2.

Instead of doing this demonstration do the following:

Open a Call Pilot Manager screen and navigate to CLID/DNIS Routing Table.  Give a brief explanation of the feature to the audience. Delete all CLID/DNIS rules.

Call Skillset 1 with Phone 1.  Make sure you indicate the CLID display to the audience, enter Call Pilot Manager and Add a rule to make the CLID of phone 1 route to skillset 2.  Terminate your call and call back into the system.  The call will now go to skillset 2!  There was no need to disable the system or Skillsets; there was no interruption to the service of your call center.  Now modify the rule to route the call to skillset 3.  Terminate your call and dial into the call center again from phone 1 and you will be routed to Skillset 3.

This helps to demonstrate how easy Call Center is to configure

#### Activity Codes

As a company grows and changes new activity codes may be needed.  When the need for new activity code arises there is no need to disable your system.  Simply enter the Call Pilot Manager and go to the Activity Codes section.  Either Insert or Import new activity codes and peg a new call in the system with the new Activity Code.

#### Greetings

Once you assign a Greeting or EWT table to a skillset you cannot change the greeting number while the skillset is enabled, but you can re-record the greetings.

For Example: Skillset 3s Day routing table is as follows

| Step | Information | Commands |
| --- | --- | --- |
| 1 | Greeting 148, Normal, Transfer | View |
| 2 | Distribute For 00:30 | View |
| 3 | Greeting 149, Normal, Transfer | View |
| 4 | Distribute For 00:30 | View |
| 5 | Greeting 150, Normal, Transfer | View |
| 6 | Distribute For 00:30 | View |
| 7 | EWT Greeting 1, Normal, Transfer | View |
| 8 | Goto Step 1 | View |
| End |  |  |

You can NOT change greetings 148, 149, or 150 while the Skillset is enabled, however you CAN re-record the greeting while the skillset is still enabled.

Open Call Pilot Manager and navigate to the Greetings menu for Contact Center.

Place a call to skillset 3 and let the audience hear the first greeting.

If Greeting 148s default dialogue is “Welcome to the Gold Skillset, Please hold for the next available agent”

Terminate the call and re-record greeting 148. You can change it to “Thank you for Calling, There is a service outage in the South Eastern Quadrant, and some customers may experience delays with service. Our support team is looking at the issue and will have it remedied in a timely manner.  If your call is urgent please stay on the line and a representative will be with you shortly”

Call skillset 3 again and demo to the callers that the greeting has changed without disabling the skillset.

#### Expected Wait Time

Expected Wait Time this is slightly more flexible than greetings are.

When assigning EWT as a routing step you assign a table.  You cannot modify which table is assigned while the skillset is active but you can modify the EWT greetings and intervals.

Demonstrating Expected Wait Time is tricky because you cannot calculate which interval will be played when you make the call for your audience.  That is, you can re-configure Call Pilot Manager but the changes will not be as obvious or as powerful as the CLID/DNIS demonstration.

To change your EWT table you may need to record new greetings first from the Greetings menu under Contact Center in Call Pilot Manager.   Once your new greetings are recorded navigate to the Expected Wait Time Table find your table and click on change.  Manipulate the table as you see fit for the demonstration (change intervals, insert intervals, or delete intervals)

#### Caller Input Rules

Caller Input Rules are only used on Step 1 of Skillset 2 by default.

The only currently configured rule is that ‘1234’ will route you to skillset 3.  You can add a new rule to transfer to extension to demonstrate to the audience how easy Caller Input is to modify.

You will need to find the DN of a telset (Feature * 0 Intercom). For this example we will assume the DN is 2004.

Place a call into skillset 2 and when you are hearing the first greeting punch in 2004 on the telset (this is the DN you found earlier).  Nothing should happen.

Enter Call Pilot Manager and navigate to Caller Input Rules under Contact Center.  Click on Rules for table 1 and add a new rule.

Change the Match string to 2004 (this is the same number as the DN you found earlier), then select the radio button beside Transfer to Extension and fill the box with the DN you want to transfer to. Submit your changes and try the above scenario again.  Your call should then be transferred from Skillset 2 to DN 2004.

Reporting for Contact Center

Under the Contact Center menu select General Properties and set the CC Reporting Server Password.  The default is CCRS (case sensitive). Ensure that Enable CC Reporting Data Stream is checked.

Obtain a copy of the Reporting for Contact Center and IP View Client install files from the Install Clients web page on the BCM.

You also need a PC with Windows 2000, NT, or XP installed as the base Operating System.  You will need the Installation CD for this OS as Internet Information Services (IIS) is a required software component.

The first step to installing RCC is to install IIS.  Place your OS Installation CD into the CD-ROM drive.

Click on Start  Settings  Control Panel, and then click on Add/Remove Programs  Add/Remove Windows Components

Find Internet Information Services (IIS) in the list of components, DO NOT check it off! Select it with a left click and then click on Details. Scroll to the bottom of the components list and Check World Wide Web Server.  Other dependant components will also automatically be checked. Click OK to close the detailed view and then click Next to install the IIS World Wide Web Server.

You shouldn’t have to do any special configurations for IIS to work.

Next, install the Reporting for Contact Center and IP View Client packages.  These packages will work with all of the default settings.

Once you install the Reporting package you will need to reboot.

When your PC (now the Webhost PC) comes back up you may access the Reporting for Call Center (RCC) WUI from:

Http://localhost/rcc

The default login is admin, password is 0000.

Change your admin password after the first login, and then click on the Contact Center Connection option on the main page.

Enter the IP address of your BCM (192.168.1.10), and the default password (CCRS).  You may want to reduce the polling interval to 5 minutes, and leave the connection security configured as HTTPS.

Once you submit this page you will get a download status indicator.  Once it reaches 100% you will be able to use the application.

If you have not already done so, please install the IPview softboard at this time.  Accept all the default settings during the install process.

Once IP view is installed launch the application, then log into RCC via http://localhost/rcc

Log in as the admin account then navigate to Reporting  Wallboard Setup  Wallboard Assignment.

Click on Add

Enter the information for the wallboard:

| IP / Network Name |  |
| --- | --- |
| Port |  |
| Wallboard Skillset |  |
| Language |  |
| Type | Software Hardware |
| Title |  |
| Title Text |  |
| Buzzer |  |
| Summary |  |

Submit this page and your wallboard should start to display system level call center statistics.

Customized CLID Display

Under the Call Center menu select General Properties and decide how you want the demo to display CLID.  Under Enable Caller ID the following choices are available: Name with number backup, Number only, and none.

#### Convertec VCM /w Outlook 2000

### Outlook

First Verify that Microsoft Outlook is functioning on the Agent PC.

None of the configuration data has to be real; we are just using the Contact Database in Outlook not its mail functionalities. Should you have an exchange server configured, you must log into CoreWan before you can use outlook. This may present a problem if your demo is isolated on a switch or must be mobile and operate anywhere.

If outlook is already configured and you would like to change it so that you can operate without a connection to CoreWan, click on Tools  Options.  Then click on the Mail Setup Tab, and then the E-mail Accounts button.

Select the View or change existing e-mail accounts radio button and hit next.  Click on the Add button and in the next screen select POP3.

Enter miscellaneous information until the Next button lights up. You will be returned to the list of your E-mail accounts, select the new bogus account and click on move up, then Set as Default if it is not Default.  When the Demo is over reverse these steps to restore your CoreWan e-mail to be your primary.

If Microsoft Outlook is not installed obtain your departments Microsoft Office CD’s and install from there.  Follow the prompts for the installation and when you come to the Mail configuration select a POP/SMTP server and enter miscellaneous configuration data (names, server addresses, passwords).

Test that you can open/close outlook after your configuration and it does not attempt to connect to CoreWan (you will receive connection errors if your isolated and on a switch).

### LAN CTE

Install a LAN CTE Desktop keycodes for your BCM and install the Nortel Networks LAN CTE Client software on your agent PC.  Configure the LAN CTE client to use the IP address of your BCM.

The next step is to start up Unified Manager and go to Services  LAN CTE Configuration  IP Address. On the top of Unified Manager click on Configuration and then Add entry to Privileges list.  Enter in the IP address of your Agent’s PC and the DN you would like to control.

### VCM

Obtain the Convertec VCM client and server install files (http://www.convertecinc.com/condown.htm).  Install the VCM Server software first.  Follow the prompts filling in the data for your BCM and reboot if requested.

Once complete go to the systems control Panel then Telephony or Phones and Modems.  Under the advanced tab add the BCM LAN CTE TSP and the CONVERTEC VCM TSP.

Next is the installation of Convertec VCM Client side software.  Install the software package and follow the prompts, if prompted enter the same DN from your Unified Manager configurations. If prompted please reboot.

Next go to Start  Programs  VCM  VCM DBviewer.  Click on setup  database and select Outlook. If any data appears click on Import  Import…

A pop-up window will appear, click on Import and when successfully finished click OK. Now Exit the VCM Dbviewer.

Click on Start  Programs  VCM Server  VCM Administrator. Click on the Item Admin button.  Add your Agent DN’s to the list and remove miscellaneous DN’s you will not need.  If you want add your VM DN and CDNs. Click on save and exit the VCM Admin.

Launch the VCM Server, and then the VCM Client.

From the VCM Client click on View and ensure that Extended is checked and Extension & Name is checked.

Log in an agent on the DN you configured LAN CTE/VCM with.  Place calls to the DN and verify that CLID information is displayed. Try to click and drag the call to other DNs, the VM DN and Skillset CDNs.  Try some Voice Calls, Pages, and initiating internal calls. Try initiating an external call using the dialing plan you created for MMCC (9-769-1001)

The Demo

Now that your demo setup is complete, try a few test calls into the Contact Center, making sure to test both traditional (Voice) calls as well as Multimedia Contact Center calls.  A suggested Script of items to point out while doing the demo will be in Sea Green.

Once ready, your actual demo presentation should follow this scenario:

- The three analog phones you have setup will represent your different mock callers:

    - One phone should be labeled as “General caller”

    - One phone should be labeled as “Upscale caller”

    - The last phone should be used by the “MMCC caller” and should sit next to the MMCC caller PC

      - At this point explain to the viewers of the demonstration that the calls are entering the BCM through a simulated telephony network that provides CLID/DNIS upon which some advanced routing decisions will be made

- The three Contact Center agents should log on to their Telsets as follows:

    - The “Voice-General” agent logs into the “General” skillset on one Telset

    - The “Voice-Upscale” agent logs into the “Upscale” skillset on another Telset

    - The “Both-Gold” agent logs into the “Gold Member” skillset on the Telset sitting next to the MMCC agent PC

      - If using supervisor features in your demo log in the “Super” agent to any spare Telset, if only three Telsets are on-hand log out Agents as required and swap between the Agent accounts.

- From the “General caller” phone place a call into the Contact Center.  The caller will hear the “General” skillset 1 greetings, after which your “Voice-General” agent will answer.  Then end the call.

  - “This is a basic Contact Center configuration in which calls come in on a designated line and are routed to a specific group of trained personnel to handle that call type.  Examples would be a Support line for an electronics vendor.  Notice that on the Agents Telset interface CLID information is displayed, this data could be used by agents to greet callers by Surname if the situation of the call center permits”

- From the “Upscale caller” phone place a call into the Contact Center.  Because of the CLID/DNIS Routing step defined above, the call will be moved to “Upscale” skillset 2.  The caller will hear the first (forced) greeting associated with skillset 2, which will prompt the caller for his “Gold Skillset” password.  The caller does not enter a password and his call is routed to the “Voice-Upscale” agent assigned to skillset 2.  Then complete the call.

  - “This call was a little different from the previous scenario. In this example the Contact Center used CLID information to forward the call to Skillset 2, another difference from the first scenario is that the agent automatically answered the call.  This auto-answer is a feature configurable on a per-agents basis, and can be used for agents who use headsets. This feature can help to enable a totally hands free solution for Contact Center Agents. Finally, We were also prompted for our ‘Gold Skillset’ password, should we terminate this call and enter the password when prompted we will escalate to the ‘Gold Skillset”

- Again from the “Upscale caller” phone, place a call into the Contact Center.  Because of the CLID/DNIS Routing step defined above, the call will be moved to “Upscale” skillset 2.  The caller will again hear the first (forced) greeting associated with skillset 2, which will prompt the caller for their 4-digit pass code to enter the “Gold Skillset”.  Now the caller enters their ‘1234’ password, upon which he is routed to “Gold Member” skillset 3 (due to Caller Input Table rule 1).  The caller hears the first greeting in skillset 3 and then is answered by your “Both-Gold”.  DO NOT complete the call.

  - “This time we will enter our ‘Gold Skillset’ password and to escalate our call to the ‘Gold Skillset’.  This type of routing can be used in a variety of ways, for example preferred customers, and Personal Account Information.”

- Now place another Call from into the General Skillset. Have it answered by “Voice-General” and then have “Voice-General” blind transfer the call (F70) to the CDN of Skillset 3.

  - “Agents can transfer calls between Call queues with ease. This could simulate a customer who has a lot of business to conduct on a single call, or a caller who only has the number for the General Skillset and lost the number for Sales, or Upscale”

  - Allow some time to pass until an Expected Wait Time greeting plays.

  - “This is the Expected Wait time, this feature calculates the amount of time the caller must wait before they will have their call answered by an Agent. Expected wait time is a kind of dynamic greeting that changes as the expected wait time changes, from these or any other greetings you can program basic or advanced call Transfers. If I press 9 I will transfer to the Gold Skillsets Mailbox where I can leave a message”

  - Press 9 and leave a message, If MWI is configured do the next step

  - “If you look on this Telset the display shows messages waiting for this skillset, you can specify an attendant Telset and they will be notified of any new skillset Voice mails”

- At this point on the caller PC the “MMCC caller” is viewing Mainpage.html where they navigate to the Africa.html link.  Once on this page, “MMCC caller PC” clicks the “Browser and Phone session” Multimedia Contact Center.  The caller then sees the Connection Preferences page where they choose how to connect, and click on “Connect”.  Since the only Multimedia Contact Center enabled agent (“Both-Gold”) is busy on the previous call, the “MMCC caller PC” sees Web Refresher messages 148, 149, and 150 in the lower frame of their web browser. The main frame will show some canned pictures of travel destinations that change every half-minute.

  - “Right now we are making a Multimedia Call into the Contact Center.  The Agent assigned to answer Multimedia calls is currently on call so we will have to wait until they become available before we get connected.  If you will notice this main page, it is fully customizable and is hosted on the BCM.  You can modify these refresh pages to contain company promotions, seasons greetings or anything you can think of.  These pages refresh at a configurable interval”

- Now terminate the call between the “Both-Gold” agent and the “Upscale-to-Gold-Member” regular Contact Center caller from step 5

- After “Both-Gold” break time is over his PC will notify him that he has an incoming MMCC call.  Simultaneously the “MMCC caller” phone will ring notifying them that their MMCC call has begun.

  - “Notice the CLID information on the Telset, It indicates which web-enabled skillset a caller initiated the call to. If multiple MMCC queues are configured this can help agents to distinguish between the calls, and have a general idea on the subject matter of the call”

- “Both-Gold” agent and “MMCC caller” have now established a MMCC session.  Proceed to show all the functions on the Agent and Caller Monitors.

  - Talk about screen pushing to displaying data external of the Internet, for example Excel calculations showing the total cost of a trip.

    - It may be useful to have a pre-made MS Excel spreadsheet chart which breaks down the caller’s “expenses”.  A simple pie chart is very effective.  This spreadsheet chart can then be used with the Screen Shot button functionality on the Agent Monitor at the end of the session

  - Talk about follow me browsing, and mention the flash/java limitations.

  - Demonstrate Text chat and a Browser only call

  - (**Optional**) May also be useful to have multiple Page Push Lists for hotels, car rentals, restaurants, etc. in order to show the Change List button functionality on Agent Monitor. If your Demo will not have access to the CoreWan skip this step

- Terminate the MMCC session

- (**Optional**) Silent Monitor

  - Release all calls and MMCC sessions and log in the supervisor account if they are not logged in already. Place a call from the “Upscale-caller” and allow the call to enter skillset 2.  Press F905 on the supervisor agent’s Telset. Select Skillset 2 and select Agent ID 2.  Press “OBSV” to start a monitor session.  Press the "Info" softkey to view basic stats on the call, then press the “JOIN” softkey.  Demo that you have Voice Path and then press “MUTE”. Demo that you are silent again. Release the F905 session but keep the call active.

    - Mention IP and any other set limitations

- (**Optional**) Supervisor Help

  - While still on call from the previous step, invoke F906 from the agent 02 “Voice-Upscale” Telset via the Pre-programmed key. The supervisor will have the F906 tone, when prompted on the supervisor display answer the help request. Release the help request from the supervisor set, but keep the call active on the Agent Telset.

- (**Optional**) F909

  - From any agent Telset invoke F909 to view live statistics

- (**Optional**) Convertec VCM

  - Open Convertec VCM Server and VCM client if they are not open already.  Place Call Center calls to the DN you associated VCM with, to do this ensure an agent is logged in on the DN.

    - Demo Voice Calls, Pages, Click and drag call transfers, Point out CLID information on the display in VCM, hold, transfer, conference.  Place a call to a second DN that you have configured to be displayed and show that the status changed on the side-panel.

- Release all calls and ask for questions, or if there is anything they would like a better look at.

END OF DEMO

### Notes

Call Back:

To configure a certain line on your BCM for callback (used for Multimedia Call Center callback calls), you will need to assign an outgoing line to a Line Pool.  Follow these steps, which are to be done within the Element Manager application, downloadable from your BCM:

(The Line Pool is “Pool A” in this case)

(The line numbers are “line 189” and “line 190” in this case. These represent lines 3 and 4 on the CTM8 MBM; configure both these lines to have a common pool)

- On a Telset, do F985 to get the Voicemail DN (“375” in this case)

- In the Element Manager for your BCM:

  - TelephonySetsActive Application DNs, click the Capabilities and  Preferences tab and select DN 375 in the table.  In the Details for DN: 375 pane below, ensure that Prime Line field is set to Pool A

  - TelephonyDialing PlansLine Pools, check Pool A in the table and ensure that Access code field is set to 9

  - TelephonyLines->Active Physical Lines, select Line 189 in the table. In the Details for Line 189 pane below, select the Parameters tab and ensure that the Line type field is set to Pool A

- In the Call Pilot Manager for your BCM:

  - Mailbox Administration  Change/Delete Mailbox  “Change” for the MMCC Skillset Mailbox

  - Modify the “Outdial Type” field to reflect your configuration – either to use Route or Pool.

  - Should you select “Pool” you must use the numeric representation of your pool.  For example 1=Pool A, 2=Pool B, 3=Pool C, etc…

Now callback works when a Multimedia Contact Center call is put into the system.

IP Address Scheme:

| IP address | Machine |
| --- | --- |
| 192.168.1.10 255.255.255.0 | BCM – LAN 1 |
| 192.168.1.11 255.255.255.0 | MMCC Agent PC |
| 192.168.1.12 255.255.255.0 | MMCC Caller PC |

Sample CLID Numbers:

| CLID | Analog Telephone |
| --- | --- |
| 769-1000 | General Caller |
| 769-1001 | Upscale Caller |
| 769-1002 | Gold Caller |

### DEMO HTML – MMCC test page.html

<!DOCTYPE html PUBLIC "-//W3C//DTD HTML 4.01 Transitional//EN">

<html>

<head>

<title>Multimedia Contact Center Test Page</title>

</head>

<body bgcolor="#eeede0">

<font face="Verdana, Arial, Times"><br>

</font>

<h1><font face="Verdana, Arial, Times"> Voice Button Testing Page </font></h1>

<font face="Verdana, Arial, Times"><br>

<br>

<br>

<font size="2"> This page was created for members of the Product Verification

group in Calgary, AB, for BCM Contact Center using the MMCC enabled

feature.   <br>

<br>

Developers can also use this page for designer testing. <br>

<br>

In order for this page to work on <i>your</i> BCM server, you must

edit the source HTML script.  Search on the IP address "192.168.1.10" and

replace each occurrence with the IP address of your BCM. <br>

<br>

<br>

Simply choose a skillset (which has been configured to receive MMCC calls)

from the drop-down menu. <br>

Then press the "Call this skillset" button to initiate the MMCC call.  <br>

<br>

<br>

<div align="Center"><!-- You MUST ensure the IP address below matches your BCM server IP! -->

<form action="https://192.168.1.10 /ivb-cgi-bin/vb.exe" method="Post"><input type="hidden" name="cmd" value="call"><input type="hidden" name="customer" value="acd"><input type="hidden" name="dn" value="1"><input type="hidden" name="monitor" value="on">

<select name="group">

<option value="skset01" selected=""> skillset 1 </option>

<option value="skset02"> skillset 2 </option>

<option value="skset03"> skillset 3 </option>

<option value="skset04"> skillset 4 </option>

<option value="skset05"> skillset 5 </option>

<option value="skset06"> skillset 6 </option>

<option value="skset07"> skillset 7 </option>

<option value="skset08"> skillset 8 </option>

<option value="skset09"> skillset 9 </option>

<option value="skset10"> skillset 10 </option>

<option value="skset11"> skillset 11 </option>

<option value="skset12"> skillset 12 </option>

<option value="skset13"> skillset 13 </option>

<option value="skset14"> skillset 14 </option>

<option value="skset15"> skillset 15 </option>

<option value="skset16"> skillset 16 </option>

<option value="skset17"> skillset 17 </option>

<option value="skset18"> skillset 18 </option>

<option value="skset19"> skillset 19 </option>

<option value="skset20"> skillset 20 </option>

<option value="skset21"> skillset 21 </option>

<option value="skset22"> skillset 22 </option>

<option value="skset23"> skillset 23 </option>

<option value="skset24"> skillset 24 </option>

<option value="skset25"> skillset 25 </option>

<option value="skset26"> skillset 26 </option>

<option value="skset27"> skillset 27 </option>

<option value="skset28"> skillset 28 </option>

<option value="skset29"> skillset 29 </option>

<option value="skset30"> skillset 30 </option>

<option value="skset31"> skillset 31 </option>

<option value="skset32"> skillset 32 </option>

<option value="skset33"> skillset 33 </option>

<option value="skset34"> skillset 34 </option>

<option value="skset35"> skillset 35 </option>

<option value="skset36"> skillset 36 </option>

<option value="skset37"> skillset 37 </option>

<option value="skset38"> skillset 38 </option>

<option value="skset39"> skillset 39 </option>

<option value="skset40"> skillset 40 </option>

<option value="skset41"> skillset 41 </option>

<option value="skset42"> skillset 42 </option>

<option value="skset43"> skillset 43 </option>

<option value="skset44"> skillset 44 </option>

<option value="skset45"> skillset 45 </option>

<option value="skset46"> skillset 46 </option>

<option value="skset47"> skillset 47 </option>

<option value="skset48"> skillset 48 </option>

<option value="skset49"> skillset 49 </option>

<option value="skset50"> skillset 50 </option>

</select>

&nbsp;&nbsp;&nbsp;&nbsp;&nbsp;  <input type="submit" value="Call this skillset"></form>

</div>

<br>

<br>

<!-- You MUST ensure the IP address below matches your BCM server IP! -->

To change your VB connection preferences <a href="https://192.168.1.10/ivb-cgi-bin/vb.exe?cmd=setup">

click here. </a>

</font></font>

</body>

</html>