Resetting a FastRAD to factory default settings

- Press Feature 9 * *

Display will read: "pswd"

- Press the CHANGE softkey

- Enter your desired RAD password.

- Press the OK softkey

- Display will read: "confirm pswd"

- Re-enter the password

- Press the OK softkey

- Press the GEN softkey

- Press the SysID softkey

- Enter your Main phone number

- Press the OK softkey

- Press the DATA softkey

- Answer Lines: set to "N"

- Press OK or NEXT until you see "Baud Rate"

- Press the CHANGE softkey until the speed is set to 19200

- Press RLS

FastRAD Versions A & B

Resetting a FastRAD to factory default settings

STEP 1) Determine your software Version, by clicking here

STEP 2) CHANGE the DATE on the Norstar system to 09/15/00
If your software version is DR5, R1T1, 1.0 or 1.1, click here for the procedure to change the Time & Date 
If your software version is 2.0, 3.0, 4.0, 4.1 or 5.0, click here for the procedure to change the Time & Date

STEP 3) DeBug procedure:

Press Feature 9 * *

- Display will read: "password"

- Press the left softkey 5 times

- Press the middle softkey 5 times

- Press the right softkey 4 times

- Press the HOLD button

- Press the "2" key

- Press the "1" key

- Press the RLS or release key.

Congratulations! You have successfully reprogrammed the Remote Access Device

Compact ICS

IRAD Programming

Programming the IRAD Password

To enter a programming session, follow the next few simple steps:

1) Press FEATURE * * 2 6 6 3 4 4

2) Enter Password: 2 6 6 3 4 4

Display will read: " Terminals & Sets"

Using the navigation keys, as displayed below, follow the next simple steps:

3) Press: NEXT until the Display reads: "Passwords"

4) Press: SHOW

Display will read: "COS Pswds"

5) Press: Next until the Display reads: "IRAD Pswd"

6) Press the CHANGE softkey

Enter your NEW IRAD Password

Press the "OK" softkey.

Press the RLS key

Congratulations! You now have learned to program your IRAD password!