# Firmware Upgrade

- New SMDR6 units will be programmed with current release of Global SMDR6. No firmware upgrade is necessary. Existing SMDR6 units can be upgraded to the new firmware by using the following procedures:

- Connect the SMDR6 to the PC using the Serial Port.

- Set the baud rate of SMDR6 to 9600 baud using Feature 9*2 on a Norstar set.

- Start up a PC terminal program that supports the YModem-G protocol (e.g. HyperTerminal in Windows 95). Select the appropriate COM port and configure the port with the following parameters (you may need to re-start the program for the change to be effective):

- 9600 baud

- 8 data bit

- none parity

- 1 stop bit

- hardware flow control

- Enter the following command in the PC terminal:
ATY=Nor* 		(no space before and after ‘=’ sign)

- The SMDR6 will prompt for firmware download. Transfer (send or upload) the firmware file (smrdwnld.bin) using the YModem-G protocol

- After the upgrade, the SMDR6 should re-start automatically. Verify the new firmware code using the ATI command.

- Repeat the above procedures if the upgrade failed (SMDR6 prompts for firmware download again).

Note that only the download code is upgraded. The bootcode is not changed and maintains the previous bootcode firmware ID.