Large System Advisements

The following advisements are for large systems only: 51C, 61C, 81, and 81C.

Options 51C/61C/81/81C

Option 51C/61C/81/81C systems operating with a Motorola-based Call Processor (Call Processor 68040, 68060, or 68060E) are supported via CD-ROM and require IODU/C (NT5D61).

For an Option 81C system equipped with Call Processor PII, Release 25 requires a Multi-Media Disk Unit (NT4N43). The Multi-Media Disk Unit is automatically included with an upgrade to Call Processor PII.

Memory and Mass Storage Requirements

All IODU/C cards and MMDUs have the necessary disk partition space for Release 25 for all systems (minimum Hard Drive Capacity 121 Mbytes).

Refer to Call Processor Recommended Memory Requirements in this section for minimum memory requirements on Release 25

Installation/Upgrading Advisements

Call Processor PII

Call Processor PII will be available in Europe on a limited availability basis until September to facilitate lab orders. Order codes will be published in a product bulletin closer to GA

Fiber Network Fabric

FNF will will be available in Europe on a limited availability basis until September to facilitate lab orders. Order codes will be published in a product bulletin closer to GA

Call Processor Memory Requirements

How to Meet Release 25 Memory Requirements

With regard to call processor memory requirements, the following tables define the actions necessary to be supported in Release 25 for existing system type / call processor combinations. The required actions are based on the minimum memory requirements for Release 25 as presented in the following section.

For Option 51C/61C Systems:

| Existing System Type | Existing Processor | Existing Memory Config. | Action to be supported on Release 25 |
| --- | --- | --- | --- |
| Option 51C/61C | 68040,68060, 68060E | 48 MB (32/16) | Upgrade to 80 MB (32/48) by adding one 32 MB DRAM SIMM (per card) |
| Option 51C/61C | 6804068060, 68060E | 64 MB (32/32) | Upgrade to 96 MB (32/64) by adding one 32 MB DRAM SIMM (per card) |
| Option 51C/61C | 6804068060, 68060E | 80 MB (32/48) | None - Configuration is supported. |
| Option 51C/61C | 6804068060, 68060E | 112 MB (64/48) | None - Configuration is supported. |
| Option 51C/61C | 6804068060E | 128 MB (64/64) | None - Configuration is supported. |

Note:  (xx/yy) denotes Flash memory (xx) and DRAM memory (yy) configuration.

For Option 81/81C Systems on 68060/68060E with five or fewer network groups, or Option 81/81C Systems on 68040:

| Existing System Type | Existing Processor | Existing Memory Config. | Action to be supported on Release 25 |
| --- | --- | --- | --- |
| Option 81/81C | 68040,68060, 68060E | 64 MB (32/32) | Upgrade to 96 MB (32/64) by adding one 32 MB DRAM SIMM (per card) |
| Option 81/81C | 68040,68060, 68060E | 80 MB (32/48) | Upgrade to 112 MB (32/80) by adding one 32 MB DRAM SIMM (per card) |
| Option 81/81C | 68040,68060, 68060E | 112 MB (64/48) | Upgrade to 144 MB (64/80) by adding one 32 MB DRAM SIMM (per card) |
| Option 81/81C | 68040,68060E | 128 MB (64/64) | None - Configuration is supported. |

For Option 81/81C Systems on 68060/60E with 6 or more network groups:

| Existing System Type | Existing Processor | Existing Memory Config. | Action to be supported on Release 25 |
| --- | --- | --- | --- |
| Option 81/81C | 68060, 68060E | 64 MB (32/32) | Upgrade to 128 MB (32/96) by adding two 32 MB DRAM SIMMs (per card) |
| Option 81/81C | 68060, 68060E | 80 MB (32/48) | Upgrade to 112 MB (32/80) by adding one 32 MB DRAM SIMM (per card) |
| Option 81/81C | 68060, 68060E | 112 MB (64/48) | Upgrade to 144 MB (64/80) by adding one 32 MB DRAM SIMM (per card) |
| Option 81/81C | 68060E | 128 MB (64/64) NT5D03 FA | Upgrade to 144 MB (64/80) by removing one 16 MB DRAM SIMM and adding one 32 MB DRAM SIMM (per card) |
| Option 81/81C | 68060E | 128 MB (64/64) NT5D03 FB | Upgrade to 160 MB (64/96) by adding one 32 MB DRAM SIMM (per card) |

The NTZC75AA DRAM Memory Upgrade Kit supports 32 MB DRAM memory upgrade of Call Processors 68040, 68060 and 68060E. This kit contains one 32 MB DRAM SIMM and supports the memory upgrade of one call processor card. The number of SIMMs required to upgrade a call processor to the minimum memory requirement is defined above for currently existing memory configurations.

The NTZC77BA package contains the anti-static mat and ESD wrist-strap, which is required to perform a memory upgrade. NTZC77AA is not required if the distributor/customer already possesses the anti-static mat and wrist-strap.

Release 25 Minimum Memory Requirements for 68040, 68060 or 68060E Processors:

| System Type | Flash Memory Requirement | DRAM Memory Requirement | Total Memory | Applicable Regions |
| --- | --- | --- | --- | --- |
| Option 51C/61C | 32 MB | 48 MB | 80 MB | All |
| Option 81/ 81C on 68060/68060E with 5 or fewer Network Groups, or Option 81C on 68040 | 32 MB | 64 MB | 96 MB | All |
| Option 81/ 81C on 68060/68060E with 6 or more Network Groups | 32 MB | 80MB | 112 MB | All |
| Option 81C on CP PII | 64 MB | 64 MB | 128 MB | All |

Call Processor PII is available only in the 128-Mb memory configuration.

Release 25 Recommended Memory Requirements for 68060E Processors:

| System Type | Flash Memory Requirement | DRAM Memory Requirement | Total Memory | Applicable Regions |
| --- | --- | --- | --- | --- |
| Option 51C/61C | 64 MB | 64MB | 128 MB | All |
| Option 81/ 81C (with or without Fiber Network Fabric) | 64 MB | 96 MB | 160 MB | All |
| Option 81C with CP PII | 64 MB | 64 MB | 128 MB | All |

Option 51C and 61C new systems will ship with Call Processor 68060 for standard realtime  and 68060E for advanced realtime option. For hardware upgrades that require (or elect) call processor upgrades, Call Processor 68060E is available for Options 51C, 61C and 81/81C systems. Where the processor upgrade is to the 68060 or 68060E, recommended memory configurations are as described above. The minimum memory requirements are as documented in the previous section of this document.

Call Processor PII is available only in the 128-Mb memory configuration.

Software Delivery Methods

The only supported media for large systems (Options 51C, 61C, 81 and 81C) is CD-ROM. This means that Release 25 requires an IODU/C (NT5D61) on a system operating with Call Processor 68040 (NT9D19), 68060 (NT5D10) or 68060E (NT5D03). For an Option 81C system equipped with Call Processor PII, Release 25 requires an Multi-Media Disk Unit - MMDU (NT4N43). The MMDU is automatically included with an upgrade to Call Processor PII.

Fiber Network Fabric Advisements

FIJI Card Loadware Version

For release 25.15, the FIJI card loadware should be issue 01310011.

Ensure No Groups Contain Only Phantom Loops Prior to Upgrading

Prior to upgrading a system to Release 25 software with FNF package (365), the customer must first check the configuration records to ensure there are no groups that contain ONLY phantom loops. If such a configuration exists in the system, the customer must first remove all phantom loops in that group(s) and load Overlay 43 to do an EDD (data dump) before the software upgrade takes place.