Software Packaging

New Package Numbers Introduced by R25

The following new packages are being introduced with Release 25 software

| FEATURE | Mnemonic |  | Option Pkg | Avail in Int'l | Supported on Opt 11C | Package on Cont release |
| --- | --- | --- | --- | --- | --- | --- |
| Agent greeting | FXS |  | 152 | Yes | Yes | Yes |
| MDECT 2000 - Multi-Site Mobility Networking | MSMN |  | 370 | Yes | Yes | Yes |
| 10/20 Digit ANI on 911 Calls (North America only) |  | M911_ENH | 249 | Yes | Yes | Yes |
| Business Networking Express |  | BNE | 367 | Yes | Yes | Yes |
| Core Processor PII (81C) |  | CPP_CNI | 368 | Yes | No | Yes |
| M3900 series Meridian Digital Telephones: Set-to-Set Messaging |  | STS_MSG | 380 | Yes | Yes | Yes |
| M3900 series Meridian Digital Telephones: Corporate Directory |  | CDIR | 381 | Yes | Yes | Yes |
| M3900 series Meridian Digital Telephones: Virtual Office |  | VIRTUAL_OFFICE | 382 | Yes | Yes | Yes |
| Fiber Network Fabric (81 / 81C) |  | FIBN | 365 | Yes | No | Yes |

R25 New Feature Packaging

Following is a list of the new features, the package in which they are to be found. and the pre requisites needed to obtain full feature operation

| FEATURE | MNEM | NUM | New Feature Pre requisites |
| --- | --- | --- | --- |
| Agent Greeting | FXS | 152 | 45 |
| MDECT 2000 - Multi-Site Mobility Networking | MSMN | 370 | 139,240, 254, 350 |
| Meridian Internet Telephone (i2004) |  |  | 88, 170 |
| Calling Line Identification on Analog Trunks for Singapore, Australia and Hong Kong (DXUT-A) |  |  | 349 |
| BNE / EuroISDN Call Diversion & Gateways | BNE | 367 | 14, 19 , 57, 58, 59, 95, 101, 102, 139, 145, 146, 154, 161, 184, 202, 216, 222, 233, 261, 301, 323 |
| BNE / Name and private Number Display Gateways | BNE | 367 | 14, 19 , 57, 58, 59, 95, 101, 102, 139, 145, 146, 154, 161, 184, 202, 216, 222, 233, 261, 301, 323 |
| BNE/ EuroISDN Explicit Call Transfer | BNE | 367 | 14, 19 , 57, 58, 59, 95, 101, 102, 139, 145, 146, 154, 161, 184, 202, 216, 222, 233, 261, 301, 323 |
| 10/20 Digit ANI on 911 Calls | ENH_M911 | 249 | 19,40,41,45,214,224,225,234 |
| M3900 series Meridian Digital Telephones: Set-to-Set Messaging | STS_MSG | 380 | 88, 170 |
| M3900 series Meridian Digital Telephones: Corporate Directory | CDIR | 381 | 88, 170 |
| M3900 series Meridian Digital Telephones: Virtual Office | VIRT_OFFICE | 382 | 88, 170 |
| Core Processor PII (81C) | CPP_CNI | 368 | 299 mutually exclusive with 298 Package 368 mutually exclusive with 365 (FNF) |
| Fiber Network Fabric (81 & 81C) | FIBN | 365 | 298 (81) or 299 (81C). Packages are mutually exclusive |
| D-Channel Expansion |  |  | 145, 222 146(PRA) or 147 (ISL) or 154 (PRI2) |
| Plug-in infrastructure | PLUG_IN | 366 |  |
| Plug-Ins |  |  | 366 |
| 24 Port Digital Line Card |  |  |  |
| Private to Public CLID Conversion |  |  | 57 or 58 and / or 59,145 146 (PRA) or 154 (PRI2) |
| ITG Trunkside 2.0 |  |  | 59,145,147,148,160 57 (BARS) or 58 (NARS) 222,263,305 (only for large systems) |
| Functionality based Pricing Phase II |  |  |  |
| Inventory Reporting Phase 2 |  |  |  |
| OTM |  |  | - Alarm Management 164, 242, 243, 296 - Maintenance Windows 164, 242, 243, 296 - Ethernet Connection for (Station Administration, Traffic Analysis and ESN ART) 164, 242 & 296 - SMNP Alarms (Open Alarms) 315 - Data Buffering and Access (DBA) 351 |

R25 Software ISM Parameters

Introduction

R25 is introducing seven new ISM counters : ITG ISDN Trunks, Traditional Trunks, Data Ports, Phantom Ports, CLASS Telephones, Attendant Consoles, Internet telephones and is changing the counting criteria for two existing ISM counters, Digital telephones & Analogue telephones and remove the MOPT ISM.

The ITG ISDN Trunks ISM is non-modifiable.

Operation of the remaining ISM counters is not changed.

Table below shows the differences in ISM counting between pre-R25 and R25.

| A TN configured in OVL 10/11/12/14 | Previous ISMs counted | New ISMs Counted |
| --- | --- | --- |
| Digital voice set | TNS, DIGITAL TELEPHONES | TNS, DIGITAL TELEPHONES |
| Digital data set | TNS, DIGITAL TELEPHONES | TNS, DIGITAL TELEPHONES |
| ATA | TNS, DIGITAL TELEPHONES | TNS, DIGITAL TELEPHONES |
| MCA | TNS, DIGITAL TELEPHONES | TNS, DIGITAL TELEPHONES |
| M39xx set | TNS, DIGITAL TELEPHONES | TNS, DIGITAL TELEPHONES |
| Analogue set | TNS, ANALOGUE TELEPHONES | TNS, ANALOGUE TELEPHONES |
| Internet set | None | TNS, INTERNET TELEPHONES |
| An Attendant console | DIGITAL TELEPHONE | TNS, ATTENDANT CONSOLES |
| CLASS set | TNS, ANALOGUE TELEPHONES | TNS, CLASS TELEPHONES |
| Line-Side set | TNS, ANALOGUE TELEPHONES | TNS, ANALOGUE TELEPHONES |
| Phantom analogue set | TNS | TNS, PHANTOM PORTS |
| Phantom digital set | TNS, AST | TNS, PHANTOM PORTS |
| A Data Port | TNS, DIGITAL or ANALOGUE TELEPHONES | TNS, DATA PORTS |
| MCMO (CT2) set | TNS, WIRELESS TELEPHONES | TNS, WIRELESS TELEPHONES |
| MDECT set | TNS, WIRELESS TELEPHONES | TNS, WIRELESS TELEPHONES |
| Microcellular set | TNS, WIRELESS TELEPHONES | TNS, WIRELESS TELEPHONES |
| ACD analogue set | TNS, ACD AGENTS, ANALOGUE TELEPHONES | TNS, ACD AGENTS, ANALOGUE TELEPHONES |
| ACD digital set | TNS, ACD AGENTS, DIGITAL TELEPHONES | TNS, ACD AGENTS, DIGITAL TELEPHONES |
| Meridian Mail/Call Pilot port | TNS | TNS |
| Meridian Integrated ACD Port | TNS, DIGITAL TELEPHONE, ACD AGENT | TNS, DIGITAL TELEPHONE, ACD AGENT |
| Analogue AST set | TNS, AST, ANALOGUE TELEPHONES | TNS, AST, ANALOGUE TELEPHONES |
| Digital AST set | TNS, AST, DIGITAL TELEPHONES | TNS, AST, DIGITAL TELEPHONES |
| Analogue AST & ACD set | TNS, AST, ACD AGENTS, ANALOGUE TELEPHONES | TNS, AST, ACD AGENTS, ANALOGUE TELEPHONES |
| Digital AST & ACD set | TNS, AST, ACD AGENTS, DIGITAL TELEPHONES | TNS, AST, ACD AGENTS, DIGITAL TELEPHONES |
| AST & Meridian Mail ACD | TNS | TNS |
| A Traditional Trunk | None | TRADITIONAL TRUNKS |
| An ITG Trunk | ITG ISDN TRUNKS (blocked to max) | ITG ISDN TRUNKS |

X11 Release 25.xx System Limits

The following table lists the ISM limits available on X11 Release 25.XX.

The characters is Bold specified the ISM changes

| ISM | Description | Technical | Limits | Default value | per system | type |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Min. | Max | Option 11C | Option 11C Mini | Large |
| LOOP LIMIT | Loop Limit | 1 | 1023 | N/A | N/A | 0(c ) |
| TNS | System TNs | 0 | 32767 | 2500 | 2500 | 32767 |
| ACD AGENTS (former AGNT) | ACD Agents | 0 | 32767 | 0 (Variable FBP(d)) Non FBP : 1700 | 0 Variable FBP. Non FBP 1700 | 0 (Variable FBP) 32767 |
| ACDN | ACD DNs | 0 | 24000 | 300 | 300 | 24000 |
| AST | Associate Telephone Sets | 0 | 32767 | 200 | 200 | 500 |
| BRI DSL | Digital Subscriber Loop | 0 | 32767 | 150 | 150 | 10000 |
| LTID | Logical Terminal Ids (ISDN BRI) | 0 | 32767 | 0 | 0 | 0 |
| DCH | D-Channels Handlers | 0(b) | 254 | # 0 | # N/A | # 64 |
| AML(a) | Application Module Links | 0(b) | 16 | N/A | N/A | 16 |
| MPH DSL | M1 Packet Handlers – Digital Subscriber Loop (ISDN BRI) | 0 | 32767 | N/A | N/A | 64 |
| RAN CON | RAN Connections | 0 | 32767 | 0 (variable) | 0 (variable) | 0 (variable) |
| RAN RTE | RAN Routes | 0 | 32767 | 500 | 500 | 512 |
| MUS CON | Music Broadcast Connections | 0 | 32767 | 0 (variable) | 0 (variable) | 0 (variable) |
| BRAND | Brand line display option | 0 | 2 | 0 | 0 | 0 |
| DIGITAL TELEPHONES | Digital telephones | 0 | 32767 | 0 (variable FBP) 1700 | 16 (Variable FBP) Non FBP variable 16-48 then max out at 1700 | 0 (variable FBP) 32767 |
| ANALOGUE TELEPHONES | Analogue Telephones | 0 | 32767 | 0 (variable FBP) 1700 | 0 (variable FBP) Non FBP 1700 | 0 (variable FBP) 32767 |
| TMDI D-CHANNELS | # TMDI D-channels | 0 | 32767 | 0 | 0 | N/A |
| WIRELESS TELEPHONES (former MOBTNS) | Wireless telephones | 0 | 32767 | 0 (variable FBP) 1024 | 0 (variable FBP) Non FBP 1024 | 0 (variable FBP) 1024 |
| ITG_ISDN | ITG ISDN Trunk | 0 | 32767 | 0 (variable FBP) | 0 (variable FBP) | 0 (variable FBP) |
| TRADITIONAL TRUNKS | Traditional trunks | 0 | 32767 | 2500 | 2500 | 32767 |
| DATA PORTS | DATA ports | 0 | 32767 | 2500 | 2500 | 32767 |
| PHANTOM PORTS | PHANTOM ports | 0 | 32767 | 2500 | 2500 | 32767 |
| ATTENDANT CONSOLES | ATTENDANT Console | 0 | 32767 | 2500 | 2500 | 32767 |
| INTERNET TELEPHONES | Internet telephone | 0 | 32767 | 0 (variable FBP) Non FBP : 1024 | 0 (variable FBP) Non FBP 1024 | 0 (variable FBP) Non FBP : 1024 |
| CLASS TELEPHONES | CLASS Telephone | 0 | 32767 | N/A in Europe | N/A in Europe | N/A in Europe |

(a) AML & DCH are pre-set as a fixed value for Opt 11C. The user will not be prompted for its value in Overlay 143.

(b) Not allowed

(c) unlimited number allowed

(d) FBP = Functionality Based Pricing

The values above are the technical limits but real limits are depending of the configuration of the system & network.

ISM definitions

Incremental Software Management (ISM) is a feature that provides flexibility and control over system configuration and implementation.

LOOP LIMIT : Define the number of loops for large systems

TNS : System TNs ISM will count *all* TNs allowed within the system (eg :Wireless Telephones, Analogue Telephones, and Digital Telephones, Phantom TN, BRI Line, Meridian Mail /Call Pilot ports).

ACD AGENTS : This ISM will count all ACD sets/ports as follows :

- ACD Analogue set

- ACD Digital set

- Meridian Integrated Conference Bridge

ACDN : This parameter will count all ACD DNs (queue)

AST : This ISM will count all AST sets/trunks as follows :

- Phantom Digital set

- Analogue AST set

- Digital AST set

- Associated trunk for CTI trunk monitoring and control

BRI DSL : This ISM will count the BRI unit

LTID : This ISM will count BRI Unit only if MISP or BRSC support D channel packet data.

DCH : This ISM will count the D-Channels Handlers on large systems

AML : This ISM will count the number of Application Module Links (eg : Meridian Mail / Call Pilot, Meridian Link, CCR)

MPH DSL : This parameter will count the Meridian Packet Handlers

RAN CON : This ISM will count the number of broadcast RAN connections.

RAN RTE : This parameter will count the number of broadcasting RAN routes.

MUS CON : This parameter will count the number of Music Broadcast connections.

BRAND : This parameter will control the brand display on the digital sets (North America only)

DIGITAL TELEPHONES : This ISM will count digital telephones as follows :

- Digital voice set

- Digital data set

- ATA

- MCA

- MCU

- R232 DAC

- R422 DAC

- ACD digital set

- Meridian Integrated Conference Bridge

- Digital AST set

ANALOGUE TELEPHONES : This ISM will count analogue telephones as follows :

- Analogue set

- Line side T1/E1

- ACD Analogue set

- Analogue AST set

TMDI D-CHANNELS : This ISM will control the number of 1.5M D-Channels

WIRELESS TELEPHONES : This ISM will count wireless telephones as follows :

- CT2 set

- MDECT set

ITG ISDN TRUNKS : This ISM will control the maximum number of ITG ISDN trunks allowed on the system (ITG 2.0 & later version).

INTERNET TELEPHONES : This ISM will count internet telephones as follows :

Ratio 1/1 = 1 i2004order ISM counter will increase by 1

ATTENDANT CONSOLES : This ISM will control the number of attendant consoles configured in OVL 12 (each TN occupied by an Attendant Console will be used for SYS TNs ISM counting criteria). It is important that the attendant console is not implemented on Digital extension

CLASS TELEPHONES : This counter will count every configured CLASS Telephone

DATA PORTS : This counter will count every configured Data Port as follows :

- Analogue data set (FAXA)

- Any set configured in OVL 11 as Data TNs

PHANTOM PORTS : This counter will count every Phantom Port configured in OVL 10.

TRADITIONAL TRUNKS : This counter will count each Traditional Trunk as follows :

- Analog Trunk

- Digital Trunk

- ISDN Trunk

- ITG 1.0 Trunk

R25 Software Package Dependency Reference

The following table lists all packages available on X11 Release 25.

NEW PACKAGES ARE HIGHLIGHTED IN BOLD

| PKG | MNEM | Description | Technical prerequisites | Market Availability |
| --- | --- | --- | --- | --- |
| 0 | BASIC | Basic System Features | 1 | 9.30A |
| 1 | OPTF | Optional Features | 0 | 9.30A |
| 2 | CUST | Multi Customer |  | 9.30A |
| 4 | CDR | Call Detail Recording | 5 | 9.30A |
| 5 | CTY | Call Detail Recording, Teletype Terminal | 4 | 9.30A |
| 7 | RAN | Recorded Announcement |  | 9.30A |
| 8 | TAD | Time and Date |  | 9.30A |
| 9 | DNDI | Do Not Disturb Individual | 16 | 9.30A |
| 10 | EES | End To End Signalling |  | 9.30A |
| 11 | INTR | Intercept Treatment |  | 9.30A |
| 12 | ANI | Automatic Number Identification | 13 | 9.30A |
| 13 | ANIR | Automatic Number Identification, Route Selection | 12 | 9.30A |
| 14 | BRTE | Basic Routing | 32 | 9.30A |
| 15 | RPE1.5 | 1.5 MB RPE (not Opt 11) | Not with 165 (RPE2) | 9.30A |
| 16 | DNDG | Do Not Disturb Group | 9 | 9.30A |
| 17 | MSB | Make Set Busy |  | 9.30A |
| 18 | SS25 | Special Service for 2500 sets | 73 | 9.30A |
| 19 | DDSP | Digit Display |  | 9.30A |
| 20 | ODAS | Office Data Administration System |  | 9.30A |
| 21 | DI | Dial Intercom |  | 9.30A |
| 22 | DISA | Direct Inward System Access |  | 9.30A |
| 23 | CHG | Charge Account | 52,24 | 9.30A |
| 24 | CAB | Charge Account/Authorisation Code Base | 23,52 | 9.30A |
| 25 | BAUT | Basic Authorisation Code | 23,24,52 | 9.30A |
| 26 | CASM | Centralised Attendant Service, | Mutually exclusive with pkg 159 | 9.30A |
| 27 | CASR | Centralised Attendant Service | Mutually exclusive with pkg 159 | 9.30A |
| 28 | BQUE | Basic Queuing |  | 9.30A |
| 29 | NTRF | Network Traffic |  | 9.30A |
| 32 | NCOS | Network Class Of Service |  | 9.30A |
| 33 | CPRK | Call Park |  | 9.30A |
| 34 | SSC | System Speed Call | 32 | 9.30A |
| 35 | IMS | Integrated Message Services | 40, 45, 46, 109 | 9.30A |
| 36 | ROA | Recorded Overflow Announcement | 7 | 9.30A |
| 37 | NSIG | Network Signalling | 32 | 9.30A |
| 38 | MCBQ | Network Call Back Queuing | 14, 28, 32, 37, 57 / 58 / 59, 61 | 9.30A |
| 39 | NSC | Network Speed Call | 14, 28, 32, 34, 57 / 58 / 59, 61 | 9.30A |
| 40 | BACD | Basic Automatic Call Distribution |  | 9.30A |
| 41 | ACDB | Automatic Call Distribution, Package B | 40, 45 | 9.30A |
| 42 | ACDC | Automatic Call Distribution, Package C | 40, 41, 45 | 9.30A |
| 43 | LMAN | Automatic Call Distribution, Load Management Reports | 40, 41, 42, 45 | 9.30A |
| 44 | MUS | Music | 7 | 9.30A |
| 45 | ACDA | Automatic Call Distribution, Package A | 40 | 9.30A |
| 46 | MWC | Message Waiting Centre | 40 | 9.30A |
| 47 | AAB | Automatic Answer back |  | 9.30A |
| 48 | GRP | Group Call |  | 9.30A |
| 49 | NFCR | New Flexible Code Restriction | 32 | 9.30A |
| 50 | ACDD | Automatic Call Distribution, Package D | 40, 41, 42, 45, 51 | 9.30A |
| 51 | LNK | ACDD Auxiliary Processor Link | 40, 45, 50 | 9.30A |
| 52 | FCA | Forced Charge Account | 23, 24 | 9.30A |
| 53 | SR | Set Relocation |  | 9.30A |
| 54 | AA | Attendant Administration |  | 9.30A |
| 55 | HIST | History file |  | 9.30A |
| 56 | AOP | Attendant Overflow Position | Mutually exclusive with pkg 159 | 9.30A |
| 57 | BARS | Basic Alternate Route Selection | 14, 32 | 9.30A |
| 58 | NARS | Network Alternate Route Selection | 14, 32 | 9.30A |
| 59 | CDP | Co-ordinated Dialling Plan | 14, 28, 32, 57 / 58 | 9.30A |
| 60 | PQUE | Network Priority Queuing | 14, 28, 32, 58, 61 | 9.30A |
| 61 | FCBQ | Flexible Call Back Queuing | 14, 28, 32, | 9.30A |
| 62 | OHQ | Off Hook Queuing | 14, 28, 29, 32, 61 optional: 29 (on main and/or remote) | 9.30A |
| 63 | NAUT | Network Authorisation Code | 14, 23, 24, 25, 28, 32, 52, 57 / 58 / 59, 61 | 9.30A |
| 64 | SNR | Stored Number Redial |  | 9.30A |
| 65 | TDET | Tone Detector (not Opt 11) |  | 9.30A |
| 67 | NXFR | Network Transfer | 32, 37 | 9.30A |
| 70 | HOT | Enhanced Hotline |  | 9.30A |
| 71 | DHLD | Deluxe hold |  | 9.30A |
| 72 | LSEL | Automatic Line Selection |  | 9.30A |
| 73 | SS5 | 500 Set Dial Access to Features | 18 | 9.30A |
| 74 | DRNG | Distinctive Ringing |  | 9.30A |
| 75 | PBXI | 1.5 MB DTI |  | 9.30A |
| 76 | DLDN | Department Listed Directory Number |  | 9.30A |
| 77 | CSL | Command Status Link |  | 9.30A |
| 79 | OOD | Optional Outpulsing Delay |  | 9.30A |
| 80 | SCI | Station Category Indication |  | 9.30A |
| 81 | CCOS | Controlled Class Of Service |  | 9.30A |
| 82 | RSDB | Resident Debug (only for non C_machine) |  | 9.30A |
| 83 | CDRQ | Call Detail Recording, Queue Record | 4 | 9.30A |
| 84 | ATM | Automatic Trunk Maintenance (not Opt 11) | 65 | 9.30A |
| 86 | TENS | Multi-Tenant Service |  | 9.30A |
| 87 | FTDS | Fast Tone & Digit Switch |  | 9.30A |
| 88 | DSET | M2000 Digital Sets |  | 9.30A |
| 89 | TSET | M3000 Digital Sets | 88 | 9.30A |
| 90 | LNR | Last Number Redial |  | 9.30A |
| 91 | DLT2 | M2317 Digital Sets | 88 | 9.30A |
| 92 | PXLT | Pre translation |  | 9.30A |
| 93 | SUPV | Supervisory Console |  | 9.30A |
| 95 | CPND | Calling Party Name display | 88 / 89 / 91 / 140 / 170 | 10.10B |
| 97 | JPN | Japan Central Office Trunk |  | 9.30A |
| 98 | DNIS | Dialled Number Identification Service | 40, 45 | 10.10B |
| 99 | BGD | Background Terminal |  | 10.10B |
| 100 | RMS | Room Status | 81, 99 | 10.10B |
| 101 | MR | PPM / Message Registration | 131 | 10.10B |
| 102 | AWU | Automatic Wake-Up | 7, 81, 99 | 10.10B |
| 103 | PMSI | Property Management System Interface | 81, 99, 100 / 101 / 102 | 10.10B |
| 104 | OPAO | Outpulsing of "*" and "#" |  | 10.10B |
| 105 | LLC | Line Load Control |  |  |
| 107 | MCT | Malicious Call Trace | 4 | 10.10B |
| 108 | ICDR | Internal CDR | 4 | 10.10B |
| 109 | APL | Auxiliary Processor Link | 40, 45 | 10.10B |
| 110 | TVS | Trunk Verification From Station |  | 10.10B |
| 111 | TOF | Automatic Call Distribution, Time Overflow Queuing | 40, 41, 45 | 10.10B |
| 113 | IDC | Incoming Digit Conversion | 32, 49 | 14.41D |
| 114 | AUXS | Automatic Call Distribution, Package D, Auxiliary Security | 40, 41, 42, 45, 50, 51 | 14.41D |
| 115 | DCP | Direct Call Pick Up |  | 14.41D |
| 116 | PAGT | Automatic Call Distribution, Priority Agent | 40, 45 | 14.41D |
| 117 | CBC | Call by Call Service | 145, 154 / 147 | 14.41D |
| 118 | CCDR | Calling Line ID in CDR | 4, 145, 154 / 147 | 14.41D |
| 119 | EMUS | Enhanced Music | 7, 44 | 14.41D |
| 120 | PLDN | Group Hunt / DN Access to SCL | 32, 34, 81, 131,139 | 15.58F |
| 121 | SCMP | Station Camp-On | 131 | 14.41D |
| 122 | IDA | Integrated Digital Access | 129 | 16.87G |
| 123 | DPNSS | Digital Private Network Signalling System 1 | 122, 148,129, 14, 28, 32, 57/ 58/ 59, 61, 145, 154 / 147 | 16.87G |
| 124 | DASS2 | Digital Access Signalling System 2 | 122, 123,148,129, 14, 28, 32, 57/ 58/ 59, 61, 145, 154 / 147 | 16.87G |
| 125 | FTC | Flexible Tones and Cadences | 74 | 9.30A |
| 126 | OPCB | Operator Call-back (China #1) | 131, 107 / 127 / 128 (if 107 then 4 is required) | 14.41D |
| 127 | BKI | Attendant Break-In / Trunk Offer |  | 9.30A |
| 128 | MFC | Multi-Frequency Compelled Signalling |  | 9.30A |
| 129 | DTI2 | 2MB Digital Trunk Interface |  | 10.10B |
| 131 | SUPP | International Supplementary Features |  | 9.30A |
| 132 | TBAR | Trunk Barring |  | 10.10B |
| 133 | ENS | Enhanced Night Service |  | 14.41D |
| 134 | AFNA | Attendant Forward No Answer | Not with Pkg 174 | 14.41D |
| 135 | MFE | Multi-Frequency Signalling for Socotel |  | 10.10C |
| 136 | JDMI | Japan Digital Multiplex Interface (not Opt 11) |  | 14.41D |
| 137 | LSCM | Local Steering Code Modifications | 58 / 59, 61 | 10.10C |
| 138 | DTD | Dial Tone Detector |  | 10.10C |
| 139 | FFC | Flexible Feature Codes | 81 required for Electronic Lock feature | 14.41D |
| 140 | DCON | M2250 Attendant Console | 88 | 15.56F |
| 141 | MPO | Multi-Party Operations |  | 14.41D |
| 143 | ICP | Intercept Computer Interface | 7, 17, 35, 40, 45, 46, 81, 109, 131, 139 | 10.10C |
| 144 | ABCD | 16 Button Digitone/Multi frequency Telephone |  | 14.41D |
| 145 | ISDN | Integrated Services Digital Network |  | 14.41D |
| 146 | PRA | 1.5 MB Primary Rate Access | 75, 145 | 14.41D |
| 147 | ISL | ISDN Signalling Link | 145 | 14.41D |
| 148 | NTWK | Advanced ISDN Network Services | 14, 28, 32, 58, 61, 145, 129&147/154/183/233 | 14.41D |
| 149 | IEC | Inter Exchange Carrier | 75, 145, 146 | 14.41D |
| 150 | DNXP | DN Expansion (7 Digit) | 151 required if CDR equipped | 14.41D |
| 151 | CDRE | CDR Expansion (7 Digit) | 4, 150 | 14.41D |
| 152 | FXS | Flexible Services Package | 45 | 25.0x |
| 153 | IAP3P | ISDN-AP for 3rd Party Vendors | 77 | 14.46E |
| 154 | PRI2 | 2MB Primary Rate Access | 145 | 14.46E |
| 155 | ACNT | Automatic Call Distribution, Activity Code | 40, 41, 42, 45, 50, 51, 114 | 14.46E |
| 157 | THF | Trunk Hook Flash (Centrex) |  | 14.46E |
| 158 | FGD | Feature Group D | 57, 58, 59, 151 | 18.20H |
| 159 | NAS | Network Attendant Service | 14, 32, 58, 145, 161, 129&147 /154 /233 57 required for NAS Equi-repartition 59 required for NAS routing Not with Pkg.'s 26, 27, 56 | 14.46E |
| 160 | FNP | Flexible Numbering Plan | 14, 28, 32, 58, 59 | 14.46E |
| 161 | ISDN INTL SUP | ISDN Supplementary Features | 14, 28, 32, 58, 145, 129&147/ 154& opt 202/ 233 (202 for call cx limitation, DID to network, L1/isdn gateway) | 14.46E |
| 162 | SAR | Scheduled Access Restrictions | 25 | 15.56F |
| 163 | MINT | Message Intercept | 125, 74 | 15.56F |
| 164 | LAPW | Limited Access to Overlays |  | 15.56F |
| 165 | RPE2 | 2MB Remote Peripheral Equipment (not Opt 11) | Not with 15 | 15.56F |
| 167 | GPRI | International 1.5 / 2.0 MB Gateway (DTI released PRI deferred) | 146, 154, 145 opt 75 | 18.20H |
| 168 | TMON | Traffic Monitoring (not Opt 11) |  |  |
| 169 | COOP | Console Operations |  | 14.46E |
| 170 | ARIE | Aries Digital Sets | 88 | 14.41D |
| 171 | JTDS | Japan Tone & Digit Switch | 74 | 14.41D |
| 172 | CPGS | Console Presentation Groups | 86, 162 | 15.56F |
| 173 | ECCS | Enhanced Controlled Class of Service | 81 | 15.56F |
| 174 | AAA | Attendant Alternative Answer | Not with Pkg 134 | 15.56F |
| 175 | NMS | Network Message Services | 14, 28, 32, 46, 58, 148, 202&154 / 129&147 /233 (10 required for Mmai)l | 15.56F |
| 176 | DTOT | DID to Tie | Japan only | 16.87G |
| 178 | EOVF | Automatic Call Distribution, Enhanced Overflow | 40, 41, 45, 111 | 15.56F |
| 179 | HVS | Meridian Hospitality Voice Services | 7, 10, 17, 35, 40, 45, 46, 77, 109, 180 | 15.56F |
| 180 | DKS | Digit Key Signalling | 7, 10, 17, 35, 40, 45, 46, 77, 109 | 15.56F |
| 181 | SACP | Semi-Automatic Camp-On |  | 15.56F |
| 182 | TFM | Trunk Failure Monitor |  | 15.56F |
| 183 | VNS | Virtual Network Services | 58, 32, 14, 145, 147, 148, 161 154 required for PRI2 | 16.67G |
| 184 | OVLP | Overlap Signalling | 14, 32, 58 | 15.56F |
| 185 | EDRG | Executive Distinct. Ringing | 58 , 32, 145, 161 | 16.67G |
| 186 | POVR | Priority Override / Forced Campon | 81, 139, 141 | 15.56F |
| 187 | RPA | Radio Paging | 81, 139 | 15.58F |
| 188 | L1MF | X08 to X11 Gateway | 14, 28, 32, 57/ 58/ 59, 61, 128, 145, 147 / 154, 159, 161 | 15.56F |
| 190 | UK | UK Program | 203, 204, 205 | 16.87G |
| 191 | SECL | Series Call |  | 15.56F |
| 192 | ORC_RVQ | Originator Routing Control (Dropback Busy) | 145, 148, 61 | 18.20 |
| 193 | RCK | Ringing Change Key |  | 15.56F |
| 196 | OHOL | On Hold On Loudspeaker |  | 20.06 |
| 197 | FRTA | French Type Approval | 131 | 15.59 |
| 198 | FFCSF | Boss Secretary Filtering (FCC activation) | 139 | 15.56F Note 1 |
| 200 | AINS | Automatic Installation | Option 11 only |  |
| 202 | IPRA | International Primary Rate Access(CO) | 145, 154 | 15.56F |
| 203 | XPE | Meridian 1 XPE | 131, 204, 205 | 15.56F |
| 204 | XCT0 | M1 Enhanced Conference, TDS & MFS | 205 | 15.56F |
| 205 | XCT1 | M1 Superloop Administration (LD 97) | 204 | 15.56F |
| 206 | MLWU | Multi-Language Wake-Up | 7, 81, 99, 102 | 16.67G |
| 207 | NACD | Network Automatic Call Distribution | 14, 28, 32, 37, 40, 41, 45, 58/ 59, 61, 111, 145, 148, 154 / 147, 178 | 15.56F |
| 208 | HSE | Hospitality Screen Enhancement | 170 | 18.20H |
| 209 | MLM | Meridian Link Module | 7, 8, 10, 17, 19, 35, 40, 45, 46, 77, 109, 153, 180 | 16.67G |
| 210 | MAID | Maid Identification | 81, 99, 100, 103 | 18.20H |
| 211 | MLIO | Multi-Language In-Output | 88,95 | 16.67G |
| 212 | VAWU | VIP Auto Wake Up | 102 | 18.20H |
| 214 | EAR | Enhanced ACD Routing | 41 | 18.20H |
| 215 | ECT | Customer Controlled Routing | 77, 214 247 from Release 20 on | 18.20H |
| 216 | BRI | Basic Rate Interface | 203 | 18.20H |
| 218 | IVR | Hold in queue for IVR | 215 | 18.20H |
| 219 | MWI | MWI Interworking with DMS |  | 20.06 |
| 221 | CIST | CIS Digital Trunk Interface | 125, 131, 160, 203 DTI = 129, 204, 205 3WIRE = 87, 182 | 21.1x |
| 222 | MSDL | Multi-Purpose Serial Data Link | 145, 154 (for ISDN functionality) 227 (for SDI) - Release 20 onwards | 18.20H |
| 223 | FC68 | FCC Compliance for DID Answer Supervision | not with 97 | 18.20H |
| 224 | M911 | M911 | 19, 40, 41, 45, 214, 225 | 20.06 |
| 225 | CWNT | Call Waiting Notification | 19, 40, 41, 45, 214 | 20.06 |
| 227 | MSDL SDI | MSDL Serial Data Interface (not Opt 11) | 222 | 20.06 |
| 228 | MSDL STA | Single Terminal Access (not Opt 11) | 222, 227 | 20.06 |
| 229 | SSAU | Station Specific Authorisation Codes | 25 | 20.06 |
| 230 | MDP | Manufacturing Delivered Patches (not Opt 11) |  | 20.06 |
| 231 | DNWK | DPNSS Network Services | 14, 28, 32, 57/ 58/ 59, 61, 122, 123, 129, 13,148, 145, 154 / 147 optional MCDN Gateway 131, 145, 148, 159 optional 161 | 16.90G |
| 232 | PEMD | Pulsed EAM (Indonesia, French Colisee) (No MFE functionality) | 129, 18, 73, 131, 127, 128, 101 126 required for Indonesia | 18.20H |
| 233 | BRIT | ISDN BRI Trunk Access | 216, 203, 145 | 18.20H |
| 234 | FCDR | New format CDR | 4, 5 | 18.20H |
| 235 | BRIL | BRI line application | 145, 203, 216 | 18.20H |
| 236 | ACRL | AC15 Recall | Reminded timer & Norstar transfer 131 M1 transfer 258 | 20.19 |
| 240 | MCMO | Meridian 1 Companion | 18, 139 | 20.19 |
| 242 | MULTI_USER | Multi User Login (not supported on Option 11) |  | 20.06 |
| 243 | ALRM_FILTER | Meridian 1 Fault Management | 55 | 20.06 |
| 245 | SYS_MSG_LKUP | System Message Lookup | 55 | 20.06 |
| 246 | VMBA | M Mail Voice Box Administration |  | 20.06 |
| 247 | CALL | Call Identification |  | 20.06 |
| 248 | MPH | Meridian 1 Packet Handler (not Opt 11) | 203, 216, 145, 154 | 20.06 |
| 249 | M911_ENH | M911 Enhancement Display | 19,40,41,45,214,224,225,234 | Rls 25.0x |
| 250 | DPNA | Direct Private Network Access | 7, 22, 25, 63 | 21.1x |
| 251 | SCDR | Station Activity Records | 4,5 | 20.19 |
| 252 | KD3 | Spanish KD3 DID/DOD Interface | 14, 18, 58, 59, 61, 113, 126, 129, 131, 160 | 20.06 |
| 253 | ARFW | Attendant & Network Remote Call Forward | 1, 139, 81 (18, 73 - for 500/2500) (145, 58, 59 - for NWK) | 20.06 |
| 254 | PHTN | Phantom TN Operation | 139 - for RCFW | 20.06 |
| 255 | INBD | Japan D70 PRI nB+D | 19, 75, 145, 146, 202, 222 | 20.06 |
| 256 | ADMINSET | Administration Set | 164, 242, 139 Option 11 : 200 Admin Set : 19, 88, 170 CPND : 95 Digit Display : 19 | 21.1x |
| 258 | ATX | AutoDial Tandem Transfer | 10 | 20.06 |
| 259 | CDRX | CDR Enhancement Phase 1 | 4, 5, 234 | 20.19 |
| 261 | EURO | EuroISDN | 19 (for CLID ), 145, AOC: 4, 81, 101, 131, 161 PRI: 154, 202, 222 BRI: 203, 216, 233 OVLP : 19 (for CLID ), 184, 160, 57/58 | 20.06 |
| 262 | SAMM | Standalone Meridian Mail | 122, 123, 175 | 20.19 |
| 263 | QSIG | ECMA QSIG | 19 (for CLID ), 145, 202 PRI: 154, 202, 222 BRI: 203, 216, 233 OVLP : 19 (for CLID ), 184, 160, 57/58 Name Display: 19, 95, 305 | 20.19 |
| 279 | MLMS | Multi-Language Messages | 211,245 | 20.06 |
| 283 | UIGW | Universal ISDN Gateway | 154 122, 123, 124 for DPNSS Interworking | 20.06 |
| 284 | DPNSS 189I | Enhanced DPNSS1 Gateway | 122, 123, 154 | 20.06 |
| 285 | CHINA | M1 IPE Loss Plan for China | 126,131 | 20.06 |
| 286 | REM_IPE | Remote IPE (not Opt 11) | 203 (required), 1.5 MB RPE = 15 2.0 MB RPE - 165 (mutually exclusive) | Rls 22.08 |
| 288 | DPNSS_ES | Enhanced DPNSS Services | Attendant Consoles = 127 DPNSS Network = 122, 123, 131, 145, 154 DPNSS Sup Services = 231 DPNSS - MCDN = 148, 159, 161 | 21.1x |
| 289 | ADSP | ACD Loop Start Trunk |  | 20.19 |
| 290 | CCB | Collect Call Blocking |  | 21.1x |
| 291 | NI-2 | North America National ISDN Class II Equipment | 145, 146, 222 | 21.1x |
| 292 | CHTL | China Toll | 128, 129, 131 | 21.1x |
| 294 | BTD | Busy Tone Detection | 203 | 21.1x |
| 296 | MAT | Network access to M1 management | 164, 242 optional 243 | Rls 22.08 |
| 297 | MQA | Multiple Queue Assignment | 40, 42, 45, 50, 51, 88, 170 For Agent Priority : 116 For Auto Forward of IDN calls :254 | 21.19 |
| 298 | CPIO | Option 81 (not Opt 11) | Not with 299 | 21.19 |
| 299 | CORENET | Option 81C (not Opt 11) | Not with 298 | 21.19 |
| 301 | CPP | Calling Party Privacy | 139 | 21.19 |
| 302 | MOSR | Mobility Server | 216, 254,303 | Rls 22.08 |
| 303 | MMO | Mobility Microcellular | 302, 216, 254 | Rls 22.08 |
| 304 | ARDL | Automatic Redial (not Opt 11) | 65 | Rls 22.08 |
| 305 | QSIGGF | QSig GF Transport | 263 | Rls 22.08 |
| 306 | CPRKNET | Call Park Networkwide | 33 For Network Call Park - 159 | Rls 22.08 |
| 307 | PAGENET | Call Page Networkwide |  | Rls 22.08 |
| 309 | MASTER | EuroISDN Master Mode | EuroISDN: 261 | Rls 22.08 |
| 310 | CPCI | Called Party Control | 107 | Rls 22.08 |
| 311 | NGCC | New Generation Call Center (ICCM) include Queue to NACD for ICCM | 41,42,43,50,114,115,214,215,218,247,324 | Rls 22.08 |
| 312 | TATO | Trunk Antitromboning | 145, 154, 147, 148 (recommended) | 21.1x/Rls 22.08 |
| 313 | ISPC | Australian SPCs | 202, 145, 154, 203, 222, 129, 147 | Rls 22.08 |
| 314 | MMSN | Mobility Multisite Nwk | 216, 254,302,303 | Rls 22.08 |
| 315 | OPEN ALARM | SNMP Alarm Integration |  | Rls 22.08 |
| 316 | QSIG-SS | QSig Sup Services Call Completion | 305, 1, 263, 59,160 optional 184 | Rls 22.08 |
| 321 | QTN | Queue to NACD (NCCR) for CCR. | 207, 215 | Rls 22.31 |
| 323 | ETSI_SS | EuroISDN Supplementary Services | 261, 1, 59, 160 | Rls 22.08 |
| 324 | NGEN | New Generation foundation | 77,153,164,242,243,296 | Rls 22.08 |
| 325 | DMWI | DPNSS Message Waiting | 122,123,231,10,46 | Rls 23.12A |
| 326 | CISMFS | CIS MF Shuttle Signalling | 125,129,131,160,221 | Rls 23.12A |
| 327 | RANBRD | RAN Broadcast | 7,11 | Rls23.12A |
| 328 | MUSBRD | Music Broadcast | 7,44 | Rls 23.12A |
| 329 | ESA | Emergency Services Access | 12, 20, 95, 145 | Rls 23.12A |
| 330 | ESA_SUPP | Emergency Services Access Supplementary | 329 | Rls 23.12A |
| 331 | ESA_CLMP | Emergency Services Access Calling Number Mapping | 329 | Rls 23.12A |
| 332 | CNUMB | CLASS: Calling number delivery |  | Rls 23.12A |
| 333 | CNAME | CLASS: Name Delivery | 95 | Rls 23.12A |
| 334 | NI-2 CBC | NI-2 call by call service selection | 291,145,146,222 | Rls 23.12A |
| 335 | JTTC | Japan TTC Common Channel Signalling | 145,146/154 ,202,216,233 | Rls 23.12A |
| 344 | Reserved |  | 164,353 Incompatible : (MMCS pack. 352) | Rls 24.25 |
| 347 | TWR1 | Taiwan R1 Modified Signaling | 160 | Rls 24.25 |
| 348 | MEET | MCDN End To End Transparency | ISDN PRI : 145,146,148,222,263,305 ISDN PRI2 : 145,148,154,202,222,263,305 ISDN BRIT : 145,148,216,222,233,263,305 For NAS : 14,28,32,58,59,61,86,148,159,161,192 incompatible : 26,27,56 For NACD : 14,19,28,32,40,41,58,148,178,207, (57 or 59) For NMS-MC : 46, 148, 175 ( with ACD 40,41,42,45) For NMS-MM : 10,35,40,45,46,77,148,175 | Rls 24.25 |
| 349 | ACLI | Information Notification Service for Japan | 97, (package UK(190) incompatible) | Rls 24.25 |
| 350 | MC32 | Meridian Companion Enhanced Capacity | 240 (Not option 11c) | Rls 24.25 |
| 351 | DBA | Data Buffering and Access | 296 | Rls 24.25 |
| 353 | Reserved |  | 164,344 Incompatible : (MMCS pack. 352) | Rls 24.25 |
| 362 | FDID | Flexible DID | 49,81,99,100,113, with PMS : 103 | Rls 24.25 |
| 364 | NMCE | NGenR2/Call Pilot Connectivity | 41,46,77,95,153,164,214,215,218,242,247,243,254,296,324 Optional: For Networked Messaging: 175 | Rls 24.25 |
| 365 | FIBN | Fiber Network Fabric | 298(option 81), 299 (option 81C) Only applicable to Option 81 & 81C | Rls 25 |
| 366 | PLUGIN | Plug-In Functionality |  | Rls 24.25 |
| 367 | BNE | Business Networking Express | 14, 19 , 57, 58, 59, 95, 101, 102, 139, 145, 146, 154, 161, 184, 202, 216, 222, 233, 261, 301, 323 | Rls 25 |
| 368 | CPP_CNI | CP PII | 299 Mutually exclusive with 298 Only applicable for Option 81C | Rls 25 |
| 370 | MSMN | Multi-Site Mobility Networking | 139,,240, 350 | Rls 25 |
| 380 | STS_MSG | Set-to-Set Messaging | 88, 170 | Rls 25 |
| 381 | CDIR | Corporate Directory | 88, 170 | Rls25 |
| 382 | VIRTUAL_OFFICE | Virtual Office | 88, 170 | Rls 25 |

Note 1: This package was introduced on 15.59F and is not available on 16.67G, 16.87G, and 16.90G

Key :	, indicates AND for option prerequisites

/ indicates OR for option prerequisites

Not Opt 11 indicates package not supported on option 11C

[] indicates optional feature packages for full feature functionality

Mut. Exc.. Mutually Exclusive

NEW PACKAGES ARE HIGHLIGHTED IN BOLD, Updates from previous release are in Italic.