System Advisements

X11 Release 25.15 is a global software release. This document provides the advisements applicable to North America CALA, and Europe, excluding Asia Pacific, for the system types indicated in the “Systems Supported” section.

In this document, Small Systems refer to the Option 11C and Option 11C Mini. Large Systems refer to the Option 51C, 61C, 81 and 81C machine types. System Advisements are applicable to all System Types. Sections 5 and 6 document advisements that are Small and Large system specific respectively.

Systems Supported

Generic X11 Release 25.15 supports the following machine types (Note - not all processors were made available in all markets):

- Meridian 1 Option 11C equipped with an NTDK20CA or higher Small System Controller (SSC) AND NTTK13AA software daughterboard (providing 48Mb Program Store) which is necessary to provide sufficient program store for Release 25.

- Meridian 1 Option 11C Mini equipped with a NTDK97AB Mini System Controller which provides 48Mb of memory for program store.

Meridian 1 Options 51C, 61C, 81, and 81C equipped with the Motorola 68040 (NT9D19), 68060 (NT5D10), or 68060E (NT5D03) commercial processors.   Release 25 upgrade for Options 51C, 61C, 81 and 81C is only supported via CD-ROM and require an IODU/C drive. The 68030 is not supported with Generic X11 Release 25.

- Meridian 1 Option 81C equipped with the Call Processor PII commercial processor.

Controlled Release Features

The following features are on Controlled Release effective with the initial introduction of Release 25.15 until September 2000 to facilitate laboratory orders. Contact your Nortel Sales Representative for the market availability status of Controlled Release features.

- Call Processor PII

- Fiber Network Fabric

Feature Interactions

- Microcellular features (packages 345, 346, 314, 302, 303) are not supported on any Release 25 based and later systems.

Memory

For small system memory requirements, refer to Small System Memory Requirements for Release 25 in section 5.  For large system memory requirements, refer to Call Processor Recommended Memory Requirements in section 6.

Hardware

For Base Software

X11 Global Release 25 base software may require memory upgrades as described above. Please refer to sections 5 and 6, for small and large systems.

EPE and Release 25

EPE will not be supported on systems wishing to upgrade to X11 Release 25 whose platforms reside on CP PII Call Processors. Cards on the network shelves are supported. The EPE in question are cards plug into PE, EPE and EEPE shelves (cards that are 1 1/2" spacing).  Connections from Networking Equipment to Meridian Mail and Digital Trunk/Primary Rate Interface are not impacted by this requirement and remain supported on X11 Release 25.

For New Features

New hardware is required to activate several of the new Release 25 features:

The following features introduce new hardware. Please see the descriptions of the features in section 7 for details.

• Call Processor PII (Option 81C only)

• Fiber Network Fabric (Option 81and Option 81C only)

• M3900 Series Enhancements - Introduces new vintage sets with new firmware to support Phase 2 features. The M3900 Digital Telephones have a Flash Memory download capability that allows downloading a new firmware version from the Meridian 1 to the M3900 telephone. In North America migration from current Release 1 M3900 sets that X11 Release 24 supports, to Release 2 M3900 sets that X11 Release 25 supports is done using the M3900 Flash Download capability. Once Release 2 M3900 telephones start shipping, no flash download will be required for new Release 2 M3900 telephone orders. Additional migration information related to M3900 Series Enhancements can be found in M3900 Series Enhancements Order Management & Logistics Information Package.

• ITG Trunks 2.0 with ISDN (new vintage ITG card)

• CLID on Analog trunks for Hong Kong (new DXUTA Pack)

New LD 32 Commands to Reset XPECs and XNETs

Two new commands have been added in overlay 32 to allow you to manually enable XPEC and XNET cards. The new commands are XRST and FRST.

In the event of a Meridian 1 power reset (e.g. during system upgrade to Fiber Network Fabric), there is a chance that the Meridian 1 may fail to automatically enable all the XPECs and/or FPECs in the system.  In such as a case it is recommended to use the following manual procedures to enable XPECs and/or FPECs as required:

- Identify the network loop number(s) and the loop type(s) (i.e. XNET or FNET) associated with the XPEC(s)/FPEC(s).

- Load Overlay 32.

- Disable one of the loops identified in Step 1 using the DISL command.

- Issue a hardware reset to the loop using the XRST command for an XNET loop, or FRST command for an FNET loop.

- Enable the loop using the ENLL command.

- Repeat steps 3 to 5 for each loop identified in Step 1.

Note 1- the XRST command also works to reset the Local Carrier Interface (LCI) card of a Carrier Remote superloop.

Note 2 - these new LD 32 prompts are not directly supported in MAT 6.6 or OTM 1.0 but are accessible through the Pass through window to overlay 32.

Real Time Impact of Release 25

The real time impact of Release 25 is shown in the following table. These values are based on the average basic calls measurements in combination with the real time impact on some basic market models which make extensive use of key features, such as: CPND, CDR, digital trunking, digital sets. Sites more heavily configured with these features will experience degradations at the high end of the ranges given, while those with only basic calls will experience degradations at the low end of the ranges.

Release to release change in real time capacity

| Machine type | 24 to25 |
| --- | --- |
| 11C | 6% |
| CP2: (51C/61C/81/81C) | 16% |
| CP3: (51C/61C/81/81C) | 6% |
| CP4: (51C/61C/81/81C | 6% |

CP4 to CP PII improvement in real time capacity

| Machine type | Release 25 |
| --- | --- |
| CP4 to CP PII: (Options 51C/61C/81/81C) | 206% (= 3.06X) |

Real Time Upgrade Factors

This section provides the guidelines to ensure "safe" upgrades to Release 25.

Upgrades to Later Releases - Real Time Consideration:

The table below shows the real time degradation expected when upgrading to Release 25.

This value does not include the impact of new applications or features.  The table applies to Thor Systems Call Processors going back as far as Release 18. CP2 (CP68040) going back as far as Release 21B. 11C systems going back as far as Release 22. CP3 (CP68060) systems going back as far as Release 23 and CP4 (CP68060E) systems going back as far as Release 24. Ranges - the width of the ranges compounds as the number of releases traversed increases.

Release-by-release Real Time Cost of Software Upgrade

The table applies to Thor Systems, CP going back as far as Release 18, CP2 going back as far as Release 21B, 11C systems going back as far as Release 22, CP3 systems going back as far as Release 23 and CP4 systems going back as far as Release 23C. In order to accommodate the fact that performance varies with machine type and with feature mix, ranges of performance ratio rather than isolated ratios are given. For Release 25, CP3 and CP4 machines are at the lower end of the ranges shown, 11C and CP2 at the higher end.

| From / to | 19 | 20B | 21B | 22 | 23 | 23C | 24 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | 4-6% | 23-31% | 33-47% | 49-75% | 55-91% | 58-99% | 72-169% | 82-212% |
| 19 |  | 18-24% | 27-39% | 42-65% | 48-80% | 51-87% | 65-154% | 75-195% |
| 20B |  |  | 8-12% | 21-33% | 26-45% | 29-51% | 41-104% | 49-137% |
| 21B |  |  |  | 12-19% | 16-30% | 18-35% | 30-84% | 38-113% |
| 22 |  |  |  |  | 4-9% | 6-13% | 16-54% | 23-79% |
| 23 |  |  |  |  |  | 2-4% | 11-42% | 18-65% |
| 23C |  |  |  |  |  |  | 9-36% | 16-58% |
| 24(A) |  |  |  |  |  |  | 2-20% | 8-39% |
| 24B |  |  |  |  |  |  |  | 6-16% |

To use the table, multiply “percent CPU utilisation" on the present system, (obtained from the TFS004 traffic report) and column "to" destination by 1 + a percent within the range shown in the table cell that is located at row "from" present release.

From the table the real time cost for a Release 22 to 24 Software upgrade is 14-28%.

Example: If TFS004 indicates 60% CPU utilization. Impact of cost of upgrade ranges from: 60*1.14% = 68.4% to 60*1.28% = 76.8%.

A range rather than a single number is specified because the real time effect varies significantly according to the site configuration. The range expresses the range of effects seen when looking at various market models and also at the effect on different call types - the basic calls, and the more widely used featured calls.

As a user of the table, you have the option of picking any single number within the range or of replacing all the ranges with single numbers and doing the multiplication accordingly (if you are traversing multiple releases.)

Machine to Machine Real Time Conversion Table

Machine types - the information from the real time cost of software upgrade table and the machine-to-machine conversion table can be combined.

The table below shows the machine to machine real time conversion factors. This can be used in conjunction with the table above in the case where one wants to upgrade to a different machine / processor when converting to Release 25.

| from/to | 11C | CP1 | CP2 | CP3 | CP4 | CP PII |
| --- | --- | --- | --- | --- | --- | --- |
| 11C | 1.00 | 1.29 | 0.77 | 0.58 | 0.42 | 0.14 |
| 81/51C/61C/81C CP1 | 0.77 | 1.00 | 0.59 | 0.45 | 0.32 | 0.11 |
| 81/51C/61C/81C CP2 | 1.30 | 1.68 | 1.00 | 0.75 | 0.54 | 0.18 |
| 81/51C/61C/81C CP3 | 1.73 | 2.23 | 1.33 | 1.00 | 0.72 | 0.23 |
| 51C/61C/81/81C CP4 | 2.40 | 3.10 | 1.85 | 1.39 | 1.00 | 0.33 |
| 51C/61C/81/81C CP PII | 7.35 | 9.49 | 5.65 | 4.26 | 3.06 | 1.00 |

Note:  Due to rounding, compounded conversions FROM / TO may not always equal the equivalent single conversion. E.g. 51C -> 61C CP1 (CP68030) -> CP2 (CP68040) -> 81C CP3 (CP68060) is 0.64 * 0.73 = 0.46, but 51C -> 81C CP3 (CP68060) is 0.43. The table is intended for single conversions.

To project the impact of a CPU upgrade: multiply the real time utilization by the old CPU to new CPU factor found in the table above.

NOTE: The machine-machine conversion table shows only the ratios for the current release (Release 24). If the system is going up a release(s), get the cost range for upgrading to Release 24 first, then do the conversion on this range.

Example:

If in Release 20 on a CP1 (CP68030) real time use is 60%, and the cost range for moving up to Release 24 is 38-70%. That would mean that when you upgrade software on this CP1 (CP68030) you could expect to use from 1.38 * 60% to 1.70 * 60% of real time (83-102%).

If there has been a hardware upgrade to CP68060, and the machine-machine ratio is 0.47 from CP1 (CP68030) to CP3 (CP68060)(Table 5.7.2) then the real time use in release 24 on the CP3 (CP68060) is .47 * (83-102%) or 39-48% CPU use.

Memory Upgrade Factors

Upgrades to Option 11C from 11E:

A customer upgrading to Release 25 on an 11C (from 11E) may do so without considering the memory requirements for his particular configuration. That is, if it fits on 11E, then it fits on 11C.

Upgrades to Thor machines (Options 51C/61C/81/81C CP2-4,

Options 81C CP PII) and to 11C from 11C:

A customer upgrading from an Omega (Options 21E/51/61/71) or Thor system to Release 25 on a Thor system (CP2-4, CP PII), or from an 11C to Release 25 on an 11C, must consider the size of his SL-1 database in terms of Release 25. With this knowledge he can then determine which size system and how much DRAM he will need.

To make this determination, take the following steps:

• Use MEMAVAIL display in SL-1 database user interface to determine the size of the SL-1 customer database on the system being upgraded from. The “USED” field in the MEMAVAIL display, returned by the SL-1 user interface after loading any service change overlay that can add or delete data (e.g., overlay 11), gives the present size (in memory) of the SL-1 customer data base in SL-1 words. (In this discussion, this quantity will be referred to as “SL1DB-words”.)

• Compare your SL1DB-words with the limits shown in Table 2 below to see if your memory requirement is small enough to be able to upgrade to Release 25 on the desired machine type. • If your customer database size is too large to upgrade to the desired machine type and/or DRAM size, it may be possible to make it fit by configuring fewer call registers. (This is done in Overlay 17 – the configuration record user interface - with the NCR prompt after you have answered “Yes” to the PARM (change system parameters) prompt. Note that you must re-initialize the system in order to make your call register count change take effect.) In Table 3 below are shown the recommended call register counts for each machine type, and they’re corresponding claims on SL-1 customer database memory. Use this as a guide to try for an optimal call register count. (As shown in the table, in Release 25 call registers use memory at a rate of 220 words per call register.)

• If you are upgrading to full Release 25 on an 11C or a CP2, 3 or 4 you need at least 32MB of DRAM. Most likely, if you are on a THOR machine your database won’t be sufficiently small enough to allow you to fit on 32MB, and so you will need at least 48MB. New THOR machines (CP4) will be sold with a minimum of 48MB DRAM. New 11C machines will be sold with 32MB DRAM.

Table 1: MEMAVAIL USED upper bounds for UPGRADE to Full Rls25

| Machine being upgraded TO: (columns) Release being upgraded FROM: (rows) | CP2-4 machines with 32MB DRAM | CP2-4 machines with 48MB DRAM | CP2-4 machines with 64MB or more* DRAM | CP PII machines with 128MB DRAM 81C |
| --- | --- | --- | --- | --- |
| upgrading from OMEGA (Options 21E/51/61/71) machines |  |  |  |  |
| Phase 8b | 384,000 | 1,751,000 | 2,456,000 | 4,037,000 |
| Release 18 | 470,000 | 2,143,000 | 3,006,000 | 4,940,000 |
| Release 19 | 521,000 | 2,373,000 | 3,329,000 | 5,471,000 |
| Release 20 | 814,000 | 3,710,000 | 5,204,000 | 8,552,000 |
| Release 21 | 896,000 | 4,081,000 | 5,724,000 | 9,408,000 |
| upgrading from THOR (Options 51/61/81/81C) machines |  |  |  |  |
| Phase 8b | 288,000 | 1,313,000 | 1,842,000 | 3,028,000 |
| Release 18 | 352,000 | 1,607,000 | 2,254,000 | 3,705,000 |
| Release 19 | 390,000 | 1,780,000 | 2,497,000 | 4,103,000 |
| Release 20 | 610,000 | 2,783,000 | 3,903,000 | 6,414,000 |
| Release 21 | 672,000 | 3,061,000 | 4,293,000 | 7,056,000 |
| Release 22 | 732,000 | 3,336,000 | 4,679,000 | 7,689,000 |
| Release 23C | 800,000 | 3,648,000 | 5,116,000 | 8,408,000 |
| Release 24B | 935,000 | 4,260,000 | 5,974,000 | 9,818,000 |

(Units=SL1words)

*For larger than 64MB DRAM add 4,000,000 to the “upgrading from THOR” entry that applies. (Databases upgrading from Omega will not be larger than the 64MB DRAM requirement.)

Table 2: MEMAVAIL USED upper bounds for UPGRADE (units=SL1words) on 11C.

| Machine being upgraded TO: (columns) Release being upgraded FROM: (rows) |  | 11C with 32MB DRAM |
| --- | --- | --- |
| Upgrading from 11C |  |  |
| Release 22 | 2,451,000 |  |
| Release 23C | 2,680,000 |  |
| Release 24B | 3,130,000 |  |

Table 3: Recommended Call Register Counts

|  | 11C | 51C CP2 | 61C CP2 | 51C CP3&4 | 61C CP3&4 |
| --- | --- | --- | --- | --- | --- |
| Recommended Call Register Count | 1750 | 1500 | 3000 | 2000 | 4000 |
| Memory Required (SL-1words) | 385,000 | 330,000 | 660,000 | 440,000 | 880,000 |

|  | 81/81C CP2 | 81/81C CP3&4 | 81C CP PII w/o NCE | 81C CP PII w/NCE |
| --- | --- | --- | --- | --- |
| Recommended Call Register Count | 7500 | 10000 | 20000 | 25000 |
| Memory Required (SL-1words) | 1,650,000 | 2,200,000 | 4,400,000 | 5,500,000 |

System Security

Nortel Networks strongly recommends changing the default system passwords for both Meridian 1 and Meridian Mail systems during initial installation. These passwords should be changed again when the system is placed in active service. These actions will help deter unauthorized system access which can result in toll fraud or system abuse.

For more information, please refer to the System Security Management NTP (NTLH10AA - A0804757) included with new system or system upgrade shipments.

Audit routine

As in the case of previous software releases, it is recommended that the Audit routine (Overlay 44) be specified as the background diagnostic to optimize the system capability to deal with call processing anomalies, especially in large line size and high traffic configurations.

MAT and CallPilot Installation Order

MAT 6.6 and CallPilot 1.0 concurrency and PC co-residency is supported with one installation/uninstallation related advisement. It is recommended that the following order of installation is followed when PC co-residency is desired for MAT 6.6 and CallPilot 1.0:

- Install MAT on the designated PC before installing CallPilot and MAS.

This order of installation will allow the uninstallation of either application to run successfully should a user decide to remove MAT or CallPilot from the PC. If CallPilot and MAS are installed before MAT, the uninstallation will not be successful due to the interaction between CallPilot, MAT Common Services, and the MAT directory.

System Management and Release X11 Release 25

Support for Features in Release 25.15

Release 25.15 requires Meridian Administration Tools Release 6.6 (MAT 6.6) or later or Optivity Telephony Manager 1. 0 (OTM 1.0) or later.

Customers that utilize 3rd party management systems such as Switchview must ensure that the Management System is compatible with Release 25 software.

MAT 6.6

For further information on MAT 6.6 please refer to the MAT 6.6 General Release Bulletin that is shipped with each MAT 6.6 package.

OTM 1.0

OTM 1.0 (Optivity Telephony Manager for Meridian) is a system management software product that provides a single point of connectivity to multiple Meridian 1 voice switches, Meridian Mail systems, and other applications within the portfolio. OTM supports connectivity over serial, dialup/PPP, or Ethernet. OTM includes:

• Windows based client applications with all of the functionality offered in MAT. This includes Station Admin. Traffic, Maintenance Windows, System terminal, etc.

• Improved Alarm management including ability to receives alarms other than core Meridian 1 alarms, and ability to translate serial alarms to SNMP.

• An Alarm Notification Script Wizard to assist in generating and maintaining the scripts that define conditions for alarm notification. OTM also offers Web Enabled Alarm viewing. The Web-based Maintenance Pages and Virtual Terminal Server provide users simple Browser access to devices in a site or workgroup both internally over the LAN or WAN or externally through dialup PPP connections.

• A transition toward IP-based management solutions needed to support Optivity and Unified Networks solutions of tomorrow.

OTM 1.0 integrates with Nortel's Optivity NMS (Network Management System) as part of a Unified Management solution.

For more information about OTM 1.0, please refer to the OTM 1.0 General Release Bulletin when available.

Release 25 Features that Require System Management

The following X11 Release 25 Features require MAT 6.6 or OTM 1.0:

- ITG Trunks 2.0 with ISDN- the configuration and maintenance of the IP Telephony Gateway (ITG) card is through the “ITG ISDN Trunks” application in MAT 6.6 or OTM 1.0.

- Corporate Directory functionality (one of the M3900 Series Enhancements) - the configuration and maintenance of Corporate directory requires MAT 6.6 or OTM 1.0

Conversion

As Release 25 will be deployed Internationally, automatic conversion will be available for both X11 and X11 International software streams.  Direct conversion path exists from Release 16G (Phase 7C), Release 18H (Phase 8), Release 20, Release 21, Release 22, Release 23, and Release 24 to Release 25 and apply to all Opt 11C, Opt 51C, Opt 61C, and Opt 81C machines.

| Machine Type | From | Via | To | Comments |
| --- | --- | --- | --- | --- |
| Option 11 | Phase 7A | ------ | Release 25.xx |  |
| Option 11 | Phase 7B | ------ | Release 25.xx |  |
| Option 11 | Phase 7C | ------ | Release 25.xx |  |
| Option 11 | Phase 8B.0 | ------ | Release 25.xx |  |
| Option 11 | Phase 8B.1 | ------ | Release 25.xx |  |
| Option 11 | Phase 8B.2 | ------ | Release 25.xx |  |
| Option 11E | Release 20.1x | ------ | Release 25.xx |  |
| Option 11E | Release 21.xx | ------ | Release 25.xx |  |
| Option 11C | Release 22.xx | ------ | Release 25.xx |  |
| Option 11C | Release 23.xx | ------ | Release 25.xx |  |
| Option 11C | Release 24.xx | ------ | Release 25.xx |  |
| Option 51 | Phase 7A | ------ | Release 25.xx |  |
| Option 51 | Phase 7B | ------ | Release 25.xx |  |
| Option 51 | Phase 7C | ------ | Release 25.xx |  |
| Option 51 | Phase 8B.0 | ------ | Release 25.xx |  |
| Option 51 | Phase 8B.1 | ------ | Release 25.xx |  |
| Option 51 | Phase 8B.2 | ------ | Release 25.xx |  |
| Option 51 | Release 20 | ------ | Release 25.xx |  |
| Option 51 | Release 21 | ------ | Release 25.xx |  |
| Option 51C | Release 20 | ------ | Release 25.xx |  |
| Option 51C | Release 21.xx | ------ | Release 25.xx |  |
| Option 51C | Release 22.xx | ------ | Release 25.xx |  |
| Option 51C | Release 23.xx | ------ | Release 25.xx |  |
| Option 51C | Release 24.xx | ------ | Release 25.xx |  |
| Option 61 | Phase 7A | ------ | Release 25.xx |  |
| Option 61 | Phase 7B | ------ | Release 25.xx |  |
| Option 61 | Phase 7C | ------ | Release 25.xx |  |
| Option 61 | Phase 8B.0 | ------ | Release 25.xx |  |
| Option 61 | Phase 8B.1 | ------ | Release 25.xx |  |
| Option 61 | Phase 8B.2 | ------ | Release 25.xx |  |
| Option 61 | Release 20 | ------ | Release 25.xx |  |
| Option 61 | Release 21 | ------ | Release 25.xx |  |
| Option 61C | Release 20 | ------ | Release 25.xx |  |
| Option 61C | Release 21.xx | ------ | Release 25.xx |  |
| Option 61C | Release 22.xx | ------ | Release 25.xx |  |
| Option 61C | Release 23.xx | ------ | Release 25.xx |  |
| Option 61C | Release 24.xx | ------ | Release 25.xx |  |
| Option 71 | Phase 7A | ------ | Release 25.xx |  |
| Option 71 | Phase 7B | ------ | Release 25.xx |  |
| Option 71 | Phase 7C | ------ | Release 25.xx |  |
| Option 71 | Phase 8B.0 | ------ | Release 25.xx |  |
| Option 71 | Phase 8B.1 | ------ | Release 25.xx |  |
| Option 71 | Phase 8B.2 | ------ | Release 25.xx |  |
| Option 71 | Release 20 | ------ | Release 25.xx |  |
| Option 71 | Release 21 | ------ | Release 25.xx |  |
| Option 81 | Release 20 | ------ | Release 25.xx |  |
| Option 81 | Release 21.xx | ------ | Release 25.xx |  |
| Option 81 | Release 22.xx | ------ | Release 25.xx |  |
| Option 81 | Release 23.xx | ------ | Release 25.xx |  |
| Option 81C | Release 21.xx | ------ | Release 25.xx |  |
| Option 81C | Release 22.xx | ------ | Release 25.xx |  |
| Option 81C | Release 23.xx | ------ | Release 25.xx |  |
| Option 81C | Release 24.xx | ------ | Release 25.xx |  |