- Introduction

- Generic X11 Release 25 is delivered on:

- Release 25.15 plus Performance Enhancement Products (PEPs) for Option 11C (globally) and Option 11C Mini (globally), and

- Release 25.15 for Options 51C through 81C (globally)

This document provides an overview of the Global features developed for the Meridian 1 Generic X11 Release 25 software product. It describes the new features and enhancements offered in this release. This document contains feature information that is applicable for all supported Meridian 1 system types as defined below.

X11 Global release 25.15 is a multi-purpose release designed to deliver a single global software stream to all markets. X11 Global Release 25.15 is supported on the following Meridian 1 systems with the processors indicated (NOTE: not all processor types were available in all markets). The default processor for new Release 25 Large systems in all regions will be the 68060E processor.

Option 11C and Option 11C Mini, Options 51C, 61C, 81, and 81C equipped with the Motorola 68040 (NT9D19), 68060 (NT5D10), or 68060E (NT5D03) commercial processors, and Option 81C equipped with Call Processor PII commercial processor.

68030 Call Processors are not supported on Release 25. For other system details, please refer to the Nortel Networks Publications (NTPs).

For the Option 11C and Option 11C Mini, details on the Meridian Mail Option, the Central Answering Position feature, the Autoconfiguration feature, Model Sets and Administration Sets, as well as installation and operation, can be found in the Nortel Networks Publications (NTPs) pertinent to these machine types.

For Options 51C, 61C, 81 and 81C details on feature installation and operation, and hardware upgrade procedures, refer to the X11 Release 25 Nortel Networks Publications (NTPs). Also, please refer to the Software Conversion Procedures (553-2001-320) that are included in your NTP shipment prior to loading this software.

The Meridian 1 Customer Documentation Library has been simplified for X11 Global Release 25 in the following ways:

- The North American and International libraries have been merged to comprise one global documentation library.

- Country specific information has been identified in the documents where applicable.

- Some documents have been retired as a result of merging information.

- Extraneous and irrelevant content has been removed from documents.

- Task based procedures and formats have been used where applicable.

- Overall page count for the Meridian 1 X11 - Global Release 25 Customer Documentation Library has been reduced by 35%.

- Obsolete components (i.e., QMT21 High Speed Data Module, Tape Drives, 5.25 Floppy Drives, CMDU, etc.) have been removed from the Meridian 1 - X11 Global Release 25 Customer Documentation Library.

- Obsolete components are documented in the Meridian 1 - X11 Release 24B Customer Documentation Library.  This library package will continue to be available as a separately orderable item.

- The small and large systems documentation (Options 11C-81C) have been placed on one CD-ROM (Global Meridian 1 Online Documentation - Release 25) to improve content access.

Release 25 includes the following features:

- SYSTEM FEATURES

- Agent Greetings

- Call Processor PII (CP PII) 1

- Fiber Network Fabric (FNF)

- Inventory Reporting Phase II

- CLID on Analog Trunks for Hong Kong

Note 1	 - CP PII and FNF will be introduced April through a Controlled Release 	 process.  Please contact your Nortel Sales Representative for general availability of this feature.

- DESKTOP

- M3900 Digital Telephone Enhancements

- NETWORKING

- Meridian ITG Trunks 2.0 with ISDN

- Private to Public CLID Conversion

- D-Channel Expansion

- Business Networking Express

- MDECT 2000 - Muti Site Mobility Networking

- REGULATORY

- 10/20 Digit ANI on 911 Calls

CONCURRENT SYSTEM MANAGEMENT FOR RELEASE 25.15

- Meridian Administration Tools 6.6 (MAT 6.6)

- Optivity Telephony Manager 1.0 (OTM 1.0) 3

Note 3 - 	Will be available after the General Availability of X11 Release 25. Please contact your Nortel Sales representative or refer to the OTM 1.0 Product and Sales & Marketing Bulletins for Market Availability information.

Note.  Not all features described in this document are offered in all countries, and as noted within the Bulletin, not all features are supported on all machine types. Please contact your local Nortel Networks sales representative for more information.

For information on Real Time requirements, please contact your local Nortel Networks sales representative. For information on Memory calculations, please refer to P0910790, the Technical Reference Guide for Small Systems or NTLH03AA-A0804746, the Meridian 1 Release 25 Planning and Engineering Guide for Large Systems.  (This information is also available on the documentation CD-ROM – please see chapter 4.

Important Please read all included advisements, requirements, and enhancements both common, and pertinent to your machine type prior to loading this software.