Features Overview

The following features are introduced in Release 25.15. All features are available on Release 25.15, but some features may not be available in all markets. Regional Sales representatives will be able to identify any features that are not available in a specific region.

The following note are referenced in the two tables that follow:

Note 1	 - CP PII will be introduced in July through a Controlled Release  process.  Please contact your Nortel Sales Representative for general availability of this feature.

.

Feature Overview

| Feature | New/ Changed ISMs | New Software package | New hardware |
| --- | --- | --- | --- |
| 10/20 Digit ANI on 911 | No | Yes – 249 | No |
| Agent Greetings | No | Yes – 152 | Yes |
| Business Networking Express | No | Yes – 367 | No |
| Call Processor PII (CP PII) for Option 81C only 1,2 | No | Yes - 368 | Yes |
| CLID on Analog Trunks for Hong Kong | No | No | Yes |
| D-Channel Expansion (for Option 81/81C only) | No | No | No |
| Fiber Network Fabric (FNF) (for Option 81/81C only) 2 | No | Yes – 365 | Yes |
| Inventory Reporting Ph. 2 | No | No – in base | No |
| ISM Enhancements | Yes | No | No |
| M3900 Digital Telephone Enhancements | No | Yes - 380, 381, 382 | Yes |
| MDECT 2000 - Multi Site Mobility Networking | No | Yes – 370 | No |
| Meridian ITG Trunks 2.0 with ISDN and 24-port ITG Card | Yes -ITG ISDN | Yes | Yes |
| Plug-ins 1-14 | No | No | No |
| Public to Private CLID Conversion | No | No | No |

Market Applicability of New Release 25 Features

| # Feature | # North America | # CALA | # Asia Pacific | # Europe |
| --- | --- | --- | --- | --- |
| SYSTEM FEATURES |  |  |  |  |
| Agent Greetings | N/A | N/A | N/A | All Market |
| Call Processor PII (CP PII) 1,2 (For Option 81C only) | All Markets | All Markets | All Markets | All Markets |
| CLID on Analog Trunks for Hong Kong | N/A | N/A | Hong Kong | N/A |
| Fiber Network Fabric (FNF) 2 (for Option 81/81C only) | All Markets | All Markets | All Markets | All Markets |
| Inventory Reporting Ph. 2 | All Markets | All Markets | All Markets | All Markets |
| ISM Enhancements | N/A | Use ITG ISDN Trunks | Use ITG ISDN Trunks | All Markets |
| Plug-ins 1-14 | N/A | N/A | N/A | All Markets |
| NETWORKING |  |  |  |  |
| 10/20 Digit ANI on 911 | All Markets | N/A | N/A | N/A |
| Business Networking Express | N/A | N/A | N/A | All Markets |
| D-Channel Expansion (81C only) | All Markets | All Markets | All Markets | All Markets |
| MDECT 2000 - Multi Site Mobility Networking | N/A | N/A | All Markets | All Markets |
| Meridian ITG Trunks 2.0 with ISDN and 24-port ITG Card | All Markets | All Markets | All Markets | All Markets |
| Public to Private CLID Conversion | All Markets | N/A | N/A | N/A |
| STATION FEATURES |  |  |  |  |
| M3900 Digital Telephones | All Markets | All Markets | All Markets | All Markets |

System Features

System Features are those features that do not require user station operation.

Agent Greetings

Description

The purpose of this feature is to free the agent in a call centre environment from the action of repeating a standard greeting for each individual call received.

M1 support for M1 ACD/SECC/SCCS currently does not provide a facility for each agent to pre-record their individual greeting for playback once the call is answered by the agent at their telset. This feature, known as Symposium Agent Greeting, introduces this functionality on X11 Rls 25.

The implementation of the feature involves the use of the VPS platform (ITG, MIRAN, MMCS-VP, Call Recording) on which the greeting recordings are stored and accessed every time the agent answers a call at their set. The card currently supports 24 ports (24 TNs per card) and this will increase to 32 ports in a further release of the feature. The configuration of the ports as Multiple Appearance DNs facilitates the availability of any port to any agent at any time i.e. there exists no inherent discrimination between ports, outside of any administrator configuration.

After logging into SCCS or standard ACD the agent may dial the service DN(s) of the VPS AG pack.  The agent is prompted to record a greeting for each language skillset of which they are a member. A deliberate effort is made to make the telephone user interface (TUI) as similar to Meridian Mail/Call Pilot as possible.

The greeting played to the caller on answer by the agent is dependant on the login-id of the agent or in the absence of login-ids being in use, the position id of the agent’s set.  It is then further defined by the language or function skillset of the agent in question (Symposium environment). The greeting is terminated from the VPS AG port when the recording reaches a finish, or may be aborted at any time by the agent by a hitting a predefined key sequence (defined in the Browser User Interface administration tool) on the set, provided DTMF EES is configured.  Once the greeting has terminated, the call proceeds as a normal agent call.

Hardware

VPS AG pack. This pack currently supports 24 voice ports. The pack resides on the M1 IPE shelf (large systems) / Opt 11c cabinet.

XCT Conference pack This pack provides additional conference resources for greeting delivery. Two XCT conference packs are provisioned with a 24 port VPC card. The conference packs reside in the M1 network shelf.

Package Requirements

• 	X11 Rls 25 software with new Flexible Services Port Package #152 enabled.

Applicable Systems

The greeting played to the caller on answer by the agent is dependant on the login-id of the agent or in the absence of login-ids being in use, the position id of the agent’s set.  It is then further defined by the language or function skillset of the agent in question (Symposium environment). The greeting is terminated from the VPS AG port when the recording reaches a finish, or may be aborted at any time by the agent by a hitting a predefined key sequence (defined in the Browser User Interface administration tool) on the set, provided DTMF EES is configured.  Once the greeting has terminated, the call proceeds as a normal agent call

Market Availability

Agent Greeting will be available on a Controlled Release basis from June WkIV 2000

Target Region

Global

Call Processor PII

Description

Call Processor PII represents the next generation processor complex for Option 81C systems and results in significantly higher real-time performance capability for high-end, complex real-time intensive customer applications. The new Call Processor Complex incorporates industry standard embedded computing components consisting of the Intel® Pentium II microprocessor, a compact Peripheral Controller Interconnect (CPOS) bus architecture (back plane), and a new version of VxWorks® real-time operating system software. The new core complex continues the tradition of high reliability rating afforded Meridian 1 systems while increasing the busy hour call completion performance three-fold.  Redundant call processor components are employed to ensure fail-safe operation.

The new call processor components reside in a core/network shelf similar to the shelf design used in Option 81C systems today. The new processor card is an off-the-shelf single board computer design from Motorola that utilizes the Intel Pentium II microprocessor.  Other new processor interfaces are introduced such as a new system utility interface and new IDE compatible software media drives (hard drive, floppy drive and CD-ROM).  The core network interface (CNI) previously used in Option 81C systems is retained, but redesigned to conform to the new cPCI back plane. Four CNI card slots per shelf are provided and allow up to eight network groups to be configured using the new Fiber Network Fabric interface cards. A new vintage of the existing shelf power supply is introduced to provide the additional electrical voltage required by the new call processor card.  The network card slots within the new core/network shelf continue to support the same network cards supported on Option 81C today.

Because of the core/network shelf design approach, most system upgrades will only require card cage exchanges.  For systems that preserve their network group equipment in the older style SL-1 cabinets, the use of Universal Equipment Modules (UEM) can be used to upgrade to Call Processor PII.

For additional product and feature information, please refer to the product bulletin that announces the introduction of Call Processor PII.

Hardware

System upgrades to Call Processor PII are possible under three scenarios:

Module Level Upgrade

Card Cage Level Upgrade

Combination of Module and Card Cage

EPE will not be supported on systems wishing to upgrade to X11 Release 25 whose platforms reside on CP PII Call Processors.  For more information, please refer to “Hardware” in section 4 of this document.

Package Requirements

This feature introduces one new package: CPP-CNI (package 368).

Prerequisite for package 368: package 299.

Applicable Systems

Call Processor PII is offered on Option 81C system only.

Market Availability

CP PII will be introduced on Controlled Release commencing in July 2000 until September 2000 to facilitate Lab orders.

Target Region

Global

CLID on Analog Trunks for Hong Kong (DXUT-A)

Description

CLID (Calling Line Identification), both caller's number and / or name along with date and time information is a service provided by the local exchange to the end user/subscriber in which the identity of the calling party is transmitted to the called party prior to the answering of the call.  In case of the calling number/name being absent, the CO may send a reason for absence of the same.  The DXUT-A card pack collects this information and gives it to the Software.  If the data received is not erroneous then call will be terminated with CLI information on the display.  If the received data is erroneous then call is terminated without displaying anything.

Package Requirements

This feature does not introduce any new packages.

This feature uses the existing ACLI package (349)

Applicable Systems

All system types supported by Release 25.

Target Region

Hong Kong

Fiber Network Fabric

Description

Fiber Network Fabric (FNF) allows the expansion of Meridian 1 Option 81 and 81C systems from five Network groups to eight Network groups, a 60% increase in port and trunk capacity.  A Dual Ring fiber optic network replaces the Inter-group cards and module in current Meridian 1 systems. This Fiber Network provides complete non-blocking communication between the network groups, eliminating the incidence of busy signals for calls switched between groups. A Fiber Network of eight Network groups provides 7680 timeslots for 3840 simultaneous conversations.

For further information about FNF please refer to Fiber Network Fabric Product Bulletin.

Package Requirements

FNF requires a minimum of X11 Release 25 software, with software package 365, FIBN (Fiber Network), installed.

Applicable Systems

This feature is applicable to Option 81 and 81C machines.

Market Availability

FNF will remain  under control release to facilitate lab orders until Sept 2000.

Target Region

Global

Inventory Reporting- Phase II

Description

The Inventory Reporting Phase II feature enhances the Rls 24 Inventory Reporting feature by adding several new cards to list of cards that can be "inventoried".

The Inventory Reporting feature (Rls 24), takes advantage of the intelligence built into the Meridian 1 PBX, to provide an automated tool for customers and support personnel to produce a hardware inventory report. This report will list the cards and telsets installed in the switch for business and support purposes.  The Inventory Reporting feature will run on the Meridian 1 PBX using the evolved Graphical User Interface (GUI) for System Management or using a TTY device providing a Command Line Interface (CLI) to the switch.

The Inventory Reporting feature will allow a MAT6.5 GUI user to download inventory information from a file resident on the PBX hard-drive to the PC for manipulation in a PC resident database. Many End-Users have inventory tools and applications for asset management but currently, they must manually enter inventory data into their inventory tool.

Uses for this feature include but are not limited to:

- Upgrade Engineering

- Inventory Control

- Fault Isolation

The following cards are now included in the inventory report with Release 25:

| Card Description | Eng. Code | Vintage | Applicable Market |
| --- | --- | --- | --- |
| CIS Trunk for Option 11C | NTCG02 | BA, BB | CIS |
| CIS Trunk for Meridian 1 | NTCG01 | BA, BB | CIS |
| System Utility Card | NT4N67 | AA | Global |
| System Utility Transition | NT4N68 | AA | Global |
| LED/LCD Display Panel | NT4N71 | AA | Global |
| PCNI Card | NT4N65 | AA | Global |
| CPU Card | (A0786611) | N/A | Global |
| High Speed Pipe (HSP) | (A0786671) | N/A | Global |
| Digital Trunk, DTI/PRI, | NT5D12 | AF | North America |
| Digital Trunk, DTI/PRI, | NT5D97 | AB | International |
| Digital Trunk, PRI2, | NTCK43 | AC | International |
| 2.0 MB DTI | NTAK10 | DC | Global |
| 1.5 MB DTI/PRI | NTAK09 | DA | North America |
| 24 Port DLC | NTRD24 | AA | Global |
| 24 Ports ISDN | NTZC44 AA, | BA | Global |
| Fiber in Junctor Interface Motherboard | NTRB3301 | N/A | Global |
| Fiber in Junctor Interface Daughterboard | NTRB3303 | N/A | Global |
| 3 Ports CNI | NTRB34 | AA | Global |
| 2.0 MB PRI | NTAK79 | BC | International |
| 2.0 MB PRI | NTBK50 | AA | Global |
| TMDI (1.5 MB PRI/DTI) | NTRB21 | AA | North America |

The additional cards supported by the Inventory Reporting feature as described above are supported on Release 25 and later and on MAT 6.6 and later.

Package Requirements

None - this feature is included in the X11 base software.

Applicable Systems

All system types supported by Release 25.

Target Region

Global

ISM Enhancements

Description

Release 25 introduces 7 new ISM parameters and changes the counting for two existing ISM parameters. The new ISM parameters are not used in all regions with Release 25. Refer to section 9 of this document for ISM parameter settings for Release 25.15.

The seven new ISM counters for R25 are as follows:

- Attendant Consoles

- CLASS Telephones

- Phantom Ports

- Data Ports

- Traditional Trunks

- Internet Telephones **

- ITG ISDN Trunks

** INTERNET TELEPHONE ISM is for the ITG Line-side and i2004 Internet Telephone product which will be available at a later date.

Counting of the existing two ISMs, Analogue Telephones and Digital Telephones, are changed:

CLASS sets are excluded from counting as Analogue Telephones.

- Data ports configured in Overlay 10, Analogue (500/2500) Telephone Administration, are excluded from counting as Analogue Telephones.

- Data ports configured in Overlay 11, Meridian Digital Telephone Administration, are excluded from counting as Digital Telephones.

Operation of the remaining ISM counters is not changed and these ISMs operate the same as they are today.  The existing System TNs ISM continues to count every TN configured in the system.

ISM parameters that are not being used in Release 25 are set to the maximum values (32767 for Large systems, 2500 for small systems) which means:

- Large Systems - the new “maximum set” ISM parameters will not appear on the keycode sheet or in the LD 22 print outs.

- Small Systems - the new “maximum set” ISM parameters will appear on the keycode sheet, during the software installation and in the LD 22 & LD 143 print outs. On the Option11C and Option 11C Mini, please ensure that the values listed on the keycode sheets are followed during upgrades.

The following table is a summary of how various TNs are counted against the new/changed ISMs  for markets using the counters in Release 25.

| A TN configured in Meridian 1 | Existing ISM (*) | ISM in R25 (*) |
| --- | --- | --- |
| An Attendant console | None | Attendant Consoles |
| A PC console | None | Attendant Consoles |
| A Phantom Analogue set | None | Phantom Ports |
| A Phantom Digital set | Wireless Telephones | Wireless Telephones |
| A CLASS set | Analogue Telephones | CLASS Telephone |
| An Analogue Data Set (FAXA) | Analogue Telephones | Data Ports |
| A Digital Cordless Set (DCS) | None | Wireless Telephones |
| A Digital Data set | Digital Telephones | Data Ports |
| An ATA set | Digital Telephones | Data Ports |
| An MCA set | Digital Telephones | Data Ports |
| An MCU | Digital Telephones | Data Ports |
| An R232 DAC | Digital Telephones | Data Ports |
| An R422 DAC | Digital Telephones | Data Ports |
| An analogue trunk | None | Traditional Trunks |
| Line-Side T1/E1 | Analogue Telephones | Analogue Telephones |
| An ITG 1.0 trunk | None | Traditional Trunks |
| An ITG 2.0 trunk | None | ITG ISDN Trunks |
| A 1.5 Mb DTI trunk | None | Traditional Trunks |
| A 2.0 Mb DTI trunk | None | Traditional Trunks |
| An ISL trunk | None | Traditional Trunks |
| A VNS trunk | None | Traditional Trunks |
| A 1.5 Mb PRI trunk | None | Traditional Trunks |
| A 2.0Mb PRI trunk | None | Traditional Trunks |
| An IDA trunk | None | Traditional Trunks |
| An ISA trunk | None | Traditional Trunks |
| A BRI trunk | None | Traditional Trunks |
| An i2004 Telephone | None | Internet Telephones |

Note 	 (*) - The System TNs ISM is not included in the table for comparison. The system TN ISM continues to count every TN configured in the system.

Package Requirements:

No new Packages are introduced with this feature.

Target Region:

Europe

CALA - will use ITG Trunks with intitial Release 25 introduction

CALA & North aMerica - will use INTERNET TELEPHONES when product is available.

Plug-ins

Description

The purpose of this new process is to make it easier to deliver and faster to integrate patch Product Improvements (PIs) by delivering integrated solutions to customer requests.  Plug-ins are integrated into the software, and are selected according to the Plug-in process limitations and specifications.

These Plug-ins will also create a library of new PI integrated into X11 software which could be use by distributors to answer customer requests in a more effective manner.

This process will be, within Release 25 timeframe, only available in Europe and could be potentially extend to others regions at a later date.

| Plug-in n° | Rls dependency | PRS number | Patch reference | Keywords | Originator | Potential Market | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | R24B | BV67946 | CHTHOR_04 |  | Switzerland | Global | The Special dial tone for Switzerland is not compliant with Switz specification. This PI must be install on each sites for this country. |
| 1 | R25 | BV63269 | MPLR09048 | BRI | Germany | Europe | The German ISDN specification notified than when a D-channel is dropped the B-channel status should be Idle. The others countries specification specified than in this case the B-channel status should be Busy. This PI deliver this functionality for Germany to ensure compliancy. |
| 2 | R25 | BV69389 | CHTHOR_199 | HOTL | Switzerland | Global | In case of INIT the AWU features is automatically turn off and without any alarm to notify it. The AWU must be reset up up manually. The patch ensure than the AWU is automatically turn on after an INIT. |
| 3 | R25 | BV69237 | MPLR11516 | HOTL | Germany | Global | This PI has been request for the Two way hotline only. In an hotline environment when an agent is on call, the others agent don't know it and could try to contact him and disturb him. With this Plug-in the others agent have a visual indication on their set when some others agent are involved in a call. |
| 4 | R25 | BV19192 | MPLR01862 | DSC | Europe | Europe | It is not possible to dial a DSC code after an LSC code. The Plug-in is offering this facility. |
| 5 | R25 | BV71211 | ch58f_245 | NRGA | Switzerland | Global | If RGA or NRGA is used on PRA trunk where digit manipulation is done by the M1, the feature is not working. This PI allows RGA or NRGA to work in this situation |
| 6 | R25 | BV28124 | 502056 | ATTN | Germany | Global | When a set want to transfer a call to the Attendant, only the attendant can release the call to coonclude the transfer. With this PI the set could complete the transfer by itself. |
| 7 | R25 | BV29410 | CHTHOR_62 | DNIS | Switzerland | Global | System has DID defined with DNIS=YES and IDC=YES. The DID route uses the IDC table X defined in OVL49. In OVL57 the PLDN XX is configured using SCLU number YY which is configured in OVL18. The entry 00 of SCL YY has stored the DN of set A. Set A is any set. When one of the DID trunks dials XX00 (PLDN + SCL entry) the system returns OVF and the ERR183 is printed in the TTY. |
| 8 | R25 | BV31300 | CHTHOR_26 | PLDN | Switzerland | Global | Set A is any set with CFW allowed. The SCL used by PLDN has DN of set B stored on entry 01. Set A presses the CFW key or in case of 5/25 set dial the CFW FFC and then dials the PLDN and the ENTRY 01 in order to be call forwarded. In the 5/25 case after dialling PLDN gets OVF and in case of BCS set it gets OVF when it presses CFW key the second time. |
| 9 | R25 | BV41730 / BV40679 / BV40044 | CHTHOR_159 | DISP | Switzerland | Global | On a standalone situation, if a set A calls the Attendant, and the ATTN calls set B. The set B display will show the set A DN instead of the ATTN. In case of network call (set B remote), the set B dispay ATTN DN. This PI ensure than the network and Standalone situation are consistent and show the ATTN DN. |
| 10 | R25 | BV42928 | mplr07615 | NAS | Switzerland | Global | In a certain scenario the NAS feature and the Nite treatment will be in conflict. This PI resolve thi situation. |
| 11 | R25 | BV62151 | mplr11339 | RAN | Switzerland | Global | In a private network (2 nodes linked through MCDN) with one switch equipped within a RAN, if a remote caller try to reach a local caller ( where the RAN is connected) which is forwarded to the RAN, the remote caller will not heard the RAN and not heard as well any tones. This PI ensure than the remote caller could listen the RAN annoucement |
| 12 | R25 | BV63726 | mplr08777 | TTY | Switzerland | Global | The problem is maintenance messages flooding the TTY. It makes the TTY unusable and hence the maintenance engineer is unable to access the switch to investigate/fix the cause of message. This PI allow to break this message display to allow engineer to investigate from the TTY. |
| 13 | R25 | BV71206 | CHTHOR_195 | MCT | Switzerland | Global | On an 500 set the Mallicious Call trace feature could not be used same if the MCT prompt is configure to Yes. This PI ensure than the MCT will be output as well within an analog set. |
| 14 | R25 | BV70355 | mplr09806 | DISP | Germany | Global | In a network call (MCDN) from set A to set B, on set B an 'H' is displayed to indicate than this is a network call. The purpose of this PI is to remove this 'H'. |

Package Requirements

The PLUGIN (366) package is needed for activating this Product Improvment.

Applicable Systems

All system types supported by Release 25.

Target Region

Europe

Networking Features

Networking Features are those features that operate in a networking environment.

10 / 20 Digit ANI on 911

Description

This feature is being developed to address Blocker Requirement GR 2953 and FCC ruling: Communications Commission Docket No. 94-102, RM-8143, which will require a PBX acting as a PSAP (Public Safety Answering Point) to accept a 10 or 20 digit ANI when terminating 9-1-1 calls. This FCC ruling addresses two separate industry issues hence the two separate features. The 10 digit ANI feature addresses the increasing number of Naps in North America. The 20 digit ANI feature addresses the ability to locate a 9-1-1 caller who is using wireless service.

These two issues are resolved with two new ANI formats. Both new formats use two II (Information Indicator) digits as opposed to the single NPD (Numbering Plan Digit) digit currently used. The new formats are II+10 digit ANI for whirling service and II+10+10 digit ANI for wireless service. These new formats will be referred as the 10 digit ANI format and the 20 digit ANI format respectively.

Due to the increasing demand for additional telephone numbers, the reserve of unassigned numbers within an NPA is quickly being exhausted. This results in many new Naps being introduced, sometimes by splitting or overlaying an already existing NPA. This in turn requires that a single PSAP must now be able to handle multiple Naps within its jurisdiction. The old signaling interface could only support a maximum of four Naps. The old format of NPD+7-digit ANI would use the single digit NPD values of 0-3 to translate into an NPA via a look-up table. The new 10 digit ANI format solves this issue by including the NPA in the 10 digit ANI field, so any number of valid Naps will now be accepted within a single PSAP.

The second issue of accurately determining the physical location of a wireless caller dialing 9-1-1, is addressed by the 20 digit ANI format. The first 10 ANI digits would provide the CSN (Calling Station Number). This number could be used to call back the originator in cases where the 9-1-1 call was disconnected. The second 10 ANI digits, or pseudo ANI, would provide the cell site/sector information to best define the wireless caller's location so assistance could be dispatched to the correct area. This function is not supported in any way by the old format.

Engineering Guidelines

The following packaging requirements exist for this feature:

| Package Mnemonic | Package Number | Package Description | Package Type (New or Existing or Dependency) | Applicable Market |
| --- | --- | --- | --- | --- |
| ENH_M911 | 249 | Enhanced M911 | New | Global |
| M911 | 224 | Meridian 911 | Existing | Global |
| DIGDSP | 19 | Digit Display | Existing | Global |
| ACD_BAS | 40 | Basic ACD-Package A | Existing | Global |
| ACDB | 41 | ACD Package B | Existing | Global |
| ACD | 45 | Extended ACD Package | Existing | Global |
| EAR | 214 | Enhanced ACD Routing | Existing | Global |
| CWNT | 225 | Call Waiting Notification | Existing | Global |

M911 format is configured on a per route basis. Overlay 16 contains one new prompt, M911_NPID_FORM, for configuring what format is expected on the M911 trunk route. M911_NPID_FORM is only prompted if the Meridian 911 and Enhanced M911 packages are equipped and an M911 trunk route is being defined.

Package Requirements

A new package 249 has been added to enable this feature.

Applicable Systems

All system types supported by Release 25.

Target Region

North America

Business Networking Express (BNE)

It is a new NORTEL VPN solution for connecting several Meridian 1 through a Resodding interface. “Business Networking Express” solution consists of an X11 software package that mixes Resodding public services as well as some Meridian1 proprietary features. The objective is to deliver a homogenous set of services that should address 90% of the basic requests for customers who want to build a small private network at a very efficient cost. BNE includes a number of frequently used Resodding supplementary services to deliver the following functionality's between Meridian 1 sites:

- The Resodding Call Completion to Busy subscriber supplementary service, introduced in release 22.

- The Name and Private number (calling and connected numbers) display using User to User Service 1, introduced in release 25.

- The Resodding Explicit Call Transfer supplementary services introduced in release 25.

- The Resodding Call Diversion supplementary services.

Business Network Express - Name and Private CLID

Description

The Business Network Express Name and Private CLID feature is useful when a user dials a private number in order to reach another Meridian 1 site through the public network. The existing ESN feature translates the dialed number to a public number so that the called user can be reached through the PSTN. The BNE feature inserts the calling Name and the Private CLI in the User-to-User IE carrying by the SETUP message. On destination switch, the private CLI is displayed along the calling name on the alerted set. The alerted name is delivered to the calling user in a User-to-User IE carried in the ALERT message and displayed on the calling set. When the call is answered, the connected name and the private Connected Number is provided to the calling user in a User-to-User carried in a CONNECT message. Note that to be consistent with MCDN and QSIG, an H is displayed in front the private number.

Includes gateways for MCDN, QSIG and DPNSS. This will ensure the Name display inside a network with several protocols (EISDN, MCDN, QSIG, and DPNSS).

Business Network Express - EISDN Call Transfer

Description

The feature allows transfer notifications for supervised & unsupervised transfer, and depending on configuration, network optimization is preferred. The feature gives the possibility to the Private Network:

- To notify the Public network that a transfer has been performed within the Private Network.

- To optimize the call, by asking the public network to perform the transfer. This is also called transfer invocation with the public network.

The EISDN Call Transfer supports supplementary s according to the standard ETS 300-367/368/369. This includes the Hero-ISDN-MCDN, Resodding-QSIG, and Resodding-DPNSS gateways.

Package Requirements

One new package (367) is required.

Applicable Systems

All system types supported by Release 25.

Target Region

Europe - Only applicable in countries where the EISDN supplementary services described above are supported.

D-Channel Expansion

Description

The D-Channel Expansion feature increases the total number of possible D-channels in a multi-group Meridian 1 system. The D-Channel Expansion feature increases the number of physical I/O addresses permitted for D-channel application to 16 per network group. For each physical I/O address, up to four ports are available for D-channel use. With the D-Channel Expansion feature, the X11 software supports up to 255 D-channels.

Feature Interactions

Incremental Software Management

The maximum number of D-Channels in a Meridian 1 system is one of the ISM limits in the system. The keycode file defines the ISM limits in an IODU/C based Meridian 1 system. The DCH limit is set in the keycode generation process. If the DCH limit is 64, the Keycode Generation group can change the DCH limit to a maximum of 255 (0-254).

Fiber Network Fabric

The D-Channel expansion feature increases the number of physical I/O addresses for DCH to 16 per network group.

The limit of physical I/O addresses in a Meridian 1 multiple group system depends on the number of groups in the system. The Fiber Network Fabric feature increases the maximum number of network groups allowed in a Meridian 1 system to eight.

Note: With Fiber Network there would appear to be a potential maximum of 512 devices (16 physical I/O addresses x 8 groups x 4 ports) however, the actual D-Channel limit of 255 (0-254) is due to Meridian 1 software considerations.

Engineering Guidelines

The D-Channel Expansion feature retains the existing physical I/O address range of 0-15. In Overlay 17 the DNUM (Device Number) prompt represents the physical I/O address of a given card.  The D-Channel Expansion feature allows these DNUM addresses to be duplicated providing the cards reside in separate network groups. As a general rule the duplicate device numbers must be DDCH or MSDL cards (with DCH applications only).  The actual limitation is that when duplicate device numbers are configured, no more than one of the duplicate devices can be a non-MSDL device (or MSDL with any non-DCH applications). Regardless of the device type, no duplicate device numbers may be provisioned within the same network group.

| Device/ Application | MSDL (DCH only) DNUM x GROUP z | MSDL (non-DCH) DNUM x GROUP z | Non-MSDL DNUM x GROUP z |
| --- | --- | --- | --- |
| MSDL (DCH only)DNUM xGROUP y | valid | valid | valid Note: see Adjacent devices |
| MSDL (non-DCH)DNU M xGROUP y | valid | not valid | not valid |
| Non-MSDLDNU M xGROUP y | valid Note: see Adjacent devices | not valid | not valid |

Where:

x = I/O device number

y = group number

z = alternate group number

Adjacent Devices: Non-MSDL cards usually appropriate one or more pairs of physical device numbers based on hardware switch settings.  The second address of the pair is known as the adjacent device.  When one address of the pair is configured in software, the other is then reserved for the same type of device.  This is consistent with current operation but may cause exceptions to the table above.

Example: MSDL 4 in group 0 is DCH only, MSDL 5 in group 0 has an SDI (non-DCH) on port 0.  Configuring TTY 4, using an SDI2 card, in group 1 is not allowed even though MSDL 4 is DCH only.  This is due to the fact that TTY 4 has an adjacent device of TTY 5, and TTY 5 would conflict with MSDL 5 (non-DCH) in group 0.

Feature Configuration

D-Channels are configured the same as with current operation.  The difference being that Overlay 17 will now allow duplicate device numbers in separate network groups provided the engineering guidelines are followed.

Applicable Systems

D-Channel Expansion is supported on Options 81 and 81C machines; these systems can support multiple groups.  D-Channel expansion is not supported on Option 11C, Option 11C Mini, Option 51C or Option 61C at this time.

Package Requirements

The D-Channel Expansion feature requires the following packages:

- Multi -purpose Serial Data Link (MSDL) package 222

- Integrated Services Digital Network (ISDN) package 145

- One or more of:

- ISDN Primary Rate Access (CO) (PRA) package 146

- ISDN Signaling Link (ISL) package 147

- 2.0 Mb/s Primary Rate Interface (PRI2) package 154

Target Region

Global

MDECT 2000 - Multi Site Mobility Networking

Description

The initial Meridian DECT program was introduced in June of 1998. This project provides an enhancement to this existing product.

This feature introduces Multi-Site Mobility Networking (MSMN) for an MDECT cordless system. MSMN provides the user with the ability to roam between Meridian sites, connected via an MCDN network, and use their DECT handset to make and receive calls as if the user is located in the home node within that network. Recognition of their handset at the visited location and the subsequent routing of calls is automatic and does not require user interaction.

A distinct component of this feature is the introduction of concentration for MDECT handsets. MSMN is supported only on concentrated handsets within a DECT system.

With the introduction of the 32 port MDECT laundered in R24, the number of DECT portables which could be configured on a single MDECT system was limited to 1024 (32 units x 32 landlords) on Option 51C - 81C or 640 (32 units x 20 landlords) on Opt.11C. These figures correspond to a limit of 2 fully populated IPE shelves on a large system or 2 fully populated cabinets on a small system.

To allow the ability to configure more portables than is limited by the MDECT system hardware constraints, concentration is introduced.

A concentrated MDECT system is a blocking system where the number of portables which can be configured is greater than the maximum number of simultaneous calls which can be supported by the available hardware. In order to configure a greater number of portables than physical resources exist, virtual Tons configured on phantom loops are used to represent the portables.

Package Requirements:

A new package has been created for the Mobility Networking feature. The package acronym is MSMN and the package number is 370.

The following table describes the total required X11 packaging for this feature to be operable.

| Package Mnemonic | Package Number | Package Description | Package Type | Applicable Market |
| --- | --- | --- | --- | --- |
| MSDN | 370 | Mobility Networking | NEW – dependency on 350 | Europe, Asia Pacific |
| MC32 | 350 | Introduce octal density laundered for wireless sets | Dependant on 240 | GLOBAL |
| MCMO | 240 | Wireless specific (MCMO and MDECT) overlay and call processing functionality | EXISTING | GLOBAL |
| FFC | 139 | Flexible Feature Codes | EXISG | GLOBAL |

Meridian ITG Trunks 2.0 with ISDN and 24 port ITG card

Description

The ITG compresses voice and demodulates Group 3 Fax. The ITG then routes the packetized data over a private IP network. Connections are thus made between Meridian 1 nodes, bypassing circuit-switched trunking facilities.

The ITG is an intelligent Peripheral Equipment (IPE) trunk card referred to as the IP Telephony Gateway (ITG) card. An ISDN Signaling Link D-channel (ISL DCH) provides DCH connectivity to the Meridian 1 and provides signaling control for the 24 ports on the ITG card. The DCH connection expands the signaling path between the Meridian 1 system and the gateway. ITG allows Meridian 1 systems to be networked together using ISDN networking features, while transmitting signaling and voice media over a standard IP signaling stack.

The ITG delivers an ISDN signaling interface between the Meridian 1 and the Voice and Fax over IP (XoIP) interface. The high signaling bandwidth of this ISDN interface expands the feature functionality for XoIP trunks. It provides, for example, Calling Line Identification (CLID) and Calling Party Name Display (CPND).

To implement an ITG, the customer must have a corporate IP network, and routers must be available for WAN connectivity between networked Meridian 1 systems.

Configuration of the ITG requires the presence of 10BaseT Ethernet interfaces and support of the IP version 4 layer and addressing in a WAN. There is no restriction on the physical medium of the WAN. 100BaseT Ethernet network connectivity is required for codecs with less compression. Voice traffic from the ITG cards is routed over a 10/100BaseT auto-sensing Ethernet interface. Inter-card signaling and communication with the Meridian Administration Tools (MAT) PC is over a 10BaseT Ethernet connection.

List of ITG ISDN Components

| Component | Product codes |
| --- | --- |
| System Packages |  |
| ITG ISDN Signaling Trunk Large Systems Package including D-Channel (NT0961AA) 24-Port ITG ISL Trunk with RTU and pre-installed software, I/O cables, DCH PC card, 50-pin I/O Panel Filter connector with ITG specific filtering for 100BaseTX, and NTP) | NTZC44AA A0786079 |
| ITG ISDN Signaling Trunk Small Systems (Option 11C) Package including D-Channel (ITG Trunk 2.0 card with RTU license and pre-installed software that supports 24 ports, required cables, DCH PC card, and NTP) | NTZC44BA A0786080 |
| ITG ISDN Signaling Trunk Small and Large Systems Package without DCH PC Card or NTP | NTZC45AA A0786081 |
| Upgrade Packages |  |
| Upgrade Kit for Large Systems from ITG Trunk 1.0 to 2.0 (includes required cables, DCH PC card, and NTP) | NTZC47AA A0786085 |
| Upgrade Kit for Small Systems from ITG Trunk 1.0 to 2.0 (includes required cables, DCH PC card, and NTP) | NTZC47BA A0786086 |
| Spare cards |  |
| Meridian ITG Trunk 2.0 card (24 ports) (NT0961AA 24-Port ITG ISL Trunk with RTU and pre-installed software) | NT0961AA A0786146 |
| Cables |  |
| E-LAN, T-LAN, RS232 and DCH Ports cable for the NT0961AA 24-Port ITG ISL Trunk DCHIP card. | NTCW84KA A0784208 |
| E-LAN, T-LAN, and RS232 Ports cable for the NT0961AA 24-Port ITG ISL Trunk card | NTMF94EA A0783470 |
| E-LAN, T-LAN, RS232 and DCH Ports cable for the NTCW80CA 8-Port ITG ISL Trunk DCHIP card | NTCW84LA A0784437 |
| E-LAN, T-LAN, RS232 and DCH Ports cable for the NTCW80AA 8-Port ITG ISL Trunk DCHIP card | NTCW84MA A0789752 |
| DCH PC Card Pigtail cable | NTCW84EA A0744403 |
| MSDL DCH cable (included in Large System package): 6 ft. 18 ft. 35 ft. 50 ft. | NTND26AA NTND26AB NTND26AC NTND26AD |
| 50 ft. MSDL DCH Extender cable | NTMF04AB A0774842 |
| 10 ft. Inter cabinet cable NTCW84KA to SDI/DCH cable | NTWE04AC A0794156 |
| 1 ft. Intra cabinet cable NTCW84KA to SDI/DCH cable | NTWE04AD A0794157 |
| Shielded four-port SDI/DCH cable for the NTAK02BB SDI/DCH card (included in Small System package) | NTAK19FB A0403450 |
| PC Maintenance cable (for faceplate RS232 maintenance port to local terminal access) | NTAG81CA A0655007 |
| Maintenance Extender cable | NTAG81BA |
| Large Systems filter connector |  |
| 50 pin I/O Panel Filter Connector Block with ITG specific filtering for 100BaseTX (included in Large Systems package) | NTCW84JA |
| A0783483 |  |
| Backplane to I/O Panel ribbon cable assembly compatible with NTCW84JA I/O Panel Filter Connector Block with ITG-specific filtering for 100BaseTX T-LAN connection (replaces NT8D81BA Backplane to I/O Panel ribbon cable assembly equipped with non-removable Molded Filter Connectors) | NT8D81AA |
| A0359946 |  |
| Documentation |  |
| Meridian Internet Telephony Gateway (ITG) Trunk 2.0/ISDN Signaling Link NTP | P0912540 |
| PC Cards |  |
| C7LIU DCH PC Card with Layer 2 DCH Software | NTWE07AA A0794155 |
| ITG Trunk 2.0 24-Port Software Upgrade on 8Mb ATA Flash Rom PC Card | NT0963AA A0786148 |
| ITG Trunk 2.0 8-Port Software Upgrade on 8Mb ATA Flash ROM PC Card | NT0962AA A0786147 |

Private to Public CLID Conversion

Description

The Private to Public CLID Conversion feature addresses situations where an incorrect CLID displays when a call hops off the private network to the public network (the PSTN) at a tandem node.

This feature is applicable to Electronic Switched Network (ESN) networks using private network numbering plans that can be either one of the following:

- Uniform Dialing Plan (UDP)

- Coordinated Dialing Plan (CDP)

On systems without this feature, if a call is sent to a PSTN route at a tandem node, the private CLID of the originating telephone is sent to the Central Office (CO). There are Cost that modify the CLID (adding an NPA and NXX); therefore, the terminating telephone displays an incorrect CLID.

This feature will be applicable to interface types such as DMS 100, DMS 250, #4 & #5 ESS, S100 and NI-2 TR-1268 interfaces.

This feature introduces a prompt (CPUB) in the Route Data Block (LD 16). This prompt controls what option applies when the tandem node builds the public format CLID.

The choices for the prompt CPUB are: ON, OFF and LDN

ON

Means the feature is enabled. The software checks the CLID of the Calling Party in the setup message to extract the LOC or DSC of the originating caller.

The system then references the LOC or DSC at the tandem switch to get the NPA and NXX of the caller to build the CLID for the outgoing call to the public network.

OFF

Means the feature is disabled. The CLID is built as it was prior to the introduction of this feature.

LDN

The tandem node sends its LDN0 (from LD 15) to the CO as the CLID. The CLID is constructed by coupling the HNPA and HNXX in CLID entry 0 in the Customer Data Block (LD 15) with the LDN0 from LD 15 at the tandem node.

Feature Interactions

Automatic Call Distribution (ACD)

When a private call is presented to an ACD DN and the call flows to the PSTN due to the ACD Night Call Forward or the Underflow or the Overflow feature, then the Private to Public CLID Conversion feature can operate.

Alternate Routing

The Private to Public CLID Conversion feature applies for Network Alternate Routing (NARS), QSIG Alternate Routing, and MCDN Alternate Routing.

Billing Display Feature (BDSP)

At the tandem node, the BDSP and Private to Public CLID Conversion features are mutually exclusive. However, if they exist together at one node, BDSP takes precedence over the Private to Public CLID Conversion feature.

- Call Forward All Types (External Calls)/Hunting

- Call Forward All Types includes the following call forward scenarios:

- Call Forward All Calls

- Call Forward Busy

- Call Forward by Busy Type

- Call Forward External Deny

- Call Forward No Answer / Flexible Call Forward No Answer

- Call Forward No Answer, Second Level

Call Redirection by Time of Day

When a call forwards to an external telephone, if the call is sent out on a PSTN route, then the Private to Public Conversion feature configures the originating CLID in a public format, if the option is configured at the node that is redirecting the call. The same applies to Hunting if the Hunt DN is an external number.

Call Transfer

The Private to Public Conversion feature has no interaction with Call Transfer. The prior functionality continues.

Calling Party Privacy (CPP) and Calling Party Privacy Override (CPPO)

A call marked as a CPP or CPPO call, can hopeful the private network to the public network. Even though the Private to Public Conversion feature modifies the private CLID to a public format CLID, the presentation indicators indicating whether this is CPP/CPPO call are not modified. After hopeful occurs, the call continues to be identified as a CPP or CPPO call.

CDR

This feature has no interaction with CDR. Even after the CLID is converted, the CDR remains the same as without conversion.

Meridian Mail

In a case where a call terminates on Meridian Mail with a converted CLID, the private greeting is not given. Either an unknown origin or public greeting is given.

Call Sender feature

This feature has no interaction with the Call Sender feature of Meridian Mail.

Remote Virtual Queuing

Remote Virtual Queuing continues to work normally. During a call re-initiation (when a public network trunk becomes available), if a hop off to the public network takes place, this feature converts the originating CLID to a public format.

Networking feature interactions

CLID Enhancements

The CLID Enhancement feature provides flexibility in the way the CLID at the origination node is built. The Private to Public Conversion feature works at the tandem node, and therefore does not have any interaction with the CLID Enhancement feature. If the configuration at the tandem node is to send the LDN of the tandem node as the CLID, CLAD entry “0” is used to build the tandem node LDN CLID.

Network ACD

When Network ACD routes a call over a PSTN route, then the Private to Public CLID Conversion feature sets the originating CLID to a public format, if this option is configured at the node that diverted the call.

Network Ring Again

The Network Ring Again feature continues to work normally. During a call re-initiation, if a hop off to the public network takes place, this feature converts the origination CLID to a public format.

Internet Telephony Gateway (ITG)

ITG 2.0 implements ITG with ISDN as ITG ISL. When an ITG ISL trunk call hops off to the public network, and if this feature is configured, the originating CLID is modified to a public network format.

Engineering Guidelines

At the tandem switch the NPA and NXX must be configured for all of the LOC or DSC codes which might originate a call within the private network.

Package Requirements

- The following software packages are required:

- Basic Automatic Route Selection (BARS) package 57 or Network Alternate Route Selection (NARS) package 58 and/or Coordinated Dialing Plan (CDP) package 59

- Integrated Services Digital Network (ISDN) package 145

- Primary Rate Access (PRA) package 146 or 2.0 Mbps Primary Rate Interface (PRI2) package 154

Applicable Systems

All system types supported by Release 25.

Target Region

North America

Station Features

Station Features are those features that require the user to perform certain steps from their station in order for the feature to function.

M3900 Digital Telephone Enhancements

Description

These enhancements to the M3900 Digital Telephones in Meridian X11 Release 25 bring additional functionality to the M3902, M3903, M3904 phones.

The new features include:

Context Sensitive Soft Keys

The Context Sensitive Soft Keys support the most frequently used call processing features.  Soft keys for these features appear during Call Processing in appropriate call states.

Set-to-Set Messaging

This feature allows an M3900 user to define a one-line text message to be displayed on a caller's phone at the time the call is established. User turns the feature ON/OFF and edits the Set-to-Set Message text using the dialpad keys.

Corporate Directory

This capability provides the user access to a directory of names (data derived from MAT).  Users can then search by last name, navigate through the directory, copy entries to their Personal Directory, and dial entries.

Virtual Office (Hot Desking)

This feature provides the capability for a user to login to a designated phone and have the user's own custom profile determine the configurable features of the phone in use.  The Virtual Office capability is useful for telecommuters, for visitors, and for workers who are infrequently in the office.

Flash Download

M3900 Flash Download provides the capability to download a new firmware version from the Meridian 1 to the M3900 telephone. Flash download provides a way for installed M3900 telephones to be updated to the appropriate firmware release level for supporting features on the Meridian 1.

Flash Download can be invoked for one M3900 telephone, for a group of M3900 telephones, or all telephones on the Meridian 1. It can be invoked locally or remotely for maintenance purposes.

The download capability includes flexible reporting capabilities for the flash download process. A report can be generated for a group of phones based on parameters specified in a table.  These parameter include the following:

- Set type can be specified  (M3902, M3903, M3904, M3905, All)

- TN Range can be specified (start TN, end TN)

- DN Range can be specified (start DN, end DN)

- Firmware version can be specified (all, specific)

The download capability also includes flexible and automated firmware downloading capabilities:

- Set type can be specified  (M3902, M3903, M3904, M3905, All)

- Day(s) of week can be specified

Up to four intervals per day can be specified (start time, length)

- TN Range can be specified (start TN, end TN)

- DN Range can be specified (start DN, end DN)

- Force Download can be specified (yes,no)

The new accessories include:

Display Based Expansion Module

The Display-Based Expansion Module is a hardware module containing eight soft-labeled keys for DNs or features.  The Expansion Modules "Page" key provides access to two additional pages allowing up to 24 DNs or features to be programmed.

A summary of the changes is included in the table below:

| Requirement | M3901 | M3902 | M3903 | M3904 | M3905 |
| --- | --- | --- | --- | --- | --- |
| Flash Download Flexibility Enh. | No | Yes | Yes | Yes | Yes |
| # of Context Sensitive Soft Keys | 0 | 0 | 4 | 4 | 0 |
| Virtual Office (Hot Desking) | No | No | Yes | Yes | No |
| Corporate Directory | No | No | Yes | Yes | No |
| Set-to-Set Messaging | No | No | Yes | Yes | No |
| Display Based Expansion Module support | No | No | No | Yes | No |

Package Requirements

The ARIES set package (170) and the DSET package (88) are required for the M3900 sets to work on the Meridian 1 switch.

The following new packages are required with this development:

Package 380 - Set to Set Messaging

Package 381 - Corporate Directory

Package 382 - Virtual Office

Applicable Systems

All system types supported by Release 25.

Market Availability of Features

The M3900 Digital Telephone Enhancements will be supported on the M3905 Meridian Digital set in a future X11 Release

Target Region

Global