Small System Advisements

The following advisements are for small systems (Option 11C and 11C Mini) only.

Small System Memory Requirements for Release 25

Option 11C Mini

The Option 11C Mini comes with 48Mb Total Memory (32 Mb Program Store and 16 Mb C-Drive space) on the Mini System Controller in order to run Release 25 software. The supported Release 25 System Controller card for Option 11C Mini:

- NTDK97AB (48 Meg - no memory upgrade needed) Mini System Controller (MSC)

Please ensure that you have upgraded the bootcode on the MSC to NTDK34FA Rel 06.  Refer to NTP Upgrades Guide – section 10 Use the flash boot ROM utility.

Option 11C

The Option 11C requires a 48Mb Total Memory (32 Mb Program Store and 16 Mb C-Drive space) in order to run Release 25 software.

The supported Release 25 call processor card for Option 11C:

- NTDK20EA or higher Small System Controller (SSC)

- NTTK13AA Software Daughterboard

An existing Option 11C system equipped with a controller card NTDK20AB can be upgraded to an NTDK20EA with a field upgrade kit NTDK19AA.  As of July 1999, all controller cards shipped to the field have been NTDK20DA, which is compatible with Release 25.

A system upgrading to Release 25 must replace its NTDK21 (32 Mb) or NTDK81 (40 Mb) based software daughterboard, with an NTTK13AA (48 Mb) daughterboard.   Please ensure that that you have upgraded the bootcode on the SSC prior to upgrading the software daughterboard.  To verify the size of the software daughterboard and for installation instructions, refer to NTP upgrade manual section 10.

New 48 MB PCMCIA for Option 11C / 11C Mini

40 Mb PCMCIA cards for Option 11C and Option 11C Mini have been replaced by a new smaller 48Mb compact flash and card holder (PCMCIA form factor). The older 40 MB cards will no longer be sold by Nortel, but existing 40 Mb cards are supported on Release 25 small systems.

Note - This document still refers to the new compact flash device in the holder as a PCMCIA card.

Software Delivery Methods

Option 11C Mini

- Pre-Programmed Mini System Controller (MSC) for new systems

- PCMCIA Card for upgrades

Option 11C

- Software Daughter board for new systems

- PCMCIA Card for upgrades

Upgrades to Release 25 from Option 11/11E

The Option 11/11E systems running on pre-release 22 software require a hardware upgrade in order to upgrade to Release 25. The Option 11C system offers a menu driven installation and upgrade method. Please refer to Option 11C Installation and or Upgrade Procedures Guide for additional information.

Please read the Option 11C NTPs thoroughly before performing any hardware/software changes. All upgrade procedures should be strictly followed step by step.

Upgrades to Release 25 from Option 11C

The Boot Code on the SSC may require updating before upgrading software to release 25.15. Refer to the “Small Systems Memory Requirements for Release 25” earlier in this section for the requirements. Updating of the SSC Boot Code is a manual process that uses the Flash Boot ROM Utility. Refer to NTP 553-3021-250 Upgrade procedures, chapter 13, or 553-3021-310 Software Installation Program Guide, chapter 11 for detailed procedures.

Upgrades to Release 25 from Option 11C Mini

Option 11C Mini was introduced with Release 24 on the NTDK97AA in Asia Pacific. This vintage of the Mini System Controller does not have sufficient memory for Release 25. To upgrade to Release 25 the minimum vintage of MSC is NTDK97AB or higher.

The Boot Code on the MSC may require updating before upgrading software to release 25.15. Refer to the "Small System Memory Requirements for Release 25" earlier in this section for the requirements.

Updating the MSC Boot code is a manual process that requires a PCMCIA card programmed with R25.15. The upgrade is done using the Flash Boot ROM Utility in LD 143. Refer to NTP 553-3021-250 Upgrade Procedures Chapter 13.

Basic Configuration Data

On Release 22.08, the "Basic Configuration" default data option provided only a configuration record and no other customer data.

With X11 Release 22.16 and later, the "Basic Configuration" data option is expanded to include default data such as XPECs, Superloops, and other default data blocks.  It doesn't include Model sets, routes, TN's etc.  For complete default data including model sets etc., choose the Pre-Configured data option.

Use of BKO command in LD 43

The BKO command is used to backup the customer data to an external data card (blank PCMCIA card) located in slot "B" on the CPU faceplate.

Warning: If the pre-programmed software PCMCIA card is used during BKO operation, the card cannot be used to install software without first removing the backup data, reformatting the disk, and reprogramming with the appropriate software.

Backwards Compatible Daughterboard - NTDK26

The backward compatible daughterboard allows Option 11/11E/11C two cabinet (copper) systems to be upgraded to Release 25 maintaining copper connectivity.

The NTDK26 has a hardware key that prevents installation when the ethernet jumper (J7) is installed.

The ethernet jumper plug (J7) on the NTDK20 (Small System Controller) pack MUST be removed before the NTDK26 daughterboard is installed.

Note: Ethernet is not supported in this configuration.

Software PEPs

Manufactured Software PEPs

There are ?? PEPs manufacture installed on the Option 11C/11C Mini software of the pre-programmed daughterboard or PCMCIA card.

For Canada and US:

Product Issue 14 includes four manufacture installed PEPs on Release25.15,

- 	MP01719 (MPLR12019) to allow the configuration of the Message Waiting Indicator by DN feature on the new M3900 Series Sets.

For Asia Pacific:

Product Issue 12 includes the four PEPs, and Product Issue 13 includes the additional two mentioned above.

For CALA:

There is only one released Product Issue (14) which includes all six of the PEPs mentioned above.

For Europe:

There is only o

Installing PEPs

All Option 11C PEP files exist in the Global Patch Database. Other PEPs which may need to be installed must be placed in the following directory: c:/u/patch

There are 5 ways to get a PEP file into this directory.

- PEPs can be downloaded to the switch by FTP over an ethernet connection.

- PEPs can be downloaded to the switch by FTP over a serial line using SLIP.

- PEPs can be downloaded to the switch by FTP over a serial line using PPP.

- Program the PEP file onto a PCMCIA card. Install the PCMCIA card in drive a. In pdt copy the PEP file from the PCMCIA card to the c drive. e.g.:  cp a:newpep.p c:/u/patch/newpep.p

- PEPs can be downloaded to the switch using XMODEM file transfer over a serial line.

The following is the description of the pdt commands to perform a file transfer using the XMODEM protocol.

rx - command for receiving a file

sx - command for sending a file

To use rx, PDT Level 1 or Level 2 password login is required.

To use sx, PDT Level 2 password login is required. This is done for security purposes so that you can't get any data out of the system unless you know the PDT Level 2 password.

To transfer a file from a PC/workstation to the switch

pdt> rx [path/]filename.ext

You then enter the appropriate commands to invoke xmodem file transfer on the PC/workstation

To transfer a file from the switch

pdt> sx [path/]filename.ext

You then enter the appropriate commands to invoke xmodem file transfer on the PC/workstation.

For binary files (e.g., PEP files and database files), please ensure that the files are transferred in binary mode.

When the transfer is completed, a transmission summary is displayed and the pdt prompt is shown.

total packets                       	       		20

number of retries			0

receive timeouts 			1

system errors 			0

unknown characters                 			0

transfer cancelled      			0

packets received out of sequence   	 		0

packets with corrupted sequence    	 		0

packets failed checksum/crc check  	 		0

incomplete packets			0

duplicate packets 			0

pdt>

The following is an example in a unix environment:

Use tip to connect to the switch (if you telnet to the switch you can't use umodem)

To transfer a PEP to the switch in pdt

cd c:/u/patch

rx newpep.p

When the system prompts “Ready to receive...”, invoke local command mode by typing ~C (tilde C) and issue the umodem (s)end (b)inary command.

~C       ( tilde C to enter local command)

umodem -sb  ~mydir/peps/newpep.p

To transfer a file to the workstation in pdt

cd to directory   eg c:/p/sl1

sx direct.rec

When the system prompts “Ready to send...”, invoke local command mode by typing ~C (tilde C) and issue the umodem (r)eceive (b)inary command.

~C        ( tilde C to enter local command)

umodem -rb ~mydir/backup/direct.rec

The following is an example in a PC/Window 95 environment:

Use the HyperTerminal application to dial up to the switch

To transfer a PEP to the switch in pdt

cd c:/u/patch

rx newpep.p

When the system prompts "Ready to receive...", invoke file transfer on the PC side using the (T)ransfer pull-down menu and selecting the (S)end File option.

Select the file to be sent and select XMODEM as the Protocol. Then start the transfer on the PC side.

To transfer a file to the PC in pdt

cd to directory   eg c:/p/sl1

sx direct.rec

When the system prompts "Ready to send...", invoke file transfer on the PC side using the (T)ransfer pull-down menu and selecting the (R)eceive File option.

Select or create a file to be received as and select XMODEM as the Protocol. Then start the transfer on the PC side.

PEP Installation Steps:

1)	 In pdt use the pload command to load the PEPs. To make sure that these PEPs remain in service you must enter the pload command without the PEP name. It will then prompt you for the PEP name and ask the following questions:

Days patch vulnerable to sysload [3]- set this to 0

In-service initialize threshold [5]- enter a carriage return

In-service days to monitor inits[7]- set this to 0

2) After using the pload command, use the pins command to put the PEPs in.

PCMCIA Card Software Structure

When properly programmed, the Option 11C and 11C Mini software on the PCMCIA card will have the following directory and file structure:

bootrom/

dflt_db/

p/

u/

dramos

dramos.sym

dramoscc.sym

readme.txt