### 1.44 Meg Floppy disk upgrade method to V03.00

The information listed below has been taken from one of the upgrades that I did, Using the standard 1.44 Meg Floppy disk.

Before you start the upgrade, You must first obtain the Succession Keycode.kcd file. This can be obtained from the Nortel Web Site, Keycode section, Or via the Meridian Order Group / Option 11 Configuration Centre.

Load in the software from new, using normal procedures with the install disk.

Insert the correct V03.00 Installation floppy disk in first, before you reboot for the type of Succession system that you have the keycodes for!

#### The different versions of install disks are listed below.

#### Option 61c

#### 1)  CP3 (Type 25.21) & CP4 (Type 29.21)

#### 2)  CPP (Type 32.21)

#### Option 81c

#### 1)  CP3 (Type 26.21) & CP4 (Type 30.21)

#### 2)  CPP (Type 33.21)

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

#####

#########   ########

########`` `#############

########      ``    `#############

#######`         ######```####

#######      ######``     `###           NORTEL NETWORKS

#######    #####``          ####

####### #####`              ####   Succession Enterprise Software

`#########`                ####`

#######`                 #####       Install Tool version 11

`#######               #####`

##########            `###`          Copyright 1992 - 2003

##``##########            #

##   `########################

`#     `########################

###########################``

`````````  ````####````

Please press <CR> when ready ...

*****************************************************************

WARNING:

This software does not support TNs configured on PE/EPE

shelves. Upgrading to this software release will permanently

disable all TNs configured on PE/EPE and will not allow new

TNs to be configured.

Proceed with the upgrade? (Y/N) Y

>Obtaining and checking system configuration ...

>Validate hard disk partitions

>Copying "/f0/disk2421.sys" to "/u/disk2421.sys" - 24310 bytes copied.

>Copying "/f0/disk2521.sys" to "/u/disk2521.sys" - 24310 bytes copied.

>Copying "/f0/disk2621.sys" to "/u/disk2621.sys" - 24182 bytes copied.

>Copying "/f0/disk2821.sys" to "/u/disk2821.sys" - 24310 bytes copied.

>Copying "/f0/disk2921.sys" to "/u/disk2921.sys" - 24310 bytes copied.

>Copying "/f0/disk3021.sys" to "/u/disk3021.sys" - 24182 bytes copied.

>System date and time is: Friday 06-04-2004, 11:44:16

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Does this system have a Signaling Server.....? (Default - No)

Please enter:

<CR> -> <n> - No

<y> - Yes

Enter choice> Y

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

M A I N    M E N U

The Software Installation Tool will install or upgrade Succession

Enterprise System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <u> - To Install Menu.

<t> - To Tools Menu.

<q> - Quit.

Enter choice> U

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Please insert the diskette with the keycode file into the floppy drive.

Please enter:

<CR> -> <a> - Continue with the keycode validation.

(The keycode diskette is in the floppy drive on Core 0.)

<q> - Quit.

Enter choice> A

>Validating keycode ...

>Copying "/f0/keycode.kcd" to "/u/keycode" - 1114 bytes copied.

>The provided keycode authorizes the install

>of X210300 software (all subissues)

>for machine type 2921 (68060E processor on Succession 1000M Single Group System).

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Please confirm that this keycode matches the CDROM release.

Please enter:

<CR> -> <y> - Yes, the keycode matches. Go on to Install Menu.

<n> - No, the keycode does not match. Try another keycode diskette.

Enter choice> Y

>Obtaining database file names ...

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Succession

Enterprise System Software, Database and the PE-ROM (both CP and

IOP ROM).

You will be prompted throughout the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools Menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 Set Languages.

<q> - Quit.

Enter choice> B

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Please insert the Software CDROM into the drive on Core 0.

The labeled side of the CDROM should be side up in the CDROM tray.

Please enter:

<CR> -> <a> - CDROM is now in drive. Continue with s/w checking.

<q> - Quit.

Enter choice> A

Follow the instructions as normal for loading in the Succession software!

##### Until you get to the information listed below.

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Software release: 0300 was installed successfully on Core 0.

All files were copied from CDROM to the hard disk.

Please press <CR> when ready ...

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You will now perform the database installation.

Note: If you are installing the database from a floppy disk,

please insert the correct disk now.

Please enter:

<CR> -> <a> - Install CUSTOMER database.

(the customer database diskette must be in the Core 0 disk drive)

.

<b> - Install DEFAULT database.

(The Software CDROM must be in the Core 0 disk drive.)

<c> - Transfer the previous system database.

<d> - Copy database from the redundant disk.

<e> - Check the database that exists on the hard disk.

<f> - Transfer database from MDU.

<q> - Quit.

Enter choice> A             (Use the Floppy disk with the V25.40b database on!)

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | Yes | Ok | From 2540B to 0300 |
| SW: disk to ROM | Yes | Ok | From 2540B to x210300 |
| Database | Yes |  | From 2540B to 2540B |
| CP-BOOTROM | Yes |  |  |
| IOP-ROM | Yes |  |  |

Please enter:

<CR> -> <y> - Yes, start installation.

<n> - No, stop installation. Return to the Main Menu.

Enter choice> Y

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You selected to transfer the database from the floppy disk - release: 2540B

to the hard disk on Core 0. release: 2540B.

This will erase the database on the hard disk.

The database diskette has been inserted into the floppy disk drive.

If you quit now, the database will be left unchanged.

Please enter:

<CR> -> <a> - Continue with database install.

<q> - Quit.

Enter choice> A

Copying "/u/db/hi/eth.tmp" to "/u/db/hi_tmp/eth.tmp" - 0 bytes copied.

Copying "/u/db/hi/inet.db" to "/u/db/hi_tmp/inet.db" - 131 bytes copied.

Copying "/u/db/hi/ipb.db" to "/u/db/hi_tmp/ipb.db" - 71 bytes copied.

Copying "/u/db/hi/cnib.db" to "/u/db/hi_tmp/cnib.db" - 39 bytes copied.

Copying "/u/db/hi/iop.db" to "/u/db/hi_tmp/iop.db" - 33 bytes copied.

Copying "/u/db/hi/simm.db" to "/u/db/hi_tmp/simm.db" - 18 bytes copied.

Copying "/u/db/hi/cp.db" to "/u/db/hi_tmp/cp.db" - 31 bytes copied.

Copying "/u/db/hi/hi.db" to "/u/db/hi_tmp/hi.db" - 255 bytes copied.

* * *    Succession Enterprise Software Database ARCHIVE Tool    * * *

====================================================

You have chosen to restore database dated: Jun 04 11:19:00 2004

Please confirm.

Please enter:

<CR> -> <y> - Yes, load.

<n> - No, DO NOT load.

Enter Choice> Y

/f0/config.rec

>Restoring "/f0/config.rec" to "/u/db/config.bak"..OK

/f0/database.rec

Let the Meridian load in all the files & wait until the next menu appears.

Once this has been completed, Then remove the Floppy disk & insert a new one labeled as Succession copy!.

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

An  upgrade from  machine type 30(CP_NET) to 29(CP_NET) has  been detected.

The  Hardware  Infrastructure (HI) data base from the restored

system is not applicable. It will be erased and the current one

will be used.

This appears because the software at V25.40b is now changing from a Option 81c, to a 61c for the keycodes listed at the Succession release!

Please press <CR> when ready ...

>Copying "/u/db/hi_bak/iop.db" to "/u/db/hi/iop.db" - 33 bytes copied.

>Copying "/u/db/hi_bak/cp.db" to "/u/db/hi/cp.db" - 31 bytes copied.

>Copying "/u/db/hi_bak/cnib.db" to "/u/db/hi/cnib.db" - 39 bytes copied.

>Copying "/u/db/hi_bak/ipb.db" to "/u/db/hi/ipb.db" - 71 bytes copied.

>Copying "/u/db/hi_bak/hi.db" to "/u/db/hi/hi.db" - 255 bytes copied.

>Copying "/u/db/hi_bak/simm.db" to "/u/db/hi/simm.db" - 18 bytes copied.

>Removing file "/u/db/hi_tmp/eth.tmp"

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You have chosen to upgrade CP-BOOTROM in card slot 14 from

the context: 2540b to the context: x210300.

This will replace old CP-BOOTROM with the ROM image files:

"/p/os/cprom".

You may continue with ROM upgrade or quit now and leave ROM unchanged.

Please enter:

<CR> -> <a> - Continue with ROM upgrade.

<q> - Quit.

Enter choice> A

>Install CP flash ROM file from "/p/os/cprom"

Burning the flash on a CP4 board.

Last address   : 0x403b32c

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

The release and issue of IOP-ROM, card slot number 16, are the

same as the release and issue of the ROM image file

"/p/os/ioprom".

Are you sure you'd like to continue installing IOP-ROM?

Please enter:

<CR> -> <y> - Yes, start installation.

<n> - No, stop installation.

Enter choice> Y

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You will now upgrade the IOP-ROM in card slot 16 from Release: 02.00 to

Release: 02.00, or quit and leave the ROM unchanged.

Please enter:

<CR> -> <a> - Continue with ROM upgrade.

<q> - Quit.

Enter choice> Q

>Install IOP-ROM from file "/p/os/ioprom"

Zeroing out EEPROM...

Erasing EEPROM, take 0010

>System resetting IOP card at slot 16

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | Yes | Ok | From 2540B to 0300 |
| SW: disk to ROM | Yes | Ok | From 2540B to x210300 |
| Database | Yes | Ok | From 2540B to 2540B |
| CP-BOOTROM | Yes | Ok | From x112540b to x210300 |
| IOP-ROM | Yes | Ok | From 02.00 to 02.00 |

Please press <CR> when ready ...

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Succession

Enterprise System Software, Database and the PE-ROM (both CP and

IOP ROM).

You will be prompted throughout the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools Menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 Set Languages.

<q> - Quit.

Enter choice> Q

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You selected to quit. Please confirm.

Please enter:

<CR> -> <y> - Yes, quit.

<n> - No, DON'T quit.

Enter choice> Y

At this stage It’s best to have a capture file to record any messages / errors that might appear & get them checked out.

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You selected to quit the Install Tool.

You may reboot the system or return to the Main Menu.

Remove all disks from the system before rebooting.

------------------------------------------------------------

DO NOT REBOOT USING RESET BUTTON!!!

------------------------------------------------------------

Please enter:

<CR> -> <a> - Reboot the system.

<m> - Return to the Main Menu.

Enter choice> A

>Removing temporary file "/u/disk2421.sys"

>Removing temporary file "/u/disk2521.sys"

>Removing temporary file "/u/disk2621.sys"

>Removing temporary file "/u/disk2821.sys"

>Removing temporary file "/u/disk2921.sys"

>Removing temporary file "/u/disk3021.sys"

>Rebooting system ...

Sys|

SYS700 02 - ROM created on Friday October 3 16:35:41 EDT 2003

SYS706 26 - Full Reset by Software Install

SYS702 64 - Total Flash size (in megs), all banks populated

SYS702 64 - DRAM size (in megs) - Banks 1 3 5 7  populated

SYS703 0DE8 - Self Tests completed

SYS700 04 - Selecting Boot Source

Succession Enterprise ROM System Software

Release     : x210300

Created     : Friday October 3 16:35:41 EDT 2003

Copyright(C) 1972-2003 Nortel Networks.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1997 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'os' from "/flash/flashos" ... 1276696: 512000 + 512000 + 252696 + 108300

Starting at 0x4100020...

Succession Enterprise System Software

Release     : x210300

Created     : Friday October 3 16:35:57 EDT 2003

Loaded      : 4/6/04 12:04:23 from '/flash/flashos'

Copyright(C) 1972-2003 Nortel Networks.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1997 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Initialize Network Interface.

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

Network Interface is up.

Host         : MERIDIAN

IP Address   : 220.0.0.100

Sub-netmask  : 255.255.255.0

Loading symbols from "/flash/flashos.sym"

Executing startup script "/p/startup"

# Empty startup file for use in the field.

cd "/p/etc"

value = 0 = 0x0

0x8c20f04 (tRptD):  (Boot ROM)              SRPT4504 Single CP configuration

Done executing startup script /p/startup

PDT Ready!

Loading SL1 .0x8c20f04 (tRptD):  (Boot ROM)              SRPT720 ROM OS 0: Cold

Start. Running on CP 0

Loading 'res' from "/flash/sl1res"

16154628 + 406872 + 1717828

Loading 'ovl' from "/flash/ovlres"

5666716 + 0 + 0

0x8c20f04 (tRptD):  (Boot ROM)              SRPT728 ROM OS 0: Loading 'diskos' from /flash/flashos

Unable to remove /p/data/ERRSEV.MSG

Start SNMP agent

Processed SNMP configuration file /p/data/view.cfg

0x8c20f04 (tRptD):  (4/6/04 12:04:30.796) SRPT730 OS 0: Cold Start

Release:  x210300

Created:  Friday October 3 16:35:57 EDT 2003

Loading OTM script files........

Loading HI Data from '/u/db/hi' ...

Loading SL1 data

SYS000 000800 000000 000000 000000 CPU 0

SYSDB RLS/ISS: 2540B DATE/TIME: 4/6/2004 11:19:28 RECS: 323 SEQNO: 2209

DATA CONVERSION

X11 ISSUE 25.40B TO SUCCESSION X21 RELEASE 3.00

CONFIG:  IDLE_SET_DISPLAY Pizza Factory Inc

PHYS

BCS TEMPLATE

PBX TEMPLATE

CUST

P_AUX_CUST

ICI

ACUST

CLID

ROUTE

LTN

LTN LINK

TN BLKS

SCL

ESN  0

ESN_ROUTE_LIST

ESN DM

NCTL

ACD

CPK

NFCR_TREES

IDC_TREES

ASYNCH

AML/ELAN

VAS

BG_TIME

BG_CAT

DCH

ARIES

SYS_PARAM

XTDT

FTC

MCAD

FCAD

FDCT

DTI2SYTI

DTI2SCAT

DTI2PDCA

FFC

LAPW

DTSL

PLUGIN

CPND DATA

CPND NAME

DONEinterrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

INI000 00000000 12 05 20 00000000 00000000 00000000 00000000 0 00000000

XMI101 12

DSET000 DOWN LOAD 0 0 0 0 0 0 0 216547

INI014 000000E0 00000030 000000E8

INI002 00000080 00000081 00000082 00000083 0000008F

DTM338 8

Total number of files 0

Warning: No corporate directory files were found!

SRPT752 INI 0: INI completed in 82 seconds

Let the Meridian do a normal system Dump “EDD” onto the new floppy disk. Login & do a general check to make sure that the customers data is intact!. Once that you are happy, Then zip up the Floppy disk (Use save folder option!) & email it out to the engineer.

I would suggest that the current PWD2 passcode be given out to the field engineer, Just in case the Meridian passcodes have been changed on site.