SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11

VERSION 16.83G

LOGI

OVL111 044 BKGD

PASS? ____

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:981876

S/W CART ID:985501

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE E.G. “4695”

DBG000

#P 1 1325 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 1325 : B8B9 B8B1 B6B7   -----------  MERIDIAN PRINTOUT IN HEX

8 9  8 1  6 7  --------   THIS TRANSLATES TO 981876

#W 1 1325  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 1325 : B8B9 /B8B9  TO ALTER THE SECOND & FIRST DIGIT

#W 1 1326

01 1326 : B8B1 /B2B1  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 1327

01 1327 : B6B7 /B4B3  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 1325 3  ---------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 1325 : B8B9 B2B1 B4B3  ----  MERIDIAN PRINTOUT IN HEX

8 9  2 1  4 3  ----   NOW CHANGED TO 981234

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:981234

S/W CART ID:985501

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11

VERSION 16.90G

LOGI

OVL111 044 BKGD

PASS? ____

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:981876

S/W CART ID:985501

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 132D 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 132D : B8B9 B8B1 B6B7   -----------  MERIDIAN PRINTOUT IN HEX

8 9  8 1  6 7  --------   THIS TRANSLATES TO 981876

#W 1 132D  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 132D : B8B9 /B8B9  TO ALTER THE SECOND & FIRST DIGIT

#W 1 132E

01 132E : B8B1 /B2B1  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 132F

01 132F : B6B7 /B4B3  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 132D 3  ---------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 132D : B8B9 B2B1 B4B3  ----  MERIDIAN PRINTOUT IN HEX

8 9  2 1  4 3  ----   NOW CHANGED TO 981234

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:981234

S/W CART ID:985501

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11

VERSION 18.20H

LOGI

OVL111 044 BKGD

PASS? ____

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:981876

S/W CART ID:985501

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 143A 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 143A : B8B9 B8B1 B6B7   -----------  MERIDIAN PRINTOUT IN HEX

8 9  8 1  6 7  --------   THIS TRANSLATES TO 981876

#W 1 143A  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 143A : B8B9 /B8B9  TO ALTER THE SECOND & FIRST DIGIT

#W 1 143B

01 143B : B8B1 /B2B1  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 143C

01 143C : B6B7 /B4B3  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 143A 3  ---------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 143A : B8B9 B2B1 B4B3  ----  MERIDIAN PRINTOUT IN HEX

8 9  2 1  4 3  ----   NOW CHANGED TO 981234

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:981234

S/W CART ID:985501

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11

VERSION 18.40H

LOGI

OVL111 044 BKGD

PASS? ____

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:981876

S/W CART ID:985501

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 143C 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 143C : B8B9 B8B1 B6B7   -----------  MERIDIAN PRINTOUT IN HEX

8 9  8 1  6 7  --------   THIS TRANSLATES TO 981876

#W 1 143C  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 143C : B8B9 /B8B9  TO ALTER THE SECOND & FIRST DIGIT

#W 1 143D

01 143D : B8B1 /B2B1  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 143E

01 143E : B6B7 /B4B3  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 143C 3  ---------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 143C : B8B9 B2B1 B4B3  ----  MERIDIAN PRINTOUT IN HEX

8 9  2 1  4 3  ----   NOW CHANGED TO 981234

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:981234

S/W CART ID:985501

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11

VERSION 18.42H

LOGI

OVL111 044 BKGD

PASS? ____

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:981876

S/W CART ID:985501

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 143E 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 143E : B8B9 B8B1 B6B7   -----------  MERIDIAN PRINTOUT IN HEX

8 9  8 1  6 7  --------   THIS TRANSLATES TO 981876

#W 1 143E  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 143E : B8B9 /B8B9  TO ALTER THE SECOND & FIRST DIGIT

#W 1 143F

01 143F : B8B1 /B2B1  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 1440

01 1440 : B6B7 /B4B3  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 143E 3  ---------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 143E : B8B9 B2B1 B4B3  ----  MERIDIAN PRINTOUT IN HEX

8 9  2 1  4 3  ----   NOW CHANGED TO 981234

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:981234

S/W CART ID:985501

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11E

VERSION 20.19

LOGI

OVL111 044 BKGD

PASS? ____

TTY #00 LOGGED IN 12:09  21/11/1996

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:986343

S/W CART ID:986489

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 164F 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 164F : 00B8B9 00B3B6 00B3B4   -----  MERIDIAN PRINTOUT IN HEX

8 9    3 6    3 4  ----   THIS TRANSLATES TO 986343

#W 1 164F  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 164F : 00B8B9 /00B8B9  TO ALTER THE SECOND & FIRST DIGIT

#W 1 1650

01 1650 : 00B3B6 /00B4B6  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 1651

01 1651 : 00B3B4 /00B9B8  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 164F 3  ------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 164F : 00B8B9 00B4B6 00B9B8  ----  MERIDIAN PRINTOUT IN HEX

8 9    4 6    9 8  ----   NOW CHANGED TO 986489

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:986489

S/W CART ID:986489

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11E

VERSION 21.19

LOGI

PASS? ____

TTY #00 LOGGED IN 12:14  21/11/1996

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:972317

S/W CART ID:972317

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 16BD 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 16BD : 00B7B9 00B3B2 00B7B1   -----  MERIDIAN PRINTOUT IN HEX

7 9    3 2    7 1  ----   THIS TRANSLATES TO 972317

#W 1 16BD  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 16BD : 00B7B9 /00B0B0  TO ALTER THE SECOND & FIRST DIGIT

#W 1 16BE

01 16BE : 00B3B2 /00B0B0  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 16BF

01 16BF : 00B7B1 /00B0B0  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 16BD 3  ------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 16BD : 00B0B0 00B0B0 00B0B0  ----  MERIDIAN PRINTOUT IN HEX

0 0    0 0    0 0  ----   NOW CHANGED TO 000000

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:000000

S/W CART ID:972317

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11E

VERSION 21.35

LOGI

PASS? ____

TTY #00 LOGGED IN 12:14  10/04/1997

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:994976

S/W CART ID:972317

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 1721 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 1721 : 00B9B9 00B9B4 00B6B7   -----  MERIDIAN PRINTOUT IN HEX

9 9    9 4    6 7  ----   THIS TRANSLATES TO 994976

#W 1 1721  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 1721 : 00B9B9 /00B2B1  TO ALTER THE SECOND & FIRST DIGIT

#W 1 1722

01 1722 : 00B9B4 /00B4B3  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 1723

01 1723 : 00B6B7 /00B6B5  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 1721 3  ------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 1721 : 00B2B1 00B4B3 00B6B5  ----  MERIDIAN PRINTOUT IN HEX

2 1    4 3    6 5  ----   NOW CHANGED TO 123456

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:123456

S/W CART ID:972317

REQ  ****

>

OVL000

>LOGO

SITE_ID

PRINTOUT OF HOW TO CHANGE THE SITE ID ON A OPTION 11E

VERSION 21.54

LOGI

PASS? ____

TTY #00 LOGGED IN 12:14  10/04/1997

>LD 22         WRITE DOWN THE EXISTING "TID" DETAILS

PT2000                 AS SHOWN BELOW...

REQ  TID

SITE ID:994976

S/W CART ID:972317

REQ

****

>

OVL000

>$$  ---------------     ENTER DEBUG SECTION "$$"

PSWD? ____  --------     ENTER LEVEL 2 PASSCODE

DBG000

#P 1 1721 3  -------     ENTER THIS ADDRESS TO PRINT EXISTING ID

01 1721 : 00B9B9 00B9B4 00B6B7   -----  MERIDIAN PRINTOUT IN HEX

9 9    9 4    6 7  ----   THIS TRANSLATES TO 994976

#W 1 1721  --------      ENTER THIS ADDRESS TO ALTER THE SITE ID

01 1721 : 00B9B9 /00B2B1  TO ALTER THE SECOND & FIRST DIGIT

#W 1 1722

01 1722 : 00B9B4 /00B4B3  TO ALTER THE FOURTH & THIRD DIGIT

#W 1 1723

01 1723 : 00B6B7 /00B6B5  TO ALTER THE SIXTH & FIFTH DIGIT

DBG000

#P 1 1721 3  ------      ENTER THIS ADDRESS TO CHECK CHANGE OF ID

01 1721 : 00B2B1 00B4B3 00B6B5  ----  MERIDIAN PRINTOUT IN HEX

2 1    4 3    6 5  ----   NOW CHANGED TO 123456

#

****

>LD 22          CHECK THAT THE SITE ID HAS CHANGED.

PT2000          IF IT IS OK THEN DO A EDD IN LD 43.

REQ  TID

SITE ID:123456

S/W CART ID:972317

REQ  ****

>

OVL000

>LOGO