This procedure lets you change the PABX IP addresses without Warm or Cold starting the PABX.

If there is a Callpilot, Symposium, Tapi server etc this will cause an

Outage of these devices and keep in mind if your changing the IP well

They need to know also. So its much more involved than just this!

Neil Teo 30/5/2003

This example is from an 11C system on 25.30

>ld 117

OAM000

MEM AVAIL: (U/P): 1038524    USED U P: 233655 104076    TOT: 1376255

DISK RECS AVAIL: 401

SURVIVABILITY AVAIL:     0    USED:     0    TOT:     0

=> new host DARWIN_11C 10.42.38.4

INET Data Added

=> pr host

Cabinet # 0

ID  Hostname         IP Address

1  PRIMARY_ENET     137.135.128.253

2  SECONDARY_ENET   137.135.128.254

3  LOCAL_PPP_IF     137.135.192.4

4  MERIDIAN1        10.42.38.130

# 5  DARWIN_11C       10.42.38.4

No database file exists for cab#1

No database file exists for cab#2

No database file exists for cab#3

No database file exists for cab#4

OK

=> stat host

*** Active Internet Host Table ***

ID  Hostname         IP Address

--  localhost        127.0.0.1

1  PRIMARY_ENET     137.135.128.253

2  SECONDARY_ENET   137.135.128.254

3  LOCAL_PPP_IF     137.135.192.4

4  MERIDIAN1        10.42.38.130

=> enl host 5

HOST: "DARWIN_11C" "10.42.38.4" enabled

OK

=> chg elnk active DARWIN_11C

INET Database updated

=> prt elnk

ACTIVE   ETHERNET: "DARWIN_11C" "10.42.38.4"

INACTIVE ETHERNET: "SECONDARY_ENET" "137.135.128.254"

OK

=> chg mask 255.255.255.128

INET Data Changed

=> prt mask

SUBNET MASK: "255.255.255.128"

OK

=> pr ro

Cabinet # 0

Route entry(ies) not found.

No database file exists for cab#1

No database file exists for cab#2

No database file exists for cab#3

No database file exists for cab#4

OK

=> stat ro

*** Active Internet Routing Table ***

ID  Destination      Gateway           Port

--  137.135.128.128  137.135.128.253    0 [qu0]

--  127.0.0.1        127.0.0.1         -- [lo0]

=> ****

OVL000

>ld 137

CIOD000

.stat elnk

ELNK   ENABLED

Ethernet (qu unit number 0):

Host: PRIMARY_ENET

Internet address: 137.135.128.253

Broadcast address: 137.135.128.255

Ethernet address: 00:00:75:45:f8:fd

Netmask: 0xffff0000; Subnetmask: 0xffffff80

0 packets received; 8 packets sent

0 input errors; 8 output errors

0 collisions

.dis elnk

OK

.****

>

OVL000

>ld 48

LNK000

.stat elan

SERVER TASK:  DISABLED

.dis elan test

OK

.

TIM000 17:43  29/5/2003 CPU 0

BUG480

BUG480 +   1066E05C 1067B7A0 1093DC2C 115B0A3E 115B0780

BUG480 + 115ACA36 115A9704 115A735C 10959A58 10958878

BUG480 + 1095812A 109565A6 10953A0A 10E3FFC2 10E3F2A8

BUG480 + 10E3EF50

.enl elan

OK

.

BUG480

BUG480 +   1066E05C 1067B7A0 1093DC2C 115B082A 115B0780

BUG480 + 115ACD36 115A9704 115A735C 10959A58 10958878

BUG480 + 1095812A 109565A6 10953A0A 10E3FFC2 10E3F2A8

BUG480 + 10E3EF50

ELAN020 ELAN Server enabled by LD 48

.stat elan

SERVER TASK:  DISABLED

.*

.*

.*

.*

OVL000

>ld 137

CIOD000

.enl elnk

Initialize Network Interface.

Network Interface is up.

Host         : DARWIN_11C

IP Address   : 10.42.38.4

Sub-netmask  : 255.255.255.128

OK

.****

>ld 117

OAM000

MEM AVAIL: (U/P): 1038524    USED U P: 233655 104076    TOT: 1376255

DISK RECS AVAIL: 401

SURVIVABILITY AVAIL:     0    USED:     0    TOT:     0

=> ping 10.42.38.1

PING 10.42.38.1 SUCCESSFUL

=> ping 10.42.112.95

PING 10.42.112.95 FAILED

=> new route 10.42.112.95 10.42.38.1	Basically how is the OTM IP reached, by going to gateway at 10.42.38.1

INET Data Added

=> pr ro

Cabinet # 0

ID  Destination      Gateway           Port

2  10.42.112.95     10.42.38.1         0

No database file exists for cab#1

No database file exists for cab#2

No database file exists for cab#3

No database file exists for cab#4

OK

=> enl ro 2	Where “2” is the ID no obtained when printing it above.

ROUTE: "10.42.112.95" "10.42.38.1" enabled

OK

=> st ro

*** Active Internet Routing Table ***

ID  Destination      Gateway           Port

--  0.0.0.0          10.42.38.1         0 [qu0]

--  10.42.38.0       10.42.38.4         0 [qu0]

--  10.42.0.0        10.42.38.1         0 [qu0]

--  10.42.112.0      10.42.38.1         0 [qu0]

--  127.0.0.1        127.0.0.1         -- [lo0]

2  10.42.112.95     10.42.38.1         0 [qu0]

=> ping 10.42.112.95

# PING 10.42.112.95 SUCCESSFUL				 Woo Hoo!

=>

To tidy things up at the end do a

=> update dbs

This renumbers the Ids to make it pretty basically.

OVL000

>ld 43

EDD000

.ddd

DB SEQ NUM = 210

CONFIG

PHYSICAL MAP

BCS TEMPLATE

PBX TEMPLATE

CUST

ACUST

CLID

ROUTE

DAPC

LTN TN

LTN LNK

TN

SCL