How to change the ITG card passcode back to default codes

Login: itgadmin

Password: (itgadmin)

Welcome to the IP Line command line.

Software Version: IPL-3.10.81

Management IP:

Host Type: Succession Media Card

Firmware Version: Firmware Version Not Available

SysName:

SysLocation:

SysContact:

OS Time:  Date (02/07/2002) Time (16:04:19)

Use "logout" to logout.

Idle session timeout = 20 minutes.

IPL> shellPasswordSet (Use this command to change the Passwords on the ITG Card)

Enter current username:itgadmin

Enter current password:(itgadmin)

Enter new username:David

username must be between 8 and 12 characters long

value = -1 = 0xffffffff = _func_taskRegsShowRtn + 0xff77219b

IPL> shellPasswordSet

Enter current username:itgadmin

Enter current password:(itgadmin)

Enter new username:Meridian

Enter new password:

Enter new password again to confirm:value = 0 = 0x0

IPL>

IPL> logout

PRESS THE REBOOT BUTTON ON THE ITG CARD TO CHECK THAT THE NEW PASSWORDS WORK!

MC Firmware Rls 6.7

8051XA Firmware Version 4.11 07 August 2003

(C) Nortel Inc. 2001

EPLD Version: 3.0

32K External RAM detected

8K DPRAM detected

All FPGAs are configured

No dongle detected

Bank 1 Checksum - B2C8H

SRAM test okay

SDRAM Addr test okay

SDRAM blank over

Early serial debug initialized

VxWorks System Boot

Copyright 1984-1998  Wind River Systems, Inc.

CPU: IXP1200

Version: 5.4

BSP version: 5.0

Creation date: Nov 22 2001, 18:21:11

Enter jkl to force boot to BootROM vxWorks prompt

auto-booting...

boot device          : ata=

unit number          : 1

processor number     : 0

file name            : /C:/exec

flags (f)            : 0x0

other (o)            : ixpMac

Attaching to ATA disk device... Found device : Cirrus CL-PD672x

PC Card Detected: Socket 1

cisGet : pcCard recognized

/C:/  - Volume is OK

3582048 + 884880 + 4498808

Starting at 0x1000...

Early serial debug initialized

Found device : Cirrus CL-PD672x

PC Card Detected: Socket 1

ipMethodInit, IP_DEFAULT_METHOD checksum

Invalid IP Address in EEPROM: 255.255.255.255

Host Name: bootHost

Target Name: vxTarget

User: target

MAC Address for Port 0 is : 00:60:38:bd:ce:a0

SysConf: uEng_intConnect to pktHandlingIsr.

0 symbols bound.

Load uCode from buffer Status : Return Success

Starting Microengines.

Loading MAC_21440...

Loading MAC_21440...

MAC 21440 String : Processed all arugments

Done loading mac21440...

mac21440Start: Starting port 0

TLAN Port  set to Auto-Negotiate Speed and Duplex Settings

ELAN set to 10BaseT Operation

ELAN  set to Half Duplex Operation

Waiting For Carrier link to Return

MAC Address for Port 1 is : 00:60:38:bd:ce:a1

Carrier still down after 10 Seconds

Loading MAC_21440...

Loading MAC_21440...

MAC 21440 String : Processed all arugments

Done loading mac21440...

mac21440Start: Starting port 1

TLAN Port  set to Auto-Negotiate Speed and Duplex Settings

ELAN set to 10BaseT Operation

ELAN  set to Half Duplex Operation

Waiting For Carrier link to Return

Carrier still down after 10 Seconds

Getting IP over BOOTP via network interface ixpMac1

Enter +++ to escape to Shell for manual configuration

........................................bootpMsgSend Failed, Error = 490006

Adding 9404 symbols for standalone.

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]  ]]]]     ]]]]]]]]]]       ]]              ]]]]         (R)

]     ]]]]]]]]]  ]]]]]]     ]]]]]]]]       ]]               ]]]]

]]     ]]]]]]]  ]]]]]]]]     ]]]]]] ]     ]]                ]]]]

]]]     ]]]]] ]    ]]]  ]     ]]]] ]]]   ]]]]]]]]]  ]]]] ]] ]]]]  ]]   ]]]]]

]]]]     ]]]  ]]    ]  ]]]     ]] ]]]]] ]]]]]]   ]] ]]]]]]] ]]]] ]]   ]]]]

]]]]]     ]  ]]]]     ]]]]]      ]]]]]]]] ]]]]   ]] ]]]]    ]]]]]]]    ]]]]

]]]]]]      ]]]]]     ]]]]]]    ]  ]]]]]  ]]]]   ]] ]]]]    ]]]]]]]]    ]]]]

]]]]]]]    ]]]]]  ]    ]]]]]]  ]    ]]]   ]]]]   ]] ]]]]    ]]]] ]]]]    ]]]]

]]]]]]]]  ]]]]]  ]]]    ]]]]]]]      ]     ]]]]]]]  ]]]]    ]]]]  ]]]] ]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]       Development System

]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]       VxWorks version 5.4

]]]]]]]]]]]]]]]]]]]]]]]]]]       KERNEL: WIND version 2.5

]]]]]]]]]]]]]]]]]]]]]]]]]       Copyright Wind River Systems, Inc., 1984-1999

CPU:  IXP1200 .  Processor #0.

Memory Size: 0x3b00000.  BSP version ITGL3.10.81sa

.

/C:/  - Volume is OK

Unable to open startup script /C:/etc/startup

Retrieved old volume params with %35 confidence:

Volume Parameters: FAT type: FAT32, se

/C:/  - disk check in progress ...

/C:/  - Volume is OK

total # of clusters:  3,906

# of free clusters:  264

# of bad clusters:  0

total free space:  1,056 Kb

max contiguous free space:  1,077,248 bytes

# of files:  21

# of folders:  10

total bytes in files:  14,442 Kb

# of lost chains:  0

total bytes in lost chains:  0

Console logging enabled

JUL 02 16:13:16 SYS: Info IPL 3.10.81 - Saturday September 27 16:09:14 EDT 2003

JUL 02 16:13:16 SYS: Alert Network not configured, aborting startup

Login: itgadmin

Password:

Login: Meridian

Password:

Login: Meridian

Password:

Login: Meridian

Password:

I CANNOT LOGIN USING THE NEW & OLD PASSWORDS (I have done this by accident!)

I'M ONLY ALLOWED 3 ATTEMPTS & NO WARNING IS GIVEN!

PRESS THE REBOOT BUTTON AGAIN!

MC Firmware Rls 6.7

8051XA Firmware Version 4.11 07 August 2003

(C) Nortel Inc. 2001

EPLD Version: 3.0

32K External RAM detected

8K DPRAM detected

All FPGAs are configured

No dongle detected

Bank 1 Checksum - B2C8H

SRAM test okay

SDRAM Addr test okay

SDRAM blank over

Early serial debug initialized

VxWorks System Boot

Copyright 1984-1998  Wind River Systems, Inc.

CPU: IXP1200

Version: 5.4

BSP version: 5.0

Creation date: Nov 22 2001, 18:21:11

Enter jkl to force boot to BootROM vxWorks prompt

jkl                   YOU MUST ENTER jkl to go into BIOS WITHIN 10 SECONDS!

Uncompressing Default vxWorks image .... Complete

Jumping to 0x1000

Early serial debug initialized

Setting tLogTask priority to 150

Found device : Cirrus CL-PD672x

PC Card Detected: Socket 1

cisGet : pcCard recognized

0x3affe64 (tRootTask): Resetting the onboard Real Time Clock

ipMethodInit, IP_DEFAULT_METHOD checksum

lnIsaNvramInfoGet: ipMethod = 1, Boot Line not updated

Host Name: bootHost

Target Name: vxTarget

User: target

MAC Address for Port 0 is : 00:60:38:bd:ce:a0

Initializing Semaphore

SysConf: uEng_intConnect to pktHandlingIsr.

0 symbols bound.

Load uCode from buffer Status : Return Success

Starting Microengines.

0x3affe64 (tRootTask): Loading MAC_21440...

0x3affe64 (tRootTask): mac21440Load: Init String[0] NULL: ixpMac

0x3affe64 (tRootTask): Loading MAC_21440...

0x3affe64 (tRootTask): MAC 21440 String : Processed all arugments

0x3affe64 (tRootTask): mac21440InitMem: Net Pool ID is 53209672

0x3affe64 (tRootTask): Done loading mac21440...

0x3affe64 (tRootTask): mac21440Start: Starting port 0

interrupt:  ixpMac0 carrier down

interrupt:  ixpMac1 carrier down

0x3affe64 (tRootTask): TLAN Port  set to Auto-Negotiate Speed and Duplex Settings

0x3affe64 (tRootTask): ELAN set to 10BaseT Operation

0x3affe64 (tRootTask): ELAN  set to Half Duplex Operation

MAC Address for Port 1 is : 00:60:38:bd:ce:a1

0x3affe64 (tRootTask): Loading MAC_21440...

0x3affe64 (tRootTask): mac21440Load: Init String[0] NULL: ixpMac

0x3affe64 (tRootTask): Loading MAC_21440...

0x3affe64 (tRootTask): MAC 21440 String : Processed all arugments

0x3affe64 (tRootTask): mac21440InitMem: Net Pool ID is 52110084

0x3affe64 (tRootTask): Done loading mac21440...

0x3affe64 (tRootTask): mac21440Start: Starting port 1

0x3affe64 (tRootTask): TLAN Port  set to Auto-Negotiate Speed and Duplex Settings

0x3affe64 (tRootTask): ELAN set to 10BaseT Operation

0x3affe64 (tRootTask): ELAN  set to Half Duplex Operation

interrupt:  ixpMac1 carrier up

Getting IP over BOOTP via network interface ixpMac1

Enter +++ to escape to Shell for manual configuration

interrupt:  ixpMac1 carrier down

..+++.                  ENTER +++ to go into BIOS!

escape sequence detected -  configure network manually...

bootpMsgSend Failed, Error = 490006

Adding 4664 symbols for standalone.

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]  ]]]]     ]]]]]]]]]]       ]]              ]]]]         (R)

]     ]]]]]]]]]  ]]]]]]     ]]]]]]]]       ]]               ]]]]

]]     ]]]]]]]  ]]]]]]]]     ]]]]]] ]     ]]                ]]]]

]]]     ]]]]] ]    ]]]  ]     ]]]] ]]]   ]]]]]]]]]  ]]]] ]] ]]]]  ]]   ]]]]]

]]]]     ]]]  ]]    ]  ]]]     ]] ]]]]] ]]]]]]   ]] ]]]]]]] ]]]] ]]   ]]]]

]]]]]     ]  ]]]]     ]]]]]      ]]]]]]]] ]]]]   ]] ]]]]    ]]]]]]]    ]]]]

]]]]]]      ]]]]]     ]]]]]]    ]  ]]]]]  ]]]]   ]] ]]]]    ]]]]]]]]    ]]]]

]]]]]]]    ]]]]]  ]    ]]]]]]  ]    ]]]   ]]]]   ]] ]]]]    ]]]] ]]]]    ]]]]

]]]]]]]]  ]]]]]  ]]]    ]]]]]]]      ]     ]]]]]]]  ]]]]    ]]]]  ]]]] ]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]       Development System

]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]       VxWorks version 5.4

]]]]]]]]]]]]]]]]]]]]]]]]]]       KERNEL: WIND version 2.5

]]]]]]]]]]]]]]]]]]]]]]]]]       Copyright Wind River Systems, Inc., 1984-2000

CPU: IXP1200.  Processor #0.

Memory Size: 0x3b00000.  BSP version 5.0.

PRESS "Return" to get to the "->" (BIOS)

-> shellPasswordNvramClear  (Enter this command to clear out the old Passwords!)

value = 0 = 0x0

->

PRESS THE REBOOT BUTTON AGAIN!

MC Firmware Rls 6.7

8051XA Firmware Version 4.11 07 August 2003

(C) Nortel Inc. 2001

EPLD Version: 3.0

32K External RAM detected

8K DPRAM detected

All FPGAs are configured

No dongle detected

Bank 1 Checksum - B2C8H

SRAM test okay

SDRAM Addr test okay

SDRAM blank over

Early serial debug initialized

VxWorks System Boot

Copyright 1984-1998  Wind River Systems, Inc.

CPU: IXP1200

Version: 5.4

BSP version: 5.0

Creation date: Nov 22 2001, 18:21:11

Enter jkl to force boot to BootROM vxWorks prompt

auto-booting...

boot device          : ata=

unit number          : 1

processor number     : 0

file name            : /C:/exec

flags (f)            : 0x0

other (o)            : ixpMac

Attaching to ATA disk device... Found device : Cirrus CL-PD672x

PC Card Detected: Socket 1

cisGet : pcCard recognized

/C:/  - Volume is OK

3582048 + 884880 + 4498808

Starting at 0x1000...

Early serial debug initialized

Found device : Cirrus CL-PD672x

PC Card Detected: Socket 1

ipMethodInit, IP_DEFAULT_METHOD checksum

Invalid IP Address in EEPROM: 255.255.255.255

Host Name: bootHost

Target Name: vxTarget

User: target

MAC Address for Port 0 is : 00:60:38:bd:ce:a0

SysConf: uEng_intConnect to pktHandlingIsr.

0 symbols bound.

Load uCode from buffer Status : Return Success

Starting Microengines.

Loading MAC_21440...

Loading MAC_21440...

MAC 21440 String : Processed all arugments

Done loading mac21440...

mac21440Start: Starting port 0

TLAN Port  set to Auto-Negotiate Speed and Duplex Settings

ELAN set to 10BaseT Operation

ELAN  set to Half Duplex Operation

Waiting For Carrier link to Return

MAC Address for Port 1 is : 00:60:38:bd:ce:a1

Carrier still down after 10 Seconds

Loading MAC_21440...

Loading MAC_21440...

MAC 21440 String : Processed all arugments

Done loading mac21440...

mac21440Start: Starting port 1

TLAN Port  set to Auto-Negotiate Speed and Duplex Settings

ELAN set to 10BaseT Operation

ELAN  set to Half Duplex Operation

Waiting For Carrier link to Return

Carrier still down after 10 Seconds

Getting IP over BOOTP via network interface ixpMac1

Enter +++ to escape to Shell for manual configuration

........................................bootpMsgSend Failed, Error = 490006

Adding 9404 symbols for standalone.

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]  ]]]]     ]]]]]]]]]]       ]]              ]]]]         (R)

]     ]]]]]]]]]  ]]]]]]     ]]]]]]]]       ]]               ]]]]

]]     ]]]]]]]  ]]]]]]]]     ]]]]]] ]     ]]                ]]]]

]]]     ]]]]] ]    ]]]  ]     ]]]] ]]]   ]]]]]]]]]  ]]]] ]] ]]]]  ]]   ]]]]]

]]]]     ]]]  ]]    ]  ]]]     ]] ]]]]] ]]]]]]   ]] ]]]]]]] ]]]] ]]   ]]]]

]]]]]     ]  ]]]]     ]]]]]      ]]]]]]]] ]]]]   ]] ]]]]    ]]]]]]]    ]]]]

]]]]]]      ]]]]]     ]]]]]]    ]  ]]]]]  ]]]]   ]] ]]]]    ]]]]]]]]    ]]]]

]]]]]]]    ]]]]]  ]    ]]]]]]  ]    ]]]   ]]]]   ]] ]]]]    ]]]] ]]]]    ]]]]

]]]]]]]]  ]]]]]  ]]]    ]]]]]]]      ]     ]]]]]]]  ]]]]    ]]]]  ]]]] ]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]]]       Development System

]]]]]]]]]]]]]]]]]]]]]]]]]]]]

]]]]]]]]]]]]]]]]]]]]]]]]]]]       VxWorks version 5.4

]]]]]]]]]]]]]]]]]]]]]]]]]]       KERNEL: WIND version 2.5

]]]]]]]]]]]]]]]]]]]]]]]]]       Copyright Wind River Systems, Inc., 1984-1999

CPU:  IXP1200 .  Processor #0.

Memory Size: 0x3b00000.  BSP version ITGL3.10.81sa

.

/C:/  - Volume is OK

Unable to open startup script /C:/etc/startup

Retrieved old volume params with %35 confidence:

Volume Parameters: FAT type: FAT32, se

/C:/  - disk check in progress ...

/C:/  - Volume is OK

total # of clusters:  3,906

# of free clusters:  264

# of bad clusters:  0

total free space:  1,056 Kb

max contiguous free space:  1,077,248 bytes

# of files:  21

# of folders:  10

total bytes in files:  14,442 Kb

# of lost chains:  0

total bytes in lost chains:  0

Console logging enabled

JAN 01 12:26:15 SYS: Info IPL 3.10.81 - Saturday September 27 16:09:14 EDT 2003

JAN 01 12:26:15 SYS: Alert Network not configured, aborting startup

Login: itgadmin            (You can now use the old default passwords!)

Password:(itgadmin)

Welcome to the IP Line command line.

Software Version: IPL-3.10.81

Management IP:

Host Type: Succession Media Card

Firmware Version: Firmware Version Not Available

SysName:

SysLocation:

SysContact:

OS Time:  Date (01/01/2000) Time (12:27:17)

Use "logout" to logout.

Idle session timeout = 20 minutes.