DDSL Monitor.doc

How to provide DDSL monitoring on a Option 11 System       Version 18 40H

>LOGI

>LD 22

REQ  ISS

VERSION 1411

RELEASE 18

ISSUE 40 + H

>$$

PSWD?  (PWD2 Password!)

DBG000

#V                                         (  V =Prints out useful monitoring address’s)

TRUNK_MONITOR @ 1EA4 = 0000

TRUNK_TN @ 1EA5 = 0000 0000 0000 0000 0000 0000 0000

DTSL_MONITOR @ 23E9 = 0000          (23E9 = Address to turn on the DDSL Monitor)

DTSL_MON_WORDS @ 23EA = 0000 0000 0000 0000 0000 0000 0000 0000 0000 0000

#W 0 23E9

00 23E9 : 0000 /E0               (E0 = Monitor ON & 00 = Turn Monitor OFF)

00 23EA : 0000 /0F               (0F = DDSL 15 to be Monitored)

(0E = DDSL 14 to be Monitored)

(0D = DDSL 13 to be Monitored)

(0C = DDSL 12 to be Monitored)

(0B = DDSL 11 to be Monitored)

(0A = DDSL 10 to be Monitored)

#V

TRUNK_MONITOR @ 1EA4 = 0000

TRUNK_TN @ 1EA5 = 0000 0000 0000 0000 0000 0000 0000

DTSL_MONITOR @ 23E9 = 00E0          (Monitor is now active on DDSL 15)

DTSL_MON_WORDS @ 23EA = 000F 0000 0000 0000 0000 0000 0000 0000 0000 0000

#

INCOMING CALL MADE VIA A PBX LINE GROUP WITH CLID NUMBER LISTED

DAI R 2 2  0  CS

DAI R 2 2  8  ICI-C 00 *86*01482210001#      (INCOMING CALL INDICATOR COMPLETED)

DAO R 2 2  9  CAM                                         (CALL ACCEPTED MESSAGE)

DAI R 2 2  9  CA                                              (CALL ACCEPTED)

DAO R 2 2  5  CCM                                          (CALL CONNECTED MESSAGE)

DAO R 2 2  4  CRM 30                                     (CALL REQUESTED MESSAGE)

DAI R 2 2  4  CIM 30 *AA*CLEARED#          (CALL CLEARED)

INCOMING CALL MADE VIA A DDI LINE GROUP NO CLID NUMBER LISTED

DAI R 2 18  0  CS

DAI R 2 18  8  ICI-C 00 005

DAO R 2 18  9  CAM *86*6005#                     (CLID NOT WORKING TO EXTN 6005)

DAI R 2 18  9  CA

DAO R 2 18  5  CCM

DAO R 2 18  4  CRM 30

DAI R 2 18  4  CIM 30 *AA*CLEARED#

NOTE!

DAI  R 2 2 0 CS  =  CHANNEL 2 ON CARD SLOT 2 IS IN USE

*86*01482210001#  =  INCOMING CALLING LINE ID OF NUMBER MADE TO MERIDIAN

CAM *86*6005#  =  INCOMING CALLINE LINE ID TO EXTN 6005 IS NOT LISTED

How to provide DDSL Monitoring on Nortel Meridian / Succession systems Version V24.xx onwards via LD 75

Note!

You must have "BUG" enabled against the TTY port that you are using, Otherwise you won't get any output monitoring messages!.

>LD 75

DTM000

.STAT DDSL                                 (Check first which DASS / DPNSS circuits require monitoring)

DDSL 015 ENBL ACTIVE

.ENL MON 15                               (Enter "ENL MON XX" XX = DDSL number to be monitored)

OK

.ENL MSGI                           (This command will now monitor all incoming  DASS / DPNSS calls)

OK

.

DAI R 5 2  0  CS

DAI R 5 2  8  ICI-C 00 *86*01482210000#6752

DAO R 5 2  9  CAM *86*6752#

DAI R 5 2  9  CA

DAI R 5 2  5  CIM 30 *AA*CLEARED#

DAO R 5 2  4  CCF 30 *AA*CLEARED#

DTM000

.DIS MSGI      (This command will now stop all incoming DASS / DPNSS calls from being monitored)

OK

.

DAO R 5 2  9  CAM *86*6752#

DAO R 5 2  4  CCF 30 *AA*CLEARED#

DAO R 5 2  9  CAM *86*6753#

DAO R 5 2  5  CCM *86*6753#

DAO R 5 2  4  CCF 30 *AA*CLEARED#

DTM000

DAO R 5 1  4  CCF 30 *AA*CLEARED#

DTM000

.ENL MSGO                      (This command will now monitor all outgoing DASS / DPNSS calls)

OK

.

DAO R 5 15  1  ISRM-I 10 *86*6709#0800800154

DAO R 5 14  1  ISRM-I 10 *86*6740#01912379537

DAO R 5 15  4  CRM 30

DTM000

.DIS MSGO    (This command will now stop all outgoing DASS / DPNSS calls from being monitored)

OK

.

.DIS MON 15         (Enter "DIS MON XX" XX = DDSL number to be removed from monitoring)

OK

.****

>