PRINTOUT OF BRIGHTON CONFIG CTR CALLING LINE ID FOR EURO ISDN 30E      23/5/2001

THIS IS AN EXAMPLE TAKEN FROM THE BRIGHTON CONFIG CENTRE OPTION 11C SYSTEM

& IT IS USED TO SHOW HOW TO SET UP A CALLING LINE I.D. ON OUT GOING CALLS.

NOTE!

PLEASE CHECK FIRST IF THE CUSTOMER IS USING A CARRIER FOR THEIR OUTGOING CALLS AS ONLY FIRST NUMBER IN THE DDI RANGE COULD BE SENT OUT TO LINE.

>LD 21

REQ: LTM

CUST 0

TYPE TLST

TKTP DID

ROUT 0

DES  01273546700

TN  001 01 MBER  1  01273546700-DDI     TN  001 02 MBER  2  01273546700-DDI

TO

TN  001 29 MBER  29  01273546700-DDI    TN  001 30 MBER  30  01273546700-DDI

>LD 20

REQ: PRT

DES  01273546700-DDI

TN   001 01

TYPE DID

CDEN SD

CUST 0

TRK  PRI2

PDCA 2

PCML A

NCOS 0

RTMB 0 1

B-CHANNEL SIGNALING

NITE

AST  NO

IAPG 0

CLS  UNR DTN CND WTA LPR APN THFD BARD

P10 VNL

TKID

DTCR NO

DATE 26 AUG 2000

DES  01273546700-DDI

TN   001 30

TYPE DID

CDEN SD

CUST 0

TRK  PRI2

PDCA 2

PCML A

NCOS 0

RTMB 0 30

B-CHANNEL SIGNALING

NITE

AST  NO

IAPG 0

CLS  UNR DTN CND WTA LPR APN THFD BARD

P10 VNL

TKID

DTCR NO

DATE 26 AUG 2000

>LD 22

REQ  PRT

TYPE ADAN DCH

ADAN     DCH 15

CTYP MSDL

CARD 01

PORT 1

DES  EURO_ISDN

USR  PRI

DCHL 1

OTBF 32

PARM RS422  DTE

DRAT 64KC

CLOK EXT

IFC  E403

CNTY ETSI

PINX_CUST 0

ISDN_MCNT 300

CLID OPT1

PROG NCHG

CO_TYPE  STD

SIDE USR

CNEG 1

RLS  ID  **

RCAP COLP

MBGA NO

OVLR NO

OVLS NO

T310 120

INC_T306 120

OUT_T306 120

T200 3

T203 10

N200 3

N201 260

K    7

>LD 22

REQ  ISS

VERSION 2111

RELEASE 25

ISSUE 15

>LD 49

REQ  PRT

TYPE IDC

CUST 0

DCNO

DCNO   1

SDID YES

IDGT    CDGT

546700    6700

546701    6701

TO

546798    6798

546799    6799

>LD 21

REQ: PRT

TYPE: CLID

CUST 0

SIZE 799

RNGE

INTL

MOST OF THE EXTNS WILL HAVE USED "0" AFTER THE DN NUMBER

THIS RELATES TO CLID ENTRY TABLE "0"

FOR THE CALLING LINE I.D. TO WORK ON OUTGOING CALLS

THE PROMPT "DIDN" MUST BE SET TO "YES"

THE FIRST TWO DIGITS OF THE LOCAL DDI NUMBER

MIGHT NEED TO IN INSERTED UNDER THE PROMPT "HLCL"

FOR AN EXAMPLE PLEASE REFER TO TABLE "0"

BRIGHTON CONFIG CENTRE'S DDI RANGE IS "5467XX"

IF THIS DOESN'T WORK THEN MAKE SURE THE NOU

HAS SET THE PROMPT "COLP" & “CLIP”

ENTRY 0

HNTN

HLCL 54

DIDN YES      (MUST BE SET TO YES FOR DDI ON OUTGOING CALLS TO WORK)

HLOC

LSC

ENTRY 1

HNTN

HLCL 546767   (THIS EXAMPLE SHOWS THE MAIN DIRECTORY NUMBER ON OUTGOING CALLS)

DIDN NO             I.E. 01273 546767 CLID IS SENT OUT TO LINE.

HLOC

LSC

>LD 21

REQ: PRT

TYPE: LDN

TYPE LDN_DATA

CUST 0

TYPE LDN_DATA

CUST 00

OPT XLDN

DLDN YES

LDN0 549999        NOTE!

LDA0 01

LDN1           LDN0 (549999) MUST BE SET TO 6 DIGITS IN LENGTH &

LDA1              IT CAN BE ANY NUMBER, BUT IT IS USUALLY THE

LDN2                     FIRST NUMBER IN THE DDI RANGE

LDA2                          FOR THE CONSOLE

LDN3

LDA3

LDN4

LDA4

LDN5

LDA5

LDBZ

>LD 21

REQ: PRT

TYPE: NET

TYPE NET_DATA

CUST 0

TYPE NET_DATA

CUST 00

OPT RTD

AC2 LOC

FNP YES

ISDN YES

PNI 1

PINX_DN

MBG 0

BSGC 65535

PFX1

PFX2

HLOC

LSC 520

RCNT 5

PSTN NO

TNDM 15

PCMC 15

SATD 1

OCLI NO

TIDM NO

DASC

ROPT NRO

DITI YES

TRNX NO

EXTT NO

FTOP FRES

APAD 0  0

VNR NO

NIT 8

NAS ATCL YES

NAS ACTV NO

FOPT 0

CNDN

CNAT

CNIP YES

DMWM NO

MWNS NO

NOTE!

TO MAKE CHANGES OR PROVIDE NEW ENTRIES TO “CLID” TABLES IN LD 15, YOU NEED TO TYPE …

REQ: CHG

TYPE: NET

TYPE NET_DATA

CUST 0

OPT

AC2

FNP

CLID “YES”

(UNDER ENTRY KEY IN EXISTING OR NEW TABLE NUMBER)

>LD 21

REQ: PRT

TYPE: RDB

CUST 0

ROUT 0

TYPE RDB

CUST 00

DMOD

ROUT 0

DES  01273 546700

TKTP DID

NPID_TBL_NUM   0

SAT  NO

RCLS EXT

DTRK YES

BRIP NO

DGTP PRI2

ISDN YES

MODE PRA

IFC  E403

CNTY ETSI

SBN  NO

PNI  00000

NCNA NO

NCRD NO

ISAR NO

CPFXS YES

DAPC YES

TBL 1

INTC NO

DSEL VOD

PTYP DCO

AUTO NO

DNIS YES

NDGT 6

WDGT L

DCDR NO

ICOG IAO

RANX NO

SRCH LIN

TRMB YES

STEP

ACOD 10900

TCPP NO

TARG 01

BILN NO

OABS

INST

IDC  YES

DCNO 1

NDNO 1 *

DNAM YES

MFC  NO

ICIS YES

OGIS YES

TIMR ICF 256

OGF 512

EOD 13952

LCT 256

NRD 10112

DDL 70

ODT 4096

RGV 640

FLH 510

GTO 896

GTI 0

SFB 3

NBS 2048

NBL 4096

IENB 5

TFD 0

VSS 0

VGD 6

DTD  NO

SCDT NO

2 DT NO

DDO  NO

DRNG YES

NDRI 01

CDR  YES

INC  YES

LAST YES

TTA  YES

ABAN YES

CDRB YES

QREC NO

OAL  YES

AIA  YES

OAN  YES

OPD  YES

NDP  EXC 0

CDRX NO

NATL YES

SSL

CFWR NO

IDOP NO

VRAT NO

MUS  YES

MRT  91

MR   NO

PANS YES

EQAR NO

FRL  0 0          FRL  1 1              FRL  2 2          FRL  3 3

FRL  4 4          FRL  5 5              FRL  6 6          FRL  7 7

OHQ  NO

OHQT 00

TTBL 0

PLEV 0

OPR  NO

PRDL YES

EOS  NO

DNSZ 0

RCAL NO

MCTS NO

ALRM NO

BTT  30

ACKW NO

ART  0

PECL NO

DCTI 0

TIDY 5201090 0    SGRP 0     AACR NO