Instructions on how to format a pcmcia card from within a Option 11c

TTY 00 SCH MTC   23:54

OVL111 IDLE   0

PDT:  login on /sio/0

Password:

PDT in Progress. Please Wait....

Done!

pdt> cd a:     (You can use either the A or B card slots on the 11c!)

pdt> ll

Directory of 'a:':

SIZE          DATE       TIME         NAME

----------   -----------  --------   ------------

512   Jul-12-2004  12:48:36   BACKUP                    <DIR>

512   Jul-12-2004  18:23:10   ARCH_DB                   <DIR>

512   Jul-12-2004  12:16:46   BOOTROM                   <DIR>

512   Jul-12-2004  12:16:48   DFLT_DB                   <DIR>

1479908   Sep-25-2003  18:08:20   DRAMOS

204010   Sep-25-2003  18:08:22   DRAMOS.SYM

661   Sep-25-2003  18:08:06   DRAMOSCC.SYM

512   Jul-12-2004  12:17:08   P                         <DIR>

892   Oct-01-2003  10:40:02   README.TXT

512   Jul-12-2004  12:18:56   U                         <DIR>

pdt> pcmciaformat a:

PCMCIA 'a:' will be formatted (y/n) ?y   (It only takes a couple of seconds!)

Formatting PCMCIA 'a:'

Format complete

pdt> ll

Directory of 'a:':      (The pcmcia card is now formatted & blank!)

SIZE          DATE       TIME         NAME

----------   -----------  --------   ------------

pdt> chkdsk

Volume in drive 'a:' has no label

47908864 bytes free on 'a:'

pdt> cd c:

pdt> exit

PDT in Progress. Please Wait....

Done!

TTY 00 SCH MTC   23:56

OVL111 IDLE   0

How to repair a PCMCIA / Compact flash card

that won't work in Windows 95/98 or on some Nortel applications

Most PCMCIA or Compact flash size cards can be reused. If by any chance it was formatted by accident with Windows 2000 or XP etc, it's possible to recover the card back into full working order by putting it into a digital camera that accepts that type of card & using the camera's own format facility.

I had a Canon Power shot A70 digital camera, which I found worked OK for me. I was then able to reuse my 64 Meg Sandisk CF card again.