How to change an Option 11c TID

This example is from a 2121 / V04.00t system, & it’s only intended as a temporary workaround to get a Meridian Mail system to work in an emergency, ie...

Whilst you are waiting for a new set of software keycodes, due to some type of outage etc. Where you have used a temporary set of software that does not actually belong to the site, the system will not then be able to do a data dump. The “EDD” data dump can only be done when this setting has been modified, either from within “pdt” or via a new set of ISM keycodes in LD 143.

>LD 22

REQ  TID

# SECURITY ID:81924637

AUX ID     :138426

REQ          (Go into “pdt” by pressing the “Ctrl” key & “pdt” letters together!)

PDT:  login on /sdi/tty2               (Access “pdt” using the required password!)

Password:

PDT in Progress. Please Wait....

Done!

pdt> p 9815 5  (cr)   (Prints out the current SECURITY ID:81924637!)

2nd 1st        4th 3rd        6th 5th         8th 7th

00009815 : 0000B1B8 0000B2B9 0000B6B4 0000B7B3 00000000

pdt> p 97bf 4  (cr)   (Prints out the current AUX ID:138426 which will be changed!)

2nd 1st        4th 3rd        6th 5th       8th 7th

000097BF : 0000B3B1 0000B4B8 0000B6B2 00000000

pdt> w 97bf  (cr)

# 000097BF : 0000B3B1 /b0b1   (space)   (This would change the Aux ID

# 000097C0 : 0000B4B8 /b3b2   (space)                                     to 10 23 36 29)

# 000097C1 : 0000B6B2 /b6b3   (space)

000097C2 : 00000000 /b9b2    (space)

# 000097C3 : 00000000 /            (cr)

pdt> sl1input  (cr)  (Or use “exit” to leave “pdt”!)

tid

SECURITY ID:81924637

AUX ID     :10233629                    (AUX ID has now been changed to 10233629!)

REQ

## Other Option 11c pdt pointer Hex address values

### V22.46     Security ID = 9          Aux ID = 9

### V23.47     Security ID = 9854    Aux ID = 981f

### V24.25     Security ID = 986d    Aux ID = 9831

### V25.15     Security ID = 98ec    Aux ID = 98a0

### V25.30     Security ID = 99a4    Aux ID = 99f0

### V25.40     Security ID = 99f2     Aux ID = 99a4

### V03.00     Security ID = 9811    Aux ID = 97bf

### V04.00t    Security ID = 9815    Aux ID = 97bf

### V04.50w  Security ID = 9815    Aux ID = 97bb