Workaround for Option 81C CP PII redundant system only

Loadware patches have to be installed on both cores for CP PII systems only

First install any X11 patch if applicable

Redundant systems must be in a split mode to install LW patch in both sides

SPLIT

Wait for the standby core to finish reloading

On Inactive side:

Copy Loadware files into C:/U/LOADWARE

LWLOAD

LWINST

<INI>

CUTOVR

On the other side which is now the inactive side:

Copy Loadware files into C:/U/LOADWARE

LWLOAD

LWINST

<INI>

JOIN

This synchronizes the memories and hard disks, wait for the disk synchronization to complete

Loadware installation is complete.