- Looking for a BCM 50 r3 memory dump

thread1361-1742341

| Zman37 (IS/IT--Management) (OP) | 21 Dec 14 2:14 |
| --- | --- |

I have successfully reverse engineered the algorithm for the password of the day in the BCM 50 r3, but I am missing a "shared sec", (If the BCM can't find it, it throws the error "shared sec not provisioned"), needed in the calculation. The algorithm basically works by taking the sha1 hash of "nnsupport" + the "shared sec" + the date rc4'd with a key + the system id. Then it takes the sha1 hash and uses the bytes from it to select numbers from an array which then make up the password of the day. The shared sec is stored somewhere in an "authdb", but I have been unable to find any files that resemble that, and I think I can locate the shared sec if I have a memory dump. I have a BCM 50 on order but it won't be here for a few weeks, so I'm wondering if in the mean time anyone has a memory dump they could share. And of course, there would be passwords of the day for everyone! Thanks, Derek Cyberprog (TechnicalUser); 21 Dec 14 17:12. Very interesting. Yes, it's likely to be stored in a database somewhere, probably along with a load of other bits & bobs which could pose interesting if found. I've been playing round trying to change the system ID but haven't gotten anywhere! Where are you located Derek? I may be able to assist with getting you a BCM 50 quicker if you wanted to play with. I have a number in my stock, including one that no longer exists (it's licenses were transferred to another system ID and I then managed to bring it back to life) if you would like one to play with. I'm in the UK (South West) FWIW. Alex Threlfall Cyberprog New Media http://www.cyberprog.net Telecoms, Networks, Hosting, Alarms, CCTV etc.. Zman37 (IS/IT--Management) (OP); 21 Dec 14 20:57. David, I sent you an email. I located a "pdr" database last night, but I cannot figure out what format it is in. From browsing around in a hex editor, it seems to contain the configuration for the BCM and looks like it may have the data I'm looking for. I'm going to keep poking around and see what I can find. The system ID is stored somewhere in a "secure sector" on the BCM, which I believe is a secureflash chip. I haven't figured out if there is a way to change it on there yet, but an easy way to change would be to patch the function in the operating system that retrieves it (osGetSystemId in nn\lib\libosa.lib). Once I get my BCM, I'm going to get that one figured out. I'm located in the Washington in the U.S. so I'm pretty far away. I've also started looking into figuring out how the keycodes work. From what I've gathered, the BCM 50 r3 uses an HMAC scheme with a symmetric key. (Here's the gist of how an hmac system works http://www.drdobbs.com/licensing-using-symmetric-a...). The symmetric key means that a license generator is possible without any modification to the BCM. The BCM 50 r6 though uses an asymmetric key, which means that Nortel/Avaya holds a private key, and the BCM 50 has the public key on it. To make a license generator, either someone would have to break into Avaya and steal the private key, or a new public/private key pair would have to be generated, and the public key on the BCM 50 replaced. With the POTD, that should be a fairly straightforward process, so in the near future we may have the ability to generate our own keycodes!.... Cyberprog (TechnicalUser); 22 Dec 14 11:14. There are ways into the file system of the BCM 50 without using SSH BTW. I have telnet enabled on one of mine, as does David. It's reasonably easy to setup and remove the password of the day stuff - but that said, the POTD would be good to break as it would enable access to systems without the need to pluck the hard disk and modify files. There are far more R1 to R3 systems out there than R5 or R6, I have a dozen of systems and only one is R5. There were many changes in R6, licensing must be one of them! Shame you're in the USA, or I'd overnight you a BCM 50 to play with! :) Alex Threlfall Cyberprog New Media http://www.cyberprog.net Telecoms, Networks, Hosting, Alarms, CCTV etc. (2) Zman37 (IS/IT--Management) (OP); 24 Dec 14 3:13. Alright, I figured it out! The "shared sec" turned out to be the Nortel Support Challenge key. I tested this algorithm on a BCM 50 R3 and R6 and it works on both of them, so I believe it is the one for all BCM's. -------------------------------------------------------------- nnsupport Password of the day (POTD) algorithm for the bcm50 Three components are needed for the password of the day calculation: -The date in the form dd/mm/yyyy -The system ID -Nortel Support Challenge key (default is "trust no one", can be found in the element manager under Configuration->Administrator Access->Security Policies) 1. Enter the date The date is entered into the string "W -- POD dd/mm/yyyy --- BCM -- C" 2. Generate an SHA-1 hash of the components SHA-1("nnsupport" + challenge key + date string + system id) 3. Take every other byte of the first 16 bytes of the SHA-1 hash so you have 8 bytes total 4. Retrieve the 10's place from each byte (Ex. 6D would get 6) 5. Use as index to retrieve value from the following array: "01234567890876540" (6th index would be 6) 6. Take value and multiply by 10 (6*10 = 60) 7. Retrieve 1's place from each byte (Ex. 6D would get D) 8. Use as index to retrieve value from the same array "01234567890876540" (Dth (13th) index would be 6) 9. Add the two numbers together (60 + 6 = 66) 10. Append to password of the day output string. If number is less than 10, prepend a zero. (Ex if the number was 7, print 07) After 8 iterations, one for each byte, you end up with a 16 digit password of the day. ------------------------------------------------------------------------------------------- Complete example Date: 15/3/2013 System ID: 001122334455 Challenge key: trust no one 1. Enter date "W -- POD 15/03/2013 --- BCM -- C" 2. SHA-1 Hash SHA-1("nnsupporttrust no oneW -- POD 15/03/2013 --- BCM -- C001122334455") SHA-1 Hash 32db0ef4561237cb289587df81b3766fccdb86de 3. Select every other of the first 16 bytes 32 0e 56 37 28 87 81 76 4. Get 10's place 3 0 5 3 2 8 8 7 5. Select from array Array:01234567890876540 Index:0123456789abcdef 3 0 5 3 2 8 8 7 6. Multiply by 10 30 0 50 30 20 80 80 70 7. Get 1's place 2 e 6 7 8 7 1 6 8. Select from array Array:01234567890876540 Index:0123456789abcdef 2 5 6 7 8 7 1 6 9. Add the two numbers 30 00 50 30 20 80 80 70 2 5 6 7 8 7 1 6 ---------------------+ 32 05 56 37 28 87 81 76 10. Combine prepending 0 to all single digit numbers 32 05 56 37 28 87 81 76 3205563728878176 Congratulations, you now have the password of the day! (POTD). (2) Cyberprog (TechnicalUser); 24 Dec 14 11:25. And this little web page, thanks to the info above, will generate a password. Seems to work ok for me and my systems. http://www.cyberprog.net/bcm/potd.asp?do=pass Alex Threlfall Cyberprog New Media http://www.cyberprog.net Telecoms, Networks, Hosting, Alarms, CCTV etc....