Configuration required for Symposium Call Centre to connect to a Meridian

## SOFTWARE LEVEL

M1 must be at Release 23 or later.

New Packages required for Symposium Call Centre are 	311 – NGCC

324 – NGCCELAN

I/P NETWORK INTERFACE CONFIGURATION

Configure IP addresses as shown below

LD 117

=>NEW HOST MERIDIAN 220.0.0.100

=>CHG ELNK ACTIVE MERIDIAN

=>CHG MASK 255.255.255.0

=>UPDATE DBS

To activate these modifications the Switch must be initialised

CONFIGURING THE ETHERNET AML

Define the ELAN

LD 17

REQ	CHG

TYPE	ADAN

ADAN	NEW ELAN 17	0-15 are reserved for AML Link

16-31 are reserved for ELAN Link

CTYP	ELAN

DES	NSCC1

LCTL	(NO)

_N1	<128,(512)>

Define VSID

LD 17

REQ	CHG

TYPE	VAS

VAS	NEW

VSID	17			The VSID must be between 16-31. Preferable

To have VSID and ELAN the same number.

ELAN	17

SECU

INTL

MCNT

ELAN Maintenance

LD 48

.STAT ELAN 17

reply

SERVER TASK : DISABLED or ENABLED

ELAN 17 DES :NSCC1

APPL_IP_ID : application IP address LYR7 : DOWN DISABLED or

ACTIVE EMPTY APPL ACTIVE

cc to TAUNTON and BRIGHTON Configuration Centres – Issue Date 16/3/99