MERIDIAN 1

OPTION 51-81

CONFIGURATION

GUIDE

#### AIDE MEMOIRE

##### Author

Terry Stainton has compiled this document.

The author thanks Alan Taylor and Trevor Battle for their major contributions and configuration team members past and present for their input.

HISTORY

| Issue | Date | Author | Reason |
| --- | --- | --- | --- |
| Issue 2.0 Issue 3.0 . | 14 Oct. 03 16 Jan. 04 | Terry Stainton Terry Stainton | Add issue history to documentation Incorporate changes relating to Impact Release 23.1. Incorporate changes relating to Merdian Succession Software Release 3.00 And Impact Release 23.2 |

APPENDIXES

## APPENDIX A	DEFAULT VALUES OF SWITCH

## SETTINGS REQUIRED TO BE ENTERED ONTO THE SWITCH TO	BRING IT UP TO UK STANDARDS.

## APPENDIX B	ROUTES AND TRUNKS

SETTINGS REQUIRED FOR THE MOST COMMONLY USED UK ROUTE AND TRUNK TYPES.

## APPENDIX B1	INTERNET APPLICATIONS

SETTINGS REQUIRED FOR IP APPLICATIONS ON SWITCH.

## APPENDIX C	APPLICATIONS

SETTINGS REQUIRED FOR THE MOST COMMONLY USED UK APPLICATIONS.

## APPENDIX D	ADDITIONAL INFORMATION

INFORMATION ABOUT ADDRESSING, MACHINE TYPES, LOOP–GROUP–SHELF RELATIONSHIPS.

## APPENDIX D1	ADDITIONAL INFORMATION

INFORMATION ABOUT 39xx & I2xxx TELEPHONE SETS.

## APPENDIX E	ADDITIONAL INFORMATION NON NEW SWITCHES

## HELPFUL TIPS WHEN CONFIGURING SECOND HAND, MAINTENANCE REPLACEMENTS AND OPTION 11 UPGRADES.

## CFN 177E	CONFIG OF MERIDIAN 1 SYSTEMS

## CUSTOMER FIELD NOTE 177E ISSUE 6.

STEPS REQUIRED TO COMPLETE AN

## OPTION 51 - 81 CONFIGURATION

## CONFIGURATION

- PC:		CREATE DIRECTORY FOR JOB. (I.E. D:\PCPLUS\10xxxx)

CREATE NEW IMPACT JOB.

- COPY EXCEL CRF FROM T:\MERIDIAN JOBS TO                        (I.E. D:\PCPLUS\10xxxx)

- CREATE A COPY OF SOFTWARE DATABASE TO FLOOPY DISK.

- (TDD & KEYCODES TO BE HELD ON FILE)

- SWITCH:	LOAD SOFTWARE DATABASE DISK SET 2 ONTO YOUR

CAPTIVE SWITCH.

DO BACKUPS DURING PROCESS.

- (LD43 EDD)

- SWITCH: / PC:	(APPENDIX E):

ADDITIONAL INFORMATION REQUIRED FOR THE					CONFIGURATION OF SECONDHAND SWITCHES AND

- JOBS REQUIRING SWITCH DATABASES TO BE COPIED ONTO

- A CLEAN DATABASE (I.E. CLEARING CORRUPTED DATABASES).

PRINT OUT TEST DATA:

- CFN  (LD 22 – SLT  & PRT CFN) (LD 97 – PRT SUPL & XPE)

- CDB (LD 21 – PRT CDB)

- IMPACT:	RUN GET TNs.

CHECK TNs MATCH CARD MAP.

RUN OUT TNs.

- PAPER:	(APPENDIX D):

- ADDITIONAL INFORMATION.

- ASSIGN D/CHANNELS AND SDI PORTS ON PAPER IF REQUIRED.

- SWITCH:	(APPENDIX A):

CHECK THAT DNs AND TNs HAVE BEEN DELETED.

- ONLY XTDs SHOULD REMAIN.

- XCT CHANGE LOCATION IF REQUIRED.

- ENTER DAYLIGHT SAVINGS SETTINGS.

- SWITCH:	(APPENDIX A):

- TTY 0 & TTY 1:	CHANGE FROM 2 PORT TO 4 PORT                  PADDLEBOARD IF REQUIRED.

- MTCE TTY:	DELETE ALL ENTRIES IN USER EXCEPT SCH &

- MTC.

- TTY 1:		IF CALL LOGGER CONNECTED DELETE

- ALL ENTRIES IN USER EXCEPT CTY.

- OTM TTYs:	ADD TWO NEW TTYs FOR OTM APPLICATION.

- HST:		CHANGE TO 20000.

- OVLY:		BKGD & DROL DELETE ALL ENTRIES.

- DEFAULTS:     CHECK DEFAULT CONFIGURATION.

- CAB MAP:	IF IPE WIRED OUT FOR 24 PORTS

- MOVE MIRAN CARD INTO SLOTS 0, 4, 8 OR 12.

- MOVE MGATE CARD INTO SLOTS 0, 4, 8 OR 12 OF A BLOCKING SEGMENT.

- IF IPE WIRED OUT FOR 32 PORTS

- MOVE MGATE CARD INTO A BLOCKING SEGMENT.

- SWITCH:	(LD 15 FFC):	CONFIGURE STATION CONTROL PASSWORD

LENGTH TO MATCH DN LENGTH IF REQUIRED.

(LD15 ATT):	CHANGE ATTENDANT DN IF NOT 0.

(LD 15 FTR):	CONFIGURE SPECIAL PREFIX NUMBER IF REQUIRED.

(APPENDIX B):

CONFIGURE FIXED ROUTES & TRUNKS MUSIC ROUTE FIRST		IF REQUIRED.

CONFIGURE BASIC RATE INTERFACE IF REQUIRED.

(APPENDIX B1):

CONFIGURE MERIDIAN IP MEDIA IF REQUIRED.

- CONFIGURE SUPERLOOP FOR VIRTUAL I2xxx AND PCA SET TNs AND VOICE OVER GATEWAY MEDIA CARD LINE SIDE IF REQUIRED.

- CONFIGURE IP ROUTES IF REQUIRED.

- CONFIGURE CALL PILOT IF REQUIRED.

- CONFIGURE SYMPOSIUM IF REQUIRD.

- (LD 50):	CONFIGURE CALL PARK IF REQUIRED.

- (LD 56):	CONFIGURE TBARs & RARTs IF REQUIRED.

(APPENDIX C):

CONFIGURE MAIL IF REQUIRED.

CONFIGURE ACD C2, SENIOR SUPERVISOR, ACD PRINTER,

- ACD SUPERVISOR & OR WALLBOARD FOR ACD C2 IF REQUIRED.

CONFIGURE MAX IPE, HIGH SPEED LINK AND LOAD

- MANAGEMENT IF REQUIRED.

- CONFIGURE CCR IF REQUIRED.

- CONFIGURE LINK IF REQUIRED.

CONFIGURE SUPERLOOPS FOR 500 SET PHANTOM TNs IF REQUIRED.

CREATE SUPERLOOP FOR 39xxV SET TNs IF REQUIRED.

- 	IMPACT: / PC:	EDIT CRFs AND IMPORT TO IMPACT.

- 	IMPACT:

- ACD:		RESERVE TNs FOR ACD SETS IF REQUIRED.

- AVOID USING CARD SLOTS 0 & 1 IF POSSIBLE					AND USE NON-BLOCKING LOOP IF AVAILABLE.

- TYPE = DIG AND RESERVED FOR = ACD.

- (TEST DATA / CRF)

- CONSOLE:	RESERVE 4 TNs PER CONSOLE IF REQUIRED.

- USUALLY PORT 12-15.

- DON’T USE CARD SLOTS 0 & 1.

- IF BUSY LAMP FIELD ARRAY IS FITTED

- ADD ONE POWER TN PER ARRAY ON SWITCH.

- TYPE = DIG. RESERVED FOR = CONSOLE.

- MAKE TNs NOT TO BE ALLOCATED ON IMPACT.

- (TEST DATA / CRF)

- PHANTOM:	PROVIDE AND RESERVE TNs AS TYPE = 500 AND RESERVED FOR = PHANTOM IF  REQUIRED.

- (APPENDIX C)

- COMPANION:	PROVIDE AND RESERVE TNs AS TYPE = 500 AND RESERVED FOR = COMPANION OR COMPANION32 IF REQUIRED.

- (TEST DATA / CRF)

- DECT:	PROVIDE AND RESERVE TNs AS TYPE = 500 AND RESERVED FOR = DECT OR DECT32 IF REQUIRED.

- (TEST DATA / CRF)

### CALL PILOT:	PROVIDE AND RESERVE CALL PILOT TNs AS     TYPE = DIG AND RESERVED FOR = PILOT IF REQUIRED.

- (TEST DATA / APPENDIX B1)

SYMPOSIUM:	MODIFY IMPACT IF REQUIRED.

(TEST DATA / CRF / APPENDIX B1)

SET I2xxx:	PROVIDE AND RESERVE TNs FOR IP SETS.

IF REQUIRED.

(TEST DATA / CRF / APPENDIX B1)

### MAIL:	PROVIDE AND RESERVE MAIL TNs IF REQUIRED.

- IF ONLY ONE MAIL LOOP PROVIDED CHANGE

- (ON BOTH IMPACT AND ON THE SWITCH)

- FROM AN EVEN LOOP TO AN ODD LOOP.

- (TEST DATA / CRF / APPENDIX C)

- SET 39xxV:	PROVIDE AND RESERVE TNs AS TYPE = DIG AND RESERVED FOR = PHANTOM.

- IF REQUIRED.

- (TEST DATA / CRF / APPENDIX C)

- PHANTOM:	PROVIDE AND RESERVE TNs AS TYPE = 500 AND RESERVED FOR = PHANTOM IF  REQUIRED.

- (APPENDIX C)

- MAKE FCR TABLE 0 ALOW.

- DO NOT USE RLI 0 IF POSSIBLE.

RUN VET.

RUN TIDY UP.

CREATE RUNFILES.

MODIFY RUNFILES AS REQUIRED.

MODIFY RUNFILES IF REQUIRED FOR CALL PILOT.

- (APPENDIX B1)

MODIFY RUNFILES IF REQUIRED FOR SYMPOSIUM.

- (APPENDIX B1)

. 	MODIFY RUNFILES IF REQUIRED FOR MAIL.

- (APPENDIX C)

RUN DOWNLOAD.

CHECK AND CORRECT ANY ERRORS.

CHECK CRF FOR ANY OUTSTANDING INPUT.

- SWITCH / PC:

PRODUCE JUMPERING SCHEDULE IF REQUIRED.

PRODUCE DIGITAL SET LABELS IF REQUIRED.

CHANGE PASSWORDS AND OVERLAY ACCESS.

- (RUN PASSWORD)

- SWITCH:	(APPENDIX A)

RECONFIGURE MTCE TTY AND OVLY BKGD & DROL.

- SWITCH/PC:	COMPILE DOCUMENTATION USING DOCGEN.

- SOFT COPY SECTIONS 1 TO 20.

- PC:		RUN FORM1, CREATES CRF AMENDMENT LOG,

- QUALITY ASSURANCE FORM, PRODUCT QUESTIONNAIRE

- AND RACE CONNECTION FORM.

- SWITCH:	RUN BACKUP. (LD43 EDD) ON SOFTWARE DATABASE

- DISK SET 2.

RUN BACKUP (LD43 EDD) ON SOFTWARE DATABASE

- DISK SET 1.

RE-LOAD DATABASE DISK SET 1.

RE-LOAD DATABASE DISK SET 2.

- PC:		WRITE PROTECT ALL DISKS.

- E-mail:		OR POST TO DELIVERY ADDRESS:

- DISK SET 2

- JUMPERING SCHEDULE

- DIGITAL SET LABELS

- CRF AMENDMENT LOG

- QUALITY ASSURANCE FORM

- PRODUCT QUESTIONNAIRE

- RETURN ADDRESS ENVELOPE

TO PLANNER

- CRF AMENDMENT LOG

- PASS:		PASS RACE CONNECTION FORM TO MANAGER.

PASS CRF AMENDMENT LOG TO TEAM LEADER.

- ONLY IF CAUSE FOR CONCERN.

- PC:		STORE COPY OF THE IMPACT JOB ON TO FLOPPY DISK &

TO HARD DISK.

- MAKE A COPY OF THE JOB DIRECTORY TO FLOPPY DISK.

- MAKE A COPY OF THE JUMPERING SCHEDULE

- TO FLOPPY DISK.

- MAKE A COPY OF THE DIGITAL SET LABELS TO FLOPPY DISK.

- SERVER: 	DESPATCH JOB ON MERIDIAN DATABASE.

- FILE:		FILE ALL PAPER AND SOFT COPIES UNDER CUSTOMER NAME

AND MIC NUMBER.