# APPENDIX A

# Default Values

AIDE MEMOIRE

SOURCE CFN 177E (ISSUE 7) AND NTPs

ONLY PROMPTS WHICH REQUIRE CHANGING SHOWN THROUGHOUT DOCUMENT

REFER TO TEST DATA & CRFs FOR REQUIRED VALUES

DEFAULT VALUES OF SWITCH

CONTENTS

XTD, XCT AND DAYLIGHT SAVING TIME	Page 3

TTYs	Page 4

HISTORY FILE LENGTH AND OVERLAYs	Page 6

SYSTEM PARAMETERS	Page 7

MERIDIAN LEGACY SYSTEMS 51, 61, 71, 81	Page 7

MERIDIAN 1 51C AND SUCCESSION 1000M HALF GROUP	Page 8

MERIDIAN 1 61C AND SUCCESSION 1000M SINGLE GROUP	Page 9

MERIDIAN 1 81C AND SUCCESSION 1000M MULTI GROUP	Page 10

ARIES TRANSMISSION	Page 11

SYSTEM PARAMETERS FOR PERIPHERAL EQUIPMENT	Page 12

FEATURES AND OPTIONS	Page 13

FLEXIBLE TONES AND CADENCES	Page 14

XTDs

LD 20	PRINT ROUTINE 1

REQ:	PRT

TYPE:	XTD

REQ:	****

LD 13	DIGITONE RECEIVERS, TONE DETECTORS, MULTIFREQUENCY SENDERS

AND RECEIVERS

REQ	OUT

TYPE	XTD

TN	l s c u	(Test data)

REQ	****

LD 13

REQ	NEW

TYPE	XTD

TN	l s c u	(Test data)

XTDT	0	Prompt only appears for first port on card

REQ	****

XCT

LD 17	CONFIGURATION RECORD 1

IT IS ESSENTIAL THAT THE XCT IS NOT LOCATED IN LOOP 0.

IF AT ALL POSSIBLE, ENETs SHOULD NOT BE LOCATED IN LOOP 0 EITHER.

WHERE POSSIBLE LOCATE AN MSDL IN LOOP 0.

DAYLIGHT SAVINGS TIME

LD 2	TRAFFIC

. FWTM	MAR L SUN 2

. BWTM	OCT L SUN 2

. SDST	ON

. TDST		(To check)

TTYs

TTYs 0 & 1 CHANGE FROM 2 PORT TO 4 PORT PADDLEBOARD IF REQUIRED

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	ADAN

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	OUT TTY 0

REQ	****

LD 17

REQ	CHG

TYPE	ADAN

ADAN	OUT TTY 1

REQ	****

LD 17

REQ	CHG

TYPE	ADAN

ADAN	NEW TTY 0

CYTP	SDI4

GRP	0	(Meridian 1 81C and Succession 1000M Multi Group only)

DES	System_Monitor

FLOW	NO

USER	MTC

XSM	YES

TTYLOG	0

BANR	YES

REQ	****

LD 17

REQ	CHG

TYPE	ADAN

ADAN	NEW TTY 1

CTYP	SDI4

GRP	0	(Meridian 1 81C and Succession 1000M Multi Group only)

DES	Call_Data_Record

FLOW	NO

USER	CTY

REQ	****

TTY 0 IF NOT CHANGED PREVIOUSLY CHANGE DES TO System_Monitor

LD 17

REQ	CHG

TYPE	ADAN

ADAN	CHG TTY 0

DES	System_Monitor

REQ	****

TTY 1 IF NOT CHANGED PREVIOUSLY AND A CALL LOGGER IS TO BE CONNECTED CHANGE DES TO Call_Data_Record AND DELETE ALL ENTRIES EXCEPT CTY

LD 17

REQ	CHG

TYPE	ADAN

ADAN	CHG TTY 1

DES	Call_Data_Record

USER	XMTC XTRF XSCH XBUG	(CRF/Test data)

REQ	****

OTM TTYs CREATE TWO NEW TTYs FOR OTM

LD 17

REQ	CHG

TYPE	ADAN

ADAN	NEW TTY xx	(Usually 10 and 11)

CTYP	PTY

PORT	x	(Usually 0 and 1)

DES	PTYxx

USER	MTC SCH BUG

BANR	NO

REQ	****

MTCE TTY DELETE ALL ENTRIES EXCEPT SCH & MTC

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	CHG TTY x	Where x is the maintenance TTY

DES	Maintenence

USER	XTRF XBUG	(Test data)

REQ	****

MTCE TTY ON COMPLETION OF CONFIGURATION ADD USERS

LD 17

REQ	CHG

TYPE	ADAN

ADAN	CHG TTY x	Where x is the maintenance TTY

USER	TRF BUG	(Test data)

REQ	****

HISTORY FILE LENGTH

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	CHG HST

SIZE	20000

REQ	****

OVERLAYs

DELETE ALL ENTRIES BKGD & DROL

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	OVLY

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	OVLY

BKGD	X44 X45

DROL	X30 X32 X34 X38 X43 X45 X60 X135 X137	(Test data)

REQ	****

ON COMPLETION OF CONFIGURATION ADD ENTRIES

LD 17

REQ	CHG

TYPE	OVLY

BKGD	44

DROL	30 43 44 45 (60) 135 137		(Test data)

REQ	****

DROL	60	Only required if Q931 & or QSIG & or EURO ISDN

SYSTEM PARAMETERS

MERIDIAN LEGACY SYSTEMS 51, 61, 71, 81

Option 51 Omega processor		(See CFN 177e Issue 7)

Option 61 Omega processor		(See CFN 177e Issue 7)

Option 71 Omega processor		(See CFN 177e Issue 7)

Option 81 Omega processor		(See CFN 177e Issue 7)

MERIDIAN 1 51C AND SUCCESSION 1000M HALF GROUP

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	PARM

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	PARM

LPIB	192	Pre release X11 24

LPIB	1850	Release X11 24

LPIB	2200	Release X11 25

LPIB	3500	Release X21 3.00 & 4.00

HPIB	64	Pre release X11 24

HPIB	1850	Release X11 24

HPIB	2200	Release X11 25

HPIB	3500	Release X21 3.00 & 4.00

500B	500	Pre release X11 24

500B	800	Release X11 24

500B	800	Release X11 25

500B	800	Release X21 3.00 & 4.00

SL1B	40	Pre release X11 24

SL1B	255	Release X11 24

SL1B	255	Release X11 25

SL1B	255	Release X21 3.00 & 4.00

DTOB	70	For 1 – 4 digital links

DTOB	100	For 5 or more digital links

NCR	1000	CP1

NCR	1500	CP2

NCR	2000	CP3

NCR	2000	CP4

MGCR	0	Release X21 3.00

FCDR	NEW

MSCL	MAX SCL No. + 1	(CRF)

MTRO	PPM

REQ	****

MERIDIAN 1 61C AND SUCCESSION 1000M SINGLE GROUP

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	PARM

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	PARM

LPIB	192	Pre release X11 24

LPIB	1850	Release X11 24

LPIB	2200	Release X11 25

LPIB	3500	Release X21 3.00 & 4.00

HPIB	64	Pre release X11 24

HPIB	1850	Release X11 24

HPIB	2200	Release X11 25

HPIB	3500	Release X21 3.00 & 4.00

500B	500	Pre release X11 24

500B	800	Release X11 24

500B	800	Release X11 25

500B	800	CP3 & CP4 - Release X21 3.00

500B	2000	CPP - Release X21 3.00 & 4.00

SL1B	40	Pre release X11 24

SL1B	255	Release X11 24

SL1B	255	Release X11 25

SL1B	255	Release X21 3.00 & 4.00

DTOB	70	For 1 – 4 digital links

DTOB	100	For 5 or more digital links

NCR	2000	CP1

NCR	3000	CP2

NCR	4000	CP3

NCR	4000	CP4

NCR	20000	CPP II

MGCR	0	Release X21 3.00 & 4.00

FCDR	NEW

MSCL	MAX SCL No. + 1	(CRF)

MTRO	PPM

REQ	****

MERIDIAN 1 81C AND SUCCESSION 1000M MULTI GROUP

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	PARM

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	PARM

LPIB	192	Pre release X11 24

LPIB	1850	Release X11 24

LPIB	2200	Release X11 25 (1 – 5 groups)

LPIB	3300	CP4 – Release X11 25 (6 – 8 groups)

LPIB	3500	CPP II – Release X11 25 (6 – 8 groups)

LPIB	3500	Release X21 3.00 & 4.00

HPIB	64 x No. of Groups	Pre release X11 24

HPIB	1850	Release X11 24

HPIB	2200	Release X11 25 (1 – 5 groups)

HPIB	3300	CP4 – Release X11 25 (6 – 8 groups)

HPIB	3500	CPP II – Release X11 25 (6 – 8 groups)

HPIB	3500	Release X21 3.00 & 4.00

500B	1000	Pre release X11 24

500B	800	Release X11 24

500B	800	Release X11 25

500B	800	CP3 & CP4 – Release X21 3.00 & 4.00

500B	2000	CPP - Release X21 3.00 & 4.00

SL1B	40	Pre release X11 24

SL1B	255	Release X11 24

SL1B	255	Release X11 25

SL1B	255	Release X21 3.00 & 4.00

DTOB	70	For 1 – 4 digital links

DTOB	100	For 5 or more digital links

NCR	7500	CP2

NCR	10000	CP3

NCR	10000	CP4

NCR	20000	CPP II (1 – 5 groups)

NCR	25000	CPP II (6 – 8 groups)

MGCR	0	Release X21 3.00 & 4.00

FCDR	NEW	All

MSCL	MAX SCL No. + 1	All	(CRF)

MTRO	PPM	All

REQ	****

ARIES TRANSMISSION

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	ATRN

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ATRN

AOLR	39	PRT as 39.05

REQ	****

SYSTEM PARAMETERS FOR PERIPHERAL EQUIPMENT

LD 97	CONFIGURATION RECORD 2

REQ	PRT

TYPE	SYSP

REQ	****

LD 97

REQ	CHG

TYPE	SYSP

P10R	68

REQ	****

EXTENDED PERIPHERAL EQUIPMENT SHELF DATA BLOCK

LD 97

REQ	PRT

TYPE	XPE

XPEC

REQ	****

LD 97

REQ	CHG

TYPE	XPE

XPEC	x	Where x is a valid XPE

RGTP	16

REQ	****

FEATURES AND OPTIONS

LD 21	PRINT ROUTINE 2

REQ	PRT

TYPE	CDB

CUST	0	(CRF)

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	FTR

CUST	0 	(CRF)

OPT	DBA NCA TTAA CFRA

REQ	****

LD 15

REQ	CHG

TYPE	PPM

CUST	0	(CRF)

HMTL	NO

UCST	1

ATCH	YES

REQ	****

LD 15

REQ	CHG

TYPE	TIM

CUST	0 	(CRF)

FOPT	0

REQ	****

FLEXIBLE TONES AND CADENCES

PROMPTS WILL VARY ACCORDING TO PACKAGES PROVIDED SO ONLY PROMPTS TO BE CHANGED SHOWN

LD 56	FLEXIBLE TONES AND CADENCES

REQ	CHG

TYPE	FTC

TABLE	0

HCCT	YES

SPCL

XTON

XCAD	0

MWDT

XTON	132

XCAD

CFMW

XTON	132

XCAD	17

TLP

XTON	129

XCAD

TLPT	4

REQ	****

LD 56

REQ	CHG

TYPE	FTC

TABLE	0

SCCT	YES

CAMP

XTON

XCAD

CDNC	17

LD 56

REQ	CHG

TYPE	MCAD

WCAD	16

CDNC	128

REQ	****