# APPENDIX B1

# IP

# Applications

- AIDE MEMOIR

ONLY PROMPTS WHICH REQUIRE CHANGING SHOWN THROUGHOUT DOCUMENT

- SOURCE NTPs

DEFAULT VALUES SHOWN IN BRACKETS WHERE APPLICABLE

REFER TO TEST DATA, CARD MAP & CRFs FOR REQUIRED VALUES

- CONTENTS

MERIDIAN IP MEDIA	Page 3

VIRTUAL LOOPS FOR MERIDIAN IP SETS	Page 4

DISTRIBUTION OF CARDS	Page 5

VOICE OVER GATEWAY MEDIA CARD LINESIDE	Page 6

VOICE OVER GATEWAY MEDIA CARD TRUNKSIDE	Page 7

VOICE OVER IP PEER ROUTES	Page 10

VOICE OVER OVER ITG2 CARD LINESIDE	Page 13

VOICE OVER ITG2 CARD TRUNKSIDE	Page 14

CALL PILOT	Page 17

SYMPOSIUM	Page 18

MERIDIAN IP MEDIA

- MERIDIAN INTERGRATED TELEPHONE GATEWAY, MERIDIAN GATEWAY MEDIA CARD, SYMPOSUIM AND CALL PILOT

IP NETWORK INTERFACE

LD 117	ETHERNET AND ALARM MANAGEMENT

=> NEW HOST MERIDIAN 220.0.0.100

INET Data Added

=> CHG ELNK ACTIVE MERIDIAN

INET Database Updated

=> CHG MASK 255.255.255.0

INET Data Changed

=> NEW ZONE 1 100000 BB 100000 BB

=> PRT ZONE ALL

|  |  |  | Intrazone |  |  |  |  |  | Intrazone |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zone | State |  |  |  |  | Zone | State |  |  |  |  |
|  |  | BandWidth | Strategy | Usage | Peak |  |  | BandWidth | Strategy | Usage | Peak |
|  |  | (Kbps) |  | (Kbps) | % |  |  | (Kbps) |  | (Kbps) | % |
| 0 | ENL | 1000000 | BQ | 0 | 0 | 0 | ENL | 1000000 | BQ | 0 | 0 |
| 1 | ENL | 100000 | BB | 0 | 0 | 1 | ENL | 100000 | BB | 0 | 0 |

- Number of Zones 2 (0 is default)

Zone is usually 1 but check CRF for confirmation

VIRTUAL LOOPS FOR MERIDIAN IP SETS

- CREATE VIRTUAL LOOP

LD 97	CONFIGURATION RECORD 2

REQ	CHG

TYPE	SUPL

SUPL	Vxxx	V156 or V252 For Fibre Network Fabric (Test data)

REQ	****

## PROVIDES	156 – 0 – 0 – 0	TO	156 – 1 – 15 – 31	ETC

## PROVIDES	252 – 0 – 0 – 0	TO	252 – 1 – 15 – 31	ETC

Configure Virtual Loops from highest loop first.

Check Loops to confirm availability.

RESERVE VIRTUAL TNs IN IMPACT

CARD TYPE = DIG

RESERVED FOR = VIRTUAL

DISTRIBUTION OF CARDS

PLACE VOICE GATEWAY MEDIA CARDS IN NON - BLOCKING SEGMENT.

DISTIBUTE A MAXIMUM OF 3 MEDIA CARDS (32 - PORT) PER SUPERLOOP (3 X 32 = 96, WHICH LEAVES 24 TIMESLOTS FOR ANOTHER CARD).

DISTRIBUTE A MAXIMUM OF 15 MEDIA CARDS (8 - PORT) PER SUPERLOOP (15 X 8 = 120, LEAVING NO OTHER CIRCUITS AVAILABLE).

VOICE OVER GATEWAY MEDIA CARD LINESIDE

LD 73	DIGITAL TRUNK INTERFACE

REQ	NEW

TYPE	DDB

TRSH	0

REQ	****

LD 73

REQ	CHG

TYPE	DTI

FEAT	PAD

PDCA	15

ONP	6	5

DSET	3	4

DTT	0	0

SDTT	0	0

DTO	3	19

VNL	0	0

SATT	3	19

ACO	2	16

ATO	3	19

PRI	0	0

PRI2	0	0

XUT	2	16

XEM	2	16

BRIL	0	0

BRIT	0	0

WRLS	3	19

REQ	****

LD 14	TRUNK DATA BLOCK

REQ	NEW xx	Number of entries card (Max 32) (CRF)

TYPE	VGW

TN	l s c u	Matches VGW card position (Test data)

DES 	IP_PHONES	(CRF)

XTRK	MC32

ZONE	1	(CRF)

CUST	0

REQ	****

VOICE OVER GATEWAY MEDIA CARD TRUNKSIDE

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW DCH xxx	Highest spare device number (Test data)

CTYP	MSDL

GRP	x	(Test Data)

DNUM	xx	(Test Data)

DES	IP_ROUTE

USR	ISLD

IFC	SL1

ISLM	xxx	Number of actual trunks (382 max) (CRF)

RLS	xx	Software release distant end (CRF)

RCAP	ND2 TAT	TAT if supported

NASA	YES

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	NET

CUST	0

ISDN	YES

VPNI	1

REQ	****

LD 16	ROUTE DATA BLOCK

REQ	NEW

TYPE	RDB

CUST	0

ROUT	xxx	(CRF)

DES	xxx	(CRF)

TKTP	TIE

ISDN	YES

MODE	ISLD

DCH	xxx	Matches DCH LD 17 (Test data)

IFC	SL1

PNI	1

NCNA	YES

NCRD	YES

CTYP	CDP

INAC	(NO) / YES	(CRF)

DAPC	(NO) / YES	(CRF)

AUTO	(NO) / YES	(CRF)

DNIS	(NO) / YES	(CRF)

NDGT	xxx	(CRF)

DCDR	(NO) / YES	(CRF)

ICOG	IAO / ICT / OGT	(CRF)

TRMB	NO

ACOD	xxx	(CRF)

TARG	xx	(CRF)

INST	xxx	(CRF)

IDC	(NO) / YES	(CRF)

DCNO	xxx	(CRF)

NDNO	xxx	(CRF)

DEXT	(NO) / YES	(CRF)

CNTL	YES

NEDC	ETH

FEDC	ETH

DRNG	YES

CDR	(NO) / YES	(CRF)

INC	(NO) / YES	(CRF)

LAST	(NO) / YES	(CRF)

QREC	(NO) / YES	(CRF)

OAL	(NO) / YES	(CRF)

AIA	(NO) / YES	(CRF)

OAN	(NO) / YES	(CRF)

OPD	(NO) / YES	(CRF)

MUS	(NO) / YES	(CRF)

MRT	xxx	(CRF)

FRL	0  0	(CRF)

PLEV	0

REQ	****

LD 14	TRUNK DATA BLOCK

REQ	NEW xx	Number of actual trunks (CRF) (32 max)

TYPE	IPTI

TN	l s c u	Matches VGW card position (Test data)

DES	xxx	(CRF)

XTRK	MC32	Or MC8 (Test data)

CUST	0

NCOS	7

RTMB	xxx yyy	Route followed by 1st member (CRF)

CHID	xxx	CHID matches member (CRF)

TGAR	xx	(CRF)

STRI	IMM

STRO	IMM

SUPN	YES

REQ	****

- VOICE OVER IP PEER ROUTES

- IP VIRTUAL ROUTE

- CREATE VIRTUAL LOOP

LD 97	CONFIGURATION RECORD 2

REQ	CHG

TYPE	SUPL

SUPL	Vxxx	V156 or V252 For CP PII (Test data)

REQ	****

## PROVIDES	156 – 0 – 0 – 0	TO	156 – 1 – 15 – 31	ETC

## PROVIDES	252 – 0 – 0 – 0	TO	252 – 1 – 15 – 31	ETC

Configure Virtual Loops from highest loop first.

Check Loops to confirm availability.

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW DCH xxx	Match Virtual loop (Test data)

CTYP	DCIP

DES	IP_PEER

IFC	SL1

ISLM	xxx		Number of actual trunks (382 max) (CRF)

SSRC	(1800)		Signalling Server resource capacity (4000max) (CRF)

RLS	xx		Software release distant end (CRF)

RCAP	ND2

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	NET

CUST	0

ISDN	YES

VPNI	1

CNTC          44	(International country code)

NATC          xxx	(Upto first 4 digits of local STD code after 0 i.e. 1823)

INTC           00	(International access code)

REQ	****

LD 16	ROUTE DATA BLOCK

REQ	NEW

TYPE	RDB

CUST	0

ROUT	xxx	(CRF)

DES	xxx	(CRF)

TKTP	TIE

VTRK	YES

ZONE	1	(CRF)

PCID	(H323) / SIP	(CRF)

NODE	xxx	Matches Node ID of Signalling Server (CRF)

MODE	ISLD

DCH	xxx	Matches DCH LD 17 (Test data)

IFC	SL1

PNI	1

NCNA	YES

NCRD	YES

INAC	(NO) / YES	(CRF)

DAPC	(NO) / YES	(CRF)

AUTO	(NO) / YES	(CRF)

DNIS	(NO) / YES	(CRF)

NDGT	xxx	(CRF)

DCDR	(NO) / YES	(CRF)

ICOG	IAO / ICT / OGT	(CRF)

ACOD	xxx	(CRF)

TARG	xx	(CRF)

INST	xxx	(CRF)

IDC	(NO) / YES	(CRF)

DCNO	xxx	(CRF)

NDNO	xxx	(CRF)

DEXT	(NO) / YES	(CRF)

CNTL	YES

NEDC	ETH

FEDC	ETH

DRNG	YES

CDR	(NO) / YES	(CRF)

INC	(NO) / YES	(CRF)

LAST	(NO) / YES	(CRF)

QREC	(NO) / YES	(CRF)

OAL	(NO) / YES	(CRF)

AIA	(NO) / YES	(CRF)

OAN	(NO) / YES	(CRF)

OPD	(NO) / YES	(CRF)

MUS	(NO) / YES	(CRF)

MRT	xxx	(CRF)

FRL	0  0	(CRF)

PLEV	0

REQ	****

LD 14	TRUNK DATA BLOCK

REQ	NEW xxx	Number of actual trunks (CRF)

TYPE	IPTI

TN	l s c u	Virtual loop & channel (Test data)

DES	xxx	(CRF)

CUST	0

NCOS	7

RTMB	xxx yyy	Route followed by 1st member (CRF)

CHID	xxx	CHID matches member (CRF)

TGAR	xx	(CRF)

STRI	IMM

STRO	IMM

SUPN	YES

REQ	****

VOICE OVER ITG2 LINESIDE

### CREATE A VIRTUAL ROUTE FOR THE ITG2 LINESIDE

ITG 24 PORT LINE CARD UPTO 96 INTERNET TELEPHONES

ITG 32 PORT LINE CARD UPTO 128 INTERNET TELEPHONES

MULT MEDIA 32 PORT CARD UPTO 128 INTERNET TELEPHONES

LD 73	DIGITAL TRUNK INTERFACE

REQ	NEW

TYPE	DDB

TRSH	0

REQ	****

LD 73

REQ	CHG

TYPE	DTI

FEAT	PAD

PDCA	15

ONP	6	5

DSET	3	4

DTT	0	0

SDTT	0	0

DTO	3	19

VNL	0	0

SATT	3	19

ACO	2	16

ATO	3	19

PRI	0	0

PRI2	0	0

XUT	2	16

XEM	2	16

BRIL	0	0

BRIT	0	0

WRLS	3	19

REQ	****

### CREATE A VIRTUAL ROUTE

LD 14	TRUNK DATA BLOCK

REQ	NEW xx	Number of entries card (Max 32) (CRF)

TYPE	TIE

TN	l s c u	If two slot card TNs match left slot (Test data)

DES 	IP_PHONES	(CRF)

XTRK	xxx	ITG1 or ITG2 (1 or 2 cardslot circuit card) (Test Data)

MAXU	xx	Maximum Number of units supported				(1 – 24 / 32 single slot or 1 – 32 dual slot)

(CRF/Test data)

IPTN	YES

ZONE	1	(CRF)

CUST	0

REQ	****

- VOICE OVER ITG2 TRUNKSIDE

ITG 2.0 1 SLOT (24 or 32 PORT) or 2 SLOT (32 PORT) CARDS

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW DCH xxx	Highest spare device number (Test data)

CTYP	MSDL

GRP	x	(Test data)

DNUM	xx	(Test data)

PORT	x	0 or 1

DES	IP_ROUTE

USR	ISLD

IFC	SL1

ISLM	xx	Number of actual trunks (Max 32) (CRF)

SIDE	USR

RLS	xx	Software release distant end (CRF)

RCAP	ND2

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	NET

CUST	0

ISDN	YES

PNI	1

REQ	****

LD 16	ROUTE DATA BLOCK

REQ	NEW

TYPE	RDB

CUST	0

ROUT	xxx	(CRF)

DES	xxx	(CRF)

TKTP	TIE

ISDN	YES

MODE	ISLD

DCH	xxx	Matches DCH LD 17 (Test data)

IFC	SL1

PNI	1

NCNA	YES

NCRD	YES

INAC	(NO) / (YES)	(CRF)

AUTO	(NO) / (YES)	(CRF)

DNIS	(NO) / (YES)	(CRF)

ICOG	IAO / ICT / OGT	(CRF)

TRMB	NO

ACOD	xxx	(CRF)

TARG	xx	(CRF)

INST	xxx	(CRF)

IDC	(NO) / (YES)	(CRF)

DCNO	xxx	(CRF)

NDNO	xxx	(CRF)

DEXT	(NO) / (YES)	(CRF)

CNTL	YES

NEDC	ETH

FEDC	ETH

DRNG	YES

CDR	(NO) / (YES)	(CRF)

INC	(NO) / (YES)	(CRF)

LAST	(NO) / (YES)	(CRF)

QREC	(NO) / (YES)	(CRF)

OAL	(NO) / (YES)	(CRF)

AIA	(NO) / (YES)	(CRF)

OAN	(NO) / (YES)	(CRF)

OPD	(NO) / (YES)	(CRF)

MUS	(NO) / (YES)	(CRF)

MRT	xxx	(CRF)

FRL	0  0	(CRF)

PLEV	0

REQ	****

LD 14

REQ	NEW xx	Number of entries (Max 32) (CRF)

TYPE	TIE

TN	l s c u	If two slot card TNs match left slot

DES	xxx	(CRF)

XTRK	xxx	ITG1 or ITG2 (ITG 1 or 2 cardslot circuit card)			(Test data)

MAXU 	xx	Maximum Number of units supported				(1 – 24 / 32 single slot or 2 – 32 dual slot)

(CRF/Test data)

CUST	0

NCOS	7

RTMB	xxx yyy	Route followed by 1st member (CRF)

CHID	xxx	CHID matches member (CRF)

TGAR	xx	(CRF)

STRI	WNK

STRO	WNK

SUPN	YES

REQ	****

CALL PILOT CONFIGURATION

ON SWITCH

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW ELAN xx	Usually 24

CTYP	ELAN

DES	CALL_PILOT

REQ	****

LD 17

REQ	CHG

TYPE	VAS

VAS	NEW

VSID	xx	Usually 24

ELAN	xx	Usually 24

SECU	NO

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ:	CHG

TYPE:	FTR

CUST	0

OPT	MCI

REQ	****

### CALL PILOT MGATE CARD

SHOULD BE IN A BLOCKING SEGMENT

CALL PILOT WITHIN IMPACT

RESERVE PHANTOM TNs FOR NGENS

CARD TYPE = DIG

RESERVED FOR = PILOT

L S C U (U = 0 – 31) Vintage NTRB18 CA  or (U = 0 – 24)

### NCOS MUST BE MOST RESTRICTIVE IMPACT DEFAULT 1

IF NCOS 1 NOT MOST RESTRICTIVE ALTER RUNFILE

INSERT PHANTOM TNs FOR CALL PILOT PHANTOM DNs

MAKE TNs NOT TO BE ALLOCATED ON IMPACT

### NIGHT SERVICE

CREATE A PHANTOM DN WITH DCFW OF CALL PILOT VOICE CDN.

POINT NIGHT SERVICE DN IN LD 15 TO PHANTOM DN.

SYMPOSIUM

SYMPOSIUM (NOTE PACKAGES 311 AND 324 REQUIRED)

ETHERNET AML

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW ELAN xx	Prefered 17

CTYP	ELAN

DES	SYMPOSIUM

REQ	****

LD 17

REQ	CHG

TYPE	VAS

VAS	NEW

VSID	xx	Prefered 17

ELAN	xx	Prefered 17

SECU	YES

REQ	****

LD 17

REQ	CHG

TYPE	PARM

MGCR	25

CSQI	255

CSQO	255

DCUS	1

REQ	****

LD15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	FTR

CUST	0

OPT	DNI

REQ	****

WITHIN IMPACT

ACD SETUP = MAX

CDN’S

Controlled by CCR = Y

MODEL SETS

ACD 500 SETS

AST ACD Telephone = YES

ISDN/AP = 1

ACD DIGITAL SETS

Associate Set Assignment = 0     x          Where x is the DN key

CHANGE RUN FILES ON MAIL QUEUE

SDNB	YES

HOML	(NO)	Default

RDNA	YES

IVR	YES

ADD PROMPT TRDN ON RUN FILE ON MAIL QUEUE

IVR

TRDN