# APPENDIX C

AIDE MEMOIRE

ONLY PROMPTS WHICH REQUIRE CHANGING SHOWN THROUGHOUT DOCUMENT

SOURCE NTPs

DEFAULT VALUES SHOWN IN BRACKETS WHERE APPLICABLE

REFER TO TEST DATA, CARD MAP & CRFs FOR REQUIRED VALUES

CONTENTS

MAIL CONFIGURATION	Page 3

REMOTE MAIL CONFIGURATION	Page 6

ACD C2 CONFIGURATION	Page 7

ACD MAX IPE CONFIGURATION	Page 8

CCR CONFIGURATION	Page 9

LINK CONFIGURATION	Page 10

PHANTOM TN CONFIGURATION	Page 11

MAIL CONFIGURATION

MAIL ON SWITCH

IF MAIL ON EVEN PORT

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	CEQU

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	CEQU

TERD	xx	Delete if xx is even port except where 2 ports are			required (Test data)

REQ	****

LD 17

REQ	CHG

TYPE	CEQU

TERD	xx	Add xx as an odd port usually 3 (Test data)

REQ	****

CONFIGURE MAIL

LD 21	PRINT ROUTINE 2

REQ	PRT

TYPE	FTR

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	FTR

CUST	0

OPT	MCI

REQ	****

LD 21

REQ	PRT

TYPE	IMS

REQ	****

LD 15

REQ	CHG

TYPE	IMS

CUST	0

IMS	YES

IMA	YES

REQ	****

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	ADAN

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW AML xx	Usually 8 (Test data)

CTYP	MSDL

GRP	x	(Meridian 1 81C and Succession 1000M Multi Group only) (Test data)

DNUM	xx	Usually 8 (Test data)

PORT	xx	Port 0 to 3 (Test data)

DES	MAIL

BPS	4800

REQ	****

LD 22	PRINT ROUTINE 3

REQ	PRT

TYPE	VAS

REQ	****

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	VAS

VAS	NEW

VSID	xx	Usually 8 (Test data)

AML	xx	Usually 8 (Test data)

SECU	NO

CONF	DIR

REQ	****

MAIL WITHIN IMPACT

RESERVE TNs

CARD TYPE = DIG AND RESERVED FOR = MAIL

MAX MAIL TNs PER MAIL LOOP ARE 24

xx – 0 – 2 – (0 – 7)	WHERE xx IS TERD (TEST DATA)

xx – 0 – 3 – (0 – 7)

xx – 1 – 2 – (0 – 7)

IMPACT MAKES MAIL SETS NCOS 1 AND CLS TLD

###### NCOS MUST BE MOST RESTRICTIVE

REMOTE MAIL CONFIGURATION

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	FTR

CUST	0

OPT	MCI

REQ	****

LD 15

REQ	CHG

TYPE	IMS

CUST	0

IMS	YES

IMA	YES

REQ	****

ACD C2 CONFIGURATION

ACD SENIOR SUPERVISOR (STATUS DISPLAY PORT (IMPACT STIO = TTY)

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	CHG TTY xx	Usually 6 (Test data)

DES	ACD_SSU

USER	XMTC XSCH ACD

CUST	0

SSUP	YES

APRT	NO

REQ	****

ACD PRINTER (PRINTER PORT (IMPACT PRIO = PRT)

LD 17

REQ	CHG

TYPE	ADAN

ADAN	OUT TTY xx	Usually 7 (Test data)

ADAN	NEW PRT xx	Usually 7 (Test data)

CTYP	xxx	Match CTYP outed

DES	ACD_PRT

USER	ACD

CUST	0

SSUP	NO

APRT	YES

REQ	****

ACD SUPERVISOR OR WALLBOARD

LD 17

REQ	CHG

ADAN	CHG TTY xx	(Test data)

DES	xxx	ACD_SUP or ACD_WBD

USER	XMTC XSCH ACD

CUST	0

SSUP	NO

APRT	NO

REQ	****

WITHIN IMPACT

ACD SETUP = C2

ACD MAX IPE CONFIGURATION

NUMBER OF ACD – ADS CUSTOMERS

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	PARM

DCUS	1	No. OF ACD – ADS CUSTOMERS

REQ	****

SET NUMBER OF MESSAGE CALL REGISTERS

LD 17

REQ	CHG

TYPE	PARM

MGCR	25

REQ	****

LOAD MANAGEMENT

LD 17

REQ	CHG

TYPE	ADAN

ADAN	CHG TTY xx	USUALLY 6 (TEST DATA)

DES	ACD_SSU

USER	XMTC XSCH ACD

CUST	0

SSUP	YES

APRT	NO

REQ	****

HIGH SPEED LINK

LD 17

REQ	CHG

TYPE	ADAN

ADAN	CHG TTY xx	USUALLY 7 (TEST DATA)

DES	MAX_HSL

USER	XMTC XSCH HSL

REQ	****

WITHIN IMPACT

ACD SETUP = MAX

CCR CONFIGURATION

CONFIGURE CCR

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW AML xx	Usually 8, 9 or 10 (Test data)

CTYP	MSDL

DNUM	xx	Usually 8, 9 or 10 (Test data)

PORT	x	PORT 0 to 3 (Test data)

DES	CCR

BPS	19200

REQ	****

LD 17

REQ	CHG

TYPE	VAS

VAS	NEW

VSID	xx	Usually 8, 9 or 10 (Test data)

AML	xx	Usually 8, 9 or 10 (Test data)

INTL	4

MCNT	400

CONF	DIR

REQ	****

LD17

REQ	CHG

TYPE	PARM

CSQI	255

CSQO	255

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	AML

CUST	0

GP02	01 02 03 04 05 06

REQ	****

IF CCR / LINK IS ON ONE UNIT IN SKIN CONFIGURE AS CCR

LINK CONFIGURATION

CONFIGURE LINK

LD 17	CONFIGURATION RECORD 1

REQ	CHG

TYPE	ADAN

ADAN	NEW AML xx	Usually 8,9 or 10 (Test data)

CTYP	MSDL

DNUM	xx	Usually 8,9 or 10 (Test data)

PORT	x	PORT 0 to 3 (Test data)

DES	CCR

BPS	19200

REQ	****

LD 17

REQ	CHG

TYPE	VAS

VAS	NEW

VSID	xx	Usually 8,9 or 10 (Test data)

AML	xx	Usually 8,9 or 10 (Test data)

INTL	4

MCNT	400

CONF	DIR

REQ	****

LD 17

REQ	CHG

TYPE	PARM

CSQI	255

CSQO	255

REQ	****

LD 15	CUSTOMER DATA BLOCK

REQ	CHG

TYPE	AML

CUST	0

OPT	DNX

GP02	01 02 03 04 05 06

REQ	****

PHANTOM TN CONFIGURATION

CONFIGURE SUPERLOOP FOR PHANTOMS

OPTIONS 51 - 81

LD 97	CONFIGURATION RECORD 2

REQ	CHG

TYPE	SUPL

SUPL	Nxxx	N156 or N252 For Fibre Network Fabric (Test data)

REQ	****

## PROVIDES	156 – 0 – 0 – 0	TO	156 – 1 – 15 – 15	ETC

## PROVIDES	252 – 0 – 0 – 0	TO	252 – 1 – 15 – 15	ETC

Configure Virtual Loops from highest loop first.

Check Loops to confirm availability.

IF SETS ARE ACD TREAT AS NON - BLOCKING IF POSSIBLE.

OPTION 11

LD 97

REQ	CHG

TYPE	SUPL

SUPL	Nxxx	N96, N100, N104, N104, N108, N112 – RLSE 24 +

N64, N68, N72, N76, N89 – PRE RLSE 24

REQ	****

RLSE 24 +

N96   PROVIDES	61 – 0	TO	64 – 0

N100 PROVIDES	65 – 0	TO	68 – 0

N104 PROVIDES	69 – 0	TO	72 – 0

N108 PROVIDES	73 – 0	TO	76 – 0

N112 PROVIDES	77 – 0	TO	80 – 0

PRE RLSE 24

N64 PROVIDES	41 – 0	TO	44 – 0

N68 PROVIDES	45 – 0	TO	48 – 0

N72 PROVIDES	49 – 0	TO	52 – 0

N76 PROVIDES	53 – 0	TO	56 – 0

N80 PROVIDES	57 – 0	TO	60 – 0

RESERVE TNs IN IMPACT

CARD TYPE = 500 (FOR PHANTOM)

CARD TYPE = DIG ( FOR 39xxV)