## APPENDIX D1

# M39xx SERIES MERIDIAN DIGITAL TELEPHONES

###### KEY DESCRIPTIONS

### M3901

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number line must go on Key 0. Note: You can configure Key 0 as a Voice Call (VCC) Key, HotLine (HOT) Key, Single Call Non-Ringing, Private Line Non-Ringing, Multiple Call Non-Ringing, Private Line Ringing or Multiple Call Ringing. |
| Key 1 | Feature or Auto Dial |
| Key 2 | Feature or Auto Dial |
| Key 3 | Feature or Auto Dial |
| Key 4 | Feature or Auto Dial |
| Key 5 | Feature or Auto Dial |
|  | Note: You can configure Keys 1 – 5 with any features that do not require a display (DAG, DWG, DSP, and RMK). |
| # | Deactivate all activated features. |
|  | Note 1: The M3901 telephone firmware controls the Volume Control Bar, and Goodbye Keys. Do not configure on the Meridian 1 System. |
|  | Note 2: Nortel Networks recommends that you DO NOT configure the M3901 as a Call Centre DN. |

### M3902

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Directory Number line must go on Key 0. Note: You can configure Key 0 as a Voice Call (VCC) Key, HotLine (HOT) Key, Single Call Non-Ringing, Private Line Non-Ringing, Multiple Call Non-Ringing, Private Line Ringing or Multiple Call Ringing. |
| Key 1 | Feature or Auto Dial |
| Key 2 | Feature or Auto Dial |
| Key 3 | Feature or Auto Dial |
| Key 4 | Call Transfer (default)(TRN) or 3 Party Conference (AO3) or 6 Party Conference (AO6) |
| Key 5 | Message Waiting (MWK) |
|  | Note: Keys 1 – 5 DO NOT configure as: Multiple Call Non-Ringing Multiple Call Ringing Private Line Non-Ringing Private Line Ringing Single Call Non-Ringing Single Call Ringing |

M3903

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number |
| Key 1 | Secondary Directory Number or Feature or Auto Dial |
| Key 2 | Secondary Directory Number or Feature or Auto Dial |
| Key 3 | Secondary Directory Number or Feature or Auto Dial |
|  | Note: Keys 1 – 3 cannot be configured as: - 3 Party Conference - 6 Party Conference - Call Forward - Account Charge - Calling Party Number - Call Park - Privacy Release - Ring Again - Ringing Number Pickup - Call Transfer |
| - Key 4 – 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| - Key 24 | Privacy Release (PRS) |
| - Key 25 | Charge Account (CHG) |
| - Key 26 | Calling Party Number (CPN) |
| - Key 27 | Configure Callers List (CLT) |
| - Key 28 | Configure Redial List (RLT) |
| - Key 29 – 31 | NUL |

### M3904

| - KEY NUMBER | - DESCRIPTION |
| --- | --- |
| - Key 0 | - Primary Directory Number |
| - Key 1 – 11 | - Secondary Directory Number or Feature or Auto Dial |
|  | - Note: Keys 1 – 11 cannot be configured as: - Message Waiting - Transfer - 3 Party Conference - 6 Party Conference - Call Forward - Ring Again - Call Park - Ringing Number Pickup - Privacy Release - Charge Account - Call Party Number |
| - Key 12 – 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 | Configure Callers List (CLT) |
| Key 28 | Configure Redial List (RLT) |
| Key 29 – 31 | NUL |

###### 1 KBA ADD ON UNIT KEYS 32 – 53

2 KBA ADD ON UNITS KEYS 32 – 75

1 DBA ADD ON UNIT KEYS 32 - 55

### M3905

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary ACD Directory Number parallel to the *In-Calls Key |
| Key 1 – 6 | Secondary Directory Number or Feature or Auto Dial |
|  | Note: Keys 1 – 11 cannot be configured as: - Message Waiting - Transfer - 3 Party Conference - 6 Party Conference - Call Forward - Ring Again - Call Park - Ringing Number Pickup - Privacy Release - Charge Account - Call Party Number |
| - Key 7 | - Program Key (optional) or NUL |
| - Key 8 – 11 | - Configured as one of the following: - Agent Answer - Answer Emergency Call - *Call Supervisor must have CLS = AGN - *Emergency (default key 10) must have CLS = AGN - *Make Set Busy (default key 8) - *Not Ready (default key 9) must have CLS = AGN or SPV - Activity Code entry - Display Waiting Calls (used with supervisor or agent telephones) - Observe agent - Ring Agent (must have CLS = SPV) - Note: * Asterisk indicates default key label from factory. |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 | Configure Callers List (CLT) |
| Key 28 | Configure Redial List (RLT) |
| Key 29 – 31 | NUL |

###### 1 KBA ADD ON UNIT KEYS 32 – 53

2 KBA ADD ON UNITS KEYS 32 – 75

###### 1 DBA ADD ON UNIT KEYS 32 – 55

# MI2xxx SERIES MERIDIAN IP TELEPHONES

###### KEY DESCRIPTIONS

I2001

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number line must go on Key 0. Note: You can cofigure Key 0 as a Voice Call (VCC) Key, Hotline (HOT) Key, Single Call Non-Ringing, Private Line Non-Ringing, Multiple Call Non-Ringing, Private Line Ringing or Multiple Call Ringing. |
| - Keys 1 – 11 | - NUL |
| - Keys 12 - 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 – 31 | NUL |

I2002

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number line must go on Key 0. Note: You can cofigure Key 0 as a Voice Call (VCC) Key, Hotline (HOT) Key, Single Call Non-Ringing, Private Line Non-Ringing, Multiple Call Non-Ringing, Private Line Ringing or Multiple Call Ringing. |
| Key 1 | Secondary Directory Number or Feature or Auto Dial |
| - Key 2 | Secondary Directory Number or Feature or Auto Dial |
| - Key 3 | Secondary Directory Number or Feature or Auto Dial |
| - Keys 4 – 11 | - NUL |
| - Keys 12 - 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 – 31 | NUL |

I2004

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number |
| Key 1 – 11 | Secondary Directory Number or Feature or Auto Dial |
|  | Note: Keys 1 – 11 cannot be configured as: - Message Waiting - Transfer - 3 Party Conference - 6 Party Conference - Call Forward - Ring Again - Call Park - Ringing Number Pickup - Speed Call - System Speed Call - Speed Call Controller - System Speed Call Controller - Privacy Release - Charge Account - Call Party Number |
| - Key 12 – 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 – 31 | NUL |

###### 1 KEM ADD ON UNIT KEYS 32 – 55

2 KEM ADD ON UNITS KEYS 32 – 75

I2007

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number |
| Key 1 – 11 | Secondary Directory Number or Feature or Auto Dial |
|  | Note: Keys 1 – 11 cannot be configured as: - Message Waiting - Transfer - 3 Party Conference - 6 Party Conference - Call Forward - Ring Again - Call Park - Ringing Number Pickup - Speed Call - System Speed Call - Speed Call Controller - System Speed Call Controller - Privacy Release - Charge Account - Call Party Number |
| - Key 12 – 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 – 31 | NUL |

I2033

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number line must go on Key 0. Note: You can cofigure Key 0 as a Voice Call (VCC) Key, Hotline (HOT) Key, Single Call Non-Ringing, Private Line Non-Ringing, Multiple Call Non-Ringing, Private Line Ringing or Multiple Call Ringing. |
| - Keys 1 – 11 | - NUL |
| - Keys 12 - 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 – 31 | NUL |

I2050

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | Primary Directory Number |
| Key 1 – 11 | Secondary Directory Number or Feature or Auto Dial |
|  | Note: Keys 1 – 11 cannot be configured as: - Message Waiting - Transfer - 3 Party Conference - 6 Party Conference - Call Forward - Ring Again - Call Park - Ringing Number Pickup - Speed Call - System Speed Call - Speed Call Controller - System Speed Call Controller - Privacy Release - Charge Account - Call Party Number |
| - Key 12 – 15 | - Blocked |
| - Key 16 | - Message Waiting is not a default feature for this key (MWK) |
| - Key 17 | - Call Transfer (TRN) |
| - Key 18 | - 3 Party Conference (A03) or 6 Party Conference (A06) |
| - Key 19 | - Call Forward (CFW) |
| - Key 20 | - Ring Again (RGA) |
| - Key 21 | - Call Park (PRK) |
| - Key 22 | - Ringing Number Pickup (RNP) |
| - Key 23 | - Configure as: - Speed Call User (SCU) - System Speed Call User (SSU) - Speed Call Controller (SCC) - System Speed Call Controller (SSC) Note: The above services are not a default feature for this key. Can use these features on any programmable keys. |
| Key 24 | Privacy Release (PRS) |
| Key 25 | Charge Account (CHG) |
| Key 26 | Calling Party Number (CPN) |
| Key 27 – 31 | NUL |

PCA

| KEY NUMBER | DESCRIPTION |
| --- | --- |
| Key 0 | PRIMARY DIRECTORY NUMBER |
| - Keys 1 | - HOT P y xxxx |