# APPENDIX E

AIDE MEMOIRE

CONFIGURATION OF SECOND HAND

OR

MAINTENANCE REPLACEMENT

SWITCHES

DIFFERENCES IN PROCESS

Use a Default database of the same generic stored on server :

T : \ Vdi \ Group \ Meridian Jobs \ Default databases \ GEN xx.xx.

Or use a previousy configured switched Day One Database of the same type.

Load software database using Default database and Keycodes from

second hand switch or from switch requiring maintenance replacement.

If Default database the switch will have no XTD &

only two TTYs to be re-configured as required.

OPT61 will have CNI 0 12 0 & 1 12 0 to be re-configured.

OPT81 will have CNI 0 12 0, 0 12 1 & 1 12 0, 1 12 1 to be re-configured.

CDB configured, CEQU stripped and FTC configured.

To configure the FTC an XCT must first be configured.

Use a combination of the convert programs and switch listing to configure the switch.

SEE FOLLOWING PAGES FOR MORE SPECIFIC DETAILS.

CNI’s

### PRINT CNIs

### LD 22

REQ	PRT

TYPE	CEQU

EXT0	3PE     (cpu 0 – active)

CNI     012   000   000   (net 0 – connected)

CNI     012   001   001   (net 1 – connected)

EXT1	3PE    (cpu 1 – standby)

CNI     012   000   000   (net 0 – connected)

CNI     012   001   001   (net 1 – connected)

LIST STATUS OF CNIs

LD 135

.STAT  CNI

CNI  0  12  0  :  remote  =  group  0  DSBL  :

CNI  0  12  1  :  remote  =  group  1  DSBL  :

CNI  1  12  0  :  remote  =  group  0  ENBL

CNI  1  12  1  :  remote  =  group  1  ENBL

TO ADD CNIs ON BOTH CPUs.

### LD 17

REQ	CHG

TYPE	CEQU

EXT0	3PE

CNI	13  0  2

CNI	13  0  3

EXT1	3PE

CNI	13  0  2

CNI	13  1  3

PRT and STAT to confirm.

TO DELETE CNIs ON BOTH CPUs.

First disable CNIs that require deleting

LD 135

.DIS	CNI	0  12  1

.DIS	CNI	1  12  1

Then out CNIs in CEQU

### LD 17

REQ	CHG

TYPE	CEQU

EXT0	3PE

CNI	12  1  X

EXT1	3PE

CNI	12  1  X

PRT and STAT to confirm.

## LIST AN ENTIRE CONFIGURATION OF A MERIDIAN 1

| FILE NAME | TITLE | PRT OVLY | MNEMONICS |
| --- | --- | --- | --- |
| CFN.LOG | Configuration Record 1 Configuration Record 2 | 22 & 97 | LD 22 ISS/TID + PRT CFN LD 97 PRT XPE/SUPL |
| CDB.LOG | Customer Data Block | 21 | PRT CDB |
| DDSL.LOG | Dpnss Link | 74 | PRT DDSL/LSSL |
| RDB.LOG | Route Data Block | 21 | PRT RDB |
| LTM.LOG | Trunk Data Block | 21 & 20 | LD 21 LTM + LD 20 PRT AWR/COT/DID/IDA/MUS/PAG/RAN/RCD/TIE |
| FCR.LOG | New Flexible Code Restriction & Incoming Digit Conversion | 49 | PRT FCR/IDC |
| DISA.LOG | Direct Inward System Access | 24 | PRT DIS |
| ESN.LOG | Electronic Switched Network | 86 | PRT ESN/DGT/RLB/NAS |
| DSC.LOG | Electronic Switched Network 2 | 87 | PRT CDP DSC/TSC/LSC + PRT NCTL 0 99 |
| SPN.LOG | Electronic Switched Network 3 | 90 | PRT NET SUM PRT NET AC1 SPN/NET AC1 HLOC/NET AC1 LOC PRT NET AC2 SPN/NET AC2 HLOC/NET AC2 LOC |
| 2250.LOG | Attendant Consoles | 20 | PRT 2250/PWR |
| FFC.LOG | Flexible Feature Codes | 57 | PRT FFC |
| GHT.LOG | Speed/Group Call, Pretranslation, | 20 | PRT GHT/GRP |
| SCL.LOG | Speed/Group Call, Pretranslation, | 20 | PRT SCL/PRE |
| CPK.LOG | Call Park | 50 | PRT CPK |
| CPND.LOG | Call Party Name Display | 95 | PRT CPND/NAME DN ALL/NAME IDC ALL |
| ACD.LOG | Automatic Call Distribution | 23 & 20 | LD 23 PRT ACD/CDN/NACD/SCB/ADS LD 20 PRT SL1 OR 2008(OPT 11) |
| RART.LOG | Flexible Tones and Cadences | 56 | PRT RART/TBAR |
| PAD.LOG | Digital Trunk Interface | 73 | PRT PRI2 PAD/PRI2 LPTI PRT DTI2 PAD/DTI2 LPTI PRT DTI2 ABCD/DTI2 SYTI |
| SAR.LOG | Authorization Code For AUTH in Load 88 PRT PWD in Load 15 | 88 | PRT AUT/AUB/SAR |
| BRI.LOG | ISDN Basic Rate Interface BRI Administartion | 27 | PRT LAPD/MISP/BRSC/DSL/TSP |
| ARIE.LOG | Meridian Digital Telephone Administration | 20 | PRT ARIE |
| 500.LOG | Analog(500/2500) Telephone Administration | 20 | PRT 500 |
| TONE.LOG | Flexible Tones and Cadences | 56 | PRT FTC/MCAD |
| DNTN.LOG | Directory No.s & Terminal No.s | 20 | PRT DNB/TNB |
| SYMP.LOG | Ethernet | 117 | PRT ELNK/HOST/MASK/ROUTE/ZONE ALL |

NAME can be converted to a .csv file and imported into Impact.

LIST A MERIDIAN FOR CONVERT PROGRAMS

| FILE NAME | CONVERT PROGRAM | PRT OVLY | MNEMONICS |
| --- | --- | --- | --- |
| 500.TXT | T:/Group/Convert/ Mctvprog/cvt500 | 20 | PRT 500 |
| ARIE.TXT | T:/Group/Convert/ Mctvprog/cvtdig | 20 | PRT ARIE |
| NET.TXT | T:/Group/Convert/ Mctvprog/cvtnet | 20, 21, 23, 57, 49, 86, 87, 88, 90 For AUTH Load 88 PRT PWD in Load 15 | LD 20 PRT SCL/GHT LD 23 PRT ACD/CDN LD 49 PRT FCR/IDC LD 57 PRT FFC LD 86 PRT DGT/RLB LD 87 PRT CDP DSC/TSC/LSC LD 87 PRT NCTL 0 99 LD 88 PRT AUB/AUT LD 90 PRT NET AC1 SPN/NET AC1 HLOC/ NET AC1 LOC PRT NET AC2 SPN/NET AC2 HLOC/ NET AC2 LOC |

All cvtnet conversions will need to be checked against listings as not all data is transferred and all other data will need to be transferred from listing with the exception of NAME which can be converted to a .csv file and imported into Impact.