|  |  | Customer field note For BT people CFN177E Issue 7, 23-Jan-2004 Published by Convergent Solutions Not to be Shown Outside BT |
| --- | --- | --- |
| Meridian & Succession Configuration Voice Meridian & Succession |  |  |

About this document ...

If you do not want to receive future updates of this document, please de-register by sending your name, address, and OUC to the ISIS Helpdesk.

- Author

The author of this document may be contacted at:

Dave Wheeler Convergent Solutions, Technical Services Convergent Solutions Technical Services (IVQR5) Post Point pp 12 Technical Services 5 Brindley Place Birmingham West Midlands B1 2BL

Tel: 0121 230 8873 Fax: 0121 230 8645 Email: dave.wheeler@bt.com

- Content approval

This is Issue 7 of this document.

The information contained in this document was approved for use

Approver Signature

- Filing

The filing reference for this document is CFN177E.

- History

| Issue | Date | Author | Reason |
| --- | --- | --- | --- |
| Issue 7 | 23-Jan-2004 | Dave Wheeler | Update information for Release 24 & 25. Inclusion of Succession Release 3. |
| Issue 6 | 2-Jan-2003 | Mike Styles | Additional information about Option 11c Release 25.40/40B Patching information, Release 25.15 & 25.40 |
| Issue 5 | 10-Aug-2000 | Mike Styles | Correction of Option 11 Midi Overlay info |
| Issue 4 | 9-Aug-2000 | Kevin Lane | Inclusion of Release 25. |
| Issue 3 | 26-Sep-97 | Kevin Lane | Upissue by Mike Styles (MOC) |
| Issue 2 | 13-Feb-97 | K Lane | re-issued |

Contents

1	Introduction	6

1.1	Naming Convension	6

2	Option  11C Cabinet, Succession 1000M Cabinet, Option 11C Chassis and Succession 1000	7

2.1	PARM and OVLY (Overlay 17).	7

2.2	Call Registers	7

2.3	Intu (overlay 97)	8

2.4	Mail	8

2.5	Conference loop	9

2.6	Hardware compatibility	9

3	Meridian 1 51C and Succession 1000M Half Group	11

3.1	PARM and OVLY (Overlay 17)	11

3.2	Call Registers (NCR)	12

3.3	Succession 3 Memory requirements	12

4	Meridian 1 61C and Succession 1000M Single Group	13

4.1	PARM and OVLY (Overlay 17)	13

4.2	Call Registers (NCR)	14

4.3	Succession 3 Memory requirements	15

4.4	Xct (overlay 17)	15

5	Meridian 1 81C and Succession 1000M Multi Group	15

5.1	PARM and OVLY  (Overlay 17).	15

5.2	Call Registers (NCR)	16

5.3	XCT (overlay 17)	17

6	Legacy Systems	17

6.1	Option 11 and 11E	17

6.2	Option 51 Omega processor	19

6.3	Call Registers	19

6.4	Option 61, Omega Processor	19

6.5	Option 71 Omega processor (Overlay 17)	20

6.6	Option 81	22

7	Digital trunk configuration	23

7.1	DASS and DPNSS	23

7.2	PRI Alarm Threshold and Timers DASS & DPNSS (Overlay 74)	23

7.3	Q931 links.	24

7.4	PRI2 Alarm Threshold and Timers Q931 (Overlay 73)	24

7.5	Pri2 pad values (overlay 73)	24

7.6	Dti2 system timers chas (overlay 73)	25

7.7	Dti loop timers chas (overlay 73)	25

7.8	Euro – ISDN (PSTN)	25

7.9	QSIG	29

7.10	Example of Meridian rlse 25 QSIG to CISCO	29

8	Acd	35

9	Network ACD	35

10	CDR (all systems)	35

11	Traffic measurements	36

12	System security	36

13	Transmission levels	36

13.1	IP Telephony	36

13.2	IP Set SLR & RLR Values for the UK (Succession 3.00 Only)	37

14	Customer data block (cdb - overlay 15)	38

15	Meridian Mail DSP parameters	38

16	Meridian 1 supported software releases	39

# Introduction

This document has been updated in line with the recently introduced naming convention and is intended to serve as a guide to BT people who may be involved with the planning, configuration, installation or maintenance of Meridian systems. It is designed to cover those configuration parameters that are frequently misunderstood and often set incorrectly. Failure to make the recommended changes will affect service and may result in an excess of alarm messages.

NB.  Remember it is possible to set Buffers and Call Registers too high as well as too low. If call registers are set too high, applications such as CCR can slow down, or even stop completely. If set too low, delayed dial tone or system freeze can be experienced. If changes are made to buffers or call registers, the system will need to be initialised to make these changes effective. After the initialise, recheck the memory available figures (udata & pdata) to ensure sufficient memory is still available. If the figures are very low, that is less than 2000, contact the MSSC for advice. If you have any queries about this document, please contact MSSC Meridian Technical Support on Tel: 0800 622396,  Fax: 0121 633 0716.

## Naming Convension

Old Name		New Name		With Signalling Server connected

Option 11C		Option 11C Cabinet	Succession 1000M Cabinet

Option 11C Mini	Option 11C Chassis

Option 51C		Meridian 51C		Succession 1000M Half Group

Option 61C		Meridian 1 61C		Succession 1000M Single Group

Option 81C		Meridian 1 81C		Succession 1000M Multi Group

# Option  11C Cabinet, Succession 1000M Cabinet, Option 11C Chassis and Succession 1000

## PARM and OVLY (Overlay 17).

| LPIB | 150 | Release 24 for Option 11C Cabinet and Option 11C Chassis* |
| --- | --- | --- |
|  | 450 | Release 25 **and Succession 1000M Cabinet, Chassis and Succession 1000*** |

| HPIB | 150 | Release 24 for Option 11C Cabinet and Option 11C Chassis* |
| --- | --- | --- |
|  | 450 | Release 25 **and Succession 1000M Cabinet, Chassis and Succession 1000*** |

*	Source – Capacity Engineering 553-3001-149 release 24

**	Source – Capacity Engineering 553-3001-149 release 25.40

***	Source – Succession Enterprise release 3.00 GRB 2003_368

| 500B | 100 | N/A (set by system to 500 and values entered here are ingnored) |
| --- | --- | --- |
| SLIB | 16 | This is the lowest value possible. It is not used by UK systems. |
| DTIB | 70 | Use this value regardless of the number of links. |
| DTOB | 70 | Use this value regardless of the number of links. |
| PCDR | NO |  |
| BKGD | 44 |  |
| DROL | 30 43 44 60* |  |

- Overlay 60 should only be included if Q931 or QSIG or EURO ISDN or CHAS are equipped.

## Call Registers

| NCR | 1000 | Release 23 S/W |
| --- | --- | --- |
|  | 1750 | Release 24 S/W * |
|  | 1750 | Release 25 S/W ** |
|  | 1750 | Succession 1000M Cabinet/Chassis |
|  | 1750 | Succession 1000 |

*	Source – Capacity Engineering 553-3001-149 release 24

** 	Source – GRB release 25.40B

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 200 |  |
| CSQO | 200 |  |

## Intu (overlay 97)

- INTU NO

NB. Most customers prefer to have no conference warn tone (NO). However, if INTU is set to YES, the warn tone may result in corruption of coincident MF4 tones.

## Mail

Option 11’s often have Mail configured but no hardware installed, resulting in an excess of VAS and AUD messages, which may confuse attempts to trace genuine service affecting problems.

To identify sites where Mail is configured but not physically provided, proceed as follows:

Load Overlay 32 and STAT loop 10. If Mail is configured, several TNs will be shown equipped. If any of the TNs are shown as BCS MSB LOG OUT, then almost certainly, Mail is not physically provided, however :-

If remotely accessing the Meridian :  there is a possibility that Mail is there, but is faulty, (E.g. Powered down)  so to make absolutely certain, it will be necessary to confirm this detail with the customer.

If in a Configuration Centre : To confirm, check the order details.

If on site : To confirm, remove the cabinet covers and look.

If Mail is configured but not physically provided, it should be removed. The procedure is a little involved, so if unsure, please contact the MSSC for advice, otherwise proceed as follows:

- Ensure there are no system alarms.

- Load overlay 43 and dump. (LD 43, EDD)

- Release 16 LD 48, DIS ESDI 8 and DIS ESDI 9

Release 18 onwards LD 48 DIS AML 9

- LD 32 and STAT 10 to identify the TNs of the Mail agents.

- LD 11 and OUT the 2008 sets indicated in the previous step.

- LD 32 and STAT 10 to confirm that all agents have been removed.

- LD 23 and OUT the Mail voice messaging ACD queue. (Normally 7000)

- LD 17  (NB. The order is critical)

Release 16 OUT TTY 8

At the VAS prompt, enter CHG then at VSID enter 9 and at CMS enter X9 and carriage return to the end of the overlay.

- OUT TTY 9

At the VAS prompt, enter OUT then at VSID enter 9 and carriage return to the end of the overlay.

Release 18 onwards

- OUT TTY 8

- OUT AML 9

- OUT VSID 9

## Conference loop

Single cabinet Option 11’s sometimes have loop 31 configured as a Conference loop, but it should in fact only be configured if the switch is a dual cabinet system. To verify a single cabinet, if remotely accessing the system, load Overlay 32 and STAT loops 11 through to 20 in turn, they should all come back as unequipped. If in a Configuration Centre check the order details. If on site, look for a second cabinet.

To remove the configuration for loop 31, load Overlay 17, and respond X31 at the CONF prompt.

## Hardware compatibility

The following table shows Option 11C software daughterboards & the software releases that they are compatible with. New orders will receive the new NTTK25AA software daughterboard.

| Software Daughterboard: | Loaded with S/W Release | Capacity | Compatible with S/W |
| --- | --- | --- | --- |
| NTSK04AC | 22-46 | 32 Meg | 22-46 |
| NTSK04AG | 23-47 | 32 Meg | 23-47 |
| NTSK04AK | 24-25 | 40 Meg | 24-25 |
| NTDK21AA | Blank | 32 Meg | Rlse 22.08D – 23.47 |
| NTDK81AA | Blank | 40 Meg | Rlse 23.47 – 24.25 |
| NTTK13AA | Blank | 48 Meg | Rlse 24.25 – 25.XX and Succession 3 |
| NTTK25AA | Blank | 48 Meg | 25.XX and Succession 3 |

| System | Minimum SSC/MSC | Compatible with S/W |
| --- | --- | --- |
| 11C/Cabinet | NTDK20CA | Rlse 24.25 with 4 –5 cabinets – 25.XX and Succession 3 |
|  | NTDK20EA | Main cabinet when using IP Expansion and … |
|  | NTDK20CA | For each IP Expansion cabinet |
| Chassis | NTDK97AB |  |
|  | NTDK20EA | Main and IP Expansion chassis when using IP Expansion |
| Succession 1000 | NTDK20EA | Call Server and Media Gateway |
| Succession Branch Office | NTDK20FA |  |

An existing Option 11C system equipped with a controller card NTDK20AB can be upgraded to an NTDK20CA with a field upgrade kit NTDK19AA. As of July 1999, all controller cards shipped to the field have been a minimum of NTDK20DA, which is compatible with Release 25 and Succession 3.

A system upgrading to Release 25 or Successsion 3 must replace its NTDK21(32 Mb) or NTDK81(40 Mb) based software daughterboard, with an NTTK13AA (48 Mb) or NTTK25AA daughterboard. Please ensure that you have upgraded the bootcode on the SSC prior to upgrading the software daughterboard. To verify the size of the software daughterboard and for installation instructions, refer to NTP upgrade manual Chapter 9.

The Succession Branch Office Call Server requires a 48MB Total Memory (32 MB Program Store and 16 MB CDrive

space) in order to run Succession 3.0 Software.

# Meridian 1 51C and Succession 1000M Half Group

## PARM and OVLY (Overlay 17)

| < X11 24 | LPIB | HPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | 192 | 64 | 500 | 40 |
| CP2 | 192 | 64 | 500 | 40 |
| CP3/4 | 192 | 64 | 500 | 40 |

| X11 24 * | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 1850 | 1850 | 800 | 255 |
| CP3/4 | 1850 | 1850 | 800 | 255 |

| X11 25 ** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 2200 | 2200 | 800 | 255 |
| CP3/4 | 2200 | 2200 | 800 | 255 |

| X21 3.00 *** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 3500 | 3500 | 800 | 255 |

*	Source – Capacity Engineering 553-3001-149 release 24

**	Source – Capacity Engineering 553-3001-149 release 25.40

***	Source – Succession Enterprise release 3.00 GRB 2003_368

| NDIS | 20 |  |
| --- | --- | --- |
| DTIB | 70 |  |
| DTOB | 70 | 1 to 4 DPNSS/DASS links |
|  | 100 | 5 or more DPNSS/DASS links |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60 135 137 |  |

(Overlay 60 should only be included if Q931 or QSIG or EURO ISDN or CHAS are equipped).

## Call Registers (NCR)

| Release 23 S/W | CP1 | 1000 |
| --- | --- | --- |
| Release 23 S/W | CP2 | 1500 |
| Release 23 S/W | CP3 | 2000 |
| Release 23 S/W | CP4 | 2000 |
| Release 24 S/W * | CP2 | 1500 |
| Release 24 S/W * | CP3 | 2000 |
| Release 24 S/W * | CP4 | 2000 |
| Release 25 S/W ** | CP2 | 1500 |
| Release 25 S/W ** | CP3 | 2000 |
| Release 25 S/W ** | CP4 | 2000 |
| Succession Release 3 *** | CP3 and CP4 | 2000 |

CP1 - NT6D66),  CP2 - NT9D19 - CP3 - NT5D10 CP4 - NT5D03, CPP II - NT4N64

*	Source – Capacity Engineering 553-3001-149 release 24

** 	Source – GRB release 25.40

***	Source – Large System Planning and Engineering 553-3021-120

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255 |  |
| CSQO | 255 |  |

## Succession 3 Memory requirements

Succession 3 requires 64 Flash memory and 64 DRAM - 128 Total

|  | Flash | DRAM | Total |
| --- | --- | --- | --- |
| CP3/4 | 64 | 64 | 128 |

# Meridian 1 61C and Succession 1000M Single Group

## PARM and OVLY (Overlay 17)

| < X11 24 | LPIB | HPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | 192 | 64 | 500 | 40 |
| CP2 | 192 | 64 | 500 | 40 |
| CP3/4 | 192 | 64 | 500 | 40 |

| X11 24 * | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 1850 | 1850 | 800 | 255 |
| CP3/4 | 1850 | 1850 | 800 | 255 |

| X11 25 ** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 2200 | 2200 | 800 | 255 |

| X21 3.00 *** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 3500 | 3500 | 800 note 1 | 255 |
| CPP II | 3500 | 3500 | 2000 note 1 | 255 |

*	Source – Capacity Engineering 553-3001-149 release 24

**	Source – Capacity Engineering 553-3001-149 release 25.40

***	Source – Succession Enterprise release 3.00 GRB 2003_368

note 1	Source – Large System Planning and Engineering

| NDIS | 20 |  |
| --- | --- | --- |
| DTIB | 70 |  |
| DTOB | 70 100 | 1 to 4 DPNSS/DASS links 5 or more DPNSS/DASS links |
| PCDR | No |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60 135 137 | (Overlay 60 should only be included if Q931 or QSIG or EURO ISDN or CHAS are equipped). |
| AML | T1 T2 T3 N1 N2 K | 4 10 5 128 8 7 |

## Call Registers (NCR)

| Release 23 S/W | CP1 | 2000 |
| --- | --- | --- |
| Release 23 S/W | CP2 | 3000 |
| Release 23 S/W | CP3 / CP4 | 4000 |
| Release 24 S/W * | CP2 | 3000 |
| Release 24 S/W * | CP3 / CP4 | 4000 |
| Release 25 S/W ** | CP3 / CP4 | 4000 |
| Succession rlse 3 *** | CP3 / CP4 | 4000 |
| Succession rlse 3 *** | CPP II | 20000 |

CP1 - NT6D66, CP2 - NT9D19 - CP3 - NT5D10 CP4 - NT5D03, CPP - NT4N64

*	Source – Capacity Engineering 553-3001-149 release 24

** 	Source – GRB release 25.40

***	Source – Large System Planning and Engineering 553-3021-120

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255 |  |
| CSQO | 255 |  |

## Succession 3 Memory requirements

|  | Flash | DRAM | Total |
| --- | --- | --- | --- |
| CP3/4 | 64 | 64 | 128 |
| CPP |  | 256 DIMM | 256 |

## Xct (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

# Meridian 1 81C and Succession 1000M Multi Group

## PARM and OVLY  (Overlay 17).

| < X11 24 | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 64(Number of groups) | 300 | 1000 | 40 |
| CP3/4 | 64(Number of groups) | 300 | 1000 | 40 |

| X11 24 * | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 1850 | 1850 | 800 | 255 |
| CP3/4 | 1850 | 1850 | 800 | 255 |

| X11 25 ** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 1 – 5 groups | 2200 | 2200 | 800 | 255 |
| CP4 6 – 8 groups | 3300 | 3300 | 800 | 255 |
| CPP II 1 – 5 groups | 2200 | 2200 | 800 | 255 |
| CPP II 6 – 8 groups | 3500 | 3500 | 800 | 255 |

| X21 3.00 *** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 3500 | 3500 | 800 note 1 | 255 |
| CPP II 1 – 8 groups | 3500 | 3500 | 2000 note1 | 255 |

*	Source – Capacity Engineering 553-3001-149 release 24

**	Source – Capacity Engineering 553-3001-149 release 25.40

***	Source – Succession Enterprise release 3.00 GRB 2003_368

note 1	Source – Large System Planning and Engineering

| NDIS | 20 |  |
| --- | --- | --- |
| DTIB | 70 |  |
| DTOB | 70 100 | 1 to 4 DPNSS/DASS links 5 or more DPNSS/DASS links |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60 135 137 |  |

(Overlay 60 should only be included if Q931 or QSIG or EURO ISDN or CHAS are equipped).

## Call Registers (NCR)

| Release 23 S/W | CP2 | 7500 |
| --- | --- | --- |
| Release 23 S/W | CP3 / CP4 | 10000 |
| Release 24 S/W | CP2 | 7500 |
| Release 24 S/W | CP3 / CP4 | 10000 |
| Release 25 S/W | CP2 | 7500 |
| Release 25 S/W | CP3 / CP4 | 10000 |
| Release 25 S/W with 1-5 groups | CPP | 20000 |
| Release 25 S/W with 5-8 groups | CPP | 25000 |
| Succession 3 S/W with 1-5 groups | CPP | 20000 |
| Succession 3 S/W with 5-8 groups | CPP | 25000 |

CP1 - NT6D66, CP2 - NT9D19 - CP3 - NT5D10 CP4 - NT5D03, CPP - NT4N64

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255 |  |
| CSQO | 255 |  |

## XCT (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

# Legacy Systems

## Option 11 and 11E

### PARM and OVLY (Overlay 17)

| LPIB | 96 |  |
| --- | --- | --- |
| HPIB | 32 | 16 for pre 16.91 software. |
| 500B | --- | Set to lowest value (ignored by system). |
| SLIB | 16 | This is the lowest value possible. It is not used by UK systems. |
| DTIB | 0 | Not required for Option 11, but once set, it cannot be restored to zero, in which case set it to 35. If set to high values, it will use up a large amount of memory and memory is often at a premium on Option 11s. |
| DTOB | 35 | Use this value regardless of the number of links. |
| PCDR | NO |  |
| BKGD | 44 |  |
| DROL | 30 43 44 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

### Call registers.

The number configured will depend on several variables. Use the following tables as a guide but never reduce existing values without first consulting MOC Technical Support;

#### Systems without ACD

| Number of : | 40 Extns. | 80 Extns. | 160 Extns. | 208 Extns. |
| --- | --- | --- | --- | --- |
| 8 Trunks | 40 | - | - | - |
| 16 Trunks | - | 56 | - | - |
| 32 Trunks | - | - | 88 | - |
| 40 Trunks | - | - | - | 106 |

#### Systems with ACD

| Number of ACD agents | 10 | 20 | 40 | 40 | 80 |
| --- | --- | --- | --- | --- | --- |
| Number of ordinary extensions | 40 | 80 | 40 | 80 | 80 |
| Number of ACD Trunks | 6 | 12 | 0 | 8 | 0 |
| Number of non ACD Trunks | 12 | 24 | 48 | 48 | 96 |
| Number of Call Registers | 54 | 84 | 95 | 111 | 166 |
| Number of Call Registers if ACD Max | 79 | 109 | 120 | 136 | 191 |

MGCR 25 Only if ACD Max is equipped (If no Max, set to 0)

If CCR or/and Link are equipped, then two more parameters will need to be configured as follows;

| CSQI | 40 |
| --- | --- |
| CSQO | 40 |

- If Mail only is equipped, the above parameters should be configured as follows;

| CSQI | 20 | (default) |
| --- | --- | --- |
| CSQO | 20 | (default) |

All input/output devices (IOTB) should have USER configured as XBUG with the exception of the history file (i.e. ADAN = HST) which should be configured for BUG.

## Option 51 Omega processor

### PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| LPIB | 96 |  |
| HPIB | 32 |  |
| 500B | (A+B)8100 | where A = total number of 500 sets B = total number of digital sets |
| SL1B | 40 |  |
| DTIB | 35 70 | for non expanded mode DDSLs for expanded mode DDSLs |
| DTOB | 35 70 100 | for 1 or 2 digital links for 3 or 4 digital links for 5 links or more |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 35 43 44 45 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

## Call Registers

| NCR | 600 | This is a minimum value. Never reduce existing values. |
| --- | --- | --- |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 100 |  |
| CSQO | 100 |  |

## Option 61, Omega Processor

### PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| LPIB | 96 |  |
| HPIB | 32 |  |
| 500B | (A+B)8100 | where A = total number of 500 sets B = total number of digital sets |
| SL1B | 40 |  |
| DTIB | 35 70 | for non expanded mode DDSLs for expanded mode DDSLs |
| DTOB | 35 70 100 | for 1 or 2 digital links for 3 or 4 digital links for 5 links or more |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 35 43 44 45 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

### Call Registers

| NCR | 600 | This is a minimum value. Never reduce existing values. |
| --- | --- | --- |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 100 |  |
| CSQO | 100 |  |

### XCT(overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

## Option 71 Omega processor (Overlay 17)

### PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| NCR | 1200 | This is a minimum value. Never reduce existing values |
| LPIB | 96 |  |
| HPIB | 32(Number of groups) |  |
| 500B | (A+B)8100 | where A = total number of 500 sets B = total number of digital sets |
| SL1B | 40 |  |
| DTIB | 35 70 | for non expanded mode DDSLs for expanded mode DDSLs |
| DTOB | 35 70 100 | for 1 or 2 digital links for 3 or 4 digital links for 5 links or more |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 35 43 44 45 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

### Registers

| NCR | 1200 | This is a minimum value. Never reduce existing values |
| --- | --- | --- |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 200 |  |
| CSQO | 200 |  |

### XCT (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

## Option 81

### PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| LPIB | 300 |  |
| HPIB | 64(Number of groups) |  |
| 500B | 1000 |  |
| SL1B | 40 |  |
| DTIB | 70 |  |
| DTOB | 70 100 | 1 to 4 DPNSS/DASS links 5 or more DPNSS/DASS links |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60 135 137 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

### Call Registers

| NCR | Release 23 S/W | CP1 | 5000 |
| --- | --- | --- | --- |
|  | Release 23 S/W | CP2 | 7500 |
|  | Release 23 S/W | CP3 / CP4 | 10000 |
|  | Release 24 S/W | CP2 | 7500 |
|  | Release 24 S/W | CP3 / CP4 | 10000 |
|  | Release 25 S/W | CP2 | 7500 |
|  | Release 25 S/W | CP3 / CP4 | 10000 |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255 |  |
| CSQO | 255 |  |

### XCT (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

# Digital trunk configuration

NB. Some digital trunk parameters cannot be changed until the link is taken out of service.  Changes to default values are shown in bold.

## DASS and DPNSS

On big switches (Options 51,61,71,81), when using expanded mode addresing, the “D” channel interface card must be fitted in the correct network group and shelf for the DDSL address being used.

For example,

DDSL 0 to 15 Network Group 0 Shelf 0

DDSL 16 to 31 Network Group 0 Shelf 1

DDSL 32 to 47 Network Group 1 Shelf 0

DDSL 48 to 63 Network Group 1 Shelf 1

DDSL 64 to 79 Network Group 2 Shelf 0

DDSL 80 to 95 Network Group 2 Shelf 1

etc.

## PRI Alarm Threshold and Timers DASS & DPNSS (Overlay 74)

| Alarm Mnemonic | Persistence Time |  | Monitor Time (hours) |  | Count Threshold |
| --- | --- | --- | --- | --- | --- |
| TBF | 5 seconds |  | 0 |  | 1 |
| FAE | 2 seconds |  | 1 |  | 4 |
| HER | 1 minute |  | 1 |  | 10 |
| TSF | 0 minutes |  | 0 |  | 0 |
| AIS | 1 minute |  | 1 |  | 6 |
| LOI | 6 seconds |  | 1 |  | 6 |
| DAI | 1 minute |  | 1 |  | 6 |
| Counter Mnemonic |  | DASS |  | DPNSS |  |
| CRT |  | 120 |  | 120 |  |
| TMT |  | 50 |  | 50 |  |
| SCT |  | 20 |  | 20 |  |

## Q931 links.

When configuring Q931 “D” channels in overlay 17, the RCAP prompt defines the remote capabilities of the “D” channel. Items such as Call Party Named Display and Network call trace are configured here. If the remote end of the link uses an MSDL card for the “D” channel interface this will also need to be defined here.

If Basic Rate Access (BRI) is used across the link, RCAP should also specify BRI.

The Release ID (overlay 17) should be set to the lowest release of software in the network . This applies to all “D” channels on all sites in the network.

## PRI2 Alarm Threshold and Timers Q931 (Overlay 73)

| SLP | 5 | 24H | 255 | 1H |
| --- | --- | --- | --- | --- |
| BVP | 128 | 122 |  |  |
| CRC | 201 | 97 |  |  |

| FAP | 28 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| GP2 | 50 | 100S | 12S | 12S | 4S |
| MNG1 | 30S |  |  |  |  |

| NCG1 | 30s |
| --- | --- |
| OSG1 | 1M |
| MNG2 | 30S |

| NCG2 | 30S |
| --- | --- |
| OSG2 | 30S |
| PERS | 50 |
| OOSC | 0 |

## Pri2 pad values (overlay 73)

The following pad values are recommended for Release 16 software or later:-

|  | RECEIVE PAD (x) | TRANSMIT PAD (y) |
| --- | --- | --- |
| ONP | 0 | 0 |
| OPX | 0 | 0 |
| DTT | 0 | 0 |
| DCO | 0 | 0 |
| NTC | 0 | 0 |
| TRC | 0 | 0 |
| DTR | 0 |  |
| VNL | 0 | 0 |
| ACO | 0 | 0 |
| AFX | 0 | 0 |
| ADD | 0 | 0 |
| PR1 | 0 | 0 |
| DSET | 0 | 0 |
| BRIL | 0 | 0 |
| BRIT | 0 | 0 |

## Dti2 system timers chas (overlay 73)

| MAND | 1M |
| --- | --- |
| NCSD | 30S |
| OSGD | 1M |
| OOSC | 0 |
| PERS | 100 |
| CCGD | 1M |
| CCAR | 1M |

## Dti loop timers chas (overlay 73)

Group 1 Impairments

| BPV | 205 | 10S | 3S | 3S | 1S |
| --- | --- | --- | --- | --- | --- |
| FAP | 32 | 4S | 1S | 1S | 100T |
| SLP | 100 | 30S | 10S | 10S | 6S |

Group 2 Impairments

| GP2 | 50 | 100S | 12S | 12S | 4S |
| --- | --- | --- | --- | --- | --- |

## Euro – ISDN (PSTN)

A sample configuration for Option 11C is shown below:

The most important prompts are highlighted.

- LD 17

- TYPE PARM

BCAP 	SPEE  **Bearer Capability = Speech

or

- 31KH **Bearer Capability = 3.1Khz Voice

- TYPE ADAN

ADAN NEW DCH **

CTYP MSDL **Multipurpose Serial Data Link**

CARD ** Insert Card No

- PORT 1

- DES EURO_ISDN

USR PRI **D-Channel for ISDN PRI**

DCHL * Insert Loop No

- OTBF 32

- PARM RS422 DTE

DRAT 64KC

CLOK EXT

NASA NO

IFC E403 **Interface type for D-Channel**

CNTY ETSI **Country Type**

- PINX_CUST 0

- ISDN_MCNT 300

CLID OPT1

PROG NCHG **Send Progress Message**

- CO_TYPE STD

- SIDE USR **USR = default** **NET for Master Mode**

- CNEG 2

- RLS ID

RCAP COLP **Supports Connected Line ID**

- LD 17 CONT.

- MBGA NO

- OVLR NO

OVLS NO

T310 120

INC_T306 120

OUT_T306 120

T200 3

T203 10

N200 3

N201 260

K 7

LD 73

TYPE PRI2

FEAT LPTI

LOOP ** Insert loop No

MFF CRC **Multiframe Format = CRC (Cyclic Redundancy Check)**

- ACRC YES **Auto report or CRC-4 error**

ALRM ALT **Alternate firmware alarm handles**

RAIE NO

G1OS YES

**Set all timers to default until further notice**

SLP 5 24 H 30 1 H

BPV 128 122

CRC 201 97 **Cyclic Redundancy Check error counts**

FAP 28 1

RATS 10

GP2 20 100 S 12 S 12 S 4 S

MNG1 15 M

NCG1 15 M

OSG1 15 M

MNG2 15 S

NCG2 15 S

OSG2 15 S

PERS 50

CLRS 1

OOSC 5

LD 16

TYPE RDB

CUST 00

DMOD

ROUT *** Insert Route No

DES EURO_ISDN

- TKTP DID

- SAT NO

RCLS EXT

DTRK YES **Digital Trunk Route**

- BRIP NO

DGTP PRI2 **Digital trunk Type for Route**

- ISDN YES

- MODE PRA

- IFC E403 **Interface Type for Route**

CNTY ETSI **Country Type*

SBN NO

PNI 00000

NCNA NO

NCRD NO

ISAR NO

CPFXS YES

ADDP NO

DSEL VOD

PTYP DCO

AUTO NO

DNIS NO

- ICOG IAO

- RANX NO

SRCH RRB

TRMB YES

STEP

ACOD 1600

TARG **

**All other responses in RDB should be set to default or customer specific**

LD 14

TN ** Insert T.N

- TYPE DID

- CDEN SD

CUST 0

- TRK PRI2

- PDCA 1 **Pad Category table number**

- PCML A **Pulse Code Modulation Law (A Law)**

NCOS 7

RTMB 100 1

B-CHANNEL SIGNALING

NITE

CLS UNR DTN WTA LPR APN

P10 VNL

TKID

DTCR NO

DATE 4 FEB 1997

LD 15

TYPE CDB

CUST 00

LDN_DATA

OPT XLDN

DLDN YES

LDN0 **** **LDN0 must be defined for ISDN PRI DID Service .The length

LDA0 of LDN0 determines the number of trailing digits translated as the

LDN1 DN on PRI DID routes**

- LDA1

- LDN2

LDA2

LDN3

LDA3

LDN4

LDA4

LDN5

LDA5

ICI 00 LD0

ICI 01

ICI 02

ICI 03

ICI 04

ICI 05

ICI 06

ICI 07

ICI 08

ICI 09

## QSIG

Like all Meridian networking solutions, QSIG has numerous permutions in which the configuration can be applied. The essential thing to understand is that, which ever way is chosen, the remote system needs to have the ability to recognise the protocol it is receiving and visa versa, therefore the only right way to configure QSIG is one which is built on common ground between two systems.

Since release 23, all QSIG Supplementary Services have provided the user with the choice of ROSE APDU Operation Coding rule : Integer Values or Object Identifiers (for example QSIG SS Path Replacement PRI/PRO etc.). However, two supplementary services, developped in release 22, offered only the Object Identifier option :

• QSIG Supplementary Service - Name Display

• QSIG Supplementary Service - Call Completion

From release 24, these supplementary services offer the possibility of choosing Integer Value option for ROSE APDU operation coding.

There is no functional change vs Release 22 versions.

## Example of Meridian rlse 25 QSIG to CISCO

Patches must include the following however please check the latest at time of instalation

PAT#  PRS      PATCH REF #      NAME             DATE      FILENAME

p16179_1.cp4

p14000_3.cp4

p14007a3.cp4,  p14007b3.cp4

p16131a1.cp4,  p16131b1.cp4,  p16131c1.cp4,  p16131d1.cp4

p17448

Packages must include

TYPE pkg

MSDL   222

MSDL SDI   227

MSDL STA   228

QSIG   263

QSIGGF   305

QSIG-SS   316

ETSI-SS   323

MEET   348

(Use numbers and loops etc to suit site)

D Channel config example

ADAN     DCH 21

CTYP MSDL

DNUM 15

PORT 1

DES  EGF4

USR  PRI

DCHL 21

OTBF 32

PARM RS422  DTE

DRAT 64KC

CLOK EXT

IFC  EGF4

PINX_CUST 0

ISDN_MCNT 300

CLID OPT0

CO_TYPE  STD

SIDE USR (Must be USR when connected to router)

CNEG 1

RLS  ID  25

QCHID YES

RCAP NCT MWI COLP NDI CCBI CCNI PRI DV3I MQC CTI

PR_TRIGS DIV 2 3

CNG 2 3

CTR2 2 3  (Use XCTR to delete CTR1 then enter CTR2 2 3)

PR_RTN NO   (YES will retain the first leg of a redirected call)

MQC_FEAT NAS NACD NMS  (depending on packages)

MBGA NO

OVLR NO

OVLS NO

T310 120

T200 3

T203 10

N200 3

N201 260

K    7

CDB Example

NET_DATA

OPT RTD

AC2 LOC

FNP YES

ISDN YES

PNI 112		(match to existing PNI number)

PINX_DN 2415	(DN must be spare extn in site number range diallable

from remote sites, although NU will result)

MBG 0

BSGC 65535

PFX1

PFX2

HLOC

LSC	(usually 3 digit site code if applicable)

RCNT 5

PSTN NO

TNDM 15

PCMC 15

SATD 1

OCLI NO

TIDM NO

DASC

ROPT NRO

DITI YES

TRNX NO

EXTT NO

FTOP FRES

APAD 0  0

VNR NO

NIT 8

NAS ATCL YES

NAS ACTV NO

FOPT 0

CNDN

CNAT

CNIP YES

ODN8

ODN9

ASTM 30

HDOPT 0

HDTM 30

Route data block example

TYPE RDB

CUST 00

DMOD

ROUT 120

DES  QSIGGF

NPID_TBL_NUM   0

ESN  NO

RPA  NO

CNVT NO

SAT  NO

RCLS EXT

DTRK YES

BRIP NO

DGTP PRI2

ISDN YES

MODE PRA

IFC  EGF4

SBN  NO

PNI  00112  (to match existing)

NCNA yes

NCRD yes

CTYP CDP

INAC NO

ISAR NO

CPFXS YES

DAPC NO

INTC NO

DSEL VOD

PTYP DTT

AUTO NO

DNIS NO

DCDR NO

ICOG IAO

SRCH LIN

TRMB NO

STEP

ACOD 1920  (to suit)

TCPP NO

TARG 01

BILN NO

OABS

INST

IDC  YES	(	)

DCNO 2 *	(To suit)

NDNO 2	(	)

DEXT NO

SIGO STD

MFC  NO

ICIS YES

OGIS YES

TIMR ICF 1920

OGF 1920

EOD 13952

LCT 256

NRD 10112

DDL 70

ODT 4096

RGV 640

GTO 896

GTI 896

SFB 3

PRPS 800

NBS 2048

NBL 4096

IENB 5

TFD 0

VSS 0

VGD 6

DTD  NO

SCDT NO

2 DT NO

DRNG NO

CDR  YES

INC  YES

LAST YES

TTA  YES

ABAN YES

CDRB NO

QREC NO

OAL  YES

AIA  YES

OAN  YES

OPD  NO

NDP  EXC 0

CDRX NO

NATL YES

SSL

CFWR NO

IDOP NO

VRAT NO

MUS  NO

PANS YES

FRL  0 0

FRL  1 0

FRL  2 0

FRL  3 0

FRL  4 0

FRL  5 0

FRL  6 0

FRL  7 0

OHQ  NO

OHQT 00

CBQ  NO

AUTH NO

TTBL 0

PLEV 2

OPR  NO

ALRM NO

ART  0

PECL NO

DCTI 0

TIDY 1920 120

SGRP 0

AACR NO

Trunk member Example

TN   021 01

TYPE TIE

CDEN SD

CUST 0

TRK  PRI2

PDCA 3	(may require new pad table as this can amended without affecting other routes)

PCML A

NCOS 7

RTMB 120 1

B-CHANNEL SIGNALING

TGAR 1

AST  NO

IAPG 0

CLS  UNR DTN CND WTA LPR APN THFD XREP BARD

P10 VNL

TKID

***** ALL QSIGGF RLI ENTRIES MUST HAVE SBOC SET WITH COPT = 1 OR 2 *****

This is in order for alternate routing to RLI entries 1,2 3 etc to work

# Acd

It is recommended that the prompts in LD 23 for call forcing, FCFT,FADT,FADR should be treated as mutually exclusive. Having a value set for more than one prompt will cause incorrect call distribution amongst agents. FCFT is the preferred prompt to use, with FADT and FADR being set to none.

# Network ACD

When a Meridian is used in a Network ACD (NACD) Q931 environment, it is important to ensure that certain parameters are correctly configured as shown below:

- Overlay 17

- CLOK EXT

NASA YES

RLS ID 16 NB.

This parameter should be set to the lowest release of software in the network. In the example shown, this is Release 16. If all switches are at a minimum of Release 18, this parameter should be set to 18 etc.

- Overlay 16

These parameters must be set for all Q931 routes where NACD is used. Ideally, they should be altered at both ends of a link at the same time, otherwise messaging errors may occur.

- SIGO ESN5

- INAC YES

# CDR (all systems)

CDR should be configured at a minimum speed of 9600 Baud. Please see MOC Newsflash 96_016 before making this change to any Option 11E.

Within LD 16, the CDR prompt Time To Answer ( TTA ) set to YES adds an overhead to the processor load. Only set this prompt on necessary routes, it is not recommended for Music or Ran routes.

# Traffic measurements

The traffic measurement for processor load TFS 004 now has extra fields to help with indicating excessive Real Time Usage. If the Maximum Real Time Used figure exceeds 80%, the switch may suffer from call processing problems at peak times.

This enhancement is available from Release 18 software onwards on Big Switches (Omega and “C” processors) and Release 22 software onwards on Option 11 (i.e., the C processor variant.)

# System security

All recommendations on Meridian1 security can be found in the Meridian “System Security Guide”  P0806182. This guide is shipped with all new systems and Meridian Mail and is the definitive document from Nortel on Meridian1 security.

# Transmission levels

AOLR in Overlay 17 should be set to value 39.  This prompt is used in Release 22 or later software to set transmission levels for ACD agents sets.

## IP Telephony

To set the correct Loss Plan information the following Configuration / PEP insertion must be adhered to for Nortel IPT products.

| Product / sw Level | PEP Requirements | Configuration Requirements |
| --- | --- | --- |
| Succession Rls3.00 | None | LD 73 – Set DDB. Change Pad 15 to UK settings as per NTP. Set UK Specific IP Set Loss Plan on Media Cards. |
| Succession CSE 1k Rls.2.2 | MPLR - 17754 | LD 73 – Set DDB. Change Pad 15 to UK settings as per NTP. |
| IP Line on Sites with X11 25.40b software | MPLR - 16341 |  |
| IP Trunk on sites with X11 25.40b software | MPLR - 15721 |  |

Media Card Loss Plan Settings (Succession Rls.2 & 3)

**See NTP 553-3001-182 (Transmission Parameters)**

To Set DDB

LD 73

REQ NEW

TYPE DDB

TRSH 0

other prompts: <cr>

PDCA (PAD 15)

DDB will create a default PAD Table 15 which must be changed to the recommended UK settings.

LD 73

REQ PRT (CHG)

TYPE DTI

FEAT PAD

PDCA 15

ONP  	6 	5

DSET	 3	4

OPX 	n/a 	n/a

DTT 	0 	0

SDTT 	0 	0

DCO  	n/a 	n/a

DTO  	3 	19

VNL 	0 	0

SATT 	3 	19

ACO  	2 	16

ATO 	3 	19

PRI  	0 	0

PRI2 	0 	0

XUT 	2 	16

XEM  	2 	16

BRIL  	0 	0

BRIT 	0 	0

WRLS 	3 	19

## IP Set SLR & RLR Values for the UK (Succession 3.00 Only)

**See NTP 553-3001-365 (IP Line)**

When a system is installed in the UK, the command UKLossPlanSet is entered at the CLI of one Media card, once entered it automatically Syncronises all other media cards in that node. This adjusts the loss plan of the Internet Telephones to the higher transmit levels required in the UK.

Use the following steps to set the loss plan for the UK.

1 Either - telnet to the card, connect to the maintenance port,  use OTM 2.1 or Element Manager to access the Voice Gateway Media Card to gain access to the CLI.

2 Log into the IPL> shell.

3 At the IPL > CLI, enter the command UKLossPlanSet. Press <cr>.

4 Exit from the login session.

Other UK Loss Plan Commands can be found on Page 519 onwards of NTP 553-3001-365 (IP Line)**

# Customer data block (cdb - overlay 15)

RGNA in the CDB (MPO) should be set to STD STD.  It is normally AAR AAR.

This will prevent transferred calls to an extension which has FNA/EFD set to another extension from reverting back to the transferee when the forward no answer timer (ring cycles) matures rather than following the call forward treatment.

# Meridian Mail DSP parameters

During installation of Meridian Mail you have the option to change the DSP parameters.

The recommended settings are shown below:

Changes from the default settings are shown in bold.

| DSP Encoding Type | ALAW |
| --- | --- |
| Disable Silence compression: | No |
| Do you wish to change other DSP parameters? | YES |
| Transmit level: | 0 |
| Receive Level: | -7 |
| DTR Reject Level: | (-36) |
| DTR Max Accept Level: | (1) |
| Disable AGC: | No |
| AGC Centre: | -20 |
| Hook Flash Pulse: | 320 |
| Telescan Debounce: | (128) |
| Telescan Ring Time: | 1024 |
| Select operation: | Done |

# Meridian 1 supported software releases

The preferred software releases for investigation of new problems are shown below. If problems are reported on sites with earlier software releases, the MSSC may recommend that a  “bugfix upgrade” is performed to bring them up to this minimum level.  For further information please contact the MSSC.

| Machine Type and Software | Minimum Software release |
| --- | --- |
| Option 11 Release 18 | 18.42 |
| Option 11E Release 21 | 21.54 |
| Option 11C Release 23 | 23.47 |
| Option 11C Release 24 | 24.25 |
| Option 11C Release 25 | 25.15 |
| Option 51,61,71 (Omega) Release 16 | 16.92 |
| Option 51,61,71 (Omega) Rlse 18, Rlse 20 & Rlse 21 | 21.54 |
| Option 51C,61C,81,81C, Release 23 | 23.47 |
| Option 51C,61C,81,81C, Release 24 | 24.25 |
| Option 51C,61C,81,81C, Release 25 | 25.15 |

END OF DOCUMENT