- Product Support Note

Meridian & Succession Switches Configuration

- Go to Contents

- Contents

- Introduction

- Product overview

2.1 Option  11C Cabinet & Chassis, CS1000M Cabinet, CS1000M Chassis, CS1000S,MG1000E, MG1000B and MG1000T

2.1.1 PARM and OVLY (Overlay 17).

2.1.2 Minimum Recommended Call Registers

2.1.3 Intu (overlay 97)

2.2 Meridian 1 51C and CS1000M Half Group

2.2.1 PARM and OVLY (Overlay 17)

2.2.2 Call Registers (NCR)

2.2.3 Succession 3 and CS1000 Memory requirements

2.3 Meridian 1 61C and CS1000M Single Group

2.3.1 PARM and OVLY (Overlay 17)

2.3.2 Call Registers (NCR)

2.3.3 Succession 3 and CS1000 Memory requirements

2.3.4 Xct (overlay 17)

2.4 Meridian 1 81C and CS1000M Multi Group and CS1000E

2.4.1 PARM and OVLY  (Overlay 17).

2.4.2 Call Registers (NCR)

2.4.3 XCT (overlay 17)

2.5 RELEASE 4.00T and RELEASE 4.50W Memory Requirements

2.6 Legacy Systems

2.6.1 Option 11 and 11E

2.6.1.1 PARM and OVLY (Overlay 17)

2.6.1.2 Call registers.

Systems without ACD

Systems with ACD

2.6.2 Option 51 Omega processor

2.6.2.1 PARM and OVLY (Overlay17)

2.6.2.2 Call Registers

2.6.3 Option 61, Omega Processor

2.6.3.1.PARM and OVLY (Overlay17)

2.6.3.2 Call Registers

2.6.3.3. XCT(overlay 17)

2.6.4 Option 71 Omega processor (Overlay 17)

2.6.4.1  PARM and OVLY (Overlay17)

2.6.4.2 Registers

2.6.4.3 XCT (overlay 17)

2.6.5 Option 81

2.6.5.1 PARM and OVLY (Overlay17)

2.6.5.2 Call Registers

2.6.5.3 XCT (overlay 17)

2.7 Digital trunk configuration

2.7.1 DASS and DPNSS

2.7.2 PRI Alarm Threshold and Timers DASS & DPNSS (Overlay 74)

2.7.3 Q931 links.

2.7.4 PRI2 Alarm Threshold and Timers Q931 (Overlay 73)

2.7.5 Pri2 pad values (overlay 73)

2.7.6  Dti2 system timers chas (overlay 73)

2.7.7 Dti loop timers chas (overlay 73)

2.7.8 Euro – ISDN (PSTN)

2.7.9 Euro – ISDN (PSTN)

2.7.10  QSIG

2.7.11 Example of Meridian rlse 25 QSIG to CISCO

2.8 ACD

2.9 Network ACD

2.10 CDR (all systems)

2.11 Traffic measurements

2.12 System security

2.13 Transmission levels

2.13.1 IP Telephony

2.13.2 Media Card Loss Plan Settings (Succession Rls.2 & 3&4+)

2.13.3 IP Set SLR & RLR Values for the UK (Rls 3.00/4.00/4.50 Only)

2.14 Customer data block (cdb - overlay 15)

2.15 Meridian Mail DSP parameters

2.16 Meridian 1 supported software releases

2.17 Patching

- Credits and document history

- 1 Introduction

This document has been updated in line with the recently introduced naming convention and is intended to serve as a guide to BT people who may be involved with the planning, configuration, installation or maintenance of Meridian systems. It is designed to cover those configuration parameters that are frequently misunderstood and often set incorrectly. This Product Bulletin information, is taken from the current NTPs and is a convenient guide only these settings may be superseded by later NTP documentation. All planners and engineers should be familiar with NTPs and have access to them. Failure to make the recommended changes will affect service and may result in an excess of alarm messages.

NB.  Remember it is possible to set Buffers and Call Registers too high as well as too low. If call registers are set too high, applications such as CCR can slow down, or even stop completely. If set too low, delayed dial tone or system freeze can be experienced. If changes are made to buffers or call registers, the system will need to be initialised to make these changes effective. After the initialise, recheck the memory available figures (udata & pdata) to ensure sufficient memory is still available. If the figures are very low, that is less than 2000, contact the Meridian STC for advice.

If you have any queries about this document, please contact Meridian STC National Technical Support on Tel: 0800 622396, Fax: 0121 633 8988.

- Back to contents

- 2

- Product Overview

The following provides the configuration parameters required for: -

| Old Name | New Name | With Signalling Server connected |
| --- | --- | --- |
| Option 11C | Option 11C Cabinet | CS1000M Cabinet |
| Option 11C Mini | Option 11C Chassis | CS1000M Chassis |
| Option 51C | Meridian 1 51C | CS1000M HG (Half Group) |
| Option 61C | Meridian 1 61C | CS1000M SG (Single Group) |
| Option 81C | Meridian 1 81C | CS1000M Multi Group |
| CSE1K |  | CS1000S |
| SBO |  | MG1000B |
| CS1000E |  | CS1000E |
| SIP Gateway |  | MG1000T |

- Back to contents

Useful Documents to supplement Product Bulletin

All Planning and Engineering NTP’s

NTP System Security Management  553-3001-307_2.00

All Technical Bulletins (Newsflashes & Product documents)

MatrixDeplist_User Guide

AIS  Test Plan Document

Back to contents

2.1 Option  11C Cabinet & Chassis, CS1000M Cabinet, CS1000M Chassis, CS1000S,MG1000E, MG1000B and MG1000T

2.1.1 PARM and OVLY (Overlay 17).

| LPIB | 150 | Release 24 for Option 11C Cabinet and Option 11C Chassis |
| --- | --- | --- |
|  | 450 | Release 25 ² Release 4 and 4.53 ³ CS1000M Cabinet & Chassis, CS1000S, MG1000B & MG1000T |
| HPIB | 150 | Release 24 for Option 11C Cabinet and Option 11C Chassis¹ |
|  | 450 | Release 25 ² Release 3, 4 and 4.5 ³ CS1000M Cabinet & Chassis, CS1000S, MG1000B,MG1000E & MG1000T |

¹	Source – Capacity Engineering 553-3001-149 release 24

²	Source – Capacity Engineering 553-3001-149 release 25.40

³           Source – Planning & Engineering 553-3041-120 release 4.00 553-3041-120 release 4.50 Succession Enterprise release 3.00 GRB 2003_368

| 500B | 100 | N/A (set by system to 500 and values entered here are ignored) |
| --- | --- | --- |
| SLIB | 16 | This is the lowest value possible. It is not used by UK systems. |
| DTIB | 70 | Use this value regardless of the number of links. |
| DTOB | 70 | Use this value regardless of the number of links. |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 60* |  |

- Overlay 60 should only be included if Q931 or QSIG or EURO ISDN or CHAS are equipped.

- Back to contents

2.1.2 Minimum Recommended Call Registers

| NCR | 1000 | Release 23 S/W |
| --- | --- | --- |
|  | 1750 | Release 24 S/W * |
|  | 1750 | Release 25 S/W ** |
|  | 800 | Release 3/4/4.5 MG1000B*** |
|  | 1750 | Release 3/4/4.5 CS1000M Cabinet/Chassis **** |
|  | 1750 | Release 3/4/4.5 MG1000T, CS1000S ¹·² |

*	Source – Capacity Engineering 553-3001-149 release 24

** 	Source – GRB release 25.40B

***      Source – Branch Office Inst & Config NTP 553-3001-214 Rlse 4

****	Source – CS1000M & Meridian 1 Small System Planning & Engineering NTP 553-3011-120 Rlse 4

¹ Sources – CS1000E & CS1000S Planning & Engineering NTPs 553-3041-120 & 553-3031-120 Rlse 4

²              Source- CS1000M & Meridian 1 Large System Planning & Engineering NTP 553-3021-120  Rls 4.5

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 or 2525 to localise if available (pdt> commands can be used)

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255 | At Release 4 and ABOVE this Total can be up to 25% of total call registers up to a max of 4095 |
| CSQO | 255 | -------------------“----------------------------------------------- |

- Back to contents

2.1.3 Intu (overlay 97)

INTU NO

NB. Most customers prefer to have no conference warn tone (NO). However, if INTU is set to YES, warn tone may result in corruption of coincident MF4 tones.

Hardware compatibility

The following table shows Option 11C and CS software daughterboards & the software releases that they are compatible with. New orders will receive the new NTTK25BA software daughterboard.

- Back to contents

| Software Daughterboard: | Loaded with S/W Release | Capacity | Compatible with S/W |
| --- | --- | --- | --- |
| NTSK04AC | 22-46 | 32 Meg | 22-46 |
| NTSK04AG | 23-47 | 32 Meg | 23-47 |
| NTSK04AK | 24-25 | 40 Meg | 24-25 |
| NTDK21AA | Blank | 32 Meg | Rlse 22.08D – 23.47 |
| NTDK81AA | Blank | 40 Meg | Rlse 23.47 – 24.25 |
| NTTK13AA | Blank | 48 Meg | Rlse 24.25 – 25.XX and Succession 3¹ |
| NTTK25AA | Blank | 48 Meg | 25.XX and Succession 3¹ |
| NTTK25BA | Blank | 48 Meg | Release 4.00 / 4.50 |

¹          NTTK13AA and NTTK25AA will work fine right up to and including 4.5W as long as the DRAM on the CPU is changed for a 32Mb.

| System | Minimum SSC/MSC | Compatible with S/W |
| --- | --- | --- |
| 11C/Cabinet | NTDK20CA | Rlse 24.25 with 4 –5 cabinets – 25.XX and Succession 3 |
|  | NTDK20EA | Main cabinet when using IP Expansion and … |
|  | NTDK20CA | For each IP Expansion cabinet |
| Chassis | NTDK97AB |  |
|  | NTDK20EA | Main and IP Expansion chassis when using IP Expansion |
| Succession 1000 | NTDK20EA | Call Server and Media Gateway |
| Succession Branch Office | NTDK20FA |  |
| MG1000T/1000E/ MG1000B/CS1000S | NTDK20HA | Minimum vintages for the Small System Controller are - IP Expansion or CS 1000 Systems - Main Cabinet: NTDK20EA-GA SSC + NTDK19BA SSC Memory Upgrade Kit (32Mb) - Each Expansion cabinet: NTDK20AB-GA SSC + NTDK19BA SSC Memory Upgrade Kit (32Mb) - NTDK20HA - Media Gateway 1000E in CS 1000E systems - NTDK20GA SSC + NTDK19BA SSC Memory Upgrade Kit (32Mb) - NTDK20HA |

- Back to contents

An existing Option 11C system equipped with a controller card NTDK20AB can be upgraded to an NTDK20CA with a field upgrade kit NTDK19AA. As of July 1999, all controller cards shipped to the field have been a minimum of NTDK20DA, which is compatible with Release 25 and Succession 3.

A system upgrading to Release 25 or Succession 3 must replace its NTDK21 (32 MB) or NTDK81 (40 MB) based software daughterboard, with an NTTK13AA (48 MB) or NTTK25AA daughterboard. Please ensure that you have upgraded the bootcode on the SSC prior to upgrading the software daughterboard. To verify the size of the software daughterboard and for installation instructions, refer to NTP upgrade manual Chapter 9.

The Succession Branch Office Call Server requires a 48MB Total Memory (32 MB Program Store and 16 MB CDrive space) in order to run Succession 3.0 Software.

- Back to contents

2.2 Meridian 1 51C and CS1000M Half Group

2.2.1 PARM and OVLY (Overlay 17)

| < X11 24 | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | 64 | 192 | 500 | 40 |
| CP2 | 64 | 192 | 500 | 40 |
| CP3/4 | 64 | 192 | 500 | 40 |

| X11 24 * | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 1850 | 1850 | 800 | 255 |
| CP3/4 | 1850 | 1850 | 800 | 255 |

| X11 25 ** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 2200 | 2200 | 800 | 255 |
| CP3/4 | 2200 | 2200 | 800 | 255 |

| X21 3.00 *** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 3500 | 3500 | 800 | N/A |

| X21 4.00**** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP3/4 | 1850 | 1850 | 2000 | N/A |

| X21 4.50¹ | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP3/4 | 1850 | 1850 | 2000 | N/A |

*	Source – Capacity Engineering 553-3001-149 release 24

**	Source – Capacity Engineering 553-3001-149 release 25.40

***	Source – Succession Enterprise release 3.00 GRB 2003_368

****   Source – CS1000E & CS1000S Planning & Engineering NTPs 553-3041-120 & 553-3031-120 Rlse 4

¹   Source- CS1000M & Meridian 1 Large System Planning & Engineering NTP 553-3021-120  Rls 4.5

| NDIS | 20 |  |
| --- | --- | --- |
| DTIB | 70 |  |
| DTOB | 70 | 1 to 4 DPNSS/DASS links |
|  | 100 | 5 or more DPNSS/DASS links |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60* 135 137 |  |

* Overlay 60 should only be included if Q931 or QSIG or EURO ISDN or CHAS are equipped.

- Back to contents

2.2.2 Call Registers (NCR)

| Release 23 S/W | CP1 | 1000 |
| --- | --- | --- |
| Release 23 S/W | CP2 | 1500 |
| Release 23 S/W | CP3 | 2000 |
| Release 23 S/W | CP4 | 2000 |
| Release 24 S/W * | CP2 | 1500 |
| Release 24 S/W * | CP3 | 2000 |
| Release 24 S/W * | CP4 | 2000 |
| Release 25 S/W ** | CP2 | 1500 |
| Release 25 S/W ** | CP3 | 2000 |
| Release 25 S/W ** | CP4 | 2000 |
| Succession Release 3 *** | CP3/4 | 2000 |
| Release 4/4.5**** | CP3/4 | 2000 |

CP1 - NT6D66), CP2 - NT9D19 - CP3 - NT5D10 CP4 - NT5D03, CP PII - NT4N64

*	Source – Capacity Engineering 553-3001-149 release 24

** 	Source – GRB release 25.40

***	Source – Large System Planning and Engineering 553-3021-120

****   Source – CS1000E & CS1000S Planning & Engineering  NTPs 553-3041-120 & 553-3031-120 Rlse 4

****  Source- CS1000M & Meridian 1 Large System Planning & Engineering NTP 553-3021-120  Rls 4.5

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255* | At Release 4 and ABOVE this Total can be up to 25% of total call registers up to a max of 4095 |
| CSQO | 255* | -------------------------“-------------------------------- |

- Back to contents

2.2.3 Succession 3 and CS1000 Memory requirements

Succession 3 requires 64 Flash memory and 64 DRAM - 128 Total

|  | Flash | DRAM | Total |
| --- | --- | --- | --- |
| CP3/4 | 64 | 64 | 128 |

RELEASE 4.00T and RELEASE 4.50W Memory Requirements

| System | Flash | Dram | Total |
| --- | --- | --- | --- |
| With 68060 (CP3) or 68060E (CP4) processors* |  |  |  |
| CS 1000M SG/PBX 51C/61C | 64MB | 64MB | 128MB |

- Back to contents

2.3 Meridian 1 61C and CS1000M Single Group

2.3.1 PARM and OVLY (Overlay 17)

| < X11 24 | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | 192 | 64 | 500 | 40 |
| CP2 | 192 | 64 | 500 | 40 |
| CP3/4 | 192 | 64 | 500 | 40 |

| X11 24 * | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 1850 | 1850 | 800 | 255 |
| CP3/4 | 1850 | 1850 | 800 | 255 |

| X11 25 ** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 2200 | 2200 | 800 | 255 |

| X21 3/4/4.5 ***/**** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 3500 | 3500 | 800 note 1 | 255 |
| CP PII/IV | 3500 | 3500 | 2000 note 1 | N/A |

*	Source – Capacity Engineering 553-3001-149 release 24

**	Source – Capacity Engineering 553-3001-149 release 25.40

***	Source – Succession Enterprise release 3.00 GRB 2003_368

****   Source – CS1000E & CS1000S Planning & Engineering  NTPs 553-3041-120 & 553-3031-120 Rlse 4

****   Source- CS1000M & Meridian 1 Large System Planning & Engineering NTP 553-3021-120  Rls 4.5

note 1	Source – Large System Planning and Engineering

CP1 - NT6D66), CP2 - NT9D19 - CP3 - NT5D10 CP4 - NT5D03, CP PII - NT4N64 CP PIV- NT4N39

| NDIS | 20 |  |
| --- | --- | --- |
| DTIB | 70 |  |
| DTOB | 70 100 | 1 to 4 DPNSS/DASS links 5 or more DPNSS/DASS links |
| PCDR | No |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60*135** 137 |  |
| AML | T1 | 4 |
|  | T2 | 10 |
|  | T3 | 8 |
|  | N1 | 5 |
|  | N2 | 128 |
|  | K | 7 |

*Overlay 60 should only be included if Q931 or QSIG or EURO ISDN or CHAS are equipped.

**  If Ovly 135 is set ensure prompt MID_SCPU=NO on CP PII/CP IV m/c’s

- Back to contents

2.3.2 Call Registers (NCR)

| Release 23 S/W | CP1 | 2000 |
| --- | --- | --- |
| Release 23 S/W | CP2 | 3000 |
| Release 23 S/W | CP3 / CP4 | 4000 |
| Release 24 S/W * | CP2 | 3000 |
| Release 24 S/W * | CP3 / CP4 | 4000 |
| Release 25 S/W ** | CP3 / CP4 | 4000 |
| Succession rlse 3 *** | CP3 / CP4 | 4000 |
| Succession rlse 3 *** | CP PII | 20000 |
| Release 4/ 4.5**** | CP3/ CP4 | 4000 |
| Release 4/ 4.5 | CP PII / CP PIV | 20000 |

CP1 - NT6D66, CP2 - NT9D19 - CP3 - NT5D10 CP4 - NT5D03, CP PII - NT4N64

CP PIV- NT4N39

*	Source – Capacity Engineering 553-3001-149 release 24

** 	Source – GRB release 25.40

***	Source – Large System Planning and Engineering 553-3021-120

****   Source – CS1000E & CS1000S Planning & Engineering NTPs 553-3041-120 & 553-3031-120 Rlse 4

****   Source- CS1000M & Meridian 1 Large System Planning & Engineering NTP 553-3021-120  Rls 4.5

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 later vers 2525 to localise if available (pdt> commands can be used).

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255* | At Release 4 and ABOVE this Total can be up to 25% of total call registers up to a max of 4095 |
| CSQO | 255* | --------------------“----------------------------------- |

- Back to contents

2.3.3 Succession 3 and CS1000 Memory requirements

|  | Flash | DRAM | Total |
| --- | --- | --- | --- |
| CP3/4 | 64 | 64 | 128 |
| CP PII |  | 256 DIMM | 256 |

RELEASE 4.00T and RELEASE 4.50W Memory Requirements

| System | Flash | Dram |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| With 68060 (CP3) or 68060E (CP4) processors* |  |  |  |  |  |  |
| CS 1000M SG/PBX 51C/61C | 64MB | 64MB |  | 128MB |  |  |
| With Pentium CP PII processors |  |  |  |  |  |  |
| CS 1000M SG/PBX 61C CP PII N/A 256 MB 256 MB |  | N/A | 256MB |  | 256MB |  |
| With Pentium CP PIV processors |  |  |  |  |  |  |
| CS 1000M SG/PBX 61C CP PIV |  | N/A | 256MB |  |  | 256MB |

- Back to contents

2.3.4 Xct (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

- Back to contents

2.4 Meridian 1 81C and CS1000M Multi Group and CS1000E

2.4.1 PARM and OVLY  (Overlay 17).

| < X11 24 | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 64(Number of groups) | 300 | 1000 | 40 |
| CP3/4 | 64(Number of groups) | 300 | 1000 | 40 |

| X11 24 * | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | 1850 | 1850 | 800 | 255 |
| CP3/4 | 1850 | 1850 | 800 | 255 |

| X11 25 ² | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 2200 | 2200 | 800 | 255 |
| CP4 6 – 8 groups | 3300 | 3300 | 800 | 255 |
| CP PII | 2200 | 2200 | 800 | 255 |
| CP PII 6 – 8 groups | 3500 | 3500 | 800 | 255 |

| X21 3.00 ³ | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP1 | N/A | N/A | N/A | N/A |
| CP2 | N/A | N/A | N/A | N/A |
| CP3/4 | 3500 | 3500 | 800 note 1 | 255 |
| CP PII 1 – 8 groups | 3500 | 3500 | 2000 note1 | 255 |

| X21 4/4.5**** | HPIB | LPIB | 500B | SL1B |
| --- | --- | --- | --- | --- |
| CP3/4 | 3500 | 3500 | 2000 | N/A |
| CP PII | 3500 | 3500 | 2000 | N/A |
| CP PIV | 3500 | 3500 | 2000 | N/A |

*	Source – Capacity Engineering 553-3001-149 release 24

²	Source – Capacity Engineering 553-3001-149 release 25.40

³	Source – Succession Enterprise release 3.00 GRB 2003_368

****   Source – CS1000E & CS1000S Planning & Engineering NTPs 553-3041-120 & 553-3031-120 Rlse 4

CS1000M & Meridian 1 Large System Planning & Engineering NTP 553-3021-120  Rls 4.5

note 1	Source – Large System Planning and Engineering

| NDIS | 20 |  |
| --- | --- | --- |
| DTIB | 70 |  |
| DTOB | 70 100 | 1 to 4 DPNSS/DASS links 5 or more DPNSS/DASS links |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60*135** 137 |  |

- Overlay 60 should only be included if Q931/QSIG/EURO ISDN/ CHAS equipped.

**  If Ovly 135 is set ensure prompt MID_SCPU=NO on CP PII/CP IV m/c’s

- Back to contents

2.4.2 Call Registers (NCR)

| Release 23 S/W | CP2 | 7500 |
| --- | --- | --- |
| Release 23 S/W | CP3 / CP4 | 10000 |
| Release 24 S/W | CP2 | 7500 |
| Release 24 S/W | CP3 / CP4 | 10000 |
| Release 25 S/W | CP2 | 7500 |
| Release 25 S/W | CP3 / CP4 | 10000 |
| Release 25 S/W with 1-5 groups | CP PII | 20000 |
| Release 25 S/W with 5-8 groups | CP PII | 25000 |
| Succession 3 S/W with 1-5 groups | CP PII | 20000 |
| Succession 3 S/W with 6-8 groups | CP PII | 25000 |
| Meridian 1 81C CS1000M MG 1-5 groups | CP3/CP4 | 10000 |
| Meridian 1 81C CS1000M MG 1-5 groups CS1000E | CP PII/CP PIV | 20000 |
| Meridian 1 81C CS1000M MG 6-8 groups CS1000E | CP PII/CP PIV | 25000 |

CP1 - NT6D66, CP2 - NT9D19 - CP3 - NT5D10 CP4 - NT5D03, CP PII - NT4N64

CP PIV- NT4N39

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 or 2525 to localise if available (pdt> commands can be used)

.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255 | At Release 4 and ABOVE this Total can be up to 25% of total call registers up to a max of 4095 |
| CSQO | 255 | --------------------“----------------------------- |

- Back to contents

2.4.3 XCT (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

- Back to contents

2.5 RELEASE 4.00T and RELEASE 4.50W Memory Requirements

| System | Flash | Dram |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| With 68060 (CP3) or 68060E (CP4) processors* |  |  |  |  |  |  |
| CS 1000M MG/PBX 81/81C (withor without Fibre Network Fabric) | 64MB | 96MB |  | 160MB |  |  |
| With Pentium CP PII processors |  |  |  |  |  |  |
| CS 1000M SG/PBX 61C CP PII N/A 256 MB 256 MB |  | N/A | 256MB |  | 256MB |  |
| CS 1000M MG/PBX 81/81C CP PII (with or without Fibre Network Fabric) |  | N/A | 256MB |  | 256MB |  |
| With Pentium CP PIV processors |  |  |  |  |  |  |
| CS 1000M SG/PBX 61C CP PIV |  | N/A | 256MB |  |  | 256MB |
| CS 1000M MG/PBX 81C CP PIV |  | n/A | 256MB |  |  | 256MB |

CS1000E Release 4.50W

| Processor | FLASH | DRAM | TOTAL |
| --- | --- | --- | --- |
| Core Call Server (CP PII) | N/A | 256MB | 256MB |
| Core Call Server (CPPIV) | N/A | 512MB | 512MB |
| MG 1000E (SSC) | 48MB | 32MB | 80MB |
| MG 1000T (SSC) | 48MB | 32MB | 80MB |

- Back to contents

2.6 Legacy Systems

2.6.1 Option 11 and 11E

2.6.1.1 PARM and OVLY (Overlay 17)

| LPIB | 96 |  |
| --- | --- | --- |
| HPIB | 32 | 16 for pre 16.91 software. |
| 500B | --- | Set to lowest value (ignored by system). |
| SLIB | 16 | This is the lowest value possible. It is not used by UK systems. |
| DTIB | 0 | Not required for Option 11, but once set, it cannot be restored to zero, in which case set it to 35. If set to high values, it will use up a large amount of memory and memory is often at a premium on Option 11s. |
| DTOB | 35 | Use this value regardless of the number of links. |
| PCDR | NO |  |
| BKGD | 44 |  |
| DROL | 30 43 44 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

- Back to contents

2.6.1.2 Call registers.

The number configured will depend on several variables. Use the following tables as a guide but never reduce existing values without first consulting MOC Technical Support.

Systems without ACD

| Number of : | 40 Extns. | 80 Extns. | 160 Extns. | 208 Extns. |
| --- | --- | --- | --- | --- |
| 8 Trunks | 40 | - | - | - |
| 16 Trunks | - | 56 | - | - |
| 32 Trunks | - | - | 88 | - |
| 40 Trunks | - | - | - | 106 |

Systems with ACD

| Number of ACD agents | 10 | 20 | 40 | 40 | 80 |
| --- | --- | --- | --- | --- | --- |
| Number of ordinary extensions | 40 | 80 | 40 | 80 | 80 |
| Number of ACD Trunks | 6 | 12 | 0 | 8 | 0 |
| Number of non ACD Trunks | 12 | 24 | 48 | 48 | 96 |
| Number of Call Registers | 54 | 84 | 95 | 111 | 166 |
| Number of Call Registers if ACD Max | 79 | 109 | 120 | 136 | 191 |

MGCR 25 Only if ACD Max is equipped (If no Max, set to 0)

If CCR or/and Link are equipped, then two more parameters will need to be configured as follows:

| CSQI | 40 |
| --- | --- |
| CSQO | 40 |

- If Mail only is equipped, the above parameters should be configured as follows:

| CSQI | 20 | (default) |
| --- | --- | --- |
| CSQO | 20 | (default) |

All input/output devices (IOTB) should have USER configured as XBUG with the exception of the history file (i.e. ADAN = HST) which should be configured for BUG.

- Back to contents

2.6.2 Option 51 Omega processor

2.6.2.1 PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| LPIB | 96 |  |
| HPIB | 32 |  |
| 500B | (A+B)8100 | where A = total number of 500 sets B = total number of digital sets |
| SL1B | 40 |  |
| DTIB | 35 70 | for non expanded mode DDSLs for expanded mode DDSLs |
| DTOB | 35 70 100 | for 1 or 2 digital links for 3 or 4 digital links for 5 links or more |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 35 43 44 45 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

- Back to contents

2.6.2.2 Call Registers

| NCR | 600 | This is a minimum value. Never reduce existing values. |
| --- | --- | --- |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 100 |  |
| CSQO | 100 |  |

- Back to contents

2.6.3 Option 61, Omega Processor

2.6.3.1.PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| LPIB | 96 |  |
| HPIB | 32 |  |
| 500B | (A+B)8100 | where A = total number of 500 sets B = total number of digital sets |
| SL1B | 40 |  |
| DTIB | 35 70 | for non expanded mode DDSLs for expanded mode DDSLs |
| DTOB | 35 70 100 | for 1 or 2 digital links for 3 or 4 digital links for 5 links or more |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 35 43 44 45 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

- Back to contents

2.6.3.2 Call Registers

| NCR | 600 | This is a minimum value. Never reduce existing values. |
| --- | --- | --- |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 100 |  |
| CSQO | 100 |  |

- Back to contents

2.6.3.3. XCT(overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

- Back to contents

2.6.4 Option 71 Omega processor (Overlay 17)

2.6.4.1  PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| NCR | 1200 | This is a minimum value. Never reduce existing values |
| LPIB | 96 |  |
| HPIB | 32(Number of groups) |  |
| 500B | (A+B)8100 | where A = total number of 500 sets B = total number of digital sets |
| SL1B | 40 |  |
| DTIB | 35 70 | for non expanded mode DDSLs for expanded mode DDSLs |
| DTOB | 35 70 100 | for 1 or 2 digital links for 3 or 4 digital links for 5 links or more |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 35 43 44 45 60 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

- Back to contents

2.6.4.2 Registers

| NCR | 1200 | This is a minimum value. Never reduce existing values |
| --- | --- | --- |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 200 |  |
| CSQO | 200 |  |

- Back to contents

2.6.4.3 XCT (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

- Back to contents

2.6.5 Option 81

2.6.5.1 PARM and OVLY (Overlay17)

| NDIS | 20 |  |
| --- | --- | --- |
| LPIB | 300 |  |
| HPIB | 64(Number of groups) |  |
| 500B | 1000 |  |
| SL1B | 40 |  |
| DTIB | 70 |  |
| DTOB | 70 100 | 1 to 4 DPNSS/DASS links 5 or more DPNSS/DASS links |
| PCDR | NO |  |
| CY45 | 01 |  |
| BKGD | 44 |  |
| DROL | 30 43 44 45 60 135 137 |  |

(Overlay 60 should only be included if Q931 or CHAS are equipped).

- Back to contents

2.6.5.2 Call Registers

| NCR | Release 23 S/W | CP1 | 5000 |
| --- | --- | --- | --- |
|  | Release 23 S/W | CP2 | 7500 |
|  | Release 23 S/W | CP3 / CP4 | 10000 |
|  | Release 24 S/W | CP2 | 7500 |
|  | Release 24 S/W | CP3 / CP4 | 10000 |
|  | Release 25 S/W | CP2 | 7500 |
|  | Release 25 S/W | CP3 / CP4 | 10000 |

If call register overflows are suspected, use System traffic reports or Diagnostic patch 1516 to localise.

| MGCR | 25 | Only if ACD Max is equipped (If no Max, set to 0) |
| --- | --- | --- |
| CSQI | 255 |  |
| CSQO | 255 |  |

- Back to contents

2.6.5.3 XCT (overlay 17)

It is essential that the XCT is not located in loop 0. (XCT 000 in Overlay 17) If it has been configured in this loop, then it must be moved to another loop. Failure to comply may result in spurious initialises, delays to dial tone and other service affecting symptoms. If at all possible, ENETs should not be located in loop 0 either. However, do not change existing systems unless service difficulties are experienced with loop 0.

- Back to contents

2.7 Digital trunk configuration

NB. Some digital trunk parameters cannot be changed until the link is taken out of service.  Changes to default values are shown in bold.

- Back to contents

2.7.1 DASS and DPNSS

On big switches (Options 51,61,71,81), when using expanded mode addressing, the “D” channel interface card must be fitted in the correct network group and shelf for the DDSL address being used.

For example,

DDSL 0 to 15 Network Group 0 Shelf 0

DDSL 16 to 31 Network Group 0 Shelf 1

DDSL 32 to 47 Network Group 1 Shelf 0

DDSL 48 to 63 Network Group 1 Shelf 1

DDSL 64 to 79 Network Group 2 Shelf 0

DDSL 80 to 95 Network Group 2 Shelf 1

Etc.

- Back to contents

2.7.2 PRI Alarm Threshold and Timers DASS & DPNSS (Overlay 74)

| Alarm Mnemonic | Persistence Time |  | Monitor Time (hours) |  | Count Threshold |
| --- | --- | --- | --- | --- | --- |
| TBF | 5 seconds |  | 0 |  | 1 |
| FAE | 2 seconds |  | 1 |  | 4 |
| HER | 1 minute |  | 1 |  | 10 |
| TSF | 0 minutes |  | 0 |  | 0 |
| AIS | 1 minute |  | 1 |  | 6 |
| LOI | 6 seconds |  | 1 |  | 6 |
| DAI | 1 minute |  | 1 |  | 6 |
| Counter Mnemonic |  | DASS |  | DPNSS |  |
| CRT |  | 120 |  | 120 |  |
| TMT |  | 50 |  | 50 |  |
| SCT |  | 20 |  | 20 |  |

- Back to contents

2.7.3 Q931 links.

When configuring Q931 “D” channels in overlay 17, the RCAP prompt defines the remote capabilities of the “D” channel. Items such as Call Party Named Display and Network call trace are configured here. If the remote end of the link uses an MSDL card for the “D” channel interface this will also need to be defined here.

If Basic Rate Access (BRI) is used across the link, RCAP should also specify BRI.

The Release ID (overlay 17) should be set to the lowest release of software in the network. This applies to all “D” channels on all sites in the network.

- Back to contents

2.7.4 PRI2 Alarm Threshold and Timers Q931 (Overlay 73)

| SLP | 5 | 24H | 255 | 1H |
| --- | --- | --- | --- | --- |
| BVP | 128 | 122 |  |  |
| CRC | 201 | 97 |  |  |

| FAP | 28 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| GP2 | 50 | 100S | 12S | 12S | 4S |
| MNG1 | 30S |  |  |  |  |

| NCG1 | 30s |
| --- | --- |
| OSG1 | 1M |
| MNG2 | 30S |

| NCG2 | 30S |
| --- | --- |
| OSG2 | 30S |
| PERS | 50 |
| OOSC | 0 |

- Back to contents

2.7.5 Pri2 pad values (overlay 73)

The following pad values are recommended for Release 16 software or later: -

|  | RECEIVE PAD (x) | TRANSMIT PAD (y) |
| --- | --- | --- |
| ONP | 0 | 0 |
| OPX | 0 | 0 |
| DTT | 0 | 0 |
| DCO | 0 | 0 |
| NTC | 0 | 0 |
| TRC | 0 | 0 |
| DTR | 0 |  |
| VNL | 0 | 0 |
| ACO | 0 | 0 |
| AFX | 0 | 0 |
| ADD | 0 | 0 |
| PR1 | 0 | 0 |
| DSET | 0 | 0 |
| BRIL | 0 | 0 |
| BRIT | 0 | 0 |

- Back to contents

2.7.6  Dti2 system timers chas (overlay 73)

| MAND | 1M |
| --- | --- |
| NCSD | 30S |
| OSGD | 1M |
| OOSC | 0 |
| PERS | 100 |
| CCGD | 1M |
| CCAR | 1M |

- Back to contents

2.7.7 Dti loop timers chas (overlay 73)

Group 1 Impairments

| BPV | 205 | 10S | 3S | 3S | 1S |
| --- | --- | --- | --- | --- | --- |
| FAP | 32 | 4S | 1S | 1S | 100T |
| SLP | 100 | 30S | 10S | 10S | 6S |

Group 2 Impairments

| GP2 | 50 | 100S | 12S | 12S | 4S |
| --- | --- | --- | --- | --- | --- |

- Back to contents

2.7.9 Euro – ISDN (PSTN)

The following is a sample configuration for Option 11C is shown below:

The most important prompts are highlighted. This is not a definitive configuration as this will depend on the functionality supported by the service provider and the software level of the M1, therefore reference must be made to the NTP and the customer requirements.

LD 17

TYPE PARM

BCAP 	SPEE  **Bearer Capability = Speech

or

31KH **Bearer Capability = 3.1Khz Voice

TYPE ADAN

ADAN NEW DCH **

CTYP MSDL **Multipurpose Serial Data Link**

CARD ** Insert Card No

PORT 1

DES EURO_ISDN

USR PRI **D-Channel for ISDN PRI**

DCHL * Insert Loop No

OTBF 32

PARM RS422 DTE

DRAT 64KC

CLOK EXT

NASA NO

*IFC EURO or E403 **Interface type for D-Channel** see note 1

*CNTY UK  0r ETSI **Country Type** see note 1

PINX_CUST 0

ISDN_MCNT 300

CLID OPT1

PROG NCHG **Send Progress Message**

CO_TYPE STD

SIDE USR **USR = default** **NET for Master Mode**

CNEG 2

RLS ID

RCAP COLP **Supports Connected Line ID**

LD 17 CONT.

MBGA NO

OVLR NO

OVLS NO

T310 120

INC_T306 120

OUT_T306 120

T200 3

T203 10

N200 3

N201 260

K 7

LD 73

TYPE PRI2

FEAT LPTI

LOOP ** Insert loop No

MFF CRC **Multiframe Format = CRC (Cyclic Redundancy Check)**

ACRC YES **Auto report or CRC-4 error**

ALRM ALT **Alternate firmware alarm handles**

RAIE NO

G1OS YES

**Set all timers to default until further notice**

SLP 5 24 H 30 1 H

BPV 128 122

CRC 201 97 **Cyclic Redundancy Check error counts**

FAP 28 1

RATS 10

GP2 20 100 S 12 S 12 S 4 S

MNG1 15 M

NCG1 15 M

OSG1 15 M

MNG2 15 S

NCG2 15 S

OSG2 15 S

PERS 50

CLRS 1

OOSC 5

LD 16

TYPE RDB

CUST 00

DMOD

ROUT *** Insert Route No

DES EURO_ISDN

TKTP DID

SAT NO

RCLS EXT

DTRK YES **Digital Trunk Route**

BRIP NO

DGTP PRI2 **Digital trunk Type for Route**

ISDN YES

MODE PRA

*IFC EURO **E403** see note 1

*CNTY UK **ETSI**  see note 1

SBN NO

PNI 00000

NCNA NO

NCRD NO

ISAR NO

CPFXS YES

ADDP NO

DSEL VOD

PTYP DCO

AUTO NO

DNIS NO

ICOG IAO

RANX NO

SRCH RRB

TRMB YES

STEP

ACOD 1600

TARG **

**All other responses in RDB should be set to default or customer specific**

LD 14

TN ** Insert T.N

TYPE DID

CDEN SD

CUST 0

TRK PRI2

PDCA 1 **Pad Category table number**

PCML A **Pulse Code Modulation Law (A Law)**

NCOS 7

RTMB 100 1

B-CHANNEL SIGNALING

NITE

CLS UNR DTN WTA LPR APN

P10 VNL

TKID

DTCR NO

DATE 4 FEB 1997

LD 15

TYPE CDB

CUST 00

LDN_DATA

OPT XLDN

DLDN YES

LDN0 **** **LDN0 must be defined for ISDN PRI DID Service .The length

LDA0 of LDN0 determines the number of trailing digits translated as the

LDN1 DN on PRI DID routes**

LDA1

LDN2

LDA2

LDN3

LDA3

LDN4

LDA4

LDN5

LDA5

ICI 00 LD0

ICI 01

ICI 02

ICI 03

ICI 04

ICI 05

ICI 06

ICI 07

ICI 08

ICI 09

Note 1

These have to be set the same on both the DCH and RDB or the switch will not accept it.

Below should only be used in rare cases where the exchange does not support newer Euro features. All BT exchanges do so we would only ever set as EURO UK when it is a non BT link and even then most of the other providers support the E403 ETSI settings.

IFC EURO

CNTY UK

- Back to contents

2.7.10  QSIG

Like all Meridian networking solutions, QSIG has numerous permutations in which the configuration can be applied. The essential thing to understand is that, which ever way is chosen, the remote system needs to have the ability to recognise the protocol it is receiving and visa versa, therefore the only right way to configure QSIG is one which is built on common ground between two systems.

Since release 23, all QSIG Supplementary Services have provided the user with the choice of ROSE APDU Operation Coding rule: Integer Values or Object Identifiers (for example QSIG SS Path Replacement PRI/PRO etc.). However, two supplementary services, developed in release 22, offered only the Object Identifier option:

• QSIG Supplementary Service - Name Display

• QSIG Supplementary Service - Call Completion

From release 24, these supplementary services offer the possibility of choosing Integer Value option for ROSE APDU operation coding.

There is no functional change vs Release 22 versions.

- Back to contents

2.7.11 Example of Meridian rlse 25 QSIG to CISCO

Patches must include the following however these are not definitive and are subject to upissue or withdrawal, please check the latest at time of installation (see patching section 17)

PAT#  PRS      PATCH REF #      NAME             DATE      FILENAME

p16179_1.cp4

p14000_3.cp4

p14007a3.cp4,  p14007b3.cp4

p16131a1.cp4,  p16131b1.cp4,  p16131c1.cp4,  p16131d1.cp4

p17448

Packages must include

TYPE pkg

MSDL   222

MSDL SDI   227

MSDL STA   228

QSIG   263

QSIGGF   305

QSIG-SS   316

ETSI-SS   323

MEET   348

(Use numbers and loops etc to suit site)

D Channel config example

ADAN     DCH 21

CTYP MSDL

DNUM 15

PORT 1

DES  EGF4

USR  PRI

DCHL 21

OTBF 32

PARM RS422  DTE

DRAT 64KC

CLOK EXT

IFC  EGF4

PINX_CUST 0

ISDN_MCNT 300

CLID OPT0

CO_TYPE  STD

SIDE USR (Must be USR when connected to router)

CNEG 1

RLS  ID  25

QCHID YES

RCAP NCT MWI COLP NDI CCBI CCNI PRI DV3I MQC CTI

PR_TRIGS DIV 2 3

CNG 2 3

CTR2 2 3  (Use XCTR to delete CTR1 then enter CTR2 2 3)

PR_RTN NO   (YES will retain the first leg of a redirected call)

MQC_FEAT NAS NACD NMS  (depending on packages)

MBGA NO

OVLR NO

OVLS NO

T310 120

T200 3

T203 10

N200 3

N201 260

K    7

CDB Example

NET_DATA

OPT RTD

AC2 LOC

FNP YES

ISDN YES

PNI 112		(match to existing PNI number)

PINX_DN 2415	(must be spare DN in site number range diallable

from remote sites, although NU will result)

MBG 0

BSGC 65535

PFX1

PFX2

HLOC

LSC	(usually 3 digit site code if applicable)

RCNT 5

PSTN NO

TNDM 15

PCMC 15

SATD 1

OCLI NO

TIDM NO

DASC

ROPT NRO

DITI YES

TRNX NO

EXTT NO

FTOP FRES

APAD 0  0

VNR NO

NIT 8

NAS ATCL YES

NAS ACTV NO

FOPT 0

CNDN

CNAT

CNIP YES

ODN8

ODN9

ASTM 30

HDOPT 0

HDTM 30

Route data block example

TYPE RDB

CUST 00

DMOD

ROUT 120

DES  QSIGGF

NPID_TBL_NUM   0

ESN  NO

RPA  NO

CNVT NO

SAT  NO

RCLS EXT

DTRK YES

BRIP NO

DGTP PRI2

ISDN YES

MODE PRA

IFC  EGF4

SBN  NO

PNI  00112  (to match existing)

NCNA yes

NCRD yes

CTYP CDP

INAC NO

ISAR NO

CPFXS YES

DAPC NO

INTC NO

DSEL VOD

PTYP DTT

AUTO NO

DNIS NO

DCDR NO

ICOG IAO

SRCH LIN

TRMB NO

STEP

ACOD 1920  (to suit)

TCPP NO

TARG 01

BILN NO

OABS

INST

IDC  YES	(	)

DCNO 2 *	(To suit)

NDNO 2	(	)

DEXT NO

SIGO STD

MFC  NO

ICIS YES

OGIS YES

TIMR ICF 1920

OGF 1920

EOD 13952

LCT 256

NRD 10112

DDL 70

ODT 4096

RGV 640

GTO 896

GTI 896

SFB 3

PRPS 800

NBS 2048

NBL 4096

IENB 5

TFD 0

VSS 0

VGD 6

DTD  NO

SCDT NO

2 DT NO

DRNG NO

CDR  YES

INC  YES

LAST YES

TTA  YES

ABAN YES

CDRB NO

QREC NO

OAL  YES

AIA  YES

OAN  YES

OPD  NO

NDP  EXC 0

CDRX NO

NATL YES

SSL

CFWR NO

IDOP NO

VRAT NO

MUS  NO

PANS YES

FRL  0 0

FRL  1 0

FRL  2 0

FRL  3 0

FRL  4 0

FRL  5 0

FRL  6 0

FRL  7 0

OHQ  NO

OHQT 00

CBQ  NO

AUTH NO

TTBL 0

PLEV 2

OPR  NO

ALRM NO

ART  0

PECL NO

DCTI 0

TIDY 1920 120

SGRP 0

AACR NO

Trunk member Example

TN   021 01

TYPE TIE

CDEN SD

CUST 0

TRK  PRI2

PDCA 3	(may require new pad table as this can be amended without affecting other routes)

PCML A

NCOS 7

RTMB 120 1

B-CHANNEL SIGNALING

TGAR 1

AST  NO

IAPG 0

CLS  UNR DTN CND WTA LPR APN THFD XREP BARD

P10 VNL

TKID

***** ALL QSIGGF RLI ENTRIES MUST HAVE SBOC SET WITH COPT = 1 OR 2 *****

This is in order for alternate routing to RLI entries 1,2 3 etc to work

- Back to contents

2.8 ACD

It is recommended that the prompts in LD 23 for call forcing, FCFT, FADT, FADR should be treated as mutually exclusive. Having a value set for more than one prompt will cause incorrect call distribution amongst agents. FCFT is the preferred prompt to use, with FADT and FADR being set to none.

- Back to contents

2.9 Network ACD

When a Meridian is used in a Network ACD (NACD) Q931 environment, it is important to ensure that certain parameters are correctly configured as shown below:

Overlay 17

CLOK EXT

NASA YES

RLS ID 16 NB.

This parameter should be set to the lowest release of software in the network. In the example shown, this is Release 16. If all switches are at a minimum of Release 18, this parameter should be set to 18 etc.

Overlay 16

These parameters must be set for all Q931 routes where NACD is used. Ideally, they should be altered at both ends of a link at the same time, otherwise messaging errors may occur.

SIGO ESN5

INAC YES

- Back to contents

2.10 CDR (all systems)

CDR should be configured at a minimum speed of 9600 Baud. Please see MOC Newsflash 96_016 before making this change to any Option 11E.

Within LD 16, the CDR prompt Time to Answer (TTA) set to YES adds an overhead to the processor load. Only set this prompt on necessary routes, it is not recommended for Music or Ran routes.

- Back to contents

2.11 Traffic measurements

The traffic measurement for processor load TFS 004 now has extra fields to help with indicating excessive Real Time Usage. If the Maximum Real Time Used figure exceeds 80%, the switch may suffer from call processing problems at peak times.

This enhancement is available from Release 18 software onwards on Big Switches (Omega and “C” processors) and Release 22 software onwards on Option 11 (i.e., the C processor variant.)

- Back to contents

2.12 System security

All recommendations on Meridian1 security can be found in the Meridian “System Security Guide”  P0806182. This guide is shipped with all new systems and Meridian Mail and is the definitive document from Nortel on Meridian1 security.

With the new release of 4.50 Nortel have introduced a new range of system security measures in line with Industry Standards. NTP System Security Management

553-3001-307_2.00 . This new standard is very exacting and all staff who work on Nortels CS switches should familiarise themselves with the rules.

To assist engineers with certain aspects of Password Security a newsflash as been issued. see  Technical Bulletin   05_23

- Back to contents

2.13 Transmission levels

AOLR in Overlay 17 should be set to value 39.  This prompt is used in Release 22 or later software to set transmission levels for ACD agent sets.

- Back to contents

2.13.1 IP Telephony

To set the correct Loss Plan information the following Configuration / PEP insertion must be adhered to for Nortel IPT products.

| Product / sw Level | PEP Requirements | Configuration Requirements |
| --- | --- | --- |
| Succession Rls3.00 | None | LD 73 – Set DDB. Change Pad 15 to UK settings as per NTP. Set UK Specific IP Set Loss Plan on Media Cards. |
| Succession CSE 1k Rls.2.2 | MPLR - 17754 | LD 73 – Set DDB. Change Pad 15 to UK settings as per NTP. |
| IP Line on Sites with X11 25.40b software | MPLR - 16341 | As above |
| IP Trunk on sites with X11 25.40b software | MPLR - 15721 | As above |
| CS1000 Release 4/4.5 | None | As above |

- Back to contents

2.13.2 Media Card Loss Plan Settings (Succession Rls.2 & 3&4+)

**See NTP 553-3001-182 (Transmission Parameters)**

To Set DDB

LD 73

REQ NEW

TYPE DDB

TRSH 0

Other prompts: <CR>

PDCA (PAD 15)

DDB will create a default PAD Table 15 which must be changed to the recommended UK settings.

LD 73

REQ PRT (CHG)

TYPE DTI

FEAT PAD

PDCA 15

ONP  	6 	5

DSET	 3	4

OPX 	N/A 	N/A

DTT 	0 	0

SDTT 	0 	0

DCO  	N/A 	N/A

DTO  	3 	19

VNL 	0 	0

SATT 	3 	19

ACO  	2 	16

ATO 	3 	19

PRI  	0 	0

PRI2 	0 	0

XUT 	2 	16

XEM  	2 	16

BRIL  	0 	0

BRIT 	0 	0

WRLS 	3 	19

- Back to contents

2.13.3 IP Set SLR & RLR Values for the UK (Rls 3.00/4.00/4.50 Only)

**See NTP 553-3001-365 (IP Line)**

When a system is installed in the UK, the command UKLossPlanSet is entered at the CLI of one Media card, once entered it automatically Synchronises all other media cards in that node. This adjusts the loss plan of the Internet Telephones to the higher transmit levels required in the UK.

Use the following steps to set the loss plan for the UK.

1 Either - telnet to the card, connect to the maintenance port, use OTM 2.1 or Element Manager to access the Voice Gateway Media Card to gain access to the CLI.

2 Log into the IPL> shell.

3 At the IPL > CLI, enter the command UKLossPlanSet. Press <CR>.

4 Exit from the login session.

The loss plan is automatically set to the required values for release 4.5 software     and  above. But for release 3 and 4 software, use the following steps to set the loss plan correctly for the  UK.

Setting the loss plan for the UK:

1 Access the CLI by either:-

· Telnet to the card,

· Connect to the maintenance port,

· OTM 2.1

· Element Manager: telnet access the Voice Gateway Media Card.

2 Log into the IPL> vxWorksShell

windRiver

developer

3 At the prompt, IPL > UKLossPlanSet. (This increases the handset / headset gain by 5dB on the IP Set).

(Use UKLossPlanClr if you want to unset it)

IPL> lossPlanPrt (to print the setting).

Parameter          Default    Offset    Result

---------------    -------    ------    ------

HandsetRLR              2         0         2

HandsetSLR             11        -5         6

HeadsetRLR              0         0         0

HeadsetSLR             11        -5         6

HandsfreeRLR           13         0        13

HandsfreeSLR           16         0        16

value = 46 = 0x2e = '.'

Note the Offset is -5dB which equates to a gain of 5dB for both Headsets and Handsets. This can sometimes be too high and result in reports of echo on IP sets.

On the SigServer, you can modify the default 5 dB gain with the command:

oam> lossPlanSet "handset",0,<-gain>

where: lossPlanSet "handset",0,-3 gives a gain of 3 dB for handsets.

where: lossPlanSet "headset",0,-3 gives a gain of 3 dB for headsets.

Use lossPlanPrt to show the changes:

Parameter          Default    Offset    Result

---------------    -------    ------    ------

HandsetRLR              2         0         2

HandsetSLR             11        -3         8

HeadsetRLR              0         0         0

HeadsetSLR             11        -3         8

HandsfreeRLR           13         0        13

HandsfreeSLR           16         0        16

- Back to contents

2.14 Customer data block (cdb - overlay 15)

RGNA in the CDB (MPO) should be set to STD STD.  It is normally AAR AAR.

This will prevent transferred calls to an extension which has FNA/EFD set to another extension from reverting back to the transferee when the forward no answer timer (ring cycles) matures rather than following the call forward treatment.

- Back to contents

2.15 Meridian Mail DSP parameters

During installation of Meridian Mail you have the option to change the DSP parameters.

The recommended settings are shown below:

Changes from the default settings are shown in bold.

| DSP Encoding Type | ALAW |
| --- | --- |
| Disable Silence compression: | No |
| Do you wish to change other DSP parameters? | YES |
| Transmit level: | 0 |
| Receive Level: | -7 |
| DTR Reject Level: | (-36) |
| DTR Max Accept Level: | (1) |
| Disable AGC: | No |
| AGC Centre: | -20 |
| Hook Flash Pulse: | 320 |
| Telescan Debounce: | (128) |
| Telescan Ring Time: | 1024 |
| Select operation: | Done |

- Back to contents

2.16 Meridian 1 supported software releases

The preferred software releases for investigation of new problems are highlighted below. If problems were reported on sites with earlier software releases, the Meridian National Technical Support group may recommend that an upgrade be performed to bring them up to this minimum level. There may be circumstances when a “bug fix” upgrade is appropriate, however definitive authority must come from Nortel Networks. Upgrades from an un-sustained release must be classed as a commercial upgrade at cost to the customer, even to eradicate a fault condition.

For further information please contact the Meridian STC.

| Machine Type and Software | Software release |
| --- | --- |
| Option 11C X11 Release 24 | 24.25 |
| Option 11C X11 Release 25 | 25.15 |
| Option 11C X11 Release 25 | 25.40 |
| Option 11C X11 Release 25 | 25.40b |
| Option 11C X21 Release 3 | 3 |
| Option 51C,61C,81,81C X11 Release 24 | 24.25 |
| Option 61C , 81C X11 Release 25 | 25.15 |
| Option 61C , 81C X11 Release 25 | 25.40 |
| Option 61C , 81C X11 Release 25 | 25.40b |
| Option 61C , 81C X21 Release 3 | 3 |
| CSK 1000 X21 Release 3 | 3 |
| CSK1000 X21 Release 2 | 2.2 |
| CS1000S/SG/HG/MG X21 Release 4 | .00T and .50W |
| CS1000E, MG1000E/B/T X21 Release 4 | .00T and .50W |
| Meridian 1 pbx 51C, 61C, 81C X21 Release 4 | .00T and .50W |
| Option 11 Release 18 | 18.42 * (H/W Break point) |
| Option 11E Release 21 | 21.54 * (H/W Break point) |
| Option 11C Release 22 | 22.16 * (H/W Break point) |
| Option 11C Release 23 | 23.47 * |
| Option 51,61,71 (Omega) Release 16 | 16.92 * (H/W Break point) |
| Option 51,61,71 (Omega) Rlse 18, 20, 21 & 22 | 22.16 * (H/W Break point) |
| Option 51C,61C,81,81C, Release 23 | 23.47 * |

- Un-sustained from Nortel Networks, this means the platform and software will not be subject to any further development work, fault investigation or patch reworks. Existing fixes will be available for deployment to site, bugfix upgrades will not be available on these platforms.

- Back to contents

2.17 Patching

Software release 25.15 saw the introduction of Service packs which provide a single “PEP” containing several product specific PEPS. Software release 25.40 introduced Product Dependency Lists which allow quick identification and delivery of Product Enhancement Paks (PEPS) that are recommended for Meridian 1 and Succession CSE 1000 Systems.  A Dependency List PAK, which is a subset of the Dependency List, is a merge of individual PEPS available from previous issues of the Dependency List.  The PAK provides an efficient method for downloading portions of the Dependency List as the user must select only a single PEP Binary File Name.  A secondary advantage of the PAK is that when patches are merged, they consume fewer patch handles.  When using DEP PAKs, please ensure that any additional individual PEPS to complete the Dependency List are included.

Software release 4.00T and 4.50W saw the introduction of a new concept of patching  with the introduction of downloadable .zip files containing dependency list’s. Also a new command structure in LD 143 for loading these zip files to the systems.

*IMPORTANT NOTE*

The dep lists at 4.50W are dynamic, and at 4.00T Dep Vers 10 on so the recommendation is to download the zip files on a regular basis.

MatrixDeplist_User Guide’s can be obtained from the Patching Duty 0800622396 or via the web address below

BT recommends that the latest service or dependency packs and lists be loaded on ALL machines.

Failure to comply with this recommendation may delay issues being escalated to Technical Support and or Nortel Networks for advanced support.

Please visit http://minerva.nat.bt.com/meridian1/patchinfo/ for further help and latest issues of patches etc.

- Back to contents

3

- Credits and document history

| Document author: Allan McColl | Version no:8 |
| --- | --- |
| Document owner: Gordon Kinrade | Date: 16th May 2006 |
| Update: | Reason for update: Update of CFN 177 to PSN format. Also to include release 4 and 4.5 information. |