Octel 200/300

Message Server

# Configuration Note 6199   Revision 4 10/99

Nortel Meridian M1/SL1*
(NPIC Card Based)

(Europe)

## 1.0  method of integration

The NPIC controls up to 16 digital 2616 ports connected to the PBX, depending upon the number of  Octel ports required.  Each port connection to the NPIC appears to the M1 PBX as a Northern Telecom 2616 digital display telephone.  The Octel 200/300 message server receives the necessary call information from the M1 display associated with the digital ports of the NPIC.  The ports perform both the function of collecting call information and answering/transferring the call.  The NPIC commands the M2616 to read the call information from the display and transmits it to the Octel system which then answers with the appropriate personal greeting.  Message Waiting is also performed using M2616 Digital Telephones, which are dedicated to that purpose.

The NPIC interprets call information from the M2616 display and sets and cancels message- waiting indicators

## 2.0  OCTEL ORDERING INFORMATION

§	Nortel Per Port Integration Line Card (supports up to 16 ports)

Octel requirements

- Adaptive Integration Software (FP X0031)

§	Serenade Software 2.0 or higher – Please refer to Section 8.7.

§	25 pair cables with female end amphenol, one per NPIC card, long enough to connect the NPIC to the PBX mainframe which then connects to M2616 ports in the switch.  The connection to the NPIC card is through a 25-pair amphenol, male connector.

## 3.0  PBX HARDWARE REQUIREMENTS

§	NT8D02 Integrated Services Digital Line (ISDL) ports:
- Revision AB or higher
- Up to 16 ports

PBX hardware requirements

§	QPC578 Vintage C or higher

NOTE: Subscribers with 2500 sets with message-waiting lamps (NE-2500 YQAs) must have special line cards (part number NT8D09) and power supply (part number QSU22) to light the message-waiting lights.  If these components are not available, subscribers can use stutter dial tone for message-waiting indication.

## 3.1  PBX SOFTWARE REQUIREMENTS

Generic X.11, Release 14 to Release 22 with the following:

§	EES, Enhanced End-to-End Signaling, Option 10

PBX software requirements

§	SS25, Special Features for 2500 sets, Option 18

§	DDSP, Digit Display, Option 19

§	BACD, Basic Automatic Call Distribution, Option 40, see section 8.1

§	ACDA, Automatic Call Distribution, Option 45 see section 8.1

§	MWC, Message-Waiting Center, Option 46

§	DSET, Digital Set, Option 88

§	ARIE, Meridian Modular Telephone Set, Option 170

§	Subscriber class of service to call forward (including 
all-calls, ring-no-answer and hunt-on-busy)

§	Subscribers class of service for message-waiting

## 4.0  SUPPORTED FEATURES

§	Station forward to personal greeting
- all calls

Supported integration features

§	System forward to personal greeting
- busy
- ring-no-answer

§	Message-waiting
- lights
- audible message-waiting (stutter dial tone)

§	Direct call

§	Personal greeting of originally-called party on double-call forward  NOTE: Nortel software allows only two forwarding steps on a ring-no-answer

§	Reply to message left by subscriber in internal telephone-answering mode

§	Automated attendant

§	Multiple return-to-operator

§	Outcalling

## 5.0  Configuring the M-1 PBX

Refer to PRM, Integration volume, MERIDIAN-1 NPIC for complete information.

SPECIAL NOTE:  The following represents changes to the M-1 PBX that will only be essential for the Octel integration when configuring the Northern Telecom Network Services (Addendum A refers).  This basically refers to a single Overture 200 serving a network of Meridian PBX’s.  Items not addressed here do not require modification for integration with the Overture 200 and should remain as-is on the PBX.  When connecting an Ooverture 200 to a Meridian not in a network of other Meridian PBX’s, please ignore section 5.0 and go to section 5.1.

Before beginning programming, it is recommended that a hard copy of the customer data block be printed in overlay 21 to verify the existing programming.  It is also recommended that a hard copy of the terminal number block (TNB) be run for M-1 sets and for 2500 sets to show existing programming.  Use overlay 20 to retrieve this information.

If necessary, change the following parameters in overlay 15 to the underlined values.  The default is in parentheses.

REQ	CHG

Programming PBX system parameters

TYPE	CDB			Customer data block

CUST	0-31			Customer number

OPT		IDP 			Digit Display allowed

MCI,			Message center allowed

Customers can selectively forward  DID verses non-DID calls on busy or ring-no-answer condition. This can be done for calls being routed to the message center.  Program the following as requested:

MDID	YES, (NO)		No-answer DID calls are (are not)
					routed to the Message Center

NDID	YES, (NO)		No-answer non-DID calls are 
					(are not) routed to the Message
					Center

MWFB	YES, (NO)		DID calls encountering busy are
					(are not) routed to Message 
					Center

There are four ways to which calls can					be answered system-wide for non-DID 					and DID calls:

FDN = Follow forwarding DN						HNT = Hunt								ATT = Attendant

FNAD 	FDN, HNT, ATT, 	Program as required by your customer					Forward DID calls

FNAN	FDN, HNT, ATT,	Program as required by your customer					Forward non-DID calls

FNAL	FDN, HNT, ATT, 	Program as required by your customer

## ** THESE ARE SYSTEM WIDE DISPLAY CHANGES AND SHOULD BE DISCUSSED WITH THE CUSTOMER ASAP IN ORDER TO NOTIFY ALL OPERATORS AND DIGITAL DISPLAY PHONE USERS**

5.1  PROGRAMMING THE NPIC PORTS TO HUNT

A hunt **CHAIN** should be created using the HUNT field associated with each digital port.  **THE LAST PORT SHOULD HUNT TO THE FIRST PORT IN A ROUND ROBIN STYLE**.  If the customer has a Northern Telecom M-1 Option 11 or 51, the maximum number of members in a hunt group is 18.  If the customer has an M-1 Option 51, 71, or 81, the maximum members per hunt group is 30.  On any M-1 PBX, this limit can be expanded through the use of the “pseudo hunting” with Basic ACD Software.  Setting up pseudo-hunting involves directing the last port in the first group to an ACD to a second group.  This ACD DN should be night-call forwarded to the next hunt group allowing additional ports to hunt.  Hunt groups can be chained together in this manner to support up to 128 total members.  Although the switch documentation indicates 18 hunt steps for smaller switches and 30 hunt steps for larger switches, field experience indicates that 16 hunt steps should be used for smaller switches and 28 hunt steps for larger switches.  Refer to section 5.3 for further details.

PBX hunt group limitations

## 5.2  Configuring the M-1 Terminal Number(s)

One M-1 TN is configured for each M2616 required.

Following is an example of a four port Octel 200/300 message server hunt group where 3000 through 3003 are port extensions.  The pilot number of this hunt group is 7777.

Extension		Hunt Field Extension	RNA Field Extension

7777 Pilot No.		3001			3001		3001			3002			3002		3002			3003			3003

3003			**7777**

Note:  The Pilot number is the last few digits of the DDI number of the Serenade.  It must be assigned to the first port of the Serenade.  This should be programmed to hunt in a linear fashion on busy to the next available Serenade port in the hunt chain.  The example above shows a pilot number of 7777 which could be for a DDI of say 0171-998-7777 for the voice mail.  Internal users call 7777 to access voice mail.

Using overlay 11, program the M2616 sets that will be connected as links for the NPIC.  Assign key 0 **A PHANTOM OR SPARE NUMBER - THIS NUMBER WILL NEVER BE ‘RUNG’** extension number of the port.  Program one digital telephone for each Octel port.  When defining the keys for each 2616, carefully follow the key allocations listed below.

Example for Port 1

REQ	NEW

TYPE	2616			Digital set data block

TN		lll S CC U		Terminal number: loop (0-159) 
					shelf (0-1) card (1-10) unit (0-7)

CDEN	SD, (DD), 4D		Density of this card is
		D8			single, (double), quad

CUST	0-31			Customer number

DES		OCTEL1		Designation for telephone

CLS		HTA 			Allow hunting sequence on busy		ADD 			Allow automatic digit display			HPR			Allow high priority				DNDA 		Allow dialed name display allowed		CNDA 		Allow call party name display			WTD 		Warning tone denied				MWD 		Message waiting denied				**HFD**		Hands free Denied (Allows 						configuration of key 15 as 						DSP)

AHD			Automatic Hold Denied

KEY	00 SCR xxxx		Single call ringing DN key						where xxxx is the extension assigned 					to the port. (e.g. 7777 as above)

15 TRN		Transfer key

HUNT	xxxx		Hunt to next port (e.g. 3001 as above).

Example for Port 2

REQ	NEW

TYPE	2616			Digital set data block

TN		lll S CC U		Terminal number: loop (0-159) 
					shelf (0-1) card (1-10) unit (0-7)

CDEN	SD, (DD), 4D		Density of this card is
		D8			single, (double), quad

CUST	0-31			Customer number

DES		OCTEL2		Designation for telephone

CLS		HTA 			Allow hunting sequence on busy		ADD 			Allow automatic digit display			HPR 			Allow high priority				DNDA 		Allow dialed name display allowed		CNDA 		Allow call party name display			WTD 		Warning tone denied				MWD 		Message waiting denied

AHD			Automatic hold denied

KEY	00 SCR xxxx		Single call ringing DN key.						where xxxx is the extension assigned 					to port 2 (e.g. 3001 as per example)
		15 TRN		Transfer key

HUNT	xxxx			Hunt to next port (e.g. 3002)

§	It is important to ensure that the programmed SCR appearances exactly match the physical port connections to the Octel, and that the SLOTS table reflects the correct port numbers.  NOTE:  Message Waiting Indication Ports must NOT be in the hunt chain.  Please also ensure the Message Waiting Indication ports are programmed as shown below (one per MWI port):-

Programming the digital port for the M2616 sets

Example for Message Waiting Port:

REQ	NEW

TYPE	2616			Digital set data block

TN		lll S CC U		Terminal number: loop (0-159) 
					shelf (0-1) card (1-10) unit (0-7)

CDEN	SD, (DD), 4D		Density of this card is
		D8			single, (double), quad

CUST	0-31			Customer number

DES		OCTEL2		Designation for telephone

CLS		HTA 			Allow hunting sequence on busy		ADD 			Allow automatic digit display			HPR 			Allow high priority				DNDA 		Allow dialed name display allowed		CNDA 		Allow call party name display			WTD 		Warning tone denied				MWD 		Message waiting denied

AHD			Automatic hold denied

KEY	00 SCR xxxx		Single call ringing DN key.						where xxxx is the extension assigned 					to the MWI port (port 8 or 16)
		13 MIK		Message indication key

14 MCK		Message cancellation key			15 TRN		Transfer key

Run a hard copy of the terminal number block (TNB) for each M-1 set to reflect and verify the additions and changes just made.  Use overlay 20.

  - CALLING PARTY NAME DISPLAY PROGRAMMING

Calling Party Name Display must be programmed in LD 95 to show the condition of the forwarded call.  Program the Calling Party Name display as follows;

REQ		CHG							CNFG								MXLN								STAL								DFLN								DES									RESN	YES							CFWD	FWDA							CFNA	CFNA							HUNT	BUSY							PKUP	PKUP							XFER								AAA								NITC

NOTE:  Items left blank do not require changes.

5.4	OPTIONAL OVERLAY 23 PROGRAMMING

This programming is required if the Octel system has greater than 18 ports on a Meridian M-1 Option 11, 21, 51, or 61, or the Octel system has greater than 30 ports on a Meridian M-1 Option 71 or 81. ACDA, a customer-purchased option on the Meridian M-1 PBX, is required.

If the number of Octel ports exceeds the hunt-group size limit for your PBX, this programming is required

To view current programming in Overlay 23, at the initial REQ 	type PRT based on the application.

The PBX technician should assign a non-DID number as the ACDN and configure this number in the hunt field of the last member of the first hunt group (e.g. port 19 or port 31).  The datablock for this ACDN should have the next port in sequence configured as the NCFW target.

Assign the ACD-DN (Directory Number), and night-call-forward 	target as follows:  (This is done in Overlay 23)

REQ	NEW			To add an ACD data block TYPE

TYPE	ACD			Automatic Call Distribution 
 					data block

CUST	0-31			Customer number

ACDN	XXXX		ACD directory number 
					(ACD-DN)

MWC	YES			Message-Waiting Center

MAXP	1			Maximum number of positions

NCFW	XXXX		Night call forward DN for ACD
					(will be the DN of the first number of 					the next **MEMBER OF THE HUNT 					CHAIN**).

If more than two hunt **CHAINS** are required to support the number of Octel ports, repeat this process to join the third hunt group to the second.

## 6.0  Configuring the Octel Voice-Processing Module

Refer to Installation and Maintenance Manual, Integration Volume, for complete information.

Octel Message Server Configuration

¨	SYSTEM PARAMETER TABLE:

§	Set Parameter 3 to Northern Telecom Meridian 1 (Option 15 for Manufacturer Number , then Option 2 for Model Number)

§	Set System Parameter 33 to be NONE

§	Set System Parameter 45 to NONE

§	Set System Parameter 46 to NONE

§	Set System Parameter 77  to YES

§	Set System Parameter 116 to be YES

§	Set System Parameter 117 to be 5

§	Set System Parameter 170 to NO

§	Set System Parameter 198 to 1 (‘A’ Law)

§	Set System Parameter 254 if non-standard transfer codes are 	required

§	System Parameter 270 should be set to NONE.  If a change is 	required, contact Technical Support.

¨	SLOTS TABLE:

The M2616 sets connect to the Octel via a 50-pin male telco connector.  This DB25 connects to P1 through P11 at the rear of the system.  For correct functionality, each port must be set up to do either call processing or message waiting.  When links are configured in the Slots Table, assign them in the following manner:

When ports are configured for message waiting assign them in the following manner:  Start with port 16 on the last NPIC card, and using port 16 on each card, work towards the first NPIC.  If additional message waiting capacity is desired, start with port 8 on the last NPIC card and work again toward the first card.  This allocates message waiting activity evenly across processors in the NPIC card.  There are two processors on each card, one for the first 8 ports and one for the second 8 ports.  Additionally, if the links assigned to message waiting activity are spread across the cards and loops in the switch, the message waiting performance is better.  (Call processing ports should also be spread across cards/loops on the switch to enhance performance and increase reliability).

§	Configure all ports for AX answering mode.

§	Set the oucalling column to NO for ports configured for message waiting indication.  Ensure only a small proportion of the call processing ports are allowed to outcall.  If only one NPIC card is used it is recommended that only the last few ports are allowed to outcall.  If there are, for example, three NPIC cards it is strongly recommended that the first NPIC is not allowed to outcall, and only four ports per processor on the remaining NPIC cards can outcall to handle non-message waiting indication outcalls. (e.g., networking, off-site/on-site message waiting notification).

§	Configure TEST for NO on all ports.

§	The LSPTAB Table is configured for the choice 24-PIC_NT.

One NPIC in the message server must be designated as primary, to provide clock synchronization with the PBX digital card supporting the NPIC.  When more than one NPIC card is installed in the message server, the second card is designated as secondary, and the third card as tertiary.  The remaining cards installed are designated as NONE.

An example of a slots table for a 32 port NPIC integration is shown on the next page:-

SLOT CARD TYPE   PORT  EXTENSION# COS MODE OUTCALL TEST SYS

----- ------ -----   -----  ------------- ---- ------ ---------- ---------

1    PICNT		      1           7777           254    AX           NO          NO   1

2           3001           254    AX           NO          NO   2

3           3002           254    AX           NO          NO   3

4           3003           254    AX           NO          NO   4

5           3004           254    AX           NO          NO   5

6           3005           254    AX           NO          NO   6

7           3006           254    AX           NO          NO   7

8           3007           254    AX           NO          NO   8

9           3008           254    AX           NO          NO   9

10         3009           254    AX           NO          NO   10

11         3010           254    AX           NO          NO   11

12         3011           254    AX           NO          NO   12

13         3012           254    AX           NO          NO   13

14         3013           254    AX           NO          NO   14

15         3014           254    AX           NO          NO   15

16         3015           254    AX           NO          NO   16

Message Waiting Link= 8 **DEFAULT**

COS=

Message Waiting Link= 16

LSP table: PIC_NT

PRIMARY SYNC RECEIVER OF CLOCK

SLOT CARD TYPE   PORT  EXTENSION# COS MODE OUTCALL TEST SYS

----- ------ -----   -----  ------------- ---- ------ ---------- ---------

2    PICNT		      1           3016           254    AX           NO          NO   1

2           3017           254    AX           NO          NO   2

3           3018           254    AX           NO          NO   3

4           3019           254    AX           YES        NO   4

5           3020           254    AX           YES        NO   5

6           3021           254    AX           YES        NO   6

7           3022           254    AX           YES        NO   7

8           3023           254    AX           NO          NO   8

9           3024           254    AX           NO          NO   9

10         3025           254    AX           NO          NO   10

11         3026           254    AX           NO          NO   11

12         3027           254    AX           YES        NO   12

13         3028           254    AX           YES        NO   13

14         3029           254    AX           YES        NO   14

15         3030           254    AX           YES        NO   15

16         3031           254    AX           NO          NO   16

Message Waiting Link= 8

COS=

Message Waiting Link= 16

LSP table: PIC_NT

SECONDARY SYNC RECEIVER OF CLOCK

¨	COS TABLE:

§	Add attribute 6 to call extension first before playing greeting.

§	Add attribute 9 to COS for sets with message waiting indicators.

§	Add attribute 58 to PORT COS for those ports used in special 	applications that do not require integration.  These ports would be 	supported by analog line cards rather than the NPIC cards.

## 7.0  Troubleshooting — Verify M-1 TNS Are Active

To ensure that the M-1 TNs are active, use overlay 32:

Verify that the digital channels of the NPIC are active

STAT	XXXX		Where XXXX is the TN of
					the M-1 programmed as an
					Octel port

The status displayed is either:

IDLE or DSBL

IDLE status is desired. If DSBL is displayed, enable them with 	the following in overlay 32:

ENLU	XXXX		Where XXXX is the TN of 
					the disabled TN

7.1  TESTING THE INSTALLATION

Refer PRM, Integration Volume, Meridian M-1, section 13.5.

Steps to test the installation when completed

o	Complete the test procedure listed in the Serenade Integration Volume V for the NPIC Integration.

o	After completing the test procedure, delete all test mailboxes, and return any altered PBX name display programming to the desired entries.

## 8.0  CONSIDERATIONS/ALTERNATIVES

8.1	Determine if the ACD option is required. Meridian integration uses hunting and does not require ACD unless the number of ports exceeds the number of hunting steps available on your PBX model. Eighteen steps are supported on the Options 11, 21, 51, or 61. Thirty steps are available on the Options 71 and 81. If the number of ports exceeds the number of hunting steps, ACD is required to expand the hunting capability.

Important notes concerning this integration

8.2	Verified software levels are 14 through 23.xx.

- “Secretarial forwarding” rings the station instead of forwarding.  Secretarial filtering allows a caller to forward his/her telephone to a secretary, permitting the designated secretary to ring the station of the forwarded party even though the telephone is in an all-calls-forward mode.

If a subscriber has his/her station all-calls forwarded to the voice processing module, the first port of the voice- processing module hunt group will act as the secretarial position.  Therefore, any transfers from the first port in the hunt group of the voice module will not all-calls forward, and instead will receive ring-no-answer.  In other words, if the first port of the voice-processing module hunt group performs a transfer to a station that is all-calls forwarded to the voice-processing module, the station will ring instead of forwarding the call.  A workaround for this is to set all stations to forward primary number of the hunt group, but direct all trunks to the second port.  This will ensure that trunk calls will not try to be transferred on the first port.

- Network Message Services is supported with the NPIC.  The minimum software is Release 15, and requires the following software options: Network Message Services (option 175), Advanced ISDN Network Feature (option 148),  ISDN Signaling (option 145),  ISDN Primary Rate Access (PRA, option 146) or ISDN Signaling Link 1 (ISL, option 147), Message Center (option 46), End to end signaling (option10), and Coordinated Dial Plan (CDP, option 59) or Network Automatic Route Select (NARS, option 58).

- Automated Attendant/ECP applications transferring calls between NMS nodes may experience loss of integration.  When Trunk Route Optimization is enabled on NMS trunks, calls transferred to stations on remote switches that forward without first ringing the station do not pass called party ID to the NPIC port.  Currently there is no workaround for this issue.

- Network Call Re-Direct on Meridian PBX Network.  Please note that there is a parameter called ‘Network Call Re-Direct’ in the trunk route programming (overlay LD14/LD16) of the Meridian programming which will cause integration problems when set to ‘YES’.  The integration problem that has been found is when a caller contacts the operator and they ask to be connected to a remote extension user who is all calls forwarded to voice mail.  The Operator performs an unsupervised transfer to the extension number (release to ringing) and the caller basically ends up at the Company Greeting rather than the Personal Greeting of the called persons mailbox.  This problem will only surface when the customer has one voice mail system which serves the local site and remote sites using a network of Meridian PBX’s.  Please ensure this parameter is disabled, otherwise there is no workaround for the issue with the voice mail integration delays.  This also applies to SIC 8 integrations.

- NPIC on Serenade 2.0/3.0  Please note that only one LSP table is available on early versions of Serenade 2.0.  This will be evident when configuring the slots table; only the default LSP table will be available to choose.  Also note that image NPIC image 2.12 will also be the default image on S.2.0.  This image is known to have problems, one of which is the ports locking up.  Octel Recommend that this image be upgraded to 2.23 minimum.

This is known to be a good image which has the patches available for it.  Although NPIC works with Serenade 2, Octel strongly recommend that the customer has Serenade S.3.0.1-2 software installed as it has the 2.23 NPIC image incorporated within it.

- NPIC on Serenade 4.0.  Please note that Serenade 4.0 will not have line card patches.  The LSP table is now used where certain LSP bytes can be adjusted to achieve what the patches achieved for Serenade 3.0.0-1 and below.  Please contact UK TAC to get the LSP table altered if you encounter any line card related problems, like DTMF 3 of DTMF B detection.

- Adjustment of Application Delays.  Please ensure the application delays 129 and 130 are correctly adjusted.  Contact the Euro CSC in order to ensure these application delays are correctly set.  The values they should be set to are 2000ms for application delay 129 and 8000ms for application delay 130.  These application delays control the speed of message waiting indication (MWI) and message waiting notification (MWN) respectively.

- Outcalling configuration for the NPIC in the slots table.  Please ensure that only a small number of ports in the hunt group can also make outcalls.  A typical figure will be one third of all CPI ports, although this figure may need to be fine tuned.  It is recommended that only 4 ports per processor on the NPIC will be able to outcall.  If the system only has one NPIC installed then only allow the last few ports to outcall.  If there are two or more NPIC ports then 4 ports per processor (8 ports per NPIC) should anly be allowed to outcall.  The first NPIC should not be allowed to outcall in this set-up.  This will ensure we have redundancy with outcalling

®™ Registered trademark of Lucent Technologies - Octel Messaging Division.

© Copyright 1998 Lucent Technologies - Octel Messaging Division.  All rights reserved.

Printed in the United States of America

## Lucent Technologies

## Octel MEssaging Division

Octel House, Ancell’s Road
Fleet, Hampshire, GU13 8UN.

(CFN 6199)

Rev 2(3/99)

## (Part No. 001-7-6199)

## 9.0  Addendum A -

## Northern Telecom’s Network Message Services

The Northern Telecom Network Message Services-Message Center feature is designed to allow a centralized message center to serve multiple Northern Telecom PBXs.  With this feature an Octel system, acting as a message center, can provide integration for multiple Meridian SL-1 or Meridian M-1 PBXs in an ISDN network.  Remote subscribers receive all integration features.

The Northern Telecom Network Message Services-Message Center feature is designed to allow a centralized message center to serve multiple Northern Telecom PBXs

The Northern Telecom Network Message Services feature requires one of the following Northern Telecom networking capabilities: ISDN Primary Rate Access (PRA) or ISDN Signaling Link (ISL).  Both require a signaling path, called the D-channel. PRA networks utilize a dedicated channel of a T1 span for the D-channel. ISL networks utilize a dedicated data line for the D-channel, usually requiring modems.

To support an ISL network for integration, the D-channel must operate at 9600 bps or faster. Slower speeds can cause message waiting notification to become unreliable. There is no restriction for PRA networks, which support a D-channel operating at 56 kbps.

Below is a summary list of the PBX requirements. This list is not intended to be all inclusive; it is meant as a guideline only. Some options have hardware and software prerequisites that are not listed. It remains the responsibility of the customer and PBX vendor to identify all necessary hardware and software components and to correctly configure the Northern Telecom Network Message Services-Message Center.

NORTHERN TELECOM PBX REQUIREMENTS:

§	Integration Requirements for Local PBX-refer to appropriate Configuration Note

§	X11 Release 16 Software (Minimum)

§	Network Message Services (option 175)

§	Advanced ISDN Network Features (option 148)

§	ISDN Signaling (option 145)

§	ISDN Primary Rate Access (PRA, option 146) or ISDN Signaling Link1 (ISL, option 147)

§	Message Center (option 46)

§	End-to-End Signaling (option 10)

§	Coordinated Dial Plan (CDP, option 59) or Network Automatic Route Select (NARS, option 58)

¹ With ISL networking, the D-channel must operate at a minimum of 9600 bps.

®™ Registered trademark of Lucent Technologies - Octel Messaging Division.

© Copyright 1997 Lucent Technologies - Octel Messaging Division.  All rights reserved.

Printed in the United States of America

## Lucent Technologies

## Octel MEssaging Division

Octel House, Ancells Road, Fleet,
Hampshire, GU13 8UN
(+44) 1252-303000

CFN 6161

(Rev  1  12/98)

(Part No. 001-7-6161)