# OMEGA via V25.40b to Succession upgrade process

This document has been devised to list the various methods involved in upgrading X11 software from V16.xx onwards for the Meridians Option 51 onwards to X21 Succession Release V03.00. The process will differ, depending upon the type of database disk supplied to us, via the field engineer from site.

I will list the main points in Bold & use itallic wording. I will explain in detail any problems that may occur.

The reason for having to do this double conversion procedure is as follows...

- There is no direct path to convert from Omega software to CPP hardware. This is because the CPP processor has no IDE connector to attach the ribbon cable from the Omega MDU disk drive.

- Some 4 Meg disks will have “C” type software on them & they will have to be converted by copying 2 “*.rec” files onto a 1.44 disk from a 4 Meg drive.

- Certain changes will have to be made to the customers database at V25.40b, before it’s then upgraded to Succession, such as changing set type to “Wirless” etc.

- Option 51 systems must have a “CEQU” setting changed in LD 17 at V25.40b. This will be to include a second network card, such as another “PRI2” i.e.

CEQU

MPED 8D

TERM

REMO

TERD

REMD

TERQ

REMQ

SUPL  048 064 080

SUPC

SUPF

DDCS  004  005  006  007

DTCS

XCT   002  030  032  062

078  094

TDS  * 002 * 030 * 032 * 062

* 078 * 094

CONF * 003 * 031 * 033 * 063

* 079 * 095

MFSD * 002 * 030 * 032 * 062

* 078 * 094

PRI2 016 017

APVL

REQ

Although it can be done at Succession V03.00, It won’t work!. This is due to a software fault at X21 level.

Please refer to the following pages for full details on upgrades.

Page 3 lists an example of a 4 Meg V23.47 upgrade to V25.40b.

Page 14 lists an example of a 2 Meg V16.91 upgrade to V25.40b.

Page 23 lists an example of a 1.44 Meg V25.40b upgrade to Succession V03.00 CP4.

Generally speaking, Any 4 Meg Omega disk will need to be upgraded, By using the ribbon connected via the MDU & onto the CD-Rom ribbon cable.

Note!

We have reserved one Network cube, which should be used for upgrading to V25.40b only. It has the ribbon cable already attached with a default database & keycode installed. Please do not use / borrow any of these parts, as they are required to help speed up the database conversion process.

The software which has already been installed on it is a V25.40b Option 81c with a CP4 processor.

The site ID on this cube is 46379 & the ISM parameters are set to very high values to allow for any variables on the customers databases. I have used a Meridian 1 keycode generator program to create the keycodes for a Option 61c & a 81c on a CP4 processor. However it’s best to just use the 81c keycode file.

There are 3 different types of database disks, & they all need to handled differently.

2 Meg (1.44) Floppy disks were used on Meridian 1 Version 16 type software & they are of a different format to the microsoft DOS format version.

4 Meg Floppy disks were generally used on Meridians from Version 18 onwards up until Version 24. The ribbon cable method should be used for all Omega’s, except for the early “C” processors. The “C” type 4 Meg disks will require the use of a computer that has a 4 Meg disk drive for copying the database.rec & config.rec files onto a 1.44 Floppy disk.

1.44 Meg Floppy disk are used for all the “C” processors with a CD-Rom installed, from Version 23 onwards. The upgrade for these can be done on site.

It does not matter that the site ID 46379 will be different at V25.40b during the upgrade process!.

All writing listed in itallic, is information that you may find to be useful.

I would recommend that you keep a different Floppy disk copy of the various databases, completed at eash upgrade stage!. e.g. Omega, V25.40b & Succession etc to be on different Floppy disks, so that you can backtrack to them if it’s required.

## 4 Meg Floppy disk upgrade method to V25.40b

The information listed below has been taken from one of the upgrades that I did, Using a 4 Meg Floppy disk. This example will also show you how to convert the customers database via “PDT” for  “C” software .

Insert the V25.40b CP3 / CP4 Installation floppy disk in first, before you reboot!

SYS700 02 - ROM created on Monday July 29 13:50:42 EDT 2002

SYS702 64 - Total Flash size (in megs), all banks populated

SYS702 64 - DRAM size (in megs) - Banks 1234     populated

SYS703 0DEB - Self Tests completed

SYS700 04 - Selecting Boot Source

Meridian 1 X11 ROM System Software

Release     : x112540b

Created     : Monday July 29 13:50:42 EDT 2002

Copyright(C) 1972-1997 Northern Telecom, Inc.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'os' from "/f0/p/os/diskos" ... 760740: 512000 + 248740 + 53056

Starting at 0x8040020...

Meridian 1 X11 System Install Utility

Created     : Monday January 14 11:57:25 EST 2002

Copyright(C) 1972-1997 Northern Telecom, Inc.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1997 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'install' from "/f0/p/install/install" ...222116 + 112664 + 371376

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

==============================================================

**

***

****     **      ****

****   *****    **********

****  *******  ***********

**************************      Nortel Networks - Meridian 1

****     ****    ****           Install Tool (Version 38)

****     ****    ****

****     ****    ****

****     ****    ****

****     *****************

****      ******  ******

****       ***     ***

Copyright 1999 - 2001  Nortel Networks

Please press <CR> when ready ...

>OBTAIN and CHECK SYSTEM CONFIGURATION

>Validate hard disk partitions

>Copy /f0/disk2411.sys to /u/disk2411.sys - 23174 bytes copied.

>Copy /f0/disk2511.sys to /u/disk2511.sys - 23174 bytes copied.

>Copy /f0/disk2611.sys to /u/disk2611.sys - 23046 bytes copied.

>Copy /f0/disk2811.sys to /u/disk2811.sys - 23174 bytes copied.

>Copy /f0/disk2911.sys to /u/disk2911.sys - 23174 bytes copied.

>Copy /f0/disk3011.sys to /u/disk3011.sys - 23046 bytes copied.

System Date and Time now is:

00-00-0000, 00:02:14

INST0077 System Date is not correct, please set it to the current

Please press <CR> when ready ...

Current date is:  00-00-0000

Enter new date (dd mm yyyy) ? 24 05 2004

Date is set to: Monday 05-24-2004

Current time is: 00:02:22

Enter new time (hh mm ss) ? 16 16 00

Time is set to: 16:16:00

System Date and Time now is:

Monday 05-24-2004, 16:16:00

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

==============================================================

M A I N    M E N U

The Software Installation Tool will install or upgrade Meridian-1

System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout  the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <u> - To Install menu

<t> - To Tools menu.

<q> - Quit.

Enter Choice> u

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

==============================================================

Please insert the diskette with the keycode file into the floppy drive.

Please enter:

<CR> -> <a> - Continue with the keycode validation

(the keycode diskette is in the floppy drive on Core 0).

<q> - Quit.

Enter Choice> a

>Validating Keycode

1114 bytes copied.

The provided keycode authorizes the install of X112540 software

(all subissues) for machine type 3011 (68060E processor on Option 81(C)).

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

Please confirm that this keycode matches the CDROM Release

Please enter:

<CR> -> <y> - Yes, the keycode matches. Go on to Install Menu.

<n> - No, the keycode does not match. Try another keycode diskette.

Enter Choice> y                      (Use the site ID 46379 V25.40b Option 81c keycode floppy disk)

>Obtain database file names

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Meridian-1

System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout  the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 set Languages.

<q> - Quit.

Enter Choice> d     (Assuming that the V25.40b software is already installed!)

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You will now perform the database installation.

Note: If you are installing  the Database from a floppy disk,

please insert the correct disk now.

Please enter:

<CR> -> <a> - Install CUSTOMER Database

(the customer database diskette must be in the Core 0 disk drive).

<b> - Install DEFAULT Database

(the installation CDROM must be in the Core 0 disk drive).

<c> - Transfer the previous system Database.

<d> - Copy Database from the redundant disk.

<e> - Check the Database that exists on the hard disk.

<f> - Transfer Database from MDU.

<q> - Quit.

Enter Choice> f

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected Database Media Transfer from MDU.

This operation allows you to transfer the system database from a

floppy diskette in the MDU to the system hard disk.

Check that:

1. cable is connected.

2. MDU is powered-on.

3. MDU is jumpered as side 1.

4. Floppy diskette is in the MDU's floppy drive.

Please enter:

<CR> -> <a> - Continue.

<q> - Quit.

Enter Choice> a

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected Database Media Transfer. This operation allows you to

transfer the system database from a 4MB floppy diskette to the

system hard disk.

If necessary, you may check the database version on the floppy disk

before you begin the transfer.

Please enter:

<CR> -> <a> - Transfer the database from floppy to hard disk.

<b> - Display the version and issue of database on floppy drive.

<q> - Quit.

Enter Choice> a

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to transfer the Database from floppy to the hard

disk.  Please insert the source 4MB floppy disk  (B1) in the  floppy drive  now.

Please press <CR> when ready ...

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | No |  |  |
| SW: disk to ROM | No |  |  |
| Database | Yes |  | From MDU |
| CP-BOOTROM | No |  |  |
| IOP-ROM | No |  |  |

Please enter:

<CR> -> <y> - Yes, start Installation.

<n> - No, stop Installation. Return to the Main Menu.

Enter Choice> y

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to transfer the Database from the floppy disk to the

hard disk on Core 0. This will erase the database on the hard disk.

If you quit now, the database will be left unchanged.

Please insert the B1 disk into the disk drive now if you are

transferring the database.

Please enter:

<CR> -> <a> - Yes, transfer the database.

<q> - Quit.

Enter Choice> a

>Transfer Database from floppy drive to hard disk for side 1

>Remove temporary file "/u/db/database.hld"

>Remove temporary file "/u/db/database.tmp"

>Remove temporary file "/u/db/config.hld"

>Remove temporary file "/u/db/config.tmp"

>Create temporary file "/u/db/database.tmp"

>Create temporary file "/u/db/config.tmp"

>Start Database transfer from floppy to the hard disk

INST0060 Incorrect disk in drive, please insert the correct one

Please press <CR> when ready ...

Note!

If you get this message, then you must go & check the database floppy disk on a computer which has a 4 Meg disk drive, to see if it contain’s the “Database.rec” & “Config.rec” files!.

Create a directory on the hard drive in DOS & copy these two files into it. Then use a spare 1.44 floppy disk & copy them back onto that disk.

Go through the steps below to reboot the Meridian & allow it to load up as normal, with the existing database.

Follow the next steps below, to take the meridian back to it’s original state.

Otherwise carry on with the database conversion & reboot the Meridian.

Remember to insert a new spare blank Floppy disk! Om system power up.

#### Wait until the Meridian completes a “EDD”, Before extracting the system passcodes!

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

Please insert the B1 disk in the floppy drive.

Please enter:

<CR> -> <a> - B1 disk is now in floppy drive.

Continue with installation.

<q> - Quit.

Enter Choice> q

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to Quit. Please confirm.

Please enter:

<CR> -> <y> - Yes, Quit.

<n> - No, DO NOT Quit.

Enter Choice> y

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | No |  |  |
| SW: disk to ROM | No |  |  |
| Database | Yes | Quit | From MDU |
| CP-BOOTROM | No |  |  |
| IOP-ROM | No |  |  |

Please press <CR> when ready ...

>database recovery

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Meridian-1

System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout  the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 set Languages.

<q> - Quit.

Enter Choice> q

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to Quit. Please confirm.

Please enter:

<CR> -> <y> - Yes, Quit.

<n> - No, DO NOT Quit.

Enter Choice> y

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to Quit the Software Installion Tool.

You may reboot the  system or return to the Main Menu.

Before rebooting the system, remove Install diskette from the floppy drive(s).

-------------------------------------------------

DO NOT REBOOT USING BUTTON!!

-------------------------------------------------

Remove the V25.40b CP3 / CP4 Installation floppy disk, before you reboot!

Please enter:

<CR> -> <a> - Reboot the system.

<m> - Return to the Main menu.

Enter Choice> a

>Removing temporary files

>Remove /u/disk2411.sys

>Remove /u/disk2511.sys

>Remove /u/disk2611.sys

>Remove /u/disk2811.sys

>Remove /u/disk2911.sys

>Remove /u/disk3011.sys

>Quit Install. Reboot system ...

SYS700 02 - ROM created on Monday July 29 13:50:42 EDT 2002

SYS706 26 - Full Reset by Software Install

SYS702 64 - Total Flash size (in megs), all banks populated

SYS702 64 - DRAM size (in megs) - Banks 1234     populated

SYS703 0DE8 - Self Tests completed

SYS700 04 - Selecting Boot Source

Meridian 1 X11 ROM System Software

Release     : x112540b

Created     : Monday July 29 13:50:42 EDT 2002

Copyright(C) 1972-1997 Northern Telecom, Inc.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'os' from "/flash/flashos" ... 1249348: 512000 + 512000 + 225348 + 105260

Starting at 0x4100020...

Meridian 1 X11 System Software

Release     : x112540b

Created     : Monday July 29 13:50:55 EDT 2002

Loaded      : 24/5/04 16:20:12 from '/flash/flashos'

Copyright(C) 1972-2001 Nortel Networks.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Initialize Network Interface.

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

Network Interface is up.

Host         : MERIDIAN

IP Address   : 220.0.0.100

Sub-netmask  : 255.255.255.0

Loading symbols from "/flash/flashos.sym"

Executing startup script "/p/startup"

# Empty startup file for use in the field.

cd "/p/etc"

value = 0 = 0x0

Done executing startup script /p/startup

Start SNMP agent

Processed SNMP configuration file /p/data/view.cfg

Loading MAT script files........

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

INI000 00000002 16 21 33 00000000 00000000 00000000 00000000 0 ? 00000000

Total number of files 0

Warning: No corporate directory files were found!

SRPT750 INI 0: INI on side 0 due to System Cold Start

CNI200 cnib 0 12 configured but not present. (mismatch with database)

Check that CNI card is in the correct slot

CNI200 cnib 0 13 configured but not present. (mismatch with database)

Check that CNI card is in the correct slot

CNI200 cnib 0 14 configured but not present. (mismatch with database)

Check that CNI card is in the correct slot

SRPT751 INI 0: starting INI on side 0 due to System Restart

Previous INI: side 0 at 24/ 5/04 16:21:01

INIs since switch-over ( 0/ 0/00 00:00:00): 2, 0

SRPT752 INI 0: INI completed in 82 seconds

Let the Meridian run it’s “EDD” as normal, before loging into “PDT”. Follow the instructions listed in bold below (in lower case!) on transferring the files from the Floppy disk into the Meridian’s hardisk.

Use the “Enter” button after each of the “PDT” command’s listed in bold below!.

PDT:  login on /sio/0

Password:                               (Use the standard or the site specific password to login with!)

PDT in Progress. Please Wait....

Done!

pdt>

cd /u/db                                  (You will need to change the directory path in Unix to .. cd /u/db)

pdt> ll                                     (This will list out the current files within the folder db)

pdt> cp /f0/config.rec .          (Prompt to overwright the existing file?. Key “y”  to accept)

1024 bytes copied

pdt> cp /f0/database.rec .      (Prompt to overwright the existing file?. Key “y”  to accept)

194560 bytes copied

pdt> ll                     (This will list out the current files within the folder db but with 2 file changes! )

pdt> reboot –1                       (This will make the Meridian reboot from within “PDT”)

During the reboot, It’s best to have a capture file set-up to record any SYS, BUG, Messages that may appear. Some of the Messages are minor errors that are not inportant. Check out all the messages that are listed.

Remove the Floppy disk with the 2 files on it & insert  a new floppy disk.

Make a note on both disks details such as Version type, Name & Date etc.

The Meridian should now do a conversion upgrade on reload, to V25.40b Option 81c

PDT in Progress. Please Wait....DONE

SYS700 02 - ROM created on Monday July 29 13:50:42 EDT 2002

SYS706 41 - Software Full Reset - reason is 41

SYS702 64 - Total Flash size (in megs), all banks populated

SYS702 64 - DRAM size (in megs) - Banks 1234     populated

SYS703 0DE8 - Self Tests completed

SYS700 04 - Selecting Boot Source

Meridian 1 X11 ROM System Software

Release     : x112540b

Created     : Monday July 29 13:50:42 EDT 2002

Copyright(C) 1972-1997 Northern Telecom, Inc.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'os' from "/flash/flashos" ... 1249348: 512000 + 512000 + 225348 + 105260

Starting at 0x4100020...

Meridian 1 X11 System Software

Release     : x112540b

Created     : Monday July 29 13:50:55 EDT 2002

Loaded      : 24/5/04 16:24:50 from '/flash/flashos'

Copyright(C) 1972-2001 Nortel Networks.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Initialize Network Interface.

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

Network Interface is up.

Host         : MERIDIAN

IP Address   : 220.0.0.100

Sub-netmask  : 255.255.255.0

Loading symbols from "/flash/flashos.sym"

Executing startup script "/p/startup"

# Empty startup file for use in the field.

cd "/p/etc"

value = 0 = 0x0

0x8c2319c (tRptD):  (Boot ROM)              SRPT4504 Single CP configuration

Done executing startup script /p/startup

Unable to open file C1                                                   (Ignore any messages such as these)

^<3]]3".

Unable to extract patch header from C1

^<3]]3"

PDT Ready!

Loading SL1 .0x8c2319c (tRptD):  (Boot ROM)              SRPT720 ROM OS 0: Cold

Start. Running on CP 0

Loading 'res' from "/flash/sl1res"

15390860 + 395112 + 833412

Loading 'ovl' from "/flash/ovlres"

5460092 + 0 + 0

Start SNMP agent

0x8c2319c (tRptD):  (Boot ROM)              SRPT728 ROM OS 0: Loading 'diskos' from /flash/flashos

Processed SNMP configuration file /p/data/view.cfg

Loading MAT script files........

Loading HI Data from '/u/db/hi' ...

Loading SL1 data

SYS000 000849 000000 000000 000000 CPU 0

SYSDB RLS/ISS: 2347 DATE/TIME: NOT AVAIL. RECS: 191 SEQNO: 0

DATA CONVERSION

X11 ISSUE 23.47 TO X11 ISSUE 25.40B

CONFIG:  IDLE_SET_DISPLAY NORTEL

PHYS

BCS TEMPLATE

PBX TEMPLATE

CUST

P_AUX_CUST

ACUST

CLID

ROUTE

LTN

LTN LINK

TN BLKS

SCL

ESN  0

ESN_ROUTE_LIST

ESN DM

NCTL

ACD

NFCR_TREES

IDC_TREES

ASYNCH

AML/ELAN

VAS

BG_TIME

BG_CAT

ARIES

SYS_PARAM

XTDT

FTC

MCAD

FCAD

FDCT

DTI2SYTI

DTI2SCAT

FFC

LAPW

DTSL

CPND DATA

CPND NAME

DONEinterrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

INI000 00000000 16 25 38 00000000 00000000 00000000 00000000 0 ? 00000000

Total number of files 0

Warning: No corporate directory files were found!

SRPT752 INI 0: INI completed in 80 seconds

Let the Meridian do a normal system Dump “EDD” onto the new floppy disk.

Obtain via R.A.C.E , The PWD2 passcode for the site. Otherwise use the “Database.rec PWD2 Extractor program to retrieve the top two level passcodes / names etc, via the Floppy disk.

Log into the system & change the parameters in LD 17 under “PARM”

(For Option 51 upgrades, use “CEQU” & add in another network card 16 onwards!).

Use the settings that are listed within the NTP’s for Succession. It’s a good idea to check out some of the settings, such as SPN’s, FCR’s, etc. They may need to be updated.

I would recommend that you provide a “Brand” name using the customers details in LD 17 under “PARM". You will need the PWD2 code! & you only have up to 24 characters. This can be useful when the upgrade is carried out to V03.00.

## 2 Meg Floppy disk upgrade method to V25.40b

The information listed below has been taken from one of the upgrades that I did, Using a 2 Meg Floppy disk. The example shown was a version 16.91G upgrade.

Insert the V25.40b CP3 / CP4 Installation floppy disk in first, before you reboot!

SYS700 02 - ROM created on Monday July 29 13:50:42 EDT 2002

SYS702 64 - Total Flash size (in megs), all banks populated

SYS702 64 - DRAM size (in megs) - Banks 1234     populated

SYS703 0DE8 - Self Tests completed

SYS700 04 - Selecting Boot Source

Meridian 1 X11 ROM System Software

Release     : x112540b

Created     : Monday July 29 13:50:42 EDT 2002

Copyright(C) 1972-1997 Northern Telecom, Inc.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'os' from "/f0/p/os/diskos" ... 760740: 512000 + 248740 + 53056

Starting at 0x8040020...

Meridian 1 X11 System Install Utility

Created     : Monday January 14 11:57:25 EST 2002

Copyright(C) 1972-1997 Northern Telecom, Inc.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1997 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'install' from "/f0/p/install/install" ...222116 + 112664 + 371376

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

**

***

****     **      ****

****   *****    **********

****  *******  ***********

**************************      Nortel Networks - Meridian 1

****     ****    ****           Install Tool (Version 38)

****     ****    ****

****     ****    ****

****     ****    ****

****     *****************

****      ******  ******

****       ***     ***

Copyright 1999 - 2001  Nortel Networks

Please press <CR> when ready ...

>OBTAIN and CHECK SYSTEM CONFIGURATION

>Validate hard disk partitions

>Copy /f0/disk2411.sys to /u/disk2411.sys - 23174 bytes copied.

>Copy /f0/disk2511.sys to /u/disk2511.sys - 23174 bytes copied.

>Copy /f0/disk2611.sys to /u/disk2611.sys - 23046 bytes copied.

>Copy /f0/disk2811.sys to /u/disk2811.sys - 23174 bytes copied.

>Copy /f0/disk2911.sys to /u/disk2911.sys - 23174 bytes copied.

>Copy /f0/disk3011.sys to /u/disk3011.sys - 23046 bytes copied.

System Date and Time now is:

Saturday 00-07-0000, 21:07:24

INST0077 System Date is not correct, please set it to the current

Please press <CR> when ready ...

Current date is: Saturday 00-07-0000

Enter new date (dd mm yyyy) ? 03 06 2004

Date is set to: Thursday 06-03-2004

Current time is: 21:08:04

Enter new time (hh mm ss) ? 10 27 00

Time is set to: 10:27:00

System Date and Time now is:

Thursday 06-03-2004, 10:27:00

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

M A I N    M E N U

The Software Installation Tool will install or upgrade Meridian-1

System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout  the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <u> - To Install menu

<t> - To Tools menu.

<q> - Quit.

Enter Choice> U

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

Please insert the diskette with the keycode file into the floppy drive.

Please enter:

<CR> -> <a> - Continue with the keycode validation

(the keycode diskette is in the floppy drive on Core 0).

<q> - Quit.

Enter Choice> A

>Validating Keycode

1114 bytes copied.

The provided keycode authorizes the install of X112540 software

(all subissues) for machine type 3011 (68060E processor on Option 81(C)).

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

Please confirm that this keycode matches the CDROM Release

Please enter:

<CR> -> <y> - Yes, the keycode matches. Go on to Install Menu.

<n> - No, the keycode does not match. Try another keycode diskette.

Enter Choice> Y

>Obtain database file names

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Meridian-1

System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout  the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 set Languages.

<q> - Quit.

Enter Choice> D

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You will now perform the database installation.

Note: If you are installing  the Database from a floppy disk,

please insert the correct disk now.

Please enter:

<CR> -> <a> - Install CUSTOMER Database

(the customer database diskette must be in the Core 0 disk drive)

.

<b> - Install DEFAULT Database

(the installation CDROM must be in the Core 0 disk drive).

<c> - Transfer the previous system Database.

<d> - Copy Database from the redundant disk.

<e> - Check the Database that exists on the hard disk.

<f> - Transfer Database from MDU.

<q> - Quit.

Enter Choice> C

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected Database Media Transfer. This operation allows you to

transfer the system database from a 2MB floppy diskette to the

system hard disk.

If necessary, you may check the database version on the floppy disk

before you begin the transfer.

Please enter:

<CR> -> <a> - Transfer the database from floppy to hard disk.

<b> - Display the version and issue of database on floppy drive.

<q> - Quit.

Enter Choice> A

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to transfer the Database from floppy to the hard

disk.  Please insert the source 2MB floppy disk  (B1) in the  floppy

drive  now.

Please press <CR> when ready ...

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | No |  |  |
| SW: disk to ROM | No |  |  |
| Database | Yes |  | from previous system floppy |
| CP-BOOTROM | No |  |  |
| IOP-ROM | No |  |  |

Please enter:

<CR> -> <y> - Yes, start Installation.

<n> - No, stop Installation. Return to the Main Menu.

Enter Choice> Y

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to transfer the Database from the floppy disk to the

hard disk on Core 0. This will erase the database on the hard disk.

If you quit now, the database will be left unchanged.

Please insert the B1 disk into the disk drive now if you are

transferring the database.

Please enter:

<CR> -> <a> - Yes, transfer the database.

<q> - Quit.

Enter Choice> Y

>Transfer Database from floppy drive to hard disk for side 0

>Remove temporary file "/u/db/database.hld"

>Remove temporary file "/u/db/database.tmp"

>Remove temporary file "/u/db/config.hld"

>Remove temporary file "/u/db/config.tmp"

>Create temporary file "/u/db/database.tmp"

>Create temporary file "/u/db/config.tmp"

>Start Database transfer from floppy to the hard disk

>Read Database from floppy disk

>Write Configuration to "/u/db/config.tmp"

>Write Database to "/u/db/database.tmp"

>Rename "/u/db/database.bak" to "/u/db/database.hld"

>Rename "/u/db/config.bak" to "/u/db/config.hld"

>Rename "/u/db/database.rec" to "/u/db/database.bak"

>Rename "/u/db/config.rec" to "/u/db/config.bak"

>Rename "/u/db/database.tmp" to "/u/db/database.rec"

>Rename "/u/db/config.tmp" to "/u/db/config.rec"

>Success! Database transferring completed

Please press <CR> when ready ...

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | No |  |  |
| SW: disk to ROM | No |  |  |
| Database | Yes | Ok | from previous system floppy |
| CP-BOOTROM | No |  |  |
| IOP-ROM | No |  |  |

Please press <CR> when ready ...

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Meridian-1

System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout  the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 set Languages.

<q> - Quit.

Enter Choice> Q

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to Quit. Please confirm.

Please enter:

<CR> -> <y> - Yes, Quit.

<n> - No, DO NOT Quit.

Enter Choice> Y

Nortel Meridian-1 Software/Database/PEROM CDROM INSTALL Tool (Version 38)

=============================================================

You selected to Quit the Software Installion Tool.

You may reboot the  system or return to the Main Menu.

Before rebooting the system, remove Install diskette from the floppy drive(s).

-------------------------------------------------

DO NOT REBOOT USING BUTTON!!

-------------------------------------------------

Remove the V25.40b CP3 / CP4 Installation floppy disk, before you reboot!

Please enter:

<CR> -> <a> - Reboot the system.

<m> - Return to the Main menu.

Enter Choice>

>Removing temporary files

>Remove /u/disk2411.sys

>Remove /u/disk2511.sys

>Remove /u/disk2611.sys

>Remove /u/disk2811.sys

>Remove /u/disk2911.sys

>Remove /u/disk3011.sys

>Quit Install. Reboot system ...

SYS700 02 - ROM created on Monday July 29 13:50:42 EDT 2002

SYS706 26 - Full Reset by Software Install

SYS702 64 - Total Flash size (in megs), all banks populated

SYS702 64 - DRAM size (in megs) - Banks 1234     populated

SYS703 0DE8 - Self Tests completed

SYS700 04 - Selecting Boot Source

Meridian 1 X11 ROM System Software

Release     : x112540b

Created     : Monday July 29 13:50:42 EDT 2002

Copyright(C) 1972-1997 Northern Telecom, Inc.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'os' from "/flash/flashos" ... 1249348: 512000 + 512000 + 225348 + 105260

Starting at 0x4100020...

Meridian 1 X11 System Software

Release     : x112540b

Created     : Monday July 29 13:50:55 EDT 2002

Loaded      : 3/6/04 10:30:53 from '/flash/flashos'

Copyright(C) 1972-2001 Nortel Networks.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1995 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Initialize Network Interface.

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

Network Interface is up.

Host         : MERIDIAN

IP Address   : 220.0.0.100

Sub-netmask  : 255.255.255.0

Loading symbols from "/flash/flashos.sym"

Executing startup script "/p/startup"

# Empty startup file for use in the field.

cd "/p/etc"

value = 0 = 0x0

0x8c243ac (tRptD):  (Boot ROM)              SRPT4504 Single CP configuration

Done executing startup script /p/startup

0x8c243ac (tRptD):  (Boot ROM)              SRPT720 ROM OS 0: Cold Start. Running on CP 0

PDT Ready!

Loading SL1 .

Loading 'res' from "/flash/sl1res"

15390860 + 395112 + 833412

Loading 'ovl' from "/flash/ovlres"

5460092 + 0 + 0

0x8c243ac (tRptD):  (Boot ROM)              SRPT728 ROM OS 0: Loading 'diskos' from /flash/flashos

Start SNMP agent

Processed SNMP configuration file /p/data/view.cfg

Loading MAT script files........

Loading HI Data from '/u/db/hi' ...

Loading SL1 data

SYS000 000800 000000 000000 000000 CPU 0

SYSDB RLS/ISS: 1691 DATE/TIME: NOT AVAIL. RECS: 203 SEQNO: 0

DATA CONVERSION

X11 INTL RELEASE 16.91G TO X11 ISSUE 25.40B

CONFIG:  IDLE_SET_DISPLAY NORTEL

BCS TEMPLATE

PBX TEMPLATE

CUST

P_AUX_CUST

ICI

ACUST

ROUTE

LTN

LTN LINK

TN BLKS

SCL

ESN  0

ESN_ROUTE_LIST

ESN DM

NCTL

ACD

AUTH  0

NFCR_TREES

IDC_TREES

CSL

VAS

DCH

ARIES

SYS_PARAM

XTDT

FTC

XSDI

MCAD

FCAD

FDCT

DTI2SYTI

DTI2SCAT

DTI2PDCA

FFC

LAPW

SYS4202 NAS SCHD 0

DTSL

DONEinterrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

INI000 00000000 10 31 41 00000000 00000000 00000000 00000000 0 ? 00000000

Total number of files 0

Warning: No corporate directory files were found!

HWI202 NCB 0 1: failed to enable 3PE on Group 1

Check that 3PE has correct DIP switch setting

Check both connector ends of CNI-3PE cable (if applicable)

>

HWI202 NCB 0 2: failed to enable 3PE on Group 2

Check that 3PE has correct DIP switch setting

Check both connector ends of CNI-3PE cable (if applicable)

HWI202 NCB 0 3: failed to enable 3PE on Group 3

Check that 3PE has correct DIP switch setting

Check both connector ends of CNI-3PE cable (if applicable)

SRPT750 INI 0: INI on side 0 due to System Cold Start

XMI101 4

XMI101 16

XMI101 4

XMI101 16

DSET000 DOWN LOAD 0 0 0 0 0 0 0 204122

INI014 000000E0 000000E8

INI002 00000080 00000081 00000082 00000083 0000008B 0000008C 0000008D

INI007 00000050 00000052 0000006E 0000006F 0000006E 0000006F

INI109

ERR225 0 3

DTM338 11

DTM338 12

DTM338 13

SRPT752 INI 0: INI completed in 76 seconds

Let the Meridian do a normal system Dump “EDD” onto the new floppy disk.

Obtain via R.A.C.E , The PWD2 passcode for the site. Otherwise use the “Database.rec PWD2 Extractor program to retrieve the top two level passcodes / names etc, via the Floppy disk.

Log into the system & change the parameters in LD 17 under “PARM”

(For Option 51 upgrades, use “CEQU” & add in another network card 16 onwards!).

Use the settings that are listed within the NTP’s for Succession. It’s a good idea to check out some of the settings, such as SPN’s, FCR’s, etc. They may need to be updated.

I would recommend that you provide a “Brand” name using the customers details in LD 17 under “PARM". You will need the PWD2 code! & you only have up to 24 characters. This can be useful when the upgrade is carried out to V03.00.

### 1.44 Meg Floppy disk upgrade method to V03.00

The information listed below has been taken from one of the upgrades that I did, Using the standard 1.44 Meg Floppy disk.

Before you start the upgrade, You must first obtain the Succession Keycode.kcd file. This can be obtained from the Nortel Web Site, Keycode section, Or via the Meridian Order Group / Option 11 Configuration Centre.

Load in the software from new, using normal procedures with the install disk.

Insert the correct V03.00 Installation floppy disk in first, before you reboot for the type of Succession system that you have the keycodes for!

#### The different versions of install disks are listed below.

#### Option 61c

#### 1)  CP3 (Type 25.21) & CP4 (Type 29.21)

#### 2)  CPP (Type 32.21)

#### Option 81c

#### 1)  CP3 (Type 26.21) & CP4 (Type 30.21)

#### 2)  CPP (Type 33.21)

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

#####

#########   ########

########`` `#############

########      ``    `#############

#######`         ######```####

#######      ######``     `###           NORTEL NETWORKS

#######    #####``          ####

####### #####`              ####   Succession Enterprise Software

`#########`                ####`

#######`                 #####       Install Tool version 11

`#######               #####`

##########            `###`          Copyright 1992 - 2003

##``##########            #

##   `########################

`#     `########################

###########################``

`````````  ````####````

Please press <CR> when ready ...

*****************************************************************

WARNING:

This software does not support TNs configured on PE/EPE

shelves. Upgrading to this software release will permanently

disable all TNs configured on PE/EPE and will not allow new

TNs to be configured.

Proceed with the upgrade? (Y/N) Y

>Obtaining and checking system configuration ...

>Validate hard disk partitions

>Copying "/f0/disk2421.sys" to "/u/disk2421.sys" - 24310 bytes copied.

>Copying "/f0/disk2521.sys" to "/u/disk2521.sys" - 24310 bytes copied.

>Copying "/f0/disk2621.sys" to "/u/disk2621.sys" - 24182 bytes copied.

>Copying "/f0/disk2821.sys" to "/u/disk2821.sys" - 24310 bytes copied.

>Copying "/f0/disk2921.sys" to "/u/disk2921.sys" - 24310 bytes copied.

>Copying "/f0/disk3021.sys" to "/u/disk3021.sys" - 24182 bytes copied.

>System date and time is: Friday 06-04-2004, 11:44:16

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Does this system have a Signaling Server.....? (Default - No)

Please enter:

<CR> -> <n> - No

<y> - Yes

Enter choice> Y

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

M A I N    M E N U

The Software Installation Tool will install or upgrade Succession

Enterprise System Software, Database and the PE-ROM (both CP and IOP ROM).

You will be prompted throughout the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <u> - To Install Menu.

<t> - To Tools Menu.

<q> - Quit.

Enter choice> U

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Please insert the diskette with the keycode file into the floppy drive.

Please enter:

<CR> -> <a> - Continue with the keycode validation.

(The keycode diskette is in the floppy drive on Core 0.)

<q> - Quit.

Enter choice> A

>Validating keycode ...

>Copying "/f0/keycode.kcd" to "/u/keycode" - 1114 bytes copied.

>The provided keycode authorizes the install

>of X210300 software (all subissues)

>for machine type 2921 (68060E processor on Succession 1000M Single Group System).

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Please confirm that this keycode matches the CDROM release.

Please enter:

<CR> -> <y> - Yes, the keycode matches. Go on to Install Menu.

<n> - No, the keycode does not match. Try another keycode diskette.

Enter choice> Y

>Obtaining database file names ...

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Succession

Enterprise System Software, Database and the PE-ROM (both CP and

IOP ROM).

You will be prompted throughout the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools Menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 Set Languages.

<q> - Quit.

Enter choice> B

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Please insert the Software CDROM into the drive on Core 0.

The labeled side of the CDROM should be side up in the CDROM tray.

Please enter:

<CR> -> <a> - CDROM is now in drive. Continue with s/w checking.

<q> - Quit.

Enter choice> A

Follow the instructions as normal for loading in the Succession software!

##### Until you get to the information listed below.

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

Software release: 0300 was installed successfully on Core 0.

All files were copied from CDROM to the hard disk.

Please press <CR> when ready ...

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You will now perform the database installation.

Note: If you are installing the database from a floppy disk,

please insert the correct disk now.

Please enter:

<CR> -> <a> - Install CUSTOMER database.

(the customer database diskette must be in the Core 0 disk drive)

.

<b> - Install DEFAULT database.

(The Software CDROM must be in the Core 0 disk drive.)

<c> - Transfer the previous system database.

<d> - Copy database from the redundant disk.

<e> - Check the database that exists on the hard disk.

<f> - Transfer database from MDU.

<q> - Quit.

Enter choice> A             (Use the Floppy disk with the V25.40b database on!)

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | Yes | Ok | From 2540B to 0300 |
| SW: disk to ROM | Yes | Ok | From 2540B to x210300 |
| Database | Yes |  | From 2540B to 2540B |
| CP-BOOTROM | Yes |  |  |
| IOP-ROM | Yes |  |  |

Please enter:

<CR> -> <y> - Yes, start installation.

<n> - No, stop installation. Return to the Main Menu.

Enter choice> Y

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You selected to transfer the database from the floppy disk - release: 2540B

to the hard disk on Core 0. release: 2540B.

This will erase the database on the hard disk.

The database diskette has been inserted into the floppy disk drive.

If you quit now, the database will be left unchanged.

Please enter:

<CR> -> <a> - Continue with database install.

<q> - Quit.

Enter choice> A

Copying "/u/db/hi/eth.tmp" to "/u/db/hi_tmp/eth.tmp" - 0 bytes copied.

Copying "/u/db/hi/inet.db" to "/u/db/hi_tmp/inet.db" - 131 bytes copied.

Copying "/u/db/hi/ipb.db" to "/u/db/hi_tmp/ipb.db" - 71 bytes copied.

Copying "/u/db/hi/cnib.db" to "/u/db/hi_tmp/cnib.db" - 39 bytes copied.

Copying "/u/db/hi/iop.db" to "/u/db/hi_tmp/iop.db" - 33 bytes copied.

Copying "/u/db/hi/simm.db" to "/u/db/hi_tmp/simm.db" - 18 bytes copied.

Copying "/u/db/hi/cp.db" to "/u/db/hi_tmp/cp.db" - 31 bytes copied.

Copying "/u/db/hi/hi.db" to "/u/db/hi_tmp/hi.db" - 255 bytes copied.

* * *    Succession Enterprise Software Database ARCHIVE Tool    * * *

====================================================

You have chosen to restore database dated: Jun 04 11:19:00 2004

Please confirm.

Please enter:

<CR> -> <y> - Yes, load.

<n> - No, DO NOT load.

Enter Choice> Y

/f0/config.rec

>Restoring "/f0/config.rec" to "/u/db/config.bak"..OK

/f0/database.rec

Let the Meridian load in all the files & wait until the next menu appears.

Once this has been completed, Then remove the Floppy disk & insert a new one labeled as Succession copy!.

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

An  upgrade from  machine type 30(CP_NET) to 29(CP_NET) has  been detected.

The  Hardware  Infrastructure (HI) data base from the restored

system is not applicable. It will be erased and the current one

will be used.

This appears because the software at V25.40b is now changing from a Option 81c, to a 61c for the keycodes listed at the Succession release!

Please press <CR> when ready ...

>Copying "/u/db/hi_bak/iop.db" to "/u/db/hi/iop.db" - 33 bytes copied.

>Copying "/u/db/hi_bak/cp.db" to "/u/db/hi/cp.db" - 31 bytes copied.

>Copying "/u/db/hi_bak/cnib.db" to "/u/db/hi/cnib.db" - 39 bytes copied.

>Copying "/u/db/hi_bak/ipb.db" to "/u/db/hi/ipb.db" - 71 bytes copied.

>Copying "/u/db/hi_bak/hi.db" to "/u/db/hi/hi.db" - 255 bytes copied.

>Copying "/u/db/hi_bak/simm.db" to "/u/db/hi/simm.db" - 18 bytes copied.

>Removing file "/u/db/hi_tmp/eth.tmp"

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You have chosen to upgrade CP-BOOTROM in card slot 14 from

the context: 2540b to the context: x210300.

This will replace old CP-BOOTROM with the ROM image files:

"/p/os/cprom".

You may continue with ROM upgrade or quit now and leave ROM unchanged.

Please enter:

<CR> -> <a> - Continue with ROM upgrade.

<q> - Quit.

Enter choice> A

>Install CP flash ROM file from "/p/os/cprom"

Burning the flash on a CP4 board.

Last address   : 0x403b32c

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

The release and issue of IOP-ROM, card slot number 16, are the

same as the release and issue of the ROM image file

"/p/os/ioprom".

Are you sure you'd like to continue installing IOP-ROM?

Please enter:

<CR> -> <y> - Yes, start installation.

<n> - No, stop installation.

Enter choice> Y

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You will now upgrade the IOP-ROM in card slot 16 from Release: 02.00 to

Release: 02.00, or quit and leave the ROM unchanged.

Please enter:

<CR> -> <a> - Continue with ROM upgrade.

<q> - Quit.

Enter choice> Q

>Install IOP-ROM from file "/p/os/ioprom"

Zeroing out EEPROM...

Erasing EEPROM, take 0010

>System resetting IOP card at slot 16

--------------------------------------------------

INSTALLATION STATUS SUMMARY

--------------------------------------------------

| Option | Choice | Status | Comment |
| --- | --- | --- | --- |
| SW: CD to disk | Yes | Ok | From 2540B to 0300 |
| SW: disk to ROM | Yes | Ok | From 2540B to x210300 |
| Database | Yes | Ok | From 2540B to 2540B |
| CP-BOOTROM | Yes | Ok | From x112540b to x210300 |
| IOP-ROM | Yes | Ok | From 02.00 to 02.00 |

Please press <CR> when ready ...

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

I N S T A L L    M E N U

The Software Installation Tool will install or upgrade Succession

Enterprise System Software, Database and the PE-ROM (both CP and

IOP ROM).

You will be prompted throughout the installation and given the

opportunity to quit at any time.

Please enter:

<CR> -> <a> - To install Software, CP-BOOTROM, IOP-ROM.

<b> - To install Software, Database, CP-BOOTROM, IOP-ROM.

<c> - To install Software only.

<d> - To install Database only.

<e> - To install CP-BOOTROM only.

<f> - To install IOP-ROM only.

<g> - To reinstall CP-Software.

<o> - To copy System Software from the other Core.

<t> - To go to the Tools Menu.

<k> - To install Keycode only.

For Feature Expansion, use OVL143.

<p> - To install 3900 Set Languages.

<q> - Quit.

Enter choice> Q

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You selected to quit. Please confirm.

Please enter:

<CR> -> <y> - Yes, quit.

<n> - No, DON'T quit.

Enter choice> Y

At this stage It’s best to have a capture file to record any messages / errors that might appear & get them checked out.

Succession Enterprise Software/Database/PEROM CDROM Install Tool

====================================================

You selected to quit the Install Tool.

You may reboot the system or return to the Main Menu.

Remove all disks from the system before rebooting.

------------------------------------------------------------

DO NOT REBOOT USING RESET BUTTON!!!

------------------------------------------------------------

Please enter:

<CR> -> <a> - Reboot the system.

<m> - Return to the Main Menu.

Enter choice> A

>Removing temporary file "/u/disk2421.sys"

>Removing temporary file "/u/disk2521.sys"

>Removing temporary file "/u/disk2621.sys"

>Removing temporary file "/u/disk2821.sys"

>Removing temporary file "/u/disk2921.sys"

>Removing temporary file "/u/disk3021.sys"

>Rebooting system ...

Sys|

SYS700 02 - ROM created on Friday October 3 16:35:41 EDT 2003

SYS706 26 - Full Reset by Software Install

SYS702 64 - Total Flash size (in megs), all banks populated

SYS702 64 - DRAM size (in megs) - Banks 1 3 5 7  populated

SYS703 0DE8 - Self Tests completed

SYS700 04 - Selecting Boot Source

Succession Enterprise ROM System Software

Release     : x210300

Created     : Friday October 3 16:35:41 EDT 2003

Copyright(C) 1972-2003 Nortel Networks.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1997 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Loading 'os' from "/flash/flashos" ... 1276696: 512000 + 512000 + 252696 + 108300

Starting at 0x4100020...

Succession Enterprise System Software

Release     : x210300

Created     : Friday October 3 16:35:57 EDT 2003

Loaded      : 4/6/04 12:04:23 from '/flash/flashos'

Copyright(C) 1972-2003 Nortel Networks.  All Rights Reserved.

VxWorks 5.2

Copyright(C) 1984-1997 Wind River Systems, Inc.  All Rights Reserved.

Use of the above software subject to terms of a license agreement.

U.S. Government users subject to "Restricted Rights" under FAR & DFAR.

Initialize Network Interface.

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

interrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

Network Interface is up.

Host         : MERIDIAN

IP Address   : 220.0.0.100

Sub-netmask  : 255.255.255.0

Loading symbols from "/flash/flashos.sym"

Executing startup script "/p/startup"

# Empty startup file for use in the field.

cd "/p/etc"

value = 0 = 0x0

0x8c20f04 (tRptD):  (Boot ROM)              SRPT4504 Single CP configuration

Done executing startup script /p/startup

PDT Ready!

Loading SL1 .0x8c20f04 (tRptD):  (Boot ROM)              SRPT720 ROM OS 0: Cold

Start. Running on CP 0

Loading 'res' from "/flash/sl1res"

16154628 + 406872 + 1717828

Loading 'ovl' from "/flash/ovlres"

5666716 + 0 + 0

0x8c20f04 (tRptD):  (Boot ROM)              SRPT728 ROM OS 0: Loading 'diskos' from /flash/flashos

Unable to remove /p/data/ERRSEV.MSG

Start SNMP agent

Processed SNMP configuration file /p/data/view.cfg

0x8c20f04 (tRptD):  (4/6/04 12:04:30.796) SRPT730 OS 0: Cold Start

Release:  x210300

Created:  Friday October 3 16:35:57 EDT 2003

Loading OTM script files........

Loading HI Data from '/u/db/hi' ...

Loading SL1 data

SYS000 000800 000000 000000 000000 CPU 0

SYSDB RLS/ISS: 2540B DATE/TIME: 4/6/2004 11:19:28 RECS: 323 SEQNO: 2209

DATA CONVERSION

X11 ISSUE 25.40B TO SUCCESSION X21 RELEASE 3.00

CONFIG:  IDLE_SET_DISPLAY Pizza Factory Inc

PHYS

BCS TEMPLATE

PBX TEMPLATE

CUST

P_AUX_CUST

ICI

ACUST

CLID

ROUTE

LTN

LTN LINK

TN BLKS

SCL

ESN  0

ESN_ROUTE_LIST

ESN DM

NCTL

ACD

CPK

NFCR_TREES

IDC_TREES

ASYNCH

AML/ELAN

VAS

BG_TIME

BG_CAT

DCH

ARIES

SYS_PARAM

XTDT

FTC

MCAD

FCAD

FDCT

DTI2SYTI

DTI2SCAT

DTI2PDCA

FFC

LAPW

DTSL

PLUGIN

CPND DATA

CPND NAME

DONEinterrupt: COM0007 Ethernet driver: no carrier detection on device unit 0: no carrier

INI000 00000000 12 05 20 00000000 00000000 00000000 00000000 0 00000000

XMI101 12

DSET000 DOWN LOAD 0 0 0 0 0 0 0 216547

INI014 000000E0 00000030 000000E8

INI002 00000080 00000081 00000082 00000083 0000008F

DTM338 8

Total number of files 0

Warning: No corporate directory files were found!

SRPT752 INI 0: INI completed in 82 seconds

Let the Meridian do a normal system Dump “EDD” onto the new floppy disk. Login & do a general check to make sure that the customers data is intact!. Once that you are happy, Then zip up the Floppy disk (Use save folder option!) & email it out to the engineer.

I would suggest that the current PWD2 passcode be given out to the field engineer, Just in case the Meridian passcodes have been changed on site.