Information on connecting a U S Robotics 56K external Modem onto a Meridian 1 system

These settings should work for a Meridian Option 11 - 81 TTY Port

The tests were carried out using "TTY 2" with "USER" = (SCH) ONLY! On a Option 11 system.

NOTE!

Do not have "USER" = (MTC,BUG,CTY, etc) as the TTY port could become disabled!.

Also make sure that the standard TTY port settings are used i.e..

"9600" baud rate word length = 8, 1 stop bit & no parity.

ATI3

U.S. Robotics 56K Voice EXT Rev. 12.1.0   (External stand alone type modem)

ATI4

U.S. Robotics 56K Voice EXT Settings...

B0  E1  F1  L2  M0  Q1  V1  X4  Y0

SPEED=9600  PARITY=N  WORDLEN=8

DIAL=TONE    OFF LINE

&A3  &B0  &C1  &D0  &H1  &I0  &K0

&M4  &N6  &R1  &S0  &T5  &U0  &Y1

S00=002  S01=000  S02=043  S03=013  S04=010  S05=008  S06=004

S07=060  S08=002  S09=006  S10=007  S11=070  S12=050  S13=000

S15=000  S16=000  S18=000  S19=000  S21=010  S22=017  S23=019

S25=005  S27=001  S28=008  S29=020  S30=000  S31=000  S32=000

S33=000  S34=000  S35=000  S36=000  S38=000  S39=011  S40=000

S41=001  S42=126

LAST DIALLED #:

ATI9

(1.0USR9180\\Modem\PNPC107\U.S. Robotics 56K Voice EXT)FF

Some other Notes!

Use ATS0=2 C/R (Number of rings for modem to answer = 2)

(I've found that a minimum number of 2 rings is best!)

Use ATM0 C/R (Speaker on modem = OFF & ATM1 = Speaker on until connected)

Before you disconnect the line connection, Make sure that you are back into the Meridian SL1 program!.

Also key "+++" first & wait until you get the "OK" prompt.    (It takes you back to the modem!)

If you don't do this, then the TTY port can disable it's self or the modem will lock up.

How to configure the U S Robotics 56K external Modem via a RS232 lead into your  P.C.

Key in the following …

“AT&A3&B0&C1&D0&H1&I0&K0&M4&N6&R1&S0&T5&U0&Y1” C/R.

The modem will answer with “OK”

Followed by  “ATB0E1F1L2M0Q1V1X4Y0” C/R.

Enter the  “S” Registers as shown in bold above. i.e.. “ATS0=2” C/R.

Finally enter “AT&W0” C/R to store the data into memory. Power Modem "OFF" & "ON".

The prompt  “ATI4” C/R will list out the values stored in memory.

To connect the Modem at 14400 baud rate & Meridian 1  TTY port at 19200 baud rate, Then use the settings  “AT&N8” C/R followed by  “AT&W0” C/R & power off & on the Modem again.