Information on connecting a U S Robotics 56K external Modem onto a Meridian 1 system

These settings should work for a Meridian Option 11 - 81 TTY Port

The tests were carried out using "TTY 2" with "USER" = (SCH) ONLY! On a Option 11 system.

NOTE!

Do not have "USER" = (MTC,BUG,CTY, etc) as the TTY port could become disabled!.

Also make sure that the standard TTY port settings are used i.e..

"9600" baud rate word length = 8, 1 stop bit & no parity.

ATI3

U.S. Robotics 56K Voice EXT Rev. 12.1.0   (External stand alone type modem)

ATI4

U.S. Robotics 56K Voice EXT Settings...

B0  E1  F1  L2  M0  Q1  V1  X4  Y0

SPEED=9600  PARITY=N  WORDLEN=8

DIAL=TONE    OFF LINE

&A3  &B0  &C1  &D0  &H1  &I0  &K0

&M4  &N6  &R1  &S0  &T5  &U0  &Y1

S00=002  S01=000  S02=043  S03=013  S04=010  S05=008  S06=004

S07=060  S08=002  S09=006  S10=007  S11=070  S12=050  S13=000

S15=000  S16=000  S18=000  S19=000  S21=010  S22=017  S23=019

S25=005  S27=001  S28=008  S29=020  S30=000  S31=000  S32=000

S33=000  S34=000  S35=000  S36=000  S38=000  S39=011  S40=000

S41=001  S42=126

LAST DIALLED #:

ATI9

(1.0USR9180\\Modem\PNPC107\U.S. Robotics 56K Voice EXT)FF

Some other Notes!

Use ATS0=2 C/R (Number of rings for modem to answer = 2)

(I've found that a minimum number of 2 rings is best!)

Use ATM0 C/R (Speaker on modem = OFF & ATM1 = Speaker on until connected)

Before you disconnect the line connection, Make sure that you are back into the Meridian SL1 program!.

Also key "+++" first & wait until you get the "OK" prompt.    (It takes you back to the modem!)

If you don't do this, then the TTY port can disable it's self or the modem will lock up.

How to configure the U S Robotics 56K external Modem via a RS232 lead into your  P.C.

Key in the following …

“AT&A3&B0&C1&D0&H1&I0&K0&M4&N6&R1&S0&T5&U0&Y1” C/R.

The modem will answer with “OK”

Followed by  “ATB0E1F1L2M0Q1V1X4Y0” C/R.

Enter the  “S” Registers as shown in bold above. i.e.. “ATS0=2” C/R.

Finally enter “AT&W0” C/R to store the data into memory. Power Modem "OFF" & "ON".

The prompt  “ATI4” C/R will list out the values stored in memory.

To connect the Modem at 14400 baud rate & Meridian 1  TTY port at 19200 baud rate, Then use the settings  “AT&N8” C/R followed by  “AT&W0” C/R & power off & on the Modem again.

# Setting taken from configuring a NS2522b pbx Modem

Select area of interest:

<1>  Fixed Factory Options

<2>  Fixed Option Set No.1

<3>  Fixed Option Set No.2

<4>  Fixed Option Set No.3

<5>  Fixed Option Set No.4

<6>  Fixed Option Set No.5

<7>  Saved (NVRAM) Options

<8>  Current (USER) Options

<-> Previous Menu  <MM> Main Menu

<QU> Quit

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

VIEW/MODIFY OPTIONS

<1>  Save selected Options to saved (NVRAM) Area

<2>  Save selected Options to current (USER) Area

<3>  Save selected Options to both Areas

<4>  Walk through all Option values

<5>  Modify All Option Pages

<7>  PSTN Options

<8>  DTE Dialogue Options

<9>  Data Mode Options

<10> V.24 Options

<12> Auto Calling/Auto Answering Options

<13> Test Mode Options

<14> Network Management Options

<-> Previous Menu  <MM> Main Menu

<QU> Quit

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE PSTN OPTIONS

OPTION NAME                                                CURRENT VALUE

<1>  PSTN Transmission Mode                              V.22 auto mode

<2>  PSTN Bell Modes                                                 Disabled

<3>  PSTN Call/Answer Mode                                      Normal

<4>  PSTN Carrier Fail Disconnect (100mS)                    14

<5>  PSTN Long Space Disconnect                            Disabled

<6>  PSTN Auto Disconnect Timer(secs)                         60

<7>  PSTN Auto Disconnect CCT103 (TD) OFF         Disabled

<8>  PSTN Auto Disconnect CCT104 (RD) OFF         Disabled

<9>  PSTN Auto Disconnect CCT105 (RTS) OFF       Disabled

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE DTE DIALOGUE OPTIONS

OPTION NAME                                          CURRENT VALUE

<1>  DTE Dialogue & A/C Mode                      AT Command Set

<2>  AT Quiet Mode                                               Enabled

<3>  AT Messages                                                    Set 8

<4>  AT Dialogue Number Output Format             Decimal

<5>  Echoplex During Dialogue                             Disabled

<6>  CMES Character (ASCII value)                         43

<7>  CMES Guard Time (25mSec steps)                  40

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE DATA MODE OPTIONS

OPTION NAME                                        CURRENT VALUE

<1>  Data Handling                                                 V.42

<2>  Automatic Transparency                              Enabled

<3>  Data Compression                                       Disabled

<4>  Transmit Buffer Size (*10 bytes)                      50

<5>  MNP Service Class                                       Class 4

<6>  V.42 Number of Retries                                   12

<7>  MNP Auto Reliable Fallback Character        Disabled

<8>  Break Handling                                             Disabled

<9>  Flow Control (DCE)                                 CCT106 (RFS)

<10> User Flow Control (DTE)                             Disabled

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE DATA MODE OPTIONS

OPTION NAME                                               CURRENT VALUE

<1>  XON Character (ASCII value)                                  17

<2>  XOFF Character (ASCII value)                                19

<3>  Async Transmission Mode                              User Defined

<4>  User Def Async Tx Frmt Char Lngth Sel           8 Data bits

<5>  User Def Async Tx Format Stop Bit Sel            1 Stop bit

<6>  User Def Async Tx Format Parity Selct                  None

<7>  User Def Async Tx Format Speed Select               1200

<8>  Guard Tone                                                          1800 Hz

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE V.24 OPTIONS

OPTION NAME                                                        CURRENT VALUE

<1>  CCT 105 (RTS)                                                             Normal

<2>  CCT 106 (RFS                                                              Normal

<3>  CCT 106 (RFS) Delay Timer (25mSec steps)              Disabled

<4>  CCT 107 (DSR) Control                                          Permanently ON

<5>  CCT 108 (DTR)                                                        Permanently ON

<6>  CCT 108Esc (DTR) No Cmnd\CMES T/out(*25ms)      Disabled

<7>  CCT 109 (CD)                                                                Normal

<8>  CCT 109 (CD) Response Timer (*100mS)                         0

<9>  CCT 140 (RDL)                                                            Disabled

<10> CCT 141 (AL)                                                              Disabled

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE AUTOCALL/ANSWER OPTIONS

OPTION NAME                                                  CURRENT VALUE

<1>  PSTN Auto Calling                                                 Enabled

<2>  PSTN Auto Answer (Rings before AA)                  2 Rings

<3>  PSTN Auto Answer Tone                                      Enabled

<4>  Dialback Security & Autologon                        Both Disabled

<5>  PSTN Exchange Tone Detection                          Enabled

<6>  PSTN Dialling                                                      Auto Detect

<7>  Wait For Quiet (secs)                                                 40

<8>  Comma Pause Time (secs)                                        4

<9>  Calling Tone                                                           Enabled

<10> Loudspeaker Control                                   On until CCT109 (CD)

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE AUTOCALL/ANSWER OPTIONS

OPTION NAME                                 CURRENT VALUE

<1>  Loudspeaker Volume                          Low

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE TEST MODE OPTIONS

OPTION NAME                                                  CURRENT VALUE

<1>  Rx RDL & Rx Ping Pong                                        Enabled

<2>  V54 Tx Odd or Even Address Parity                         Odd

<3>  Number of Blocks (*100)                                             10

<4>  Test Timer (*5secs)                                                    180

<-> Previous Menu  <MM> Main Menu

<QU> Quit          <M> More

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *

CONFIGURE NETWORK MANAGEMENT OPTIONS

OPTION NAME                                   CURRENT VALUE

<1>  NM Address                                                  0

<2>  Local Display Driver Access               Local & Remote

<3>  Call Event Log Reporting                       Disabled

<4>  Alarm Hit Count Threshold                          1

<5>  Alarm Receive Level Threshold                   1

<6>  Alarm Signal Quality Threshold                   1

<7>  Alarm Phase Jitter Threshold                      1

<8>  Alarm Retrain Threshold                             1

<-> Previous Menu  <MM> Main Menu

<QU> Quit

Type selection and press RETURN:

* * * * * * Local NS2522b * * * * * *